3GPP TSG-CT WG1 Meeting #124-e	C1-203838

Electronic meeting, 2-10 June 2020	(was C1-203199)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0153 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Functional alias - 10.2.4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16 |  |  |  |  |  | Date: |  |  | 2 June 2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The participating MCData function needs to check whether an incoming SIP MESSAGE request contains a <functional-alias-URI> element, and if it does, check whether that FA is active. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text is added to subclause 10.2.4.3.1 for the participating MCData function to check whether a functional-alias-URI> element is present in an mcdata-info+xml MIME body. If so, then determine whether the FA is active. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inability to fully support functional alias at the MCData server. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: The ME box is unticked. |  |  |  |  |  |  |  |  |

* * * * * FIRST CHANGE * * * * *

##### 10.2.4.3.1	Originating participating MCData function procedures

Upon receipt of a "SIP MESSAGE request for FD using HTTP for originating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

2)	shall determine the MCData ID of the originating user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request, and shall authorise the calling user;

NOTE:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in subclause 7.3.

3)	if the participating MCData function cannot find a binding between the public user identity and an MCData ID or if the validity period of an existing binding has expired, then the participating MCData function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in subclause 4.9, and shall not continue with any of the remaining steps;

4)	if <mcdata-controller-psi> element is present in the application/vnd.3gpp.mcdata-info+xml, shall use its value as public service identity of the controlling MCData function. Otherwise, if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is:

a)	set to a value of "group-fd", shall determine the public service identity of the controlling MCData function hosting the group standalone FD using HTTP service, associated with the MCData group identity in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request; or

b)	set to a value of "one-to-one-fd", shall determine the public service identity of the controlling MCData function hosting the one-to-one FD using HTTP service for the calling user;

5)	if unable to identify the controlling MCData function, it shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in subclause 4.9, and shall not continue with any of the remaining steps;

6)	shall determine whether the MCData user identified by the MCData ID is authorised for MCData communications by following the procedures in subclause 11.1;

7)	if <mcdata-controller-psi> in not present in the application/vnd.3gpp.mcdata-info+xml and if the procedures in subclause 11.1 indicate that the user identified by the MCData ID:

a)	is not allowed to initiate MCData communications as determined by step 1) of subclause 11.1, shall reject the "SIP MESSAGE request for FD using HTTP for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "200 user not authorised to transmit data" in a Warning header field as specified in subclause 4.9, and shall not continue with the rest of the steps in this subclause; and

b)	is not allowed to initiate one-to-one MCData communications due to exceeding the maximum amount of data that can be sent in a single request as determined by step 7) of subclause 11.1, shall reject the "SIP MESSAGE request for FD using HTTP for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "202 user not authorised for one-to-one MCData communications due to exceeding the maximum amount of data that can be sent in a single request" in a Warning header field as specified in subclause 4.9, and shall not continue with the rest of the steps in this subclause;

8)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

9)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCData function as determined by step 4) in this subclause;

10)	shall copy all MIME bodies included in the incoming SIP MESSAGE request to the outgoing SIP MESSAGE request;

10a)	if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body that contains a <functional-alias-URI> element, shall check if the status of the functional alias is activated for the MCData ID. If the functional alias status is activated, then the participating MCData function shall set the <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP INVITE request to the received value, otherwise shall not include a <functional-alias-URI> element;

11)	shall include the MCData ID of the originating user in the <mcdata-calling-user-identity> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP MESSAGE request;

12)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]), into the P-Asserted-Service header field of the outgoing SIP MESSAGE request;

13)	shall set the P-Asserted-Identity in the outgoing SIP MESSAGE request to the public user identity in the P-Asserted-Identity header field contained in the received SIP MESSAGE request; and

14)	shall send the SIP MESSAGE request as specified to 3GPP TS 24.229 [5].

Upon receipt of a SIP 202 (Accepted) response in response to the SIP MESSAGE request in step 14):

1)	shall generate a SIP 202 (Accepted) response as specified in 3GPP TS 24.229 [5]; and

2)	shall send the SIP 202 (Accepted) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 200 (OK) response in response to the SIP MESSAGE request in step 14):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5]; and

2)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the SIP MESSAGE request in step 14) the participating MCData function:

1)	shall generate a SIP response according to 3GPP TS 24.229 [5];

2)	shall include Warning header field(s) that were received in the incoming SIP response; and

3)	shall forward the SIP response to the MCData client according to 3GPP TS 24.229 [5].

* * * * * END CHANGES * * * * *