3GPP TSG-CT Meeting #88-e	CP-201176

# Online, , 28th Jun 2020 - 1st Jul 2020													Was C4-203040

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.503 | CR | 0423 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Context in AMF Data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Hewlett-Packard Enterprise |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Hewlett-Packard Enterprise |  |  |  |  |  |  |  |  |  |
| Work item code: | UDICOM |  |  |  |  |  | Date: |  |  | 2020-06-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For UDICOM, TS 23.632 requires HSS use - Nudm_SDM_Get service operation to fetch the UE context in SMF data and the Intersystem continuity context data for the user, if any. - Nudm_SDM_Subscribe service operation to subscribes to be notified when the Intersystem continuity context data or the UE context in SMF data are modified. The Intersystem continuity context data (i.e. epsInterworkingInfo) is in the Amf3GppAccessRegistration context. Currently SMF context is exposed over UDM SDM but there is no SDM resource to convey Amf3GppAccessRegistration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the UE Context in AMF 3GPP Data so that the Intersystem continuity context data can be retrieved/subscribed by Nudm SDM service operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Intersystem continuity context data cannot be retrieved or subscribed by Nudm SDM service operations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1, 6.1.3.x, 6.1.3.x.1, 6.1.3.x.2, 6.1.3.x.3, 6.1.3.x.3.1, 6.1.6.1, 6.1.6.2.15, 6.1.6.2.y, 6.1.6.3.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR introduces backward compatible changes to the Nudm_SDM OpenAPI. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | rev1: $ref: '#/components/schemas/EpsInterworkingInfo' replaced with: $ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/EpsInterworkingInfo' |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the nudm-sdm API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name (Archetype) | Resource URI |  | HTTP method or custom operation |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| Supi (Document) | /{supi} |  | GET |  | Retrieve UE's subscription data |  |
| Nssai (Document) | /{supi}/nssai |  | GET |  | Retrieve the UE's subscribed Network Slice Selection Assistance Information |  |
| UeContextInAmfData (Document) | /{supi}/ue-context-in-amf-data |  | GET |  | Retrieve the UE's Context in AMF Data |  |
| AccessAndMobilitySubscriptionData (Document) | /{supi}/am-data |  | GET |  | Retrieve the UE's subscribed Access and Mobility Data |  |
|  | /{supi}/am-data/update-sor |  | update-sor (POST) |  | Trigger the update of Steering of Roaming Information at the UE |  |
| SorAck (Document) | /{supi}/am-data/sor-ack |  | PUT |  | Providing acknowledgement of Steering of Roaming |  |
| UpuAck (Document) |  |  | /{supi}/am-data/upu-ack |  | PUT |  | Providing acknowledgement of UE parameters update |  |
| CagAck (Document) |  |  | /{supi}/am-data/cag-ack |  | PUT |  | Providing acknowledgement of UE CAG configuration update |  |
| EnhancedCoverageRestrictionData |  |  | /{supi}/am-data/ecr-data |  | GET |  | Retrieve the UE's subscribed Enhance Coverage Restriction Data |  |
| SmfSelectionSubscriptionData (Document) | /{supi}/smf-select-data |  | GET |  | Retrieve the UE's subscribed SMF Selection Data |  |
| UeContextInSmfData (Document) | /{supi}/ue-context-in-smf-data |  | GET |  | Retrieve the UE's Context in SMF Data |  |
| SessionManagementSubscriptionData (Document) | /{supi}/sm-data |  | GET |  | Retrieve the UE's session management subscription data |  |
| SMSSubscriptionData (Document) | /{supi}/sms-data |  | GET |  | Retrieve the UE's SMS subscription data |  |
| SMSManagementSubscriptionData (Document) | /{supi}/sms-mng-data |  | GET |  | Retrieve the UE's SMS management subscription data |  |
| LcsPrivacySubscriptionData (Document) | /{ueId}/lcs-privacy-data |  | GET |  | Retrieve the UE's LCS privacy subscription data |  |
| LcsMobileOriginatedSubscriptionData (Document) | /{supi}/lcs-mo-data |  | GET |  | Retrieve the UE's LCS Mobile Originated subscription data |  |
| SdmSubscriptions (Collection) | /{ueId}/sdm-subscriptions |  | POST |  | Create a subscription |  |
| Individual subscription (Document) | /{ueId}/sdm-subscriptions/{subscriptionId} |  | DELETE |  | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |  |
|  |  |  | PATCH |  | Modify the sdm-subscription identified by {subscriptionId} |  |
| IdTranslationResult (Document) | /{ueId}/id-translation-result |  | GET |  | Retrieve a UE's SUPI or GPSI |  |
| UeContextInSmsfData (Document) | /{supi}/ue-context-in-smsf-data |  | GET |  | Retrieve the UE's Context in SMSF Data |  |
| TraceData (Document) | /{supi}/trace-data |  | GET |  | Retrieve Trace Configuration Data |  |
| SharedData (Collection) | /shared-data |  | GET |  | Retrieve shared data |  |
| SharedDataSubscriptions (Collection) | /shared-data-subscriptions |  | POST |  | Create a subscription |  |
| SharedDataIndividual subscription (Document) | /shared-data-subscriptions/{subscriptionId} |  | DELETE |  | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe |  |
|  |  |  | PATCH |  | Modify the shared data subscription identified by {subscriptionId} |  |
| GroupIdentifiers (Document) | /group-data/group-identifiers |  | GET |  | Retrieve group identifiers and the UE identifiers belong to the group identifiers. |  |
| SnssaisAck (Document) | /{supi}/am-data/subscribed-snssais-ack |  | PUT |  | Providing acknowledgement of UE for subscribed S-NSSAIs |  |

* * * Next Change * * * *

#### 6.1.3.x	Resource: UeContextInAmfData (Document)

##### 6.1.3.x.1	Description

This resource represents the allocated AMF for a SUPI.

##### 6.1.3.x.2	Resource Definition

Resource URI: {apiRoot}/nudm-sdm/{apiVersion}/{supi}/ue-context-in-amf-data

This resource shall support the resource URI variables defined in table 6.1.3.x.2-1.

Table 6.1.3.x.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| apiVersion | See clause 6.1.1 |
| supi | Represents the Subscription Permanent Identifier (see 3GPP TS 23.501 [2] clause 5.9.2) pattern: See pattern of type Supi in 3GPP TS 29.571 [7] |

##### 6.1.3.x.3	Resource Standard Methods

###### 6.1.3.x.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.x.3.1-1.

Table 6.1.3.x.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.1.3.x.3.1-2 and the response data structures and response codes specified in table 6.1.3.x.3.1-3.

Table 6.1.3.x.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.x.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeContextInAmfData | M | 1 | 200 OK | Upon success, a response body containing the UeContextInAmfData shall be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: - USER_NOT_FOUND - DATA_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 6.1.7-1 are supported. |  |  |  |  |

* * * Next Change * * * *

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the structured data types defined for the Nudm_SDM service API. For simple data types defined for the Nudm_SDM service API see table 6.1.6.3.2-1.

Table 6.1.6.1-1: Nudm_SDM specific Data Types

| Data type |  | Clause defined |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Nssai |  | 6.1.6.2.2 |  | Network Slice Selection Assistance Information |  |
| SdmSubscription |  | 6.1.6.2.3 |  | A subscription to notifications |  |
| AccessAndMobilitySubscriptionData |  | 6.1.6.2.4 |  | Access and Mobility Subscription Data |  |
| SmfSelectionSubscriptionData |  | 6.1.6.2.5 |  | SMF Selection Subscription Data |  |
| UeContextInSmfData |  | 6.1.6.2.16 |  | UE Context In SMF Data |  |
| PduSession |  | 6.1.6.2.17 |  |  |  |
| DnnInfo |  | 6.1.6.2.6 |  | Data Network Name and associated information (LBO roaming allowed flag) |  |
| SnssaiInfo |  | 6.1.6.2.7 |  | S-NSSAI and associated information (DNN Info) |  |
| SessionManagementSubscriptionData |  | 6.1.6.2.8 |  | User subscribed session management data |  |
| DnnConfiguration |  | 6.1.6.2.9 |  | User subscribed data network configuration |  |
| PduSessionTypes |  | 6.1.6.2.11 |  | Default/allowed session types for a data network |  |
| SscModes |  | 6.1.6.2.12 |  | Default/allowed SSC modes for a data network |  |
| SmsSubscriptionData |  | 6.1.6.2.13 |  |  |  |
| SmsManagementSubscriptionData |  | 6.1.6.2.14 |  | SMS Management Subscription Data |  |
| SubscriptionDataSets |  | 6.1.6.2.15 |  |  |  |
| IdTranslationResult |  | 6.1.6.2.18 |  | SUPI that corresponds to a given GPSI |  |
| IpAddress |  | 6.1.6.2.22 |  | IP address (IPv4, or IPv6, or IPv6 prefix) |  |
| 3GppChargingCharacteristics |  | 6.1.6.3.2 |  | 3GPP Charging Characteristics |  |
| IwkEpsInd |  | 6.1.6.3.2 |  | Interworking with EPS Indication |  |
| ModificationNotification |  | 6.1.6.2.21 |  |  |  |
| UeContextInSmsfData |  | 6.1.6.2.23 |  |  |  |
| SmsfInfo |  | 6.1.6.2.24 |  |  |  |
| AcknowledgeInfo |  | 6.1.6.2.25 |  |  |  |
| SorInfo |  | 6.1.6.2.26 |  | Steering Of Roaming Information |  |
| UpuInfo |  | 6.1.6.2.33 |  | UE Parameters Update Information |  |
| SharedData |  | 6.1.6.2.27 |  | Subscription Data shared by multiple UEs |  |
| PgwInfo |  | 6.1.6.2.28 |  | Information about the DNNs/APNs and PGW-C+SMF FQDNs used in interworking with EPS |  |
| TraceDataResponse |  | 6.1.6.2.29 |  | Contains Trace Data or a shared data Id identifying shared Trace Data |  |
| SteeringContainer |  | 6.1.6.2.30 |  |  |  |
| SdmSubsModification |  | 6.1.6.2.31 |  | Modification instruction for a subscription to notifications |  |
| EmergencyInfo |  | 6.1.6.2.32 |  | Information about emergency session |  |
| EpsIwkPgw |  | 6.2.6.2.11 |  | Information of the PGW-C+SMF selected by the AMF for EPS interworking with N26 interface. |  |
| GroupIdentifiers |  | 6.1.6.2.34 |  |  |  |
| ExtGroupId |  | 6.1.6.3.2 |  |  |  |
| NiddInformation |  | 6.1.6.2.35 |  | Non-IP Data Delivery information |  |
| CagData |  | 6.1.6.2.36 |  |  |  |
| CagInfo |  | 6.1.6.2.37 |  |  |  |
| DataSetName |  | 6.1.6.3.3 |  |  |  |
| PduSessionContinuityInd |  | 6.1.6.3.7 |  |  |  |
| AdditionalSnssaiData |  | 6.1.6.2.38 |  | Additional information specific to a slice |  |
| VnGroupData |  | 6.1.6.2.39 |  |  |  |
| AppDescriptor |  | 6.1.6.2.40 |  |  |  |
| AppPortId |  | 6.1.6.2.41 |  | Application Port Id |  |
| LcsPrivacyData |  | 6.1.6.2.42 |  |  |  |
| Lpi |  | 6.1.6.2.43 |  |  |  |
| UnrelatedClass |  | 6.1.6.2.44 |  |  |  |
| PlmnOperatorClass |  | 6.1.6.2.45 |  |  |  |
| ValidTimePeriod |  | 6.1.6.2.46 |  |  |  |
| LcsMoData |  | 6.1.6.2.47 |  |  |  |
| EcRestrictionData |  | 6.1.6.2.48 |  | Enhance Coverage Restriction Data |  |
| ExpectedUeBehaviourData |  | 6.1.6.2.49 |  | Expected UE Behaviour Data |  |
| MaximumResponseTime |  | 6.1.6.2.50 |  | Maximum Response Time |  |
| MaximumLatency |  | 6.1.6.2.51 |  | Maximum Latency |  |
| SuggestedPacketNumDl |  | 6.1.6.2.52 |  | Suggested Number of Downlink Packets |  |
| FrameRouteInfo |  | 6.1.6.2.54 |  | Frame Route Information |  |
| EnhancedCoverageRestrictionData |  | 6.1.6.2.56 |  | Enhanced Coverage Restriction Data |  |
| EdrxParameters |  | 6.1.6.2.57 |  | eDRX Parameters |  |
| PtwParameters |  | 6.1.6.2.58 |  | Paging Time Window Parameters |  |
| OperationMode |  | 6.1.6.3.12 |  | Operation Mode |  |
| SorUpdateIndicator |  | 6.1.6.3.13 |  | SoR Update Indicator |  |
| NonExternalUnrelatedClass |  | 6.1.6.2.59 |  |  |  |
| LcsClientNonExternal |  | 6.1.6.2.60 |  |  |  |
| AfNonExternal |  | 6.1.6.2.61 |  |  |  |
| ExternalUnrelatedClass |  | 6.1.6.2.62 |  |  |  |
| AfExternal |  | 6.1.6.2.63 |  |  |  |
| LcsClientExternal |  | 6.1.6.2.64 |  |  |  |
| LcsClientGroupExternal |  | 6.1.6.2.65 |  |  |  |
| ServiceTypeUnrelatedClass |  | 6.1.6.2.66 |  |  |  |
| codeWordInd |  | 6.1.6.3.14 |  |  |  |
| UeContextInAmfData |  | 6.1.6.2.y |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nudm_SDM service API from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudm_SDM service API.

Table 6.1.6.1-2: Nudm_SDM re-used Data Types

| Data type |  | Reference |  | Comments |  |
| --- | --- | --- | --- | --- | --- |
| Dnn |  | 3GPP TS 29.571 [7] |  | Data Network Name with Network Identifier only; this type is used as key in a map of: - DnnConfigurations; see clause 6.1.6.2.8; - EpsIwkPgws; see clause 6.2.6.2.2; - ExpectedUeBehaviourData; see clause 6.1.6.2.8; |  |
| DurationSec |  | 3GPP TS 29.571 [7] |  | Time value in seconds |  |
| ProblemDetails |  | 3GPP TS 29.571 [7] |  | Common data type used in response bodies |  |
| Snssai |  | 3GPP TS 29.571 [7] |  | Single NSSAI |  |
| Uri |  | 3GPP TS 29.571 [7] |  | Uniform Resource Identifier |  |
| Gpsi |  | 3GPP TS 29.571 [7] |  | Generic Public Subscription Identifier |  |
| RatType |  | 3GPP TS 29.571 [7] |  | Radio Access Technology Type |  |
| Area |  | 3GPP TS 29.571 [7] |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [7] |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [7] |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [7] |  | see 3GPP TS 29.500 [4] clause 6.6 |  |
| PlmnId |  | 3GPP TS 29.571 [7] |  | PLMN Identity |  |
| PduSessionType |  | 3GPP TS 29.571 [7] |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [7] |  | Subscribed Default QoS |  |
| Ambr |  | 3GPP TS 29.571 [7] |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [7] |  | PduSessionId is used as key in a map of PduSessions; see clause 6.1.6.2.16. |  |
| NfInstanceId |  | 3GPP TS 29.571 [7] |  |  |  |
| Supi |  | 3GPP TS 29.571 [7] |  |  |  |
| RfspIndex |  | 3GPP TS 29.571 [7] |  |  |  |
| SscMode |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv4Address |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Address |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [7] |  |  |  |
| SorMac |  | 3GPP TS 29.509 [24] |  |  |  |
| SteeringInfo |  | 3GPP TS 29.509 [24] |  |  |  |
| AckInd |  | 3GPP TS 29.509 [24] |  |  |  |
| CounterSor |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuMac |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuData |  | 3GPP TS 29.509 [24] |  |  |  |
| UpuAckInd |  | 3GPP TS 29.509 [24] |  |  |  |
| CounterUpu |  | 3GPP TS 29.509 [24] |  |  |  |
| TraceData |  | 3GPP TS 29.571 [7] |  | Trace control and configuration parameters |  |
| NotifyItem |  | 3GPP TS 29.571 [7] |  |  |  |
| UpSecurity |  | 3GPP TS 29.571 [7] |  |  |  |
| ServiceName |  | 3GPP TS 29.510 [19] |  |  |  |
| OdbPacketServices |  | 3GPP TS 29.571 [7] |  |  |  |
| GroupId |  | 3GPP TS 29.571 [7] |  | This type is also used as key of a map in attributes: - vnGroupInfo and sharedVnGroupDataIds; see clause 6.1.6.2.4, 6.1.6.2.8, 6.1.6.2.27; |  |
| DateTime |  | 3GPP TS 29.571 [7] |  |  |  |
| CagId |  | 3GPP TS 29.571 [7] |  |  |  |
| StnSr |  | 3GPP TS 29.571 [7] |  | Session Transfer Number for SRVCC |  |
| CMsisdn |  | 3GPP TS 29.571 [7] |  | Correlation MSISDN |  |
| OsId |  | 3GPP TS 29.519 [33] |  |  |  |
| Uint16 |  | 3GPP TS 29.571 [7] |  |  |  |
| RgWirelineCharacteristics |  | 3GPP TS 29.571 [7] |  |  |  |
| Tmbr |  | 3GPP TS 29.571 [7] |  |  |  |
| GeographicArea |  | 3GPP TS 29.572 [34] |  |  |  |
| LcsServiceType |  | 3GPP TS 29.572 [34] |  |  |  |
| ScheduledCommunicationTime |  | 6.5.6.2.9 |  |  |  |
| LocationArea |  | 6.5.6.2.10 |  |  |  |
| StationaryIndication |  | 6.5.6.3.3 |  |  |  |
| TrafficProfile |  | 6.5.6.3.4 |  |  |  |
| ScheduledCommunicationType |  | 6.5.6.3.5 |  |  |  |
| BatteryIndication |  | 6.5.6.2.14 |  |  |  |
| ScheduledCommunicationTime |  | 6.5.6.2.9 |  |  |  |
| LocationArea |  | 6.5.6.2.10 |  |  |  |
| StationaryIndication |  | 6.5.6.3.3 |  |  |  |
| TrafficProfile |  | 6.5.6.3.4 |  |  |  |
| ScheduledCommunicationType |  | 6.5.6.3.5 |  |  |  |
| BatteryIndication |  | 6.5.6.3.14 |  |  |  |
| AcsInfo |  | 3GPP TS 29.571 [7] |  | ACS Information |  |
| IPv4AddrMask |  | 3GPP TS 29.571 [7] |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [7] |  |  |  |
| NefId |  | 3GPP TS 29.510 [19] |  |  |  |
| PatchResult |  | 3GPP TS 29.571 [7] |  |  |  |
| NrV2xAuth |  | 3GPP TS 29.571 [7] |  |  |  |
| LteV2xAuth |  | 3GPP TS 29.571 [7] |  |  |  |
| BitRate |  | 3GPP TS 29.571 [7] |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.15	Type: SubscriptionDataSets

Table 6.1.6.2.15-1: Definition of type SusbcriptionDataSets

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amData | AccessAndMobilitySubscriptionData | O | 0..1 | Access and Mobility Subscription Data |
| smfSelData | SmfSelectionSubscriptionData | O | 0..1 | SMF Selection Subscription Data |
| uecAmfData | UeContextInAmfData | O | 0..1 | UE Context In AMF Data |
| uecSmfData | UeContextInSmfData | O | 0..1 | UE Context In SMF Data |
| uecSmsfData | UeContextInSmsfData | O | 0..1 | UE Context In SMSF Data |
| smsSubsData | SmsSubscriptionData | O | 0..1 | SMS Subscription Data |
| smData | array(SessionManagementSubscriptionData) | O | 1..N | Session Management Subscription Data |
| traceData | TraceData | O | 0..1 | Trace Data. The Null value indicates that trace is not active. |
| smsMngData | SmsManagementSubscriptionData | O | 0..1 | SMS Management Subscription Data |
| lcsPrivacyData | LcsPrivacyData | O | 0..1 | LCS Privacy Subscription Data |
| lcsMoData | LcsMoData | O | 0..1 | LCS Mobile Originated Subscription Data |

* * * Next Change * * * *

##### 6.1.6.2.y	Type: UeContextInAmfData

Table 6.1.6.2.y-1: Definition of type UeContextInAmfData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| epsInterworkingInfo | EpsInterworkingInfo | O | 0..1 | This IE contains the associations between APN/DNN and PGW-C+SMF selected by the AMF for EPS interworking. |

* * * Next Change * * * *

##### 6.1.6.3.3	Enumeration: DataSetName

Table 6.1.6.3.3-1: Enumeration DataSetName

| Enumeration value | Description |
| --- | --- |
| "AM" | Access and Mobility Subscription Data |
| "SMF_SEL" | SMF Selection Subscription Data |
| "UEC_SMF" | UE Context in SMF Data |
| "UEC_SMSF" | UE Context in SMSF Data |
| "SMS_SUB" | SMS Subscription Data |
| "SM" | Session Management Subscription Data |
| "TRACE" | Trace Data |
| "SMS_MNG" | SMS Management Subscription Data |
| "LCS_PRIVACY" | LCS Privacy Subscription Data |
| "LCS_MO" | LCS Mobile Originated Subscription Data |
| "UEC_AMF" | UE Context in AMF Data |

Note:	The current naming conventions for Enumerations (uppercase with underscore), when their intended usage is for query parameters is not consistent with the naming conventions for URI components (lowercase with hyphen).

* * * Next Change * * * *

## A.2	Nudm_SDM API

<...skip…>

/{supi}/nssai:

get:

summary: retrieve a UE's subscribed NSSAI

operationId: GetNSSAI

tags:

- Slice Selection Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/Nssai'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/ue-context-in-amf-data:

get:

summary: retrieve a UE's UE Context In AMF Data

operationId: GetUeCtxInAmfData

tags:

- UE Context In AMF Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/UeContextInAmfData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

/{supi}/am-data:

get:

summary: retrieve a UE's Access and Mobility Subscription Data

operationId: GetAmData

tags:

- Access and Mobility Subscription Data Retrieval

parameters:

- name: supi

in: path

description: Identifier of the UE

required: true

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

- name: supported-features

in: query

description: Supported Features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: plmn-id

in: query

description: serving PLMN ID

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

<...skip…>

SubscriptionDataSets:

type: object

properties:

amData:

$ref: '#/components/schemas/AccessAndMobilitySubscriptionData'

smfSelData:

$ref: '#/components/schemas/SmfSelectionSubscriptionData'

uecAmfData:

$ref: '#/components/schemas/UeContextInAmfData'

uecSmfData:

$ref: '#/components/schemas/UeContextInSmfData'

uecSmsfData:

$ref: '#/components/schemas/UeContextInSmsfData'

smsSubsData:

$ref: '#/components/schemas/SmsSubscriptionData'

smData:

type: array

items:

$ref: '#/components/schemas/SessionManagementSubscriptionData'

minItems: 1

traceData:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TraceData'

smsMngData:

$ref: '#/components/schemas/SmsManagementSubscriptionData'

lcsPrivacyData:

$ref: '#/components/schemas/LcsPrivacyData'

lcsMoData:

$ref: '#/components/schemas/LcsMoData'

<...skip…>

Nssai:

type: object

required:

- defaultSingleNssais

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

defaultSingleNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

singleNssais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 1

provisioningTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

additionalSnssaiData:

type: object

additionalProperties:

$ref: '#/components/schemas/AdditionalSnssaiData'

minProperties: 1

nullable: true

UeContextInAmfData:

type: object

properties:

epsInterworkingInfo:

$ref: 'TS29503_Nudm_UECM.yaml#/components/schemas/EpsInterworkingInfo'

UeContextInSmfData:

type: object

properties:

pduSessions:

description: A map (list of key-value pairs where PduSessionId serves as key) of PduSessions

type: object

additionalProperties:

$ref: '#/components/schemas/PduSession'

pgwInfo:

type: array

items:

$ref: '#/components/schemas/PgwInfo'

minItems: 1

emergencyInfo:

$ref: '#/components/schemas/EmergencyInfo'

<...skip…>

# ENUMS:

DataSetName:

anyOf:

- type: string

enum:

- AM

- SMF_SEL

- UEC_SMF

- UEC_SMSF

- SMS_SUB

- SM

- TRACE

- SMS_MNG

- LCS_PRIVACY

- LCS_MO

- UEC_AMF

- type: string

* * * End of Change * * * *