TSG-CT WG Meeting #88-e	CP-201192

E-Meeting, 29 June – 1 July 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0xxx | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the feature for ip address pool |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Supported feature is needed when the sender cannot know the behaviour of the receiver by other means and is significant for handling subsequent interaction between sender and receiver. The current feature marking IpAddrPool is redundant because: - C3-203227 agreed in CT3#110e mandates the AAA to return the selected IP address pool info in Access-Accept / DEA message which is an implicit indication for feature support. - For ip address pool info sent to DHCP server as described in clause 10.3, there is no feature needed since DHCP also returns the selected IP address pool info to the SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the feature IpAddrPool. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Over engineering for exchanging IP address pool info between SMF and AAA and mis-use of supported feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1, 12.4.0, 12.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st Change ***

### 11.3.1	General

RADIUS attributes as defined in subclause 16.4 of 3GPP TS 29.061 [5] are re-used in 5G with the following differences:

-	SMF replaces P-GW. GGSN and PPP PDP type related description are not applicable for 5G.

-	5G QoS flow replaces IP-CAN bearer and PDU session replaces IP-CAN session.

-	N6 replaces Gi/Sgi and UE replaces MS.

-	DNN replaces APN.

-	Detailed information needed for 5G compared to 3GPP TS 29.061 [5] is described below.

Table 11.3-1: Additional information needed for 5G compared to the RADIUS attributes defined in 3GPP TS 29.061 [5]

| Attr # | Attribute Name | Description | Content | Presence Requirement | Applicable message |
| --- | --- | --- | --- | --- | --- |
| 79 | EAP-Message | This attribute encapsulates EAP message (as defined in IETF RFC 3748 [6]) exchanged between the SMF and DN-AAA, see IETF RFC 3579 [7] for details. | String | Conditional NOTE | Access-Request, Access-Accept, Access-Challenge, Access-Reject, CoA-Request, CoA-ACK, Disconnect-Request, Disconnect-ACK |
| 80 | Message-Authenticator | This attribute includes the message authenticator, see IETF RFC 3579 [7] for details. | String | Conditional NOTE | Access-Request, Access-Accept, Access-Challenge, Access-Reject, CoA-Request, CoA-ACK, CoA-NAK Disconnect-Request, Disconnect-ACK, Disconnect-NAK |
| NOTE: Shall be present if EAP is used. |  |  |  |  |  |

Table 11.3-2: Different information needed for 5G compared to the RADIUS VSA defined in subclause 16.4.7 of 3GPP TS 29.061 [5]

| Sub-attr # | Sub-attribute Name | Differences |
| --- | --- | --- |
| 1 | 3GPP-IMSI | Re-used. |
| 2 | 3GPP-Charging-Id | Re-used. |
| 3 | 3GPP-PDP-Type | Re-used. For SMF, this sub-attribute represents PDU session type and only the values "0", "2", "3", "5" and "6" are applicable. |
| 4 | 3GPP-CG-Address | Re-used. |
| 5 | 3GPP-GPRS-Negotiated-QoS-Profile | Re-used. For SMF, it uses the format for Release indicator value "15" as defined in 3GPP TS 29.061 [5]. |
| 6 | 3GPP-SGSN-Address | Re-used. It includes AMF IPv4 address. |
| 7 | 3GPP-GGSN-Address | Re-used. It includes (home) SMF control plane IPv4 address. |
| 8 | 3GPP-IMSI-MCC-MNC | Re-used. |
| 9 | 3GPP-GGSN-MCC-MNC | Re-used. MCC and MNC of the network the (home) SMF belongs to. |
| 10 | 3GPP-NSAPI | Re-used. It identifies QFI with value range 0-255. |
| 11 | 3GPP-Session-Stop-Indicator | Re-used. |
| 12 | 3GPP-Selection-Mode | Re-used. SMF maps the selection mode value from the enumeration value of DnnSelectionMode in 3GPP TS 29.502 [40]. |
| 13 | 3GPP-Charging-Characteristics | Re-used. |
| 14 | 3GPP-CG-Ipv6-Address | Re-used. |
| 15 | 3GPP-SGSN-Ipv6-Address | Re-used. It includes AMF IPv6 address. |
| 16 | 3GPP-GGSN-Ipv6-Address | Re-used. It includes (home) SMF control plane IPv6 address. |
| 17 | 3GPP-Ipv6-DNS-Servers | Re-used. |
| 18 | 3GPP-SGSN-MCC-MNC | Re-used. MCC and MNC of the network the AMF belongs to |
| 19 | 3GPP-Teardown-Indicator | Re-used. |
| 20 | 3GPP-IMEISV | Re-used. |
| 21 | 3GPP-RAT-Type | Re-used. For SMF, it uses the sub-attribute definition for P-GW and only the values "3", "7" and "51" are applicable. |
| 22 | 3GPP-User-Location-Info | Re-used. For SMF, only the values "128", "129", "130", "135" and "136" of Geographic Location Type are applicable. |
| 23 | 3GPP-MS-TimeZone | Re-used. |
| 24 | 3GPP-CAMEL-Charging-Info | Not applicable. |
| 25 | 3GPP-Packet-Filter | Re-used. |
| 26 | 3GPP-Negotiated-DSCP | Re-used. |
| 27 | 3GPP-Allocate-IP-Type | Re-used. |
| 28 | External-Identifier | Re-used. |
| 29 | TWAN-Identifier | Not applicable. |
| 30 | 3GPP-User-Location-Info-Time | Re-used. |
| 31 | 3GPP-Secondary-RAT-Usage | Not applicable. |
| 110 | 3GPP-Notification | Added. |
| 111 | 3GPP-UE-MAC-Address | Added. |
| 112 | 3GPP-Authorization-Reference | Added. |
| 113 | 3GPP-Policy-Reference | Added. |
| 114 | 3GPP-Session-AMBR | Added. |
| 115 | 3GPP-NAI | Added. |
| 116 | 3GPP-Session-AMBR-v2 | Added. |
| 117 | 3GPP-Supported-Features | Added. |
| 118 | 3GPP-IP-Address-Pool-Id | Added. |
| NOTE: 5G specific RADIUS VSAs are numbered from 110. |  |  |

# 110 – 3GPP-Notification

|  |  |  | Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 |  | 7 | 6 | 5 | 4 | 3 |  |  | 2 | 1 |
| 1 |  | 3GPP type = 110 |  |  |  |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= 3 |  |  |  |  |  |  |  |  |  |  |
| 3 |  | Spare |  |  |  |  |  |  |  | ACC |  | AUTH |

# 3GPP Type: 110

Length: 3

Octet 3 is Octet String type.

For bit 1 AUTH,

-	if the value of AUTH is set to "1", and there is IPv4 address and/or IPv6 prefix change (not allocated/de-allocated by the DN-AAA itself) and the PDU session is not terminated, the SMF shall send Access-Request message to the DN-AAA with  GPSI in Calling-Station-Id or External-Identifier attribute and IP address in:

1)	Framed-IP-Address and Framed-Ipv6-Prefix, if both IPv4 address and IPv6 prefix(es) exist for the PDU session; or

2)	Framed-IP-Address, if only IPv4 address exists for the PDU session; or

3)	Framed-Ipv6-Prefix, if only IPv6 prefix(es) exists for the PDU session.

For Ethernet PDU session, if there is UE MAC address change, the SMF shall send Access-Request message to the DN-AAA with GPSI in Calling-Station-Id or External-Identifier attribute and the complete list of used UE MAC addresses in the 3GPP-UE-MAC-Address attribute.

-	if the value is set to "0", the SMF may notify authentication DN-AAA with the UE address and GPSI based on local configuration.

For bit 2 ACC,

-	if the value is set to "1", and there is IPv4 address and/or IPv6 prefix change (not allocated/de-allocated by the DN-AAA itself) and the PDU session is not terminated, the SMF shall send Accounting-Request Interim-Update message to the DN-AAA with  GPSI in Calling-Station-Id or External-Identifier attribute and IP address in:

1)	Framed-IP-Address and Framed-Ipv6-Prefix, if both IPv4 address and IPv6 prefix(es) exist for the PDU session; or

2)	Framed-IP-Address, if only IPv4 address exists for the PDU session; or

3)	Framed-Ipv6-Prefix, if only IPv6 prefix(es) exists for the PDU session.

For Ethernet PDU session, if there is UE MAC address change, the SMF shall send Accounting-Request Interim-Update message to the DN-AAA with GPSI in Calling-Station-Id or External-Identifier attribute and the complete list of used UE MAC addresses in the 3GPP-UE-MAC-Address attribute.

-	if the value is set to "0", the SMF may notify accounting DN-AAA with the UE address and GPSI based on local configuration.

# 111 – 3GPP-UE-MAC-Address

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 111 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= 8 |  |  |  |  |  |  |  |
| 3-10 |  | MAC Address (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 111

Length: 8

It is sent from the DN-AAA to authorize UE MAC addresses. Multiple 3GPP-MAC-Address sub-attributes (maximum 16) may be sent in one RADIUS CoA or Access-Accept message. The DN-AAA shall always provide the full list of allowed MAC addresses, and SMF shall replace the existing list with the newly received one. When omitted, there is no restriction and all UE MAC addresses are permitted for the Ethernet PDU session.

When sending from the SMF to the DN-AAA, it indicates UE MAC addresses in use. Multiple 3GPP-MAC-Address sub-attributes may be sent in one RADIUS Access-Request or Accounting-Request Interim-Update message.

MAC address is Octet String type.

# 112 – 3GPP-Authorization-Reference

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 112 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |
| 3-m |  | Authorization Data Reference (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 112

Length:  m

Authoriztion Data Reference: Octet String. It is sent from the DN-AAA to refer to the local authorization data in the SMF.

# 113 – 3GPP-Policy-Reference

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 113 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |
| 3-m |  | Policy Data Reference (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 113

Length:  m

Policy Data Reference: Octet String. It is sent from the DN-AAA and used by the SMF to retrieve the SM or QoS policy data from the PCF.

# 114 – 3GPP-Session-AMBR

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 114 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |
| 3-m |  | Session AMBR (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 114

Length:  m

Session AMBR: Octet String. It is sent from the DN-AAA to authorize the PDU Session AMBR. The encoding is defined as BitRate in 3GPP TS 29.571 [39].

# 115 – 3GPP-NAI

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 115 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |
| 3-m |  | NAI (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 115

Length: m

NAI: Octet String. It shall be formatted according to subclause 14.3 of 3GPP TS 23.003 [28] that describes an NAI.

# 116 – 3GPP-Session-AMBR-v2

|  |  |  | Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 |  | 7 | 6 | 5 | 4 | 3 |  |  | 2 | 1 |
| 1 |  | 3GPP type = 116 |  |  |  |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |  |  |  |
| 3 |  | Spare |  |  |  |  |  |  |  | DL |  | UL |
| 4-5 |  | UL Session-AMBR length (octet string) |  |  |  |  |  |  |  |  |  |  |
| 6-m |  | UL Session-AMBR (octet string) |  |  |  |  |  |  |  |  |  |  |
| (m+1)-(m+2) |  | DL Session-AMBR length (octet string) |  |  |  |  |  |  |  |  |  |  |
| (m+3)-n |  | DL Session-AMBR (octet string) |  |  |  |  |  |  |  |  |  |  |

# 3GPP Type: 116

Length:  m

Octet 3 is Octet String type.

Bit 1 UL and bit 2 DL indicate if the corresponding UL and DL Session-AMBR shall be present in a respective field or not. If one of these bits is set to "0", the corresponding field shall not be present at all.

UL/DL Session AMBR: Octet String. It is sent from the DN-AAA to authorize the PDU Session AMBR. The encoding is defined as BitRate in 3GPP TS 29.571 [39].

If the feature eSessionAMBR is supported and if applicable, the DN-AAA shall send this VSA; otherwise, the DN-AAA shall send the VSA 3GPP-Session-AMBR.

# 117 – 3GPP-Supported-Features

|  |  |  | Bits |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 |  | 7 | 6 | 5 | 4 | 3 |  | 2 | 1 |
| 1 |  | 3GPP type = 117 |  |  |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |  |  |
| 3-6 |  | Vendor ID (octet string) |  |  |  |  |  |  |  |  |  |
| 7-10 |  | Feature List ID (octet string) |  |  |  |  |  |  |  |  |  |
| 11-14 |  | Feature List (octet string) |  |  |  |  |  |  |  |  |  |

# 3GPP Type: 117

Length:  m

This VSA may be present in the Access-Request (initial one) message and either the Access-Challenge (initial one) or the Access-Accept message. If present, this VSA informs the destination entity about the features that the origin entity requires to successfully complete the message exchange. The Vendor ID, Feature List ID and Feature List are encoded according to 3GPP TS 29.229 [41]. See clause 12.4.1 for more detailed information regarding the general principle of the feature negotiation with the difference that RADIUS terms replace Diameter terms. The table 12.4.1-1 defines the features applicable to the RADIUS N6 interfaces for the feature lists with a Feature-List-ID of 1.

# 118 – 3GPP-IP-Address-Pool-Id

|  |  | Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |
| 1 |  | 3GPP type = 118 |  |  |  |  |  |  |  |
| 2 |  | 3GPP Length= m |  |  |  |  |  |  |  |
| 3-m |  | IP address pool id (octet string) |  |  |  |  |  |  |  |

# 3GPP Type: 118

Length: m

The SMF may determine an IP address pool ID based on UPF ID, S-NSSAI, DNN, and IP version as described in subclause 5.8.2.2.1 in 3GPP TS 23.501 [2] and includes the IP address pool ID within 3GPP-IP-Address-Pool-Id and send it to the DN-AAA. The DN-AAA assigns IPv6 prefix or IPv4 address from the requested IP address pool. Multiple 3GPP-IP-Address-Pool-Id sub-attributes may be sent in the RADIUS Access-Request message.

Table 11.3-3 describes the sub-attributes of the 3GPP Vendor-Specific attribute described above in different RADIUS messages.

Table 11.3-3: List of the 3GPP Vendor-Specific sub-attributes for N6

| Sub-attr # | Sub-attribute Name | Description | Presence Requirement | Associated attribute (Location of Sub-attr) | Applicability |
| --- | --- | --- | --- | --- | --- |
| 110 | 3GPP-Notification | It includes all notifications that the DN-AAA wants to receive from the SMF. | Optional | Access-Accept |  |
| 111 | 3GPP-UE-MAC-Address | It is sent from the DN-AAA to authorize UE MAC addresses, or it indicates UE MAC addresses in use when sending from the SMF to the DN-AAA. | Optional | Access-Request, Access-Response, Accounting-Request Interim-Update, Change-of-Authorization |  |
| 112 | 3GPP-Authorization-Reference | It is sent from the DN-AAA to refer to the local authorization data in the SMF. | Optional | Access-Accept, Change-of-Authorization |  |
| 113 | 3GPP-Policy-Reference | It is sent from the DN-AAA and used by the SMF to retrieve the SM or QoS policy data from the PCF. | Optional | Access-Accept, Change-of-Authorization |  |
| 114 | 3GPP-Session-AMBR | It is sent from the DN-AAA to authorize the PDU Session AMBR. | Optional | Access-Accept, Change-of-Authorization |  |
| 115 | 3GPP-NAI | The Network Access Identifier identifying the UE. | Optional | Access-Request, Accounting-Request START, Accounting-Request STOP, Accounting-Request Interim-Update |  |
| 116 | 3GPP-Session-AMBR-v2 | It is sent from the DN-AAA to authorize the PDU Session AMBR, it includes separate session AMBR for UL and DL. | Optional | Access-Accept, Change-of-Authorization | eSessionABMR |
| 117 | 3GPP-Supported-Features | It indicates the supported features as specified in clause 12.4.1. | Optional | Access-Request, Access-Accept, Access-Challenge, Accounting-Request START, Accounting-Response START |  |
| 118 | 3GPP-IP-Address-Pool-Id | It indicates the IP address pool identifier. | Optional | Access-Request |  |

RADIUS attributes related to the DN-AAA initiated re-authorization and authentication challenge are described in the following subclauses.

*** Next Change ***

### 12.4.0	General

Table 12.4-1 lists the Diameter AVPs re-used by the N6 reference point from existing Diameter Applications, reference to the respective specifications and a short description of the usage within the N6 reference point.

Table 12.4-1: N6 re-used Diameter AVPs

| Attribute Name | AVP Code | Section defined | Value Type (NOTE 2) | AVP Flag rules (NOTE 1) |  |  |  | May Encr. | Applicability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Must | May | Should not | Must not |  |  |
| 3GPP-IMSI | 1 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-Charging-Id | 2 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-PDP-Type | 3 | 3GPP TS 29.061 [5] (NOTE 3) | Enumerated | V | P |  | M | Y |  |
| 3GPP-CG-Address | 4 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-GPRS-Negotiated-QoS-Profile | 5 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-SGSN-Address | 6 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-GGSN-Address | 7 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-IMSI-MCC-MNC | 8 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-GGSN-MCC-MNC | 9 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-NSAPI | 10 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Selection-Mode | 12 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-Charging-Characteristics | 13 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-CG-Ipv6-Address | 14 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-SGSN-Ipv6-Address | 15 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-GGSN-Ipv6-Address | 16 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Ipv6-DNS-Servers | 17 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-SGSN-MCC-MNC | 18 | 3GPP TS 29.061 [5] (NOTE 3) | UTF8String | V | P |  | M | Y |  |
| 3GPP-IMEISV | 20 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-RAT-Type | 21 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-User-Location-Info | 22 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-MS-TimeZone | 23 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Packet-Filter | 25 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Negotiated-DSCP | 26 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Allocate-IP-Type | 27 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| External-Identifier | 28 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-User-Location-Info-Time | 30 | 3GPP TS 29.061 [5] (NOTE 3) | OctetString | V | P |  | M | Y |  |
| 3GPP-Notification | 110 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-UE-MAC-Address | 111 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-Authorization-Reference | 112 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-Policy-Reference | 113 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-Session-AMBR | 114 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-NAI | 115 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| 3GPP-Session-AMBR-v2 | 116 | 11.3.1 | OctetString | V | P |  | M | Y | eSessionABMR |
| 3GPP-IP-Address-Pool-Id | 118 | 11.3.1 | OctetString | V | P |  | M | Y |  |
| Supported-Features | 628 | 3GPP TS 29.229 [41] | Grouped | V | M |  |  | N |  |
| NOTE 1: The AVP header bit denoted as 'M', indicates whether support of the AVP is required. The AVP header bit denoted as 'V', indicates whether the optional Vendor-ID field is present in the AVP header. For further details, see IETF RFC 6733 [24]. NOTE 2: The value types are defined in IETF RFC 6733 [24]. NOTE 3: The use of Radius VSA as a Diameter vendor AVP is described in Diameter NASREQ (IETF RFC 7155 [23]) and the P flag may be set. |  |  |  |  |  |  |  |  |  |

NOTE 1:	Attribute 3GPP-CAMEL-Charging-Info (24), TWAN-Identifier (29) and 3GPP-Secondary-RAT-Usage (31) are not applicable for 5G in the present specification.

NOTE 2:	Table 11.3-2 lists the differences between the RADIUS VSAs used in 5G and the VSAs defined in subclause 16.4.7 of 3GPP TS 29.061 [5].

*** Next Change ***

### 12.4.1	Use of the Supported-Features AVP on the N6 reference point

The Supported-Features AVP is used during session establishment to inform the destination host about the required and optional features that the origin host supports. The client shall, in the first request in a Diameter session indicate the set of supported features. The server shall, in the first answer within the Diameter session indicate the set of features that it has in common with the client and that the server shall support within the same Diameter session. Any further command messages shall always be compliant with the list of supported features indicated in the Supported-Features AVPs during session establishment. Features that are not advertised as supported shall not be used to construct the command messages for that Diameter session. Unless otherwise stated, the use of the Supported-Features AVP on the N6 reference point shall be compliant with the requirements for dynamic discovery of supported features and associated error handling on the Cx reference point as defined in clause 7.2.1 of 3GPP TS 29.229 [41].

The base functionality for the N6 reference point is the 3GPP Rel-15 standard and a feature is an extension to that functionality. If the origin host does not support any features beyond the base functionality, the Supported-Features AVP may be absent from the N6 commands. As defined in clause 7.1.1 of 3GPP TS 29.229 [41], when extending the application by adding new AVPs for a feature, the new AVPs shall have the M bit cleared and the AVP shall not be defined mandatory in the command ABNF.

As defined in 3GPP TS 29.229 [41], the Supported-Features AVP is of type grouped and contains the Vendor-Id, Feature-List-ID and Feature-List AVPs. On the N6 reference point, the Supported-Features AVP is used to identify features that have been defined by 3GPP and hence, for features defined in this document, the Vendor-Id AVP shall contain the vendor ID of 3GPP (10415). If there are multiple feature lists defined for the N6 reference point, the Feature-List-ID AVP shall differentiate those lists from one another.

On receiving an initial request application message, the destination host shall act as defined in clause 7.2.1 of 3GPP TS 29.229 [41].

Once the SMF and DN-AAA have negotiated the set of supported features during session establishment, the set of common features shall be used during the lifetime of the Diameter session.

The table below defines the features applicable to the N6 interfaces for the feature lists with a Feature-List-ID of 1.

# Table 12.4.1-1: Features of Feature-List-ID 1 used in N6

| Feature bit | Feature | M/O | Description |
| --- | --- | --- | --- |
| 0 | eSessionAMBR | M | This feature indicates the support of enhanced Session AMBR function. If supported, the DN-AAA authorizes DL and/or UL Session AMBR separately. |
| Feature bit: The order number of the bit within the Feature-List AVP where the least significant bit is assigned number "0". Feature: A short name that can be used to refer to the bit and to the feature, e.g. "5GC". M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O") in this 3GPP Release. Description: A clear textual description of the feature. |  |  |  |

*** End of Change ***