3GPP TSG-CT Meeting #88e	CP-201330

# E-Meeting, 29th June – 1st July 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.121 | CR | 0332 | rev | 4 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of minor issues in 15.2.x test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Comprion GmbH, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_UEConTest |  |  |  |  |  | Date: |  |  | 2020-05-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The 15.2.x TCs need some correction of terms, list order, removal of unneded notes, duplicate words, formatting and clarification on procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of the Acceptance criteria of TC 15.2.3. Deletion of an unneded Note in TC 15.2.1. Correction of the wording in Procedure steps in TC 15.2.2 and 15.2.4. Editorial and formatting corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case content is misleading or incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.2.1.4.2, 15.2.1.5, 15.2.2.3, 15.2.2.4.2, 15.2.2.5, 15.2.3.4.2, 15.2.3.5, 15.2.4.4.1, 15.2.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of C6-200297 which has been a revision of C6-200295 which has been a revision of C6-200247 |  |  |  |  |  |  |  |  |

…

## 15.2	Authentication procedure for 5G AKA

***** First change *****

### 15.2.1	Authentication procedure for 5G AKA - Authentication is successful

…

##### 15.2.1.4.2	Procedure

- The UE is switched on.

- Upon reception of an RRCSetupRequest message from the UE, NG-SS transmits an RRCSetup message to the UE followed by reception of an RRCSetupComplete message from the UE.

- After receipt of a REGISTRATION REQUEST message from the UE during registration, the NG-SS initiates the 5G AKA authentication procedure and sends 5G AKA-Challenge message in the AUTHENTICATION REQUEST message, uses:

ngKSI:

NAS key set identifier:	'000'

TSC:	'0'

Authentication parameter RAND (5G authentication challenge): 	128 bits value

Authentication parameter AUTN (5G Authentication challenge).	128 bits value

- Using the 5G authentication challenge data received in AUTHENTICATION REQUEST message the ME pass the RAND and AUTN to the USIM.

- Upon reception of AUTHENTICATION RESPONSE message from the UE, the NG-SS sends a SECURITY MODE COMMAND message. The UE sends a SECURITY MODE COMPLETE message.

- The NG-SS sends a REGISTRATION ACCEPT message.

5G-GUTI: 	24408300010266436587

TAI:	42 34 80 00 00 01

- The UE sends a REGISTRATION COMPLETE message.

***** Next change *****

#### 15.2.1.5	Acceptance criteria

1)	After step a) the ME shall read EFUST, EF5GS3GPPNSC and EF5GAUTHKEYS

2)	During step c) the UE shall indicate within the REGISTRATION REQUEST for the NAS key set identifier that no key is available.

3)	In step d) the ME forwards the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

4)	During step e) the UE sends an AUTHENTICATION RESPONSE message contains:

Authentication response parameter:	16 octets RES* value calculated according to 3GPP TS 24.501 [42]

5)	In step e) the UE shall send SECURITY MODE COMPLETE message

6)	After step f) the ME updates EF5GS3GPPNSC and EF5GAUTHKEYS as shown below.

***** Editorial change *****

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context)

Logically:

5GS NAS Security Context:

ngKSI:	00

KAMF:	value not checked

Uplink NAS count:	any value

Downlink NAS count:	any value

Identifiers of selected NAS integrity
and encryption algorithms:	any value

Identifiers of selected EPS NAS 
integrity and encryption algorithms
for use after mobility to EPS:	any value

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 00 | 81 | xx | xx | … | xx |

EF5GAUTHKEYS (5G authentication keys)

Logically:

KAUSF:	value not checked

KSEAF: 	value not checked

| Coding: | B1 | B2 | B3 | Bx | Bx+1 | Bx+2 | Bx+3 | .. | Bxx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 80 | L1 | xx | .. | 81 | L2 | xx | .. | xx |

…

***** Next change *****

### 15.2.2	Authentication procedure for 5G AKA – Authentication is successful - GSM UICC

…

##### 15.2.2.3 Test purpose

- To verify that the ME will ignore the GPRS Kc and will not store the GPRS Kc on USIM or in ME when the USIM computes a Kc (i.e. GPRS Kc) from CK and IK.

- To verify that the ME stores the 5GS 3GPP access NAS security context parameters and the 5G authentication keys inside the ME non-volatile memory when Service n°122 and Service n°123 are not available on the USIM.

***** Next change *****

##### 15.2.2.4.2	Procedure

- The UE is switched on.

- Upon reception of an RRCSetupRequest message from the UE, NG-SS transmits an RRCSetup message to the UE followed by reception of an RRCSetupComplete message from the UE.

- After receipt of a REGISTRATION REQUEST message from the UE during registration, the NG-SS initiates the 5G AKA authentication procedure and sends 5G-AKA-Challenge message in the AUTHENTICATION REQUEST message, uses:

ngKSI:

NAS key set identifier:	'000'

TSC:		'0'

Authentication parameter RAND (5G authentication challenge):	128 bits value

Authentication parameter AUTN (5G Authentication challenge).	128 bits value

- Using the 5G authentication challenge data received in AUTHENTICATION REQUEST message the ME pass the RAND and AUTN to the USIM, the USIM calculates the response parameter KC (i.e. GPRS Kc) from CK and IK using conversion function c3.

- Upon reception of AUTHENTICATION RESPONSE message from the UE, the NG-SS sends a SECURITY MODE COMMAND message. The UE sends a SECURITY MODE COMPLETE message.

- The NG-SS sends a REGISTRATION ACCEPT message contains:

5G-GUTI:	24408300010266436587

TAI:		42 34 80 00 00 01

- The UE sends a REGISTRATION COMPLETE message, the NG-SS sends RRCRelease message.

- The UE is switched off or the UE's radio interface is switched off to perform the DEREGISTRATION procedure.

- The UE is switched on.

- The UE sends REGISTRATION REQUEST message, indicates the 5GS mobile identity information element type "5G-GUTI"

5G-GUTI:	24408300010266436587

- The NG-SS transmits a SECURITY MODE COMMAND message using the last calculated KAMF indicated by the ngKSI to activate NAS security to activate NAS security, then the UE sends SECURITY MODE COMPLETE message.

- The NG-SS sends REGISTRATION ACCEPT to the UE with:

5G-GUTI:	24408300010266436555

TAI:	42 34 80 00 00 01

- The UE sends a REGISTRATION COMPLETE message, the NG-SS sends RRCRelease message.

- The UE is switched off or the UE's radio interface is switched off to perform the DEREGISTRATION procedure.

#### 15.2.2.5	Acceptance criteria

1)	After step a) the ME shall read EFUST.

2)	During step c) the UE shall indicate within the REGISTRATION REQUEST for the NAS key set identifier that no key is available.

3)	In step d) the ME forwards the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

4)	During step e) the UE sends an AUTHENTICATION RESPONSE message contains:

Authentication result message identity	‘0101 1010’B

ngKSI:	same value in AUTHENTICATION REQUEST message

Authentication response parameter:	16 octets RES* value calculated according to 3GPP TS 24.501 [42].

5)	In step e) the UE shall send SECURITY MODE COMPLETE message.

6)	In step g) the ME acknowledge in the REGISTRATION COMPLETE message the received 5G-GUTI.

7)	During steps e) to g) the ME does not store GPRS Kc on the USIM.

8)	In step j) the UE shall indicate in the REGISTRATION REQUEST that NAS key set identifier is set to '000' and TSC is set to '0' and uses the 5GS mobile identity information element type "5G-GUTI" with value:

5G-GUTI:	24408300010266436587.

9)	In step k) the UE shall send SECURIY MODE COMPLETE message.

…

***** Next change *****

### 15.2.3	Authentication procedure 5G AKA – AUTN fails on the USIM

…

##### 15.2.3.4.2	Procedure

a)	The UE is switched on.

b)	Upon reception of an RRCSetupRequest message from the UE, NG-SS transmits an RRCSetup message to the UE followed by reception of an RRCSetupComplete message from the UE.

c)	After receipt of a REGISTRATION REQUEST message from the UE during registration, the NG-SS initiates the 5G AK' authentication procedure and sends 5G AKA challenge data in the AUTHENTICATION REQUEST message contains:

ngKSI:

NAS key set identifier:	'000'

TSC:	'0'

Authentication parameter RAND (5G Authentication challenge): 	128 bits value

Authentication parameter AUTN (5G Authentication challenge):	128 bits value

d)	Using the data received in AUTHENTICATION REQUEST message the ME pass the RAND and AUTN to the USIM.

e)	The UICC returns AUTS parameter to the ME indicating that the verification of AUTN failed due to a synchronisation failure.

f)	The UE transmits the AUTHENTICATION FAILURE message and starts the timer T3520, the NG-SS sends an AUTHENTICATION REQUEST message.

g)	After the reception of AUTHENTICATION REQUEST message from the NG-SS, the ME forwards the received RAND, AUTN to the UICC and stops the timer T3520.

h)	After reception of AUTHENTICATION RESPONSE message from the UE, the NG-SS sends a SECURITY MODE COMMAND message, the UE sends a SECURITY MODE COMPLETE message

i)	Upon reception of REGISTRATION ACCEPT message with a new 5G-GUTI, the UE sends a REGISTRATION COMPLETE message.

…

#### 15.2.3.5	Acceptance criteria

- In step d) the ME forwards the RAND and AUTN received in AUTHENTICATION REQUEST message to the USIM.

- In step f) the UE sends AUTHENTICATION FAILURE message indicating:

5GMM cause:	#21 "Synch failure"

Authentication Failure parameter:	AUTS (see 3GPP TS 33.102 [45]

- In step g) after the reception of AUTHENTICATION REQUEST message the UE shall stop the timer T3520.

- After step h) ME shall send AUTHENTICATION RESPONSE message containing the response calculated in the USIM (RES).

…

***** Next change *****

### 15.2.4	Authentication procedure for 5G AKA - after SUPI is changed

#### …

#### 15.2.4.3	Test purpose

1)	To verify that the ME generates the 5G NAS security context parameters and stores them inside the non-volatile memory in the ME together with the SUPI from the USIM.

2)	To verify that the UE deletes existing 5G NAS security context parameters from the ME’s non-volatile memory if SUPI is changed.

#### 15.2.4.4	Method of test

##### 15.2.4.4.1	Initial conditions

The NG-SS transmits on the BCCH, with the following network parameters:

-	TAI (MCC/MNC/TAC):	 244/083/000001.

-	Access control:	unrestricted.

##### The default E-UTRAN UICC is installed into the ME and the UE is powered on.

##### 15.2.4.4.2	Procedure

a)	The UE is switched on.

b)	Upon reception of an RRCSetupRequest message from the UE, NG-SS transmits an RRCSetup message to the UE followed by reception of an RRCSetupComplete message from the UE.

c)	After receipt of a REGISTRATION REQUEST message from the UE during registration, the NG-SS initiates the 5G AKA authentication procedure and sends 5G AKA challenge message in the AUTHENTICATION REQUEST message, uses:

ngKSI:

NAS key set identifier:	'000'

TSC:	'0'

Authentication parameter RAND (5G Authentication challenge): 	128 bits value

Authentication parameter AUTN (5G Authentication challenge).	128 bits value

d)	After reception of AUTHENTICATION RESPONSE message from the UE, the NG-SS sends a SECURITY MODE COMMAND message, then the UE sends a SECURITY MODE COMPLETE message.

e)	The NG-SS sends a REGISTRATION ACCEPT message.

5G-GUTI:	24408300010266436587

TAI:		42 34 80 00 00 01

f)	The UE sends a REGISTRATION COMPLETE message, the NG-SS sends RRCRelease message.

g)	The UE is switched off or the UE's radio interface is switched off to perform the DEREGISTRATION procedure.

h)	Change the UICC configuration by setting the IMSI to (24681685533963), then switch the ME on.

i)	After the receipt of REGISTRATION REQUEST message from the UE during registration, the NG-SS transmits a SECURITY MODE COMMAND message using the last calculated KAMF indicated by the ngKSI to activate NAS security.

j)	The UE sends SECURITY MODE REJECT message.

k)	The UE is switched off.

***** End of changes *****