3GPP TSG-CT1 Meeting #114	C1-190593

# Bratislava, Slovakia, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.122 | CR | 0400 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing SoR information due to mobility registration update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Blackberry UK Ltd., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-CT |  |  |  |  |  | Date: |  |  | 2019-01-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 24.501, the SoR information can be provided to the UE during initial registration procedure as well as during mobility and periodic registration update procedure. However TS 23.122 only mentions SoR information being made available to the UE during initial registration and there is no mention of SoR information being made available to UE during mobility registration update. Interoperability impact analysis - No interoperability issues are expected as the CR does not affect any NAS messages and causes no backward compatibility issue. - The current TS is confusing and inconsistent with other specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | HPLMN may provide SoR information during mobility registration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements and inconsistent specifications as 23.122 is not aligned with 24.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

# C.1	General

The purpose of the control plane solution for steering of roaming in 5GS procedure is to allow the HPLMN to update the "Operator Controlled PLMN Selector with Access Technology" list in the UE by providing the HPLMN protected list of preferred PLMN/access technology combinations via NAS signalling. The HPLMN updates the "Operator Controlled PLMN Selector with Access Technology" based on the operator policies, which can be based on the registered VPLMN, the location of the UE, etc.

The HPLMN can configure their subscribed UE's USIM to indicate that the UE is expected to receive the steering of roaming information due to initial registration in a VPLMN. At the same time the HPLMN will mark the UE is expected to receive the steering of roaming information due to initial registration in a VPLMN, in the subscription information in the UDM. In this case, it is mandatory for the HPLMN to provide the steering of roaming information to the UE during initial registration in a VPLMN. Otherwise if such configuration is not provided in the USIM, it is optional for the HPLMN to provide the steering of roaming information to the UE during initial registration (based on operator policy). The HPLMN can provide the steering of roaming information to the UE during the registration procedure for mobility and periodic registration update. In addition, the HPLMN can request the UE to provide an acknowledgement of successful reception of the steering of roaming information.

If:

-	the UE's USIM is configured to indicate that the UE shall expect to receive the steering of roaming information during initial registration procedure but did not receive it or security check on the steering of roaming information fails;

-	the current chosen VPLMN is not contained in the list of "PLMNs where registration was aborted due to SOR";

-	the current chosen VPLMN is not part of "User Controlled PLMN Selector with Access Technology" list; and

-	the UE is not in manual mode of operation;

then the UE will perform PLMN selection with the current VPLMN considered as lowest priority.

It is mandatory for the VPLMN to transparently forward to the UE the steering of roaming information received from HPLMN and to transparently forward to the HPLMN the acknowledgement of successful reception of the steering of roaming information received from UE, both while the UE is trying to register onto the VPLMN as described in subclause C.2, and after the UE has registered onto the VPLMN as described in subclause C.3.

The procedure in this annex for steering of UE in VPLMN can be initiated by the network while the UE is trying to register onto the VPLMN as described in subclause C.2, or after the UE has registered onto the VPLMN as described in subclause C.3.

Editor's note: The requirements for the case that steering of UE information is received while the UE is accessing the network for the purpose of establishing an IMS eCall is FFS.

***** No more change *****