3GPP TSG-CT WG1 Meeting #122-e	C1-200910

# Electronic meeting, 20-28 February 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.628 | CR | 0072 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing video announcement at the same time with audio conversation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, China Telecom, China Unicom, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eIMSVideo |  |  |  |  |  | Date: |  |  | 2020-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Providing announcement during the established call is supported by TS 24.628. Acturally, there are two potential scenarios, one is providing announcement when the conversation is stopped temporaly (e.g. call hold). The other is providing announcement while the conversation is going on. Some operators think the second scenario is valuable. When the caller and callee are in audio conversation, usually their screen are available. It’s good chance to provide video announcement on these idle screens. This method to tranfer the announcement and conversation media in two parrellel media stream need to be considered in TS 24.628. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add one method to provide video announcement while the audio conversationis going on. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the method for providing video announcement at the same time with the conversation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.7.2.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Next change *****

4.2.3	Providing announcements to a user during an established communication session

A service may provide an announcement during an established communication. If an announcement is provided the service shall use one of the following methods:

-	use an Call-Info header field in a re-INVITE request;

-	use the existing media stream. The media stream may have to be re-negotiated by the service to a media type suitable for the announcement; or

-	create a new media stream by SDP offer/answer mechanism for providing the announcement.

Mixing announcements into an existing media stream requires that the AS use the 3rd party call control procedure as specified by subclause 5.7.5 in 3GPP TS 24.229 [1].

***** Next change *****

4.7.2.9.1	Providing announcements during an established communication session

The AS may use the Call-Info header field according to procedures specified by IETF RFC 3261 [4] to provide an announcement during an established communication session.

The AS may send an in-band message or media using an existing media-stream to provide an announcement during an established communication session. The AS may re-negotiate the media to a media type suitable for the announcement.

The AS may create a new media stream by using an SDP offer, including a media components based on the SDP offer or answer received from terminating UE, inluding a different type media components based on announcement media information received from MRF, to negotiate with the served UE, for providing announcement during the established communication session.