3GPP TSG-CT1 Meeting #122-e 	C1-201010

# Electronic meeting,  , 20th Feb 2020 - 28th Feb 2020 						   (was C1-200923)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 1985 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update SNPN key differences |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Ericcson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the SA2 agreement in S2-2001521 (CR2084 to 23.501) roaming between SNPNs, handover between SNPNs are not supported: “Interworking with EPS is not supported for SNPN. Also, emergency services are not supported by SNPN. Furthermore, roaming between SNPNs and handover between SNPNs are not supported.” According to the SA2 agreement in S2-2001520 (CR2097 to 23.501) the case where UE that has successfully registered with a PLMN over non-3GPP access to access SNPN services is not specified in this release: “To access SNPN services, a UE that has successfully registered with a PLMN over 3GPP access may perform another registration via the PLMN User Plane with an SNPN (using the credentials of that SNPN) following the same architectural principles as specified in clause 4.2.8 and the PLMN taking the role of "Untrusted non-3GPP access" of the SNPN, i.e. using the procedures for Untrusted non-3GPP access in clause 4.12.2 of TS 23.502 [3]. Annex D, clause D.3 provides additional details. The case where UE that has successfully registered with a PLMN over non-3GPP access to access SNPN services is not specified in this release.” Accordingly, 24.501 needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Clause 4.14.2 item h), k) and l) h) when accessing SNPN services via a PLMN using 3GPP access, access to 5GCN of the SNPN is performed using 5GMM procedures for non-3GPP access and 5GMM parameter for non-3GPP access. When accessing PLMN services via a SNPN, access to 5GCN of the PLMN is performed using 5GMM procedures for non-3GPP access and 5GMM parameter for non-3GPP access. If the UE is accessing the PLMN using non-3GPP access, the access to 5GCN of the SNPN via PLMN is not specified in this release; k) roaming is not supported in SNPN access mode; l) handover between SNPNs are not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage3 specification not aligned with stage2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

### 4.14.2	Stand-alone non-public network

If the UE is not SNPN enabled, the UE is always considered to be not operating in SNPN access mode. If the UE is SNPN enabled, the UE can operate in SNPN access mode. Details of activation and deactivation of SNPN access mode at the SNPN enabled UE are up to UE implementation.

The functions and procedures of NAS described in the present document are applicable to an SNPN and an SNPN enabled UE unless indicated otherwise. The key differences brought by the SNPN to the NAS layer are as follows:

a)	instead of the PLMN selection process, the SNPN selection process is performed by a UE operating in SNPN access mode (see 3GPP TS 23.122 [5] for further details on the SNPN selection);

b)	a "permanently forbidden SNPNs" list and a "temporarily forbidden SNPNs" list are managed by a UE operating in SNPN access mode instead of forbidden PLMN lists;

c)	inter-system change to and from S1 mode is not supported;

d)	emergency services are not supported in SNPN access mode;

e)	CAG is not supported in SNPN access mode;

f)	with respect to the 5GMM cause values:

1)	5GMM cause values #74 "Temporarily not authorized for this SNPN" and #75 "Permanently not authorized for this SNPN" are supported whereas these 5GMM cause values cannot be used in a PLMN;

2)	5GMM cause values #11 "PLMN not allowed" and #73 "Serving network not authorized" are not supported whereas these 5GMM cause values can be used in a PLMN;and

3)	5GMM cause value #76 "Not authorized for this CAG or authorized for CAG cells only" is not supported whereas this 5GMM cause value can be used in a PLMN; and

Editor's note [WI: Vertical_LAN, CR#1286]:	It is FFS whether 5GMM cause values #31 "Redirection to EPC required" and # 72 "Non-3GPP access to 5GCN not allowed" are supported in an SNPN.

g)	a list of "5GS forbidden tracking areas for roaming" and a list of "5GS forbidden tracking areas for regional provision of service" are managed per SNPN (see 3GPP TS 23.122 [5]);

h)	when accessing SNPN services via a PLMN using 3GPP access, access to 5GCN of the SNPN is performed using 5GMM procedures for non-3GPP access and 5GMM parameter for non-3GPP access. When accessing PLMN services via a SNPN, access to 5GCN of the PLMN is performed using 5GMM procedures for non-3GPP access and 5GMM parameter for non-3GPP access. If the UE is accessing the PLMN using non-3GPP access, the access to 5GCN of the SNPN via PLMN is not specified in this release;

i)	when registered to an SNPN, the UE shall use only the UE policies provided by the registered SNPN;

j)	equivalent SNPN is not supported;

k)	roaming is not supported in SNPN access mode; and

l)	handover between SNPNs is not supported.

*** End change ***