# 3GPP TSG CT WG1 Meeting#124-e													C1-203003

# Electronic meeting, 02-10 June 2020

| Meeting documents by agenda item Meeting: Meeting #124-e Electronic meeting 02 - 10 June 2020 All indicated times are CEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cyan background means allocated but not available. |  |  |  |  | Yellow background means available but not yet treated document. | Green background means this document was agreed at a revious meeting in this plenary cycle. |  |  |  | White background means that the document has been handled in the meeting and a decision has been made. |
|  |  | Additional Colour coding for Tdocs in the 1st row |  |  |  |  |  |  |  |  |
|  |  | Late Papers |  |  |  |  |  |  |  |  |
|  |  | Easy and uncontroversial papers – can be presented within 2 minutes |  |  |  |  |  |  |  |  |
|  |  | Papers for common sessions |  |  |  |  |  |  |  |  |
|  |  | Low Priority |  |  |  |  |  |  |  |  |
| Agenda item | Agenda item title |  | Tdoc | Title |  |  | Source | Spec | Result |  |
|  | Opening & welcome |  | Tdoc | Title |  |  | Source | Spec | Result |  |
|  |  |  | IPR Policy Reminder to Individual Members and the persons making the technical proposals about their obligations under their respective Organizational Partners IPR Policy: I draw your attention to your obligations under the 3GPP Partner Organizations' IPR policies. Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP. |  |  |  |  |  |  |  |
|  |  |  | Antitrust & Competition I also draw your attention to the fact that 3GPP acti ities are subject to all applicable antitrust and competition laws and that compliance with said laws is therefore required of any participant of this TSG/WG meeting including the Chairman and Vice Chairman. In case of question I recommend that you contact your legal counsel. The leadership shall conduct the present meeting with impartiality and in the interests of 3GPP. Furthermore, I would like to remind you that timely submission of work items in advance of TSG/WG meetings is important to allow for full and fair consideration of such matters. |  |  |  |  |  |  |  |
|  |  |  | Usage if WiFi During 3GPP meetings, IT support staff have noticed an increasing amount of RF pollution from private, ad hoc, wireless networks (Wi-Fi Direct, hot-spots hosted on mobile phones, …), and this gives rise to reduced throughput capability of the 3GPP WLAN. I would like to remind delegates to disable all such non-3GPP Wi-Fi networks while they are in the meeting rooms or adjacent areas. This will allow the quality of connection to the 3GPP Wi-Fi network which delegates have a right to expect. |  |  |  |  |  |  |  |
|  |  |  | Statement Regarding Engagement with Companies Added to the U.S. Export Administration Regulations (EAR) Entity List in 3GPP Activities 1. Public Information is Not Subject to EAR 3GPP is an open platform where all contributions (including technology protected or not by patent) made by the different Individual Members under the membership of each respective Organizational Partner are publicly available. Indeed, contributions by all and any Individual Members are uploaded to a public file server when received and then the documents are effectively in the public domain. In addition, since membership of email distribution lists is open to all, documents and emails distributed by that means are considered to be publicly available. As a result, information contained in 3GPP contributions, documents, and emails distributed at 3GPP meetings or by 3GPP email distribution lists, because it is made available to the public without restrictions upon its further dissemination, is not subject to the export restrictions of the EAR. Meeting minutes are maintained for 3GPP meetings. Such meeting minutes for 3GPP meetings are made available to the public without restrictions upon its further dissemination. As a result, information, including information conveyed orally, contained in 3GPP meetings is not subject to the export restriction of the EAR; this would include information conveyed during side meetings that may occur during the main meetings, if these meetings are open to any participants and the results of all said meetings are publicly available without restrictions upon their further dissemination. 2. Non-Public Information Non-public information refers to the information not contained or not intended to be contained in 3GPP contributions, documents or emails. Such non-public information may be disclosed during informal meetings, exchanges, discussions or any form of other communication outside the 3GPP meetings and email distribution lists, and may be subject to the EAR. 3. Other Information Certain encryption software controlled under the International Traffic in Arms Regulations (ITAR), even if publicly available, may still be subject to US export controls other than the EAR. 4. Conduct of Meetings The situation should be considered as "business as usual" during all the meetings called by 3GPP. 5. Responsibility of Individual Members It should be remembered that contributions, meetings, exchanges, discussions or any form of other communication in or outside the 3GPP meetings are of the accountability, integrity and the responsibility of each Individual Member. In addition, Individual Members remain responsible for uring their compliance with all applicable export control regulations, including but not limited to EAR. Individual Members with questions regarding the impact of laws and regulations on their participation in 3GPP should contact their companies’ legal counsels. |  |  |  |  |  |  |  |
|  |  |  | Please remember: - to perform the electronic registration before end-of-meeting - to wear your badge |  |  |  |  |  |  |  |
|  | Agenda & Reports |  | Tdoc | Title |  |  | Source | Doctype | Result & comments |  |
|  |  |  | C1-203000 | 3GPP TSG CT1#124-e – agenda for Tdoc allocation |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203001 | 3GPP TSG CT1#124-e – agenda after Tdoc allocation deadline |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203002 | 3GPP TSG CT1#124-e – agenda with proposed LS-actions |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203003 | 3GPP TSG CT1#124-e – agenda at start of meeting |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203004 | 3GPP TSG CT1#124-e – agenda Tuesday (09 June) evening |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203005 | 3GPP TSG CT1#124-e – agenda at end of meeting |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203005 | 3GPP TSG CT1#124-e – agenda at end of meeting |  |  | CT1 chairman | agenda |  |  |
|  |  |  | C1-203006 | draft C1-123e meeting report |  |  | MCC | report |  |  |
|  |  |  |  |  |  |  |  |  | Hightest number C1-203772 |  |
|  |  |  | Agenda Start of e-meeting: Tuesday 2nd June 09:00 CEST Comment Free Time Tuesday 9th June 12:00-16:00 CEST Last revision upload: Tuesday 9th June 16:00 CEST Last comments: Wednesday 10th June 16:00 CEST Chairman’s report of the meeting: Thursday 11th June 12:00 CEST 1 Opening 2 Agenda and Reports 3 work organization 4 incoming LS (47) Rel-14 and earlier: 8.1 all work items (0) 8.2 all work items (0) 9.1 all work items (0) 9.2 all work items (0) 10.1 all work items (0) 10.2 all work items (0) 11.1 all work items (0) 11.2 all work items (0) 12.1 all work items (0) 12.2 all work items (0) 13.1 all work items (5+15) 13.2 all work items (0) 13.3 all work items (0) 14.1 all work items (7+14) 14.2 all work items (0) 14.3 all work items (1+2) Rel-15: 15.1.1 all work items (2+2) 15.1.2 all work items (0) 15.1.3 all work items (11+8) Rel-16: Agenda Items from 16.1 16.1.x (6) Agenda Items from 16.2 16.2.2 SINE_5G (1) 16.2.3 SAES16 (all aspects) (11) 16.2.4 5GProtoc16 (all aspects) (125) 16.2.5 ATSSS (16) 16.2.6 eNS (40) 16.2.7.x vertical-LAN (53) 16.2.8 5G_CIoT (41) 16.2.9 5WWC (18) 16.2.11 5G_eLCS (5) 16.2.14 RACS (5) 16.2.15 5G_SRVCC (0) 16.2.16 xBDT () 16.2.17 IAB-CT (2) 16.2.18 5GS_OTAF (1) 16.2.19 5G_URLLC (0) 16.2.21 Rel-16 non-IMS issues (37) 16.2.1 ePWS (3) 16.2.10 PARLOS (3) 16.2.12 V2XAPP (23) 16.2.13 eV2XARC (59) 16.2.20 SEAL (26) Agenda Items from 16.3 16.3.1 MCCI_CT (0) 16.3.2 MCProtoc16 (50) 16.3.5 MCSMI_CT (0) 16.3.6 eMCDATA2 (15) 16.3.10 MONASTERY2 (49) 16.3.12 enh2MCPTT-CT (0) 16.3.3 MuD (0) 16.3.4 IMSProtoc16 (1) 16.3.7 E2E_DELAY (0) 16.3.8 VBCLTE (0) 16.3.11 eIMS5G_SBA (0) 16.3.13 eIMSVideo (1) 16.3.14 IMS/MC TEI16 (7) Rel-17: Agenda Items from 17.1 17.1.1 (10) 17.1.2 (3) 17.1.3 (2) 17.1.4 (2) 18 outgoing LS (15) |  |  |  |  |  |  |  |
|  | Work organisation |  | Tdoc | Title |  |  | Source | To / CC | Result & comments |  |
|  | Meeting schedule |  |  |  |  |  |  |  |  |  |
|  |  |  |  | CT1 and CT plenary meeting dates. |  |  |  |  |  |  |
|  |  |  |  | Date |  |  | Meeting |  | Venue |  |
|  |  |  |  | 13 – 17 January |  |  | CT1-Potential Ad-Hoc |  | cancelled |  |
|  |  |  |  | 16 – 22 January |  |  | CT1#121bis-e |  | Electronic Meeting |  |
|  |  |  |  | 24 – 28 February |  |  | CT1#122 |  | cancelled |  |
|  |  |  |  | 20 – 28 February |  |  | CT1#122-e |  | Electronic Meeting |  |
|  |  |  |  | 16 – 17 March 2020 |  |  | CT plenary #87 |  | Electronic Meeting |  |
|  |  |  |  | 20 – 24 April |  |  | CT1#123 |  | cancelled |  |
|  |  |  |  | 16 – 24 April |  |  | CT1#123-e |  | Electronic Meeting |  |
|  |  |  |  | 25 – 29 May |  |  | CT1#124 |  | F2F cancelled |  |
|  |  |  |  | 02 – 10 June |  |  | CT1#124-e |  | Electronic Meeting |  |
|  |  |  |  | 29 June – 1 July. 2020 |  |  | CT plenary #88-e |  | Electronic Meeting |  |
|  |  |  |  | 13 – 17 July |  |  | CT1-Potential Ad-Hoc |  | cancelled |  |
|  |  |  |  | 24 – 28 August |  |  | CT1#125 |  | F2F cancelled |  |
|  |  |  |  | 20 – 28 August |  |  | CT1#125-e |  | Electronic Meeting |  |
|  |  |  |  | 14 – 15 September 2020 |  |  | CT plenary #89 |  | Funchal, Madeira |  |
|  |  |  |  | 12 – 16 October |  |  | CT1#126 |  | India |  |
|  |  |  |  | 16 – 20 November |  |  | CT1#127 |  | US |  |
|  |  |  |  | 7 – 8 December 2020 |  |  | CT plenary #90 |  | NAF |  |
|  |  |  |  | 25 – 29 January 2021 |  |  | CT1#127bis |  | tbd |  |
|  |  |  |  | 01- 05 March 2021 |  |  | CT1#128 |  | tbd |  |
|  |  |  |  | 22 – 23 March 2021 |  |  | CT plenary #91 |  | US |  |
|  |  |  |  | 19 – 23 April 2021 |  |  | CT1#129 |  | tbd |  |
|  |  |  |  | 24 – 28 May 2021 |  |  | CT1#130 |  | tbd |  |
|  |  |  |  | 14 – 15 June 2021 |  |  | CT plenary #92 |  | Japan |  |
|  | Work Plan and other adm. issues |  | Tdoc | Title |  |  | Source | Spec / doctype | Result & comments |  |
|  |  |  | C1-203007 | work plan |  |  | MCC | Work Plan | Revision of C1-202051 |  |
|  |  |  | C1-203072 | CT1#124-e Electronic Meeting – Process and Scope |  |  | CT1 chairman | other |  |  |
|  |  |  | C1-203080 | Decision making– electronic show of hands |  |  | CT1 chairman | other |  |  |
|  | Input Liaison statements |  | Tdoc | Title |  |  | Source | To / CC | Result & comments |  |
|  |  |  | C1-203008 | LS on Proposal to transfer the study on service-based support for SMS in 5GC to CT WGs (CP-193301) |  |  | TSG CT | Cc | Proposed Noted Mariusz, Tue, 10:06 Orange will bring a wid led by CT4 to the next meetings |  |
|  |  |  | C1-203009 | LS reply to SA2 on PLMN Selection (5GJA12_115r3) |  |  | GSMA 5G Joint Activity | To | Proposed Noted There is no action for CT1 |  |
|  |  |  | C1-203010 | LS on status of 5WWC work (LIAISE-390) |  |  | Broadband Forum | To | Proposed tbd draft LS out in C1-203474 Related CR inC1-203479 |  |
|  |  |  | C1-203011 | Reply LS on Rel-16 NB-IoT enhancements (R2-2004053) |  |  | RAN2 | To | Proposed Noted CRs already agreed |  |
|  |  |  | C1-203012 | LS response to SA3 on the security related issues for NR SL (R2-2004083) |  |  | RAN2 | Cc | Proposed Noted |  |
|  |  |  | C1-203013 | LS on Questions on onboarding requirements (S1-201087) |  |  | SA1 | Cc | Proposed Noted |  |
|  |  |  | C1-203014 | LS on PLMN selection solutions for satellite access (S2-1912551) |  |  | SA2 | To | Proposed tbd draft LS out in C1-203115 Related Disc in C1-203369 |  |
|  |  |  | C1-203015 | LS on GSMA NG.116 Attribute Area of service and impact on PLMN selection (S2-2001726) |  |  | SA2 | To | Proposed Noted GSMA replied in C1-203009 |  |
|  |  |  | C1-203016 | Questions on onboarding requirements (S2-2001729) |  |  | SA2 | Cc | Proposed Noted |  |
|  |  |  | C1-203017 | Reply LS on Questions on onboarding requirements (S2-2003216) |  |  | SA2 | Cc | Proposed Noted |  |
|  |  |  | C1-203018 | Reply LS on assistance indication for WUS (S2-2003217) |  |  | SA2 | Cc | Proposed Noted |  |
|  |  |  | C1-203019 | Reply LS on support for eCall over NR (S2-2003308) |  |  | SA2 | To | Proposed Noted Proposal for a CT1 reply to SA in C1-203221 CR related to the SA LS in C1-203038 |  |
|  |  |  | C1-203020 | Reply LS on the use of service area restriction for NSSAA (S2-2003474) |  |  | SA2 | To | Proposed Noted No action for CT1 |  |
|  |  |  | C1-203021 | Reply LS on MO exception data (S2-2003504) |  |  | SA2 | Cc | Proposed Noted |  |
|  |  |  | C1-203022 | LS on SA WG2 status of MT-EDT in Rel-16 (S2-2003505) |  |  | SA2 | To | Proposed Noted No action for CT1 |  |
|  |  |  | C1-203023 | Response LS on the Usage of Version ID (S2-2003506) |  |  | SA2 | To | Proposed Noted Do we have CRs? Our spec is already aligned, nothing needed |  |
|  |  |  | C1-203024 | Reply LS on IANA assigned values for mission critical (S3-194603) |  |  | SA3 | To | Proposed tbd draft LS out in C1-203503 Related CRs in C1-203499-502 |  |
|  |  |  | C1-203025 | SA3 Reply LS on how the IWF obtains key material for interworking group and private communications (S3-200649) |  |  | SA3 | To | Proposed tbd No CRs to the meeting |  |
|  |  |  | C1-203026 | Reply LS on AUSF role in slice specific authentication (S3-200821) |  |  | SA3 | To | Proposed Noted LS out proposal in C1-203121 Related CR in C1-203122 |  |
|  |  |  | C1-203027 | LS on Specifying the PC5-S messages that can be processed without protection (S3-200834) |  |  | SA3 | To | Proposed Noted Related CR in C1-203118 |  |
|  |  |  | C1-203028 | LS on Location information for SMS over IMS (S3i200161) |  |  | SA3 LI | Cc | Proposed Noted |  |
|  |  |  | C1-203029 | Reply on QoE Measurement Collection (S4-200241) |  |  | SA4 | To | Proposed Noted See also C1-203036 |  |
|  |  |  | C1-203030 | LS on RTP/RTCP Verification (S4-200340) |  |  | SA4 | To | Proposed Noted Simon, Wed, 00:29 No change to CT1 specs are needed |  |
|  |  |  | C1-203031 | Reply LS to Transfer the study on service-based support for SMS in 5GC to CT WGs (SP-191362) |  |  | TSG SA | Cc | Proposed Noted |  |
|  |  |  | C1-203034 | Response LS on the support for ECN in 5GS (R2-2004284) |  |  | RAN2 | Cc | Proposed Noted |  |
|  |  |  | C1-203035 | LS on Reply on QoE Measurement Collection (S5-202305) |  |  | SA5 | Cc | Proposed Noted |  |
|  |  |  | C1-203036 | LS on Reply on QoE Measurement Collection (S5-202304) |  |  | SA5 | To | Proposed Noted draft LS out in C1-203674 Related CR in C1-203670 |  |
|  |  |  | C1-203039 | Reply LS on manual CAG selection (R2-2003870) |  |  | RAN2 | To | Proposed Noted Related CR in C1-203601 |  |
|  |  |  | C1-203040 | LS on early UE capability retrieval for eMTC (R2-2003935) |  |  | RAN2 | To | Proposed tbd draft LS out in C1-203482 Related CR in C1-203483 |  |
|  |  |  | C1-203041 | LS on UAC applicability to IABs (R2- 2003941) |  |  | RAN2 | To | Proposed Noted Related CRs in C1-203226, C1-203512 |  |
|  |  |  | C1-203042 | Reply LS on suspend indication 5G NAS (R2-2003942) |  |  | RAN2 | To | Proposed Noted Related CR in C1-203289 |  |
|  |  |  | C1-203043 | LS on IAB supporting in NPN deployment (R2-2004282) |  |  | RAN2 | Cc | Proposed Noted |  |
|  |  |  | C1-203073 | Issue with FN-RG IPv6 support (LIAISE-394) |  |  | Broadband Forum | To | Proposed Noted Related CR in C1-203068 |  |
|  |  |  |  |  | C1-203333 |  | LS on the applicability of 5G NAS protocol for 5G-RG and FN-RG (LIAISE-397) |  |  |  | Broadband Forum |  | To |  | Proposed tbd draft LS out in C1-203473 |  |  |
|  |  |  |  |  | C1-203749 |  | Reply LS on selected EPS NAS algorithms for unauthenticated emergency sessions in 5GS (S3-201300) |  |  |  | SA3 |  | To |  | Proposed Noted Related CR in C1-203543 |  |  |
|  |  |  |  |  | C1-203750 |  | LS on protection of allowed CAG list against MITM Attack (S3-201414) |  |  |  | SA3 |  | Cc |  | Proposed Noted |  |  |
|  |  |  |  |  | C1-203751 |  | Reply LS on SUCI computation from an NSI (S3-201432) |  |  |  | SA3 |  | To |  | Proposed Noted No CRs needed |  |  |
|  |  |  |  |  | C1-203752 |  | LS reply on security context for 5GC to EPC mobility (S3-201453) |  |  |  | SA3 |  | To |  | Proposed Noted No CRs needed |  |  |
|  |  |  |  |  | C1-203754 |  | LS on security consideration of performance measurement function protocol (S3-201490) |  |  |  | SA3 |  | To |  | Proposed Noted Related to CR in C1-203081 |  |  |
|  |  |  |  |  | C1-203755 |  | LS on Updated User Plane Integrity Protection advice (S3-201487) |  |  |  | SA3 |  | To |  | Proposed tbd draft LS out in C1-203537 Related CRs in C1-203533, C1-203534 and C1-203535. Revision of C1-203753 |  |  |
|  |  |  | C1-203766 | Reply LS on CMAS/ETWS and emergency services for SNPNs (S1-202220) |  |  | SA1 | Cc | Proposed Noted |  |
|  |  |  | C1-203767 | Reply LS on Manual CAG ID selection and granularity of UAC parameters for PNI-NPNs (S1-202265) |  |  | SA1 | Cc | Proposed Noted |  |
|  |  |  | C1-203768 | Reply LS on Questions on onboarding requirements (S1-202266) |  |  | SA1 | Cc | Proposed Noted |  |
|  |  |  | C1-203769 | reply LS on UAC applicability to IABs (S1-202274) |  |  | SA1 |  | Proposed Noted |  |
|  |  |  | C1-203770 | Reply LS on manual CAG selection (S1-202277) |  |  | SA1 |  | Proposed Noted Related CR in C1-203601 |  |
|  |  |  | C1-203771 | reply LS on GSMA NG.116 Attribute Area of service and impact on PLMN (S1-202294) |  |  | SA1 |  | Proposed Noted |  |
|  |  |  | C1-203772 | Reply LS on limiting the number of simultaneous log ins of an MCX user (S1-202280) |  |  | SA1 |  | Proposed Noted CRs in C1-203721, C1-203722, more CRs might be needed |  |
|  |  |  | C1-203781 | Reply LS on assistance indication for WUS (R2-2005939) |  |  | RAN2 | Cc: CT1 | Late |  |
|  |  |  | C1-203780 | Reply LS on manual CAG ID selection and granularity of UAC parameters for PNI-NPNs (S2-2004335) |  |  | SA2 | Cc: CT1, | late |  |
|  |  |  | C1-203789 | LS Reply on QoE Measurement Collection (S4-200962) |  |  | SA4 | Cc | Late |  |
|  |  |  | C1-203788 | LS on Media Feature Tag for IMS Data Channel (S4-200908) |  |  | SA4 | To | late |  |
|  | void |  |  |  |  |  |  |  | Release 5 is closed |  |
|  | void |  |  |  |  |  |  |  | Release 6 is closed |  |
|  | void |  |  |  |  |  |  |  | Release 7 is closed |  |
|  | Release 8 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-8 IMS Work Items and issues: MRFC MRFC_TS UUSIW PktCbl-Intw PktCbl-Deploy PktCbl-Sec NBA OAM8-Trace Overlap PRIOR IMS_RP PNM IMSProtoc2 IMS_Corp ICSRA IMS-Cont MAINT_R1 MAINT_R2 REDOC_TIS-C1 REDOC_3GPP2 CCBS-CCNR CW-IMS FA CAT-SS TEI8 (IMS related issues) + all other IMS related issues |  |  | Jörgen – Breakout |  |  |  |  | All WIs completed AS – MRFC protocol (This covers both the study item and the work item) User – User Signalling interworking Packetcable - Protocol enhancements Packetcable - Regulatory requirements Packetcable - Security requirements NASS Bundled Authentication Service level tracing in IMS CT1 aspects of overlap signaling Multimedia priority service IMS restoration procedures Personal Network Management (stage 2 and 3) IP Multimedia Core Network Subsystem - IMS Stage3 Protocol Evolution for Rel-8 IMS corporate network access IMS centralized service control IMS Service Continuity TISPAN R1 and R2 maintenance 3GPP and 3GPP2 re-documentation IMS supplementary services: Call Completion on Busy Subscriber (CCBS) / Call Completion on Non-Reachable (CCNR) in IMS Communication Waiting in IMS Flexible alerting in IMS Customized alerting tone in IMS |  |
|  | Rel-8 non-IMS Work Items and issues: SAES SAES-CSFB SAES-SRVCC HomeNB-LTE HomeNB-3G ETWS PPACR-CT1 EData IWLANNSP EVA IWLAN_Mob TEI8 (non-IMS) + all other non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed SAE issues CS-Fallback SRVCC CSG, HomeeNB and HomeNB Earthquake and tsunami warning systems Paging Permission with Access Control Data transfer during an emergency call WLAN Network Selection Principles Enhancements for VGCS applications Mobility between 3GPP-WLAN Interworking and 3GPP Systems |  |
|  | Release 9 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-9 IMS Work Items and issues: Work Items: CRS eCAT-SS eMMTel-CC IMSProtoc3 IMS_SCC-SPI IMS_SCC-ICS IMS_SCC-ICS_I1 EMC2 MEDIASEC_CORE PAN_EPNM IMS_EMER_GPRS_EPS IMS_EMER_GPRS_EPS-SRVCC TEI9 (IMS related) + all other Rel-9 IMS issues |  |  | Jörgen – Breakout |  |  |  |  | All WIs completed IMS Supplementary services IMS Customized Ringing Signal Service Enhancements of IMS Customized Alerting Tone (CAT) Service Enhancements for Completion of Communications Supplementary service IMS Stage-3 IETF Protocol Alignment IMS Service Continuity Enhancements: Service, Policy, Interactions, and Inter UE Transfer Enhancements to IMS Centralized Services IMS Centralized Services support via I1 interface Definition of Ml interface for Control Plane LCS IMS Media Plane Security Support of Personal Area Networks and Enhancements to Personal Network Management Emergency Call Enhancements for IP& PS Based Calls – stage 3 IMS part SRVCC support for IMS Emergency Calls |  |
|  | Rel-9 non-IMS Work Items and issues: IMS_EMER_GPRS_EPS (non-IMS) SSAC VAS4SMS PWS-St3 eANDSF MUPSAP LCS_EPS-CPS EHNB-CT1 TEI9 (non-IMS issues) + all other Rel-9 non-IMS issues |  |  | Peter - Main |  |  |  |  | All WIs completed Support for IMS Emergency Calls over GPRS and EPS Service Specific Access Control Requirements Value-Added Services for Short Message Service Public Warning System (PWS) ANDSF while roaming Multiple PDN Connection to the Same APN for PMIP-based Interfaces Multiple PDN Connection to the Same APN for PMIP-based Interfaces Control Plane LCS in the EPC EHNB-issues for Rel-9 |  |
|  | Release 10 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-10 IMS Work Items and issues: Work Items: IMS_SC_eIDT CCNL eAoC OMR IESE eSRVCC aSRVCC AT_IMS IMSProtoc4 + all other Rel-10 IMS issues |  |  | Jörgen – Breakout |  |  |  |  | All WIs completed IMS Inter-UE Transfer enhancements Call Completion on Not Logged-in AoC enhancements Optimal Media Routing IMS Emergency Session Enhancements SRVCC enhancements SRVCC in alerting phase AT Commands for IMS-configuration IMS Stage-3 IETF Protocol Alignment |  |
|  | Rel-10 non-IMS Work Items and issues: Work Items: ECSRA_LAA-CN eMPS-CN NIMTC AT_UICC SMOG-St3 IFOM-CT LIPA SIPTO MAPCON-St3 TIGHTER MOCN-GERAN + all other Rel-10 non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed Enabling Coder Selection and Rate Adaptation for UTRAN and E-UTRAN for Load Adaptive Applications, CN impacts Enhancements for Multimedia Priority Service Network Improvements for Machine Type Communications AT Commands for USAT S2b Mobility based on GTP IP Flow Mobility and WLAN offload Local IP Access Selected IP Traffic Offload Multi Access PDN Connectivity Tightened Link Level Performance Requirements for Single Antenna MS Support of Multi-Operator Core Network by GERAN |  |
|  | Release 11 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-11 IMS Work Items and issues: Work Items: USSI IOI_IMS_CH RLI IPXS VINE-CT MRB GINI RAVEL-CT IOC IODB GBA-ext-St3 NWK-PL2IMS-CT MMTel_T.38_FAX vSRVCC-CT rSRVCC-CT ATURI IMSProtoc5 + all other Rel-11 IMS issues |  |  | Jörgen – Breakout |  |  |  |  | All WIs completed USSD Simulation Service IMS Interconnection Charging Enhancements for transit scenarios in multi operator environments CT1 aspects of RLI Advanced Interconnection of Services Supp. 3G Voice Interworking w. Enterprise IP-PBX Inclusion of Media Resource Broker Support of RFC 6140 in IMS Roaming Architecture for VoIMS w Local Breakout IMS Overload Control Operator Determined Barring GBA Extension for re-use of SIP Digest credentials Network Provided Location Information for IMS Enhanced T.38 FAX support SRVCC for 3G-CS SRVCC from UTRAN/GERAN to E-UTRAN/HSPA AT Commands for URI Support IMS Stage-3 IETF Protocol Alignment |  |
|  | Rel-11 non-IMS Work Items and issues: Work Items: RT_VGCS_Red SIMTC SIMTC-CS SIMTC-RAN_OC SIMTC-Reach SIMTC-Sig SIMTC-CN_Pow SIMTC-PS_Only BBAI BBAI-BBI BBAI-BBII BBAI-BBIII Full_MOCN-GERAN RT_ERGSM DIDA SAMOG_WLAN- CN eNR_EPC PROTOC_SMS_SGs SAES2 SAES2-CSFB + all other Rel-11 non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed GCSMSC and GCR Redundancy for VGCS/VBS System Improvements to Machine-Type Communications - CS aspects for CT groups - Extended Access Barring for UTRAN and E-UTRAN for CT groups - Reachability Aspects - Signalling Optimizations - "CN-based" and power considerations BroadBand Forum Accesses Interworking - Building Block I, II and III Full Support of Multi-Operator Core Network Introduction of ER-GSM band for GSM-R Data identification in ANDSF Mobility based on GTP & PMIPv6 for WLAN access to EPC enhanced Nodes Restoration for EPC Enhancement of the Protocols for SMS over SGs SAE Protocol Development |  |
|  | Release 12 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-12 IMS Work Items and issues: bSRVCC SMSMI-CT TURAN-CT IMS_TELEP eDRVCC EMC_PC IMS_RegCon-CT BusTI-CT UP6665 eIODB IMS_WebRTC IMS_Corp2 NNI_RS USSD_MS USSI-NET RFC7044 FS_NNI_RS eMEDIASEC-CT IMS_SSFDD CVO-CT SIS_CT FS_REVOLTE_IMS NETLOC_TWAN_CT ALTC PCSCF_RES EVS_codec-CT IMSProtoc6 TEI12 (IMS related issues) + all other Rel-12 IMS related issues |  |  | Jörgen – Breakout |  |  |  |  | All WIs completed Single Radio Voice Call Continuity (SRVCC) before ringing SMS submit and delivery without MSISDN in IMS Tunnelling of UE Services over Restrictive Access Networks IMS-based Telepresence (Stage 3) Dual-Radio VCC (DRVCC) enhancements IMS Emergency PSAP Callback CT aspects of IMS registration control CT Aspects of IMS Business Trunking for IP-PBX in Static Mode of Operation Updating IMS to conform to RFC 6665 Enhancements to IMS Operator Determined Barring Web Real Time Communication (WebRTC) Access to IMS Transfer of ETSI business trunking specifications Indication of NNI Routeing scenarios in SIP requests USSD method selection - stage-3 Network Initiated USSD Simulation Services in IMS SI: Evaluation and introduction of RFC 7044 (History-Info) Indication of NNI Routeing scenarios in SIP requests CT aspects of Extended IMS media plane security IM-SSF Application Server Service Data Descriptions CT Aspects of Coordination of Video Orientation CT Aspects of Signalling of Image Size Technical Aspects on Roaming End to End scenarios with VoLTE IMS and other networks CT aspects of Network Provided Location Information for IMS Trusted WLAN Access Network Support of ALT-C attribute P-CSCF restoration enhancements CT Impacts of Codec for Enhanced Voice Services IMS Stage-3 IETF Protocol Alignment |  |
|  | Rel-12 non-IMS Work Items and issues: LIMONET-LIPA REP-WMD MTCe-UEPCOP-CT ProSe-CT SINE SCM_LTE-CT UTRA_LTE_WLAN_interw-CT OPIIS-CT eSaMOG_St3 WORM-CT WLAN_NS-CT LIMONET-SIPTO Dia_SGSN_SMS GCSE_LTE-CT MSRD_VAMOS (GERAN) DMCG (GERAN) NewToN (GERAN) SAES3 SAES3-CSFB SAES3-non3GPP TEI12 (non-IMS) + all other Rel-12 non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed Core Network aspects of LIPA Mobility Reporting Enhancements in Warning Message Delivery UE Power Consumption Optimizations, stage 3 CT aspects of Proximity-based Services Signalling Improvements for Network Efficiency CT aspects of Smart Congestion Mitigation in E-UTRAN CT aspects of WLAN/3GPP Radio Interworking Operator Policies for IP Interface Selection Enhanced S2a Mobility Over Trusted WLAN access to EPC for Stage 3 Optimized Offloading to WLAN in 3GPP RAT mobility CT aspects of WLAN network selection for 3GPP terminals Core Network aspects of SIPTO at the local network Diameter based interface between SGSN and SMS central functions CT aspects of Group Communication System Enablers for LTE CT1 introduction of MS capability support for MS supporting MSRD for VAMOS CT part: Downlink Multi Carrier GERAN CT1 part of New Training Sequence Codes (TSC) for GERAN general Stage-3 SAE Protocol Development Stage-3 SAE Protocol Development related to Circuit Switched Fall Back Stage-3 SAE Protocol Development related to non-3GPP access |  |
|  | Release 13 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-13 Mision Critical Work Items and issues: MCPTT-CT MPTT-Prof |  |  | Jörgen – Breakout on MC |  |  |  |  | All WIs completed Mission Critical Push-To-Talk over LTE - MCPTT call control protocol - MCPTT floor control protocol Mission Critical general work - Group management - Identity management - Management Object (MO) - Configuration management IMS Profile to support Mission Critical Push To Talk over LTE |  |
|  |  |  | C1-203095 | Corrections to Off-network group call control |  |  | NIST, FirstNet | CR 0566 24.379 Rel-13 |  |  |
|  |  |  | C1-203096 | Corrections to Off-network group call control |  |  | NIST, FirstNet | CR 0567 24.379 Rel-14 |  |  |
|  |  |  | C1-203097 | Corrections to Off-network group call control |  |  | NIST, FirstNet | CR 0568 24.379 Rel-15 |  |  |
|  |  |  | C1-203098 | Corrections to Off-network group call control |  |  | NIST, FirstNet | CR 0569 24.379 Rel-16 |  |  |
|  |  |  | C1-203099 | Corrections to Off-network group call type control |  |  | NIST, FirstNet | CR 0570 24.379 Rel-13 |  |  |
|  |  |  | C1-203100 | Corrections to Off-network group call type control |  |  | NIST, FirstNet | CR 0571 24.379 Rel-14 |  |  |
|  |  |  | C1-203101 | Corrections to Off-network group call type control |  |  | NIST, FirstNet | CR 0572 24.379 Rel-15 |  |  |
|  |  |  | C1-203102 | Corrections to Off-network group call type control |  |  | NIST, FirstNet | CR 0573 24.379 Rel-16 |  |  |
|  |  |  | C1-203103 | Corrections to Off-Network Floor Control procedures |  |  | NIST, FirstNet | CR 0232 24.380 Rel-13 |  |  |
|  |  |  | C1-203104 | Corrections to Off-Network Floor Control procedures |  |  | NIST, FirstNet | CR 0233 24.380 Rel-14 |  |  |
|  |  |  | C1-203105 | Corrections to Off-Network Floor Control procedures |  |  | NIST, FirstNet | CR 0234 24.380 Rel-15 |  |  |
|  |  |  | C1-203106 | Corrections to Off-Network Floor Control procedures |  |  | NIST, FirstNet | CR 0235 24.380 Rel-16 |  |  |
|  |  |  | C1-203109 | Correction of unit values for T204 and T205 |  |  | NIST, FirstNet | CR 0068 24.483 Rel-13 |  |  |
|  |  |  | C1-203110 | Correction of unit values for T204 and T205 |  |  | NIST, FirstNet | CR 0069 24.483 Rel-14 |  |  |
|  |  |  | C1-203111 | Correction of unit values for T204 and T205 |  |  | NIST, FirstNet | CR 0070 24.483 Rel-15 |  |  |
|  |  |  | C1-203112 | Correction of unit values for T204 and T205 |  |  | NIST, FirstNet | CR 0071 24.483 Rel-16 |  |  |
|  |  |  | C1-203499 | Resolution of registered SAKKE parameters clash |  |  | Ericsson /Jörgen | CR 0040 24.481 Rel-13 |  |  |
|  |  |  | C1-203500 | Resolution of registered SAKKE parameters clash |  |  | Ericsson /Jörgen | CR 0041 24.481 Rel-14 |  |  |
|  |  |  | C1-203501 | Resolution of registered SAKKE parameters clash |  |  | Ericsson /Jörgen | CR 0042 24.481 Rel-15 |  |  |
|  |  |  | C1-203502 | Resolution of registered SAKKE parameters clash |  |  | Ericsson /Jörgen | CR 0043 24.481 Rel-16 |  |  |
|  | Rel-13 IMS Work Items and issues: voE-UTRAN _PPD-CT QOSE2EMTSI-CT DRuMS-CT RTCP-MUX IMSProtoc7 PCSCF_RES_WLAN INNB_IW mSRVCC eWebRTCi_CT ROI-CT TEI13 (IMS related issues) + all other Rel-13 IMS related issues |  |  | Jörgen – Breakout on IMS |  |  |  |  | All WIs completed Voice over E-UTRAN Paging Policy Differentiation QoS End to End MTSI extensions Double Resource Reuse for Multiple Media Sessions Support of RTP / RTCP transport multiplexing (signalling) in IMS IMS Stage-3 IETF Protocol Alignment for Rel-13 P-CSCF Restoration Enhancements with WLAN Interworking solution for Called IN number and original called IN number ISUP parameters Message interworking during PS to CS SRVCC Enhancements to WEBRTC interoperability stage 3 Video Enhancements by Region-Of-Interest information signalling |  |
|  | Rel-13 non-IMS Work Items and issues: eProSe-Ext-CT RISE WSR_EPS ePCSCF_WLAN SAES4 SAES4-CSFB SAES4-non3GPP EVSoCS-CT MONTE-CT MEI_WLAN ASI_WLAN NBIFOM-CT GROUPE-CT eDRX-CT SEW1-CT CIoT-CT NB_IOT EC-GSM-IoT EASE_EC_GSM DECOR-CT TEI13 (non-IMS) + all other Rel-13 non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed Enhancements to Proximity-based Services extensions Retry restriction for Improving System Efficiency Warning Status Report in EPS Enhanced P-CSCF discovery using signalling for access to EPC via WLAN general Stage-3 SAE Protocol Development Stage-3 SAE Protocol Development related to Circuit Switched Fall Back Stage-3 SAE Protocol Development related to non-3GPP access EVS in 3G Circuit-Switched Networks Monitoring Enhancements CT aspects Mobile Equipment signalling over the WLAN access Authentication Signalling Improvements for WLAN IP Flow Mobility support for S2a and S2b Interfaces Group based Enhancements CT aspects of extended DRX cycle for power consumption optimization CT aspects of Support of Emergency services over WLAN – phase 1 CT1 aspects of WIs with IoT-functionality (WIs from C, RAN & SA Dedicated Core Networks CT aspects |  |
|  | Release 14 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-14 Mision Critical Work Items and issues: MCImp-MCVIDEO-CT MCImp-MCDATA-CT MCImp-eMCPTT-CT MCPTTProtoc1 |  |  | Jörgen |  |  |  |  | All WIs completed Mission Critical Video – CT aspects Mission Critical Data – CT aspects Enhancements for Mission Critical Push To Talk – CT aspects Technical enhancements for Mission Critical Push To Talk over LTE protocol aspects |  |
|  |  |  | C1-203608 | Specification of MONP messages to support off-network MCData and MCVideo |  |  | Ericsson /Jörgen | CR 0610 24.379 Rel-14 |  |  |
|  |  |  | C1-203610 | Specification of MONP messages to support off-network MCData and MCVideo |  |  | Ericsson /Jörgen | CR 0611 24.379 Rel-15 |  |  |
|  |  |  | C1-203611 | Specification of MONP messages to support off-network MCData and MCVideo |  |  | Ericsson Jörgen | CR 0612 24.379 Rel-16 |  |  |
|  |  |  | C1-203612 | Off-network MCData support |  |  | Ericsson /Jörgen | CR 0171 24.282 Rel-14 |  |  |
|  |  |  | C1-203613 | Off-network MCData support |  |  | Ericsson /Jörgen | CR 0172 24.282 Rel-15 |  |  |
|  |  |  | C1-203614 | Off-network MCData support |  |  | Ericsson /Jörgen | CR 0173 24.282 Rel-16 |  |  |
|  |  |  | C1-203628 | Off-network MCVideo support |  |  | Ericsson LM | CR 0090 24.281 Rel-14 |  |  |
|  |  |  | C1-203630 | Off-network MCVideo support |  |  | Ericsson /Jörgen | CR 0091 24.281 Rel-15 |  |  |
|  |  |  | C1-203638 | Off-network MCVideo support |  |  | Ericsson /Jörgen | CR 0092 24.281 Rel-16 |  |  |
|  |  |  | C1-203677 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0613 24.379 Rel-16 |  |  |
|  |  |  | C1-203678 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0242 24.380 Rel-16 |  |  |
|  |  |  | C1-203679 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0614 24.379 Rel-15 |  |  |
|  |  |  | C1-203680 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0243 24.380 Rel-15 |  |  |
|  |  |  | C1-203681 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0615 24.379 Rel-14 |  |  |
|  |  |  | C1-203682 | SSRC handling for implicit floor request case |  |  | Samsung | CR 0244 24.380 Rel-14 |  |  |
|  |  |  | C1-203685 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0174 24.282 Rel-16 |  |  |
|  |  |  | C1-203686 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0012 24.582 Rel-16 |  |  |
|  |  |  | C1-203687 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0175 24.282 Rel-15 |  |  |
|  |  |  | C1-203688 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0013 24.582 Rel-15 |  |  |
|  |  |  | C1-203689 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0176 24.282 Rel-14 |  |  |
|  |  |  | C1-203690 | Adding mcdata id in signalling payload for sender of the data in MCData media plane (Session) communication. |  |  | Samsung | CR 0014 24.582 Rel-14 |  |  |
|  | Rel-14 IMS Work Items and issues: ISAT MMCMH_Enh-CT IOC_UE_conf PWDIMS-CT IMSProtoc8 V8-CT RobVoLTE-CT REAS_EXT CH14-DCCII-CT SPECTRE-CT TEI14 (IMS related issues) + all other Rel-14 IMS related issues |  |  | Jörgen – Breakout on IMS |  |  |  |  | All WIs completed IMS Signalling Activated Trace CT1 aspects of MTSI Extension on Multi-stream Multiparty Improved operator control using new UE configuration parameters Password based service activation for IMS Multimedia Telephony service IMS Stage-3 IETF Protocol Alignment for Rel-14 CT Aspects of S8 Home Routing Architecture for VoLTE CT Aspects of Robust Call Setup for VoLTE subscriber in LTE SIP Reason header extension CT Aspects of Determination of Completeness of Charging Information in IMS User Controlled Spoofed Call Treatment |  |
|  | Rel-14 non-IMS Work Items and issues: EIEI-CT NonIP_GPRS-CT EWE-CT SAES5 SAES5-CSFB SAES5-non3GPP V2X-CT eDECOR-CT AT_CIoT SEW2-CT ERP-CT AE_enTV-CT CIoT-Ext-CT PS_DATA_OFF-CT TEI14 (non-IMS) + all other Rel-14 non-IMS issues |  |  | Peter – Main |  |  |  |  | All WIs completed CT aspects of evolution to and interworking with eCall in IMS CT aspects for Non-IP for Cellular Internet of Things for 2G/3G-GPRS EIR check for WLAN access to EPC general Stage-3 SAE Protocol Development Stage-3 SAE Protocol Development related to Circuit Switched Fall Back Stage-3 SAE Protocol Development related to non-3GPP access CT aspects of V2X Services CT aspects of Enhancements of Dedicated Core Networks AT Commands for CIoT CT aspects of Support of Emergency services over WLAN – phase 2 Support of EAP Re-authentication Protocol for WLAN Interworking CT aspects of system architecture enhancements for TV service Core network aspects of extended Architecture support for CIoT CT aspects of PS data off function |  |
|  |  |  | C1-203632 | Adding the new V2X message family |  |  | CATT | CR 0025 24.386 Rel-14 | Shifted from 14 Ivo, Tue, 09:35 Not essential Frederic, Tue, 13:45 “?” to be removed form cover page Sunghoon, Tue, 14:28 QCOM wants to co-sign Christian, Thu, 11:31 Wants to co-sign, |  |
|  |  |  | C1-203633 | Adding the new V2X message family |  |  | CATT | CR 0026 24.386 Rel-15 | Shifted from 14 Ivo, Tue, 09:35 Not essential Sunghoon, Tue, 14:28 QCOM wants to co-sign Christian, Thu, 11:31 Wants to co-sign, |  |
|  |  |  | C1-203660 | Adding the new V2X message family |  |  | CATT | CR 0027 24.386 Rel-16 | Shifted from 14 Ivo, Tue, 09:35 is the document publicly available? it does not seem to be available at http://www.ccsa.org.cn Scott, Tue, 10:41 Provides the link Sunghoon, Tue, 14:28 QCOM wants to co-sign Ivo, Wed, 22:21 Fine Christian, Thu, 11:31 Wants to co-sign, |  |
|  | Release 15 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Rel-15 Mission Critical work items and issues: eMCVideo-CT eMCDATA-CT enhMCPTT-CT MCProtoc15 MONASTERY MBMS_MCservices |  |  | Jörgen |  |  |  |  | All work items complete Enhancements to Mission Critical Video – CT aspects Enhancements for Mission Critical Data – CT aspects Enhancements for Mission Critical Push-to-Talk – CT aspects Protocol enhancements for Mission Critical Services sion Critical Push-to-Talk – CT aspects Mobile Communication System for Railways MBMS usage for mission critical communication services |  |
|  |  |  | C1-203253 | Location reporting corrections |  |  | Ericsson /Jörgen | CR 0237 24.380 Rel-15 |  |  |
|  |  |  | C1-203254 | Location reporting corrections |  |  | Ericsson /Jörgen | CR 0238 24.380 Rel-16 |  |  |
|  |  |  | C1-203683 | Simultaneous reception of media at transmission control server (reception control) |  |  | Samsung | CR 0074 24.581 Rel-16 |  |  |
|  |  |  | C1-203684 | Simultaneous reception of media at transmission control server (reception control) |  |  | Samsung | CR 0075 24.581 Rel-15 |  |  |
|  | Rel-15 IMS work items and issues 5GS_Ph1-IMSo5G eCNAM-CT FS_PC_VBC (CT3) IMSProtoc9 bSRVCC_MT eSPECTRE PC_VBC (CT3) TEI15 (IMS) |  |  | Jörgen |  |  |  |  | All work items complete IMS impact due to 5GS IP-CAN CT aspects of Enhanced Calling Name Service Study on Policy and Charging for Volume Based Charging IMS Stage-3 IETF Protocol Alignment for Rel-15 SRVCC for terminating call in pre-alerting phase Enhancements to Call spoofing functionality Policy and Charging for Volume Based Charging |  |
|  |  |  | C1-202584 | Reference update for PASSporT Extension for Diverted Calls |  |  | Orange / Mariusz | CR 6416 24.229 Rel-15 | Agreed |  |
|  |  |  | C1-202585 | Reference update for PASSporT Extension for Diverted Calls |  |  | Orange / Mariusz | CR 6417 24.229 Rel-16 | Agreed |  |
|  | Rel-15 non-IMS/non-MC work items and issues 5GS_Ph1-CT EDCE5-CT ProSe_WLAN_DD_Stage3 VoWLAN-CT PS_DATA_OFF2-CT LTE_LIGHT_CON-CT AT_CIoT-Ext SAES6 INOBEAR-CT TEI15 |  |  | Peter |  |  |  |  | All work items complete CT aspects on 5G System - Phase 1 EPC enhancements to support 5G New Radio via Dual Connectivity Inclusion of WLAN direct discovery technologies as an alternative for ProSe direct discovery Complementary Features for Voice services over WLAN PS Data Off Phase 2 CT aspects of signalling reduction to enable light connection for LTE AT Commands for CIoT-Ext Stage-3 SAE Protocol Development for Rel-15 Increasing the number of EPS bearers Other Rel-15 non-IMS topics |  |
|  |  |  | C1-203044 | Remove USE_TRANSPORT_MODE in response |  |  | ZTE / Joy, Huawei, HiSilicon, Ericsson | CR 0124 24.502 Rel-15 | Revision of C1-202786 ------------------------------------------ Was agreed Revision of C1-202291 |  |
|  |  |  | C1-203045 | Remove USE_TRANSPORT_MODE in response |  |  | ZTE / Joy, Huawei, HiSilicon, Ericsson | CR 0125 24.502 Rel-16 | Revision of C1-202787 ----------------------------------------- Was agreed Revision of C1-202292 |  |
|  |  |  | C1-203237 | Revert CR 0820 |  |  | Apple | CR 2246 24.501 Rel-15 | Christian, Tue, 11:30 In short, we do object the set of CRs (C1-203237 and C1-203238) as they revert CR0820 which is wrong and leads to the problems CT1 faced in the past. A revision of C1-203238 (to Rel-16) might be progressed if only proposes the changes regarding the “last visited registered TAI” as described above and nothing else. Lena, Tue, 17:45 Same parts are not FASMO prefer to stay aligned with the wording in TS 24.301 v16.4.0 |  |
|  |  |  | C1-203238 | Revert CR 0820 |  |  | Apple | CR 2247 24.501 Rel-16 | Christian, Tue, 11:30 In short, we do object the set of CRs (C1-203237 and C1-203238) as they revert CR0820 which is wrong and leads to the problems CT1 faced in the past. A revision of C1-203238 (to Rel-16) might be progressed if only proposes the changes regarding the “last visited registered TAI” as described above and nothing else. Lena, Tue, 17:45 Same parts are not FASMO prefer to stay aligned with the wording in TS 24.301 v16.4.0 |  |
|  |  |  | C1-203356 | Correction to the URSP coding |  |  | MediaTek Inc., ZTE / JJ | CR 0076 24.526 Rel-15 | Ivo, Tue, 09:35 non-backward compatible CR is not acceptable at this late stage of Rel-15 if a backward compatible solution is possible. We believe that a backward compatible solution is possible. Only at the latest release Long explanation Lena, Tue, 17:45 - The CR is non-backwards compatible as it make existing Rel-15 filter components no longer supported in Rel-15. We would prefer to resolve this issue in a backwards compatible manner, for instance by not removing support for the old components, but adding new components in Rel-16 and having an indication of UE support - The UE behavior in case some parts of an IP 3 tuple are missing needs to be specified (does the UE discard the received IP 3 tuple parts?) JJ, Wed, 13:14 Fine to only correct rel-16 Christian, Wed, 13:50 Asking for the princiciple of the Rel-16 way forward JJ, Wed .14:11 Explaining that a new CR to 24.501 for Rel-16 would be requested Lena, Thu, 01:16 Further comments on the new to be created rel-16 CR |  |
|  |  |  | C1-203357 | Correction to the URSP coding |  |  | MediaTek Inc., ZTE / JJ | CR 0077 24.526 Rel-16 | Ivo, Tue, 09:35 non-backward compatible CR is not acceptable at this late stage of Rel-15 if a backward compatible solution is possible. We believe that a backward compatible solution is possible. Only at the latest release Long explanation Lena, Tue, 17:45 - The CR is non-backwards compatible as it make existing Rel-15 filter components no longer supported in Rel-15. We would prefer to resolve this issue in a backwards compatible manner, for instance by not removing support for the old components, but adding new components in Rel-16 and having an indication of UE support - The UE behavior in case some parts of an IP 3 tuple are missing needs to be specified (does the UE discard the received IP 3 tuple parts?) |  |
|  |  |  | C1-203409 | Discussion of LI and N3AN node selection |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | discussion | Ivo, Tue, 09:35 - Observation 4 is not correct - 24.526 states " The content of N3AN node selection information contain at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for any PLMN." and 24.502 states "The N3AN node selection information contains at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for "any_PLMN"..". - Problem 2 - such configuration is syntactially invalid and the UE needs to consider it as such. John-Luc, Tue, 16:55 When the UE detects information that is syntactically invalid, should it ignore the entire IE? Where is this specified e.g. for pre-configured information. …… I urge you to take a step back and look at the merit of mandating the “any_PLMN” entry. You may find none. In short, BlackBerry would be fine with defining that the IE is considered invalid (and thus absent) due to syntactical problems, but BlackBerry would prefer making the “any_PLMN” entry optional (and as such align with TS 24.302 even). Amer, Tue, 17:36 Problem 1: It should be addressed. Problem 2: Entry for “any PLMN” in N3AN selection information is mandatory. TS 24.501 specifies the behavior of the receiver when receiving an incorrect IE in a message: the IE is considered as not present in the message. If there is still some ambiguity left about this error handling in the receiver, it should be clarified/defined such that there is no possibility of breaching the LI requirements for a spec-compliant UE. If the N3AN configuration information is considered as not present if the “any PLMN” entry is not present, the existing procedure does not present any risks. Problem 3: It should be addressed. |  |
|  |  |  | C1-203410 | Correct N3AN node selection due to LI |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0119 24.502 Rel-15 | Revision of C1-202831 Ivo, Tue, 09:34 7.2.4.1 - there is no interworking between 5GS and GPRS so it is not clear why attach according to 24.008 needs to be considered Amer, Tue, 17:36 - Most of the changes related to problem 1 in C1-203409 are not FASMO, since the visited country can always mandate the selection of N3IWF in the visited country even if the country does not deploy any N3IWF by providing a DNS response containing at least one record with MCC corresponding to the country. The MNC of this record could be set to a ‘dummy’ valid value, e.g. a non-assigned MNC value. This would prevent the UE from selecting an N3IWF in the home country. A small correction in the text, as already proposed in the CR, would enable this configuration option. - The changes related narrowly to problem 3 in C1-203409 could be addressed as FASMO. The other changes are not FASMO and some are not needed even for Rel-16. Please see the comments on C1-203411 for more commentsj, provides what is agreeabl John-Luc, Tue, 22:12 Answering to Amer John-Luc, 22:34 Asks Ivo toc clarify Amer, Wed, 05:45 Does not agree with John luc, Ivo, Wed, 13:28 Clarifying to John-Luc John-Luc, Wed, 18:01 Rev from Amer does not work John-Luc, Wed, 23:59 Rev Amer, Thu, 09:38 Offers a new rev, which is forward comp |  |
|  |  |  | C1-203411 | Correct N3AN node selection due to LI |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0120 24.502 Rel-16 | Revision of C1-202832 Ivo, Tue, 09:34 7.2.4.1 - there is no interworking between 5GS and GPRS so it is not clear why attach according to 24.008 needs to be considered Amer, Tue, 17:37 - Changes specifically related to problem 1 in C1-203409 could be agreed. However, no need to send two DNS queries off the bat; only if the N3IWF query returns no records, the ePDG query needs to be sent to check for the LI requirements. Accordingly, many changes that assume two simultaneous queries are not applicable or needed. - Changes specifically related to problem 3 in C1-203409 could be agreed - The changes in sc. 7.2.4.2 are not needed, since ePDG selection is specified in 24.301 and the UE supporting ePDG selection will follow it. Provides rev of what is agreeable John-Luc, Wed, 17:45 Rev from Amer does not work Amer, THue, 09:04 Dummy MNC not in the rel-16 |  |
|  |  |  | C1-203412 | Correct inconsistency regarding presence of Any_PLMN entry |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0078 24.526 Rel-15 | Ivo, Tue, 09:34 - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN." - reasoning not correct - 24.526 states "The content of N3AN node selection information contain at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for any PLMN." - not essential Christian, Tue, 16:07 we object changes to Rel-15 for both set of CRs (C1-203412 and C1-203414) and Rel-16 is considered enough. We are supportive of the changes to Rel-16. Amer, Tue, 17:37 Not FASMO John-Luc, Wed, 23:59 rev Mariusz, Thu, 13:41 commenting |  |
|  |  |  | C1-203413 | Correct inconsistency regarding presence of Any_PLMN entry |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0079 24.526 Rel-16 | Ivo, Tue, 09:34 - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN." - reasoning not correct - 24.526 states "The content of N3AN node selection information contain at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for any PLMN." Christian, Tue, 16:07 we object changes to Rel-15 for both set of CRs (C1-203412 and C1-203414) and Rel-16 is considered enough. We are supportive of the changes to Rel-16. Amer, Tue, 17:37 “anyPLMN” is mandatory, Consequently, we don’t think the CR is correct in treating the “any PLMN” entry as optional and in handling the case when it is missing as a normal case within the procedure. |  |
|  |  |  | C1-203414 | Define behavior when Any_PLMN entry is missing |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0132 24.502 Rel-15 | Ivo, Tue, 09:34 - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN." - reasoning not correct - 24.502 states "The N3AN node selection information contains at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for "any_PLMN".." - "Consequences if not approved:" - incorrect due to the above - 7.2.4.3 NOTE 1 - incorrect due to the above - normative changes are for a possibility not allowed by stage-2 and not expected in 24.526 and 24.502 - not essential Andrew, Tue, 11:58 Explains to Ivo why it is essential Ivo, Tue, 12:16 If the configuration is an error (as explained by ANdrew), then UE needs to ignore this, and the CR goes in opposite direction Andrew, Tue, 13:10 Prefers explicit handling Ivo, Tue, 13:56 Explaining why ignoring errorornous entry in the UE is enough for the error case Christian, Tue, 16:07 we object changes to Rel-15 for both set of CRs (C1-203412 and C1-203414) and Rel-16 is considered enough. We are supportive of the changes to Rel-16. Amer, Tue, 17:38 Not FASMO, , we don’t think the CR is needed or correct. Additionally, correcting a misconfiguration is not a FASMO. Amer, Tue, 17:42 Agrees with Ivo, can be ignored, no LI issue Mariusz, Wed, 15:47 Supports the CR John-Luc, Wed, 23:59 rev Amer, Thu, 02:20 Explaining why this is not FASMO Mariusz, Thu, 12:51 Further discussing the optionality |  |
|  |  |  | C1-203415 | Define behavior when Any_PLMN entry is missing |  |  | BlackBerry UK Ltd., NTAC, Ministère Economie et Finances, The Police of the Netherlands | CR 0133 24.502 Rel-16 | Ivo, Tue, 09:37 - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN." - reasoning not correct - 24.502 states "The N3AN node selection information contains at least an N3AN node selection information entry with information for the HPLMN and an N3AN node selection information entry for "any_PLMN".." - "Consequences if not approved:" - incorrect due to the above - 7.2.4.3 NOTE 1 - incorrect due to the above - normative changes are for a possibility not allowed by stage-2 and not expected in 24.526 and 24.502 Christian, Tue, 16:07 we object changes to Rel-15 for both set of CRs (C1-203412 and C1-203414) and Rel-16 is considered enough. We are supportive of the changes to Rel-16. Amer, Tue, 17:37 The presence of a N3AN node selection information entry corresponding to “any_PLMN” is mandatory in stage 3 specs… We don’t think the CR is correct in treating the “any PLMN” entry as optional and in handling the case when it is missing as a normal case withing the procedure Mariusz, Wed, 15:47 Supports the CR |  |
|  |  |  | C1-203416 | Discussion on "any PLMN" entry and Home-routed Roaming architecture |  |  | BlackBerry UK Ltd. | discussion | Ivo, Tue, 09:34 This is not essential - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN." and stage-3 cannot contradict it - BlackBerry needs to raised the issue in SA2 by a company contribution. John-Luc, Tue, 16:18 Explains why this is essential |  |
|  |  |  | C1-203528 | Connected mode mobility from N1 mode to S1 mode and DL NAS COUNT handling |  |  | Nokia, Nokia Shanghai Bell | CR 2343 24.501 Rel-15 | Behrouz, Tue, 09:23 This is a Rel-15 CR (and not Rel-16). The Spec “version” and “Release” are both wrong. Christian, Tue, 11:30 In short, we do object the set of CRs (C1-203528 and C1-203544) as there is no problem to solve and the current stage 2 on security (TS 33.501) and stage 3 (TS 24.501) are aligned and correct on when the NAS COUNT counter value for downlink needs to be incremented Sung, Wed, 19:53 Explaining Mikael, Thu, 00:44 He does not agree with Sung’s conclusion n the discussion paper Sung, Thu, 01:14 Different view than Mikael |  |
|  |  |  | C1-203544 | Connected mode mobility from N1 mode to S1 mode and DL NAS COUNT handling |  |  | Nokia, Nokia Shanghai Bell | CR 2348 24.501 Rel-16 | Christian, Tue, 11:30 In short, we do object the set of CRs (C1-203528 and C1-203544) as there is no problem to solve and the current stage 2 on security (TS 33.501) and stage 3 (TS 24.501) are aligned and correct on when the NAS COUNT counter value for downlink needs to be incremented Sung, Wed, 19:53 Explaining Mikael, Thu, 00:44 He does not agree with Sung’s conclusion n the discussion paper Sung, Thu, 01:14 Different view than Mikael |  |
|  |  |  | C1-203545 | DL NAS COUNT mismatch during connected mode mobility from N1 mode to S1 mode |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-15 | Mikael, Tue, 09:58 I do not think there is any problem with current specification. SO any problem will have to be better described Sung, Wed, 19:53 Explaining Mikael, Thu, 00:44 He does not agree with Sung’s conclusion n the discussion paper Sung, Thu, 01:14 Different view than Mikael |  |
|  |  |  | C1-203742 | Corrections to UE policies specification |  |  | Orange / Mariusz | CR 0083 24.526 Rel-15 | Lazaros, Tue, 14:01 Not essential, some rewording Christian, Tue, 15:45 In short, we object changes to Rel-15 (C1-203742) and Rel-16 is considered enough. We are supportive of those changes to Rel-16 so we would like to co-sign a revision of C1-203743 with an additional change that in the Figure 5.3.2.4c the octet 23 and octet 24 should be shown optional as already indicated by the specification so the SSID related information may or not be present (in its totally). Lena, Tue, 17:45 Not FASMO, only Rel-16 |  |
|  |  |  | C1-203743 | Corrections to UE policies specification |  |  | Orange / Mariusz | CR 0084 24.526 Rel-16 | Lazaros, Tue, 14:01 Agrees with Rel-16 change, some rewording Christian, Tue, 15:45 In short, we object changes to Rel-15 (C1-203742) and Rel-16 is considered enough. We are supportive of those changes to Rel-16 so we would like to co-sign a revision of C1-203743 with an additional change that in the Figure 5.3.2.4c the octet 23 and octet 24 should be shown optional as already indicated by the specification so the SSID related information may or not be present (in its totally). Lena, Tue, 17:45 Not FASMO, only Rel-16 |  |
|  |  |  | C1-203455 | Correcting reference |  |  | Motorola Mobility, Lenovo | CR 0137 24.502 Rel-15 | SHIFTED FROM 5WWC agenda item (CR was requested with 5WWC work item) Ivo, Tue 09:24 - details of N3IWF - AMF interface are out of scope of TS 24.502 Roozbeh, Wed, 04:52 Does not agree with Ivo Ivo, Wed, 23:49 Why is this FASMO Joy, Thu, 17:33 This is not FASMO ,only for Rel-16 Roozbeh, Thu, 17:51 defending |  |
|  | Release 16 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Tdocs on Work Items |  |  |  |  |  |  |  | Papers related to Rel-16 Work Items |  |
|  | Work Item Descriptions |  |  | Peter - Main |  |  |  |  | New and revised Work Item Descritpions |  |
|  |  |  | C1-203130 | Revised WID on CT aspects of eV2XARC |  |  | Huawei, HiSilicon /Christian | WID revised Rel-16 | Revision of CP-200291 Frederic, Tue, 14:03 The target completion plenary is a copy/paste from the other specs, i.e. CT87 March 2020. Wouldn’t it be better to update to CT88e June? Or maybe work was already achieved back in March, but not documented in the WID ------------------------------------------- Was agreed Revision of C1-202166 |  |
|  |  |  | C1-203227 | Revised WID on CT aspects of support for integrated access and backhaul (IAB) |  |  | Qualcomm Incorporated / Lena | WID revised Rel-16 |  |  |
|  |  |  | C1-203245 | Revised WID of eNS |  |  | ZTE / Shuang | WID revised Rel-16 | Frederic, Tue, 14:11 Show that new spec is in CT4 |  |
|  |  |  | C1-203637 | Revised WID on Volume Based Charging Aspects for VoLTE CT |  |  | China Mobile | WID revised Rel-16 | Revision of CP-191110 Frederic, Tue, 13:52 Target completion to be updated to #88 Clarified that completion date is correct We need to endorse Scott, 05:10 Wants to be listed as co-source |  |
|  |  |  | C1-203728 | Updated WID MONASTERY2 |  |  | Nokia, Nokia Shanghai Bell | WID revised Rel-16 | Revision of C1-202570 ------------------------------------------ Was agreed |  |
|  | CRs and Discussion Documents related to new or revised Work Items |  |  | Peter - Main |  |  |  |  | CRs and Disc papers related to new Work Items |  |
|  | Status of other Work Items |  |  | Peter - Main |  |  |  |  | Status information on other relevant Rel-16 Work Items |  |
|  |  |  | C1-203488 | 5G CIoT workplan |  |  | Qualcomm Incorporated / Amer | Work Plan Rel-16 | Behrouz, Tue, 09:23 Correction related C1-103289 requested |  |
|  | Release 16 documents for information |  |  | Peter - Main |  |  |  |  | Miscellaneous documents provided for information |  |
|  | WIs for common and SAE/5G |  |  |  |  |  |  |  | WIs mainly targeted for common sessions or the SAE/5G breakout |  |
|  | ePWS |  |  | Lena – Main |  |  |  |  | CT aspects of enhancements of Public Warning System |  |
|  |  |  | C1-203261 | CR 23.041#0218 Deletion of Editor’s note in the clause 9.3.24 Warning-Type for ETWS |  |  | SyncTechno Inc. | CR 0218 23.041 Rel-16 |  |  |
|  |  |  | C1-203262 | CR 23.041#0219 Editor’s notes on Unicode baed pictograms mapping disasters considered by ePWS |  |  | SyncTechno Inc. | CR 0219 23.041 Rel-16 |  |  |
|  |  |  | C1-203263 | Workplan for ePWS-CT aspects |  |  | SyncTechno Inc. | Work Plan Rel-16 |  |  |
|  | SINE_5G |  |  | Peter – Main |  |  |  |  | Signalling Improvements for Network Efficiency in 5GS 100% |  |
|  |  |  | C1-203361 | Correction to the handling for 5GSM #27 |  |  | MediaTek Inc., Huawei, HiSilicon / JJ | CR 2286 24.501 Rel-16 | Ivo, Tue, 09:33 NOTE 1 seems to attempt to override the existing normative text. This is not possible JJ, Wed, 13:51 Provides a rev Ivo, Wed, 22:31 Co-sign |  |
|  | SAES16 WIs |  |  | Peter – Main |  |  |  |  | Stage-3 SAE protocol pevelopment for Rel-16 100% |  |
|  | SAES16 |  |  | Peter – Main |  |  |  |  | General Stage-3 SAE protocol development |  |
|  |  |  | C1-202519 | Correction to Handling of #35 |  |  | MediaTek Inc. | CR 3369 24.301 Rel-16 | Agreed |  |
|  |  |  | C1-202127 | Consistent usage of "tracking area updating procedure" |  |  | Samsung Electronics Polska | CR 3341 24.301 Rel-16 | Agreed Shifted from 16.2.21 |  |
|  |  |  | C1-202690 | Correction to Handling of #31 |  |  | MediaTek Inc. | CR 3368 24.301 Rel-16 | Agreed Revision of C1-202517 |  |
|  |  |  | C1-202688 | Correction of EMM initiated TAU procedure in EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM |  |  | MediaTek Inc. | CR 3366 24.301 Rel-16 | Agreed Revision of C1-202515 |  |
|  |  |  | C1-202824 | Error handling of precedence value conflict |  |  | MediaTek Inc. / JJ | CR 3372 24.301 Rel-16 | Agreed Revision of C1-202542 |  |
|  |  |  | C1-203241 | Updates to manual network selection mode to include Equivalent PLMN and Forbidden PLMN description |  |  | Apple | CR 0535 23.122 Rel-16 | Behrouz, Tue, 09:23 Wrong work item, needs to be TEI16 Ivo, Tue, 09:32 “temporary forbidden PLMN" and "permanently forbidden PLMNs" are unused terms in 23.122 Osama, Tue, 19:19 CR is not needed |  |
|  |  |  | C1-203315 | Correction to handling of ESM timers in abnormal cases |  |  | MediaTek Inc. / Carlson | CR 3377 24.301 Rel-16 |  |  |
|  |  |  | C1-203316 | Clarification on missing subclause in EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH |  |  | MediaTek Inc. / Carlson | CR 3378 24.301 Rel-16 |  |  |
|  |  |  | C1-203317 | Clarification on missing subclause in EMM-REGISTERED.ATTEMPTING-TO-UPDATE |  |  | MediaTek Inc. / Carlson | CR 3379 24.301 Rel-16 |  |  |
|  |  |  | C1-203318 | Clarification on procedure collision handling in paging |  |  | MediaTek Inc. / Carlson | CR 3380 24.301 Rel-16 | Osama, Wed, 00:47 LTE is a little different: it can be for CS or with IMSI. Why copy text from 5G blindly and ignore paging? Can we leave it up to implementation Carlsron, Thu, 09:35 Explaining Marko, Thu, 12:26 Explaining Vishnu, Thu, 13:00 Not agreeing with Marko |  |
|  |  |  | C1-203319 | Correction to Release of the NAS signalling connection |  |  | MediaTek Inc. / Carlson | CR 3381 24.301 Rel-16 | Behrouz, Tue, 09:23 1)In section 5.3.1.2.1: I believe it is an overkill to add “(only applicable to attach and tracking area update procedures) and #35 (not applicable to the network initiated detach procedure). However, if the rest of CT1 wants to add them, I won’t object 2) In the table: (not applicable to DETACH REQUEST) or #35 (not applicable to DETACH REQUEST) No need to specify that these cause values are NA as they won’t even be sent by the NW for those procedures Osama, Tue, 20:21 Not needed Vishnu, Wed, 12:44 Not needed |  |
|  |  |  | C1-203338 | Clarification on use of voice domain preference IE |  |  | NTT DOCOMO INC. | CR 3382 24.301 Rel-16 | Osama, Tue, 20:18 Requests rewording Maoki, Wed, 11:05 Discussing Osama, Wed, 17:17 Challenges that this is significant issue Maoki, Thu, 13:14 Still discussing |  |
|  |  |  | C1-203396 | Correction to handling of cause #31 for TAU procedure |  |  | MediaTek Inc. / Marko | CR 3397 24.301 Rel-16 | Osama, Wed, 00.50 Keep existing sub-state |  |
|  | SAES16-CSFB |  |  | Peter – Main |  |  |  |  | Stage-3 SAE protocol development related to Circuit Switched Fall Back |  |
|  |  |  | C1-202689 | Correction to Handling of MO CSFB Emergency call in EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM |  |  | MediaTek Inc. | CR 3367 24.301 Rel-16 | Agreed Revision of C1-202516 |  |
|  | SAES16-non3GPP |  |  | Peter – Main |  |  |  |  | Stage-3 SAE protocol development related to non-3GPP access |  |
|  |  |  | C1-203116 | Handover of ethernet PDN connection to ePDG not supported |  |  | ZTE / Joy | CR 0719 24.302 Rel-16 | Ivo, Tue, 09:32 Editorial Rohit, Tue, 10:17 New NOTE seems already covered, asking for clarification Joy, Thu, 04:29 Explaining to Rohit |  |
|  |  |  | C1-203339 | Correction of IKEV2 protocol RFC number from old 5996 to new 7296 |  |  | MediaTek Beijing Inc. | CR 0720 24.302 Rel-16 | Ivo, Tue, 09:32 Diff of the two rfcs should be listed on cover page, also reference to 33.210 could be done Joy, Tue, 11:06 Same as Ivo, needs to be CAT F Rohit, Wed, 10:55 Provides rev Ivo, Wed, 13:37 |  |
|  |  |  | C1-203341 | Enhancement in UE handling when error MAX_CONNECTION_REACHED is received from network. |  |  | MediaTek Beijing Inc. | CR 0721 24.302 Rel-16 | Ivo, Tue, 09:32 Prefers existing text for most of the new parts, problem in bullet ii Joy, Tue, 13:50 this CR does not bring necessary changes. JohnLuc, Tue, 18:20 Concurs with Joy and Ivo Osama, Tue, 18:24 Not needed Frederic, Wed, 16:06 Cr# is wrong Rohit, Thu, 04:44 Explaining Ivo, Thu, 13:26 commenting |  |
|  | 5GProtoc16 WIs |  |  | Peter – Main |  |  |  |  | Stage-3 5GS NAS protocol development for Rel-16 |  |
|  | 5GProtoc16 |  |  |  |  |  |  |  | General Stage-3 5GS NAS protocol development |  |
|  |  |  | C1-202535 | Indicate support of ePCO length of two octets parameter when establishing the PDU session – Alt#2 |  |  | MediaTek Inc. / JJ | CR 2204 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202017 | Correction for SoR-AF |  |  | Ericsson / Ivo | CR 0481 23.122 Rel-16 | Agreed Revision of C1ah-200189 |  |
|  |  |  | C1-202068 | SoR in HPLMN after registration |  |  | Orange, Ericsson / Mariusz | CR 0508 23.122 Rel-16 | Agreed |  |
|  |  |  | C1-202071 | Reference correction in URSP encoding |  |  | Orange / Mariusz | CR 0071 24.526 Rel-16 | Agreed |  |
|  |  |  | C1-202074 | Correction to figure |  |  | one2many B.V. | CR 0212 23.041 Rel-16 | Agreed |  |
|  |  |  | C1-202075 | Corrections to references |  |  | one2many B.V. | CR 0213 23.041 Rel-16 | Agreed |  |
|  |  |  | C1-202089 | Clarification of NAS COUNT handling in 5G |  |  | Vodafone GmbH | CR 2036 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202101 | Clarification on DL only match-all packet filter |  |  | Qualcomm Incorporated / Lena | CR 2037 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202128 | Correction of the handling of timer TG |  |  | Apple, Qualcomm Incorporated, T-Mobile USA | CR 0513 23.122 Rel-16 | Agreed |  |
|  |  |  | C1-202129 | Correction of the handling of 5GMM cause #27 |  |  | Apple, Qualcomm Incorporated, T-Mobile USA | CR 2047 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202136 | Dual-registration requirements for EHPLMNs |  |  | Intel, Qualcomm Incorporated / Vivek | CR 1974 24.501 Rel-16 | Agreed Revision of C1-200620 |  |
|  |  |  | C1-202201 | Clarification of the figure of registration procedure |  |  | Vivo | CR 2072 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202219 | UAC for MO-IMS registration related signalling EN resolution |  |  | NTT DOCOMO INC. | CR 6413 24.229 Rel-16 | Agreed Revision of C1-200684 |  |
|  |  |  | C1-202229 | Possible KSI types in EPS |  |  | Ericsson / Mikael | CR 3346 24.301 Rel-16 | Agreed |  |
|  |  |  | C1-202272 | Correct parameters included by AMF during inter-system change from S1 mode to N1 mode in 5GMM-CONNECTED mode |  |  | Qualcomm Incorporated | CR 2095 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202275 | Remove invalid cases in error handling for QoS rule operation and TFT operation |  |  | Qualcomm Incorporated | CR 2096 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202331 | Consider PDU session type IE set by UE in IP address allocation |  |  | Huawei, HiSilicon / Cristina | CR 2110 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202342 | Fixing a reference in the service request procedure |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2118 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202347 | Correcting length of extended emergency number list IE |  |  | Huawei, HiSilicon / Cristina | CR 3352 24.301 Rel-16 | Agreed |  |
|  |  |  | C1-202381 | No emergency session transfer after ESFB |  |  | Nokia, Nokia Shanghai Bell | CR 2141 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202477 | Correction on UE behaviour for service area restriction |  |  | Huawei, HiSilicon/Lin | CR 1823 24.501 Rel-16 | Agreed Revision of C1ah-200161 |  |
|  |  |  | C1-202510 | Correcting that 5G NAS integrity key is one of the input parameters for integrity protection algorithm |  |  | MediaTek Inc. | CR 2192 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202518 | Correction to Handling of #31 |  |  | MediaTek Inc. | CR 2194 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202523 | De-registration before initial registration for Emergency Services |  |  | MediaTek Inc. | CR 2197 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202526 | Correction to deletion of Allowed NSSAI |  |  | Huawei, HiSilicon / Vishnu | CR 2200 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202634 | Add handling for parameter set to “value is not used” in 5GS |  |  | Qualcomm Incorporated | CR 2093 24.501 Rel-16 | Agreed Revision of C1-202268 |  |
|  |  |  | C1-202635 | Add handling for UE configured to use timer T3245 in 5GS via 3GPP access |  |  | Qualcomm Incorporated | CR 1803 24.501 Rel-16 | Agreed Revision of C1-202278 |  |
|  |  |  | C1-202607 | OTAF renamed to SP-AF |  |  | Orange / Mariusz | CR 0510 23.122 Rel-16 | Agreed Revision of C1-202070 |  |
|  |  |  | C1-202680 | UAC exception for emergency |  |  | MediaTek Inc. | CR 2184 24.501 Rel-16 | Agreed Revision of C1-202501 |  |
|  |  |  | C1-202683 | Correction to criteria to enter 5GMM-REGISTERED.UPDATE-NEEDED substate after resumption failure |  |  | MediaTek Inc. | CR 2187 24.501 Rel-16 | Agreed Revision of C1-202505 |  |
|  |  |  | C1-202697 | Unify terms network-initiated and network-requested |  |  | Huawei, HiSilicon / Cristina | CR 2103 24.501 Rel-16 | Agreed Revision of C1-202295 |  |
|  |  |  | C1-202698 | Add MFBR as mandatory parameter in GBR QoS flow |  |  | Huawei, HiSilicon / Cristina | CR 2120 24.501 Rel-16 | Agreed Revision of C1-202344 |  |
|  |  |  | C1-202684 | T3346 handling when the UE is registered to different PLMNs over 3GPP and non-3GPP |  |  | MediaTek Inc. | CR 2190 24.501 Rel-16 | No longer Agreed Revision of C1-202508 Mikael, Thu, 12:49 Reopens the CR and OBJECTS due to further analysis, with an alternative in C1-203498 with further justification in C1-203497 |  |
|  |  |  | C1-202599 | Subscription management in PWS-IWF |  |  | one2many B.V. | CR 0214 23.041 Rel-16 | Agreed Revision of C1-202076 |  |
|  |  |  | C1-202623 | Removal of Duplicate Service Operation Details |  |  | one2many | CR 0207 23.041 Rel-16 | Agreed Revision of C1-202073 |  |
|  |  |  | C1-202793 | Modification of exchanges between SOR-AF and UDM |  |  | Orange, Ericsson / Mariusz | CR 0509 23.122 Rel-16 | Agreed Revision of C1-202069 |  |
|  |  |  | C1-202812 | Restricting handling of cause #9 to the access on which it was received |  |  | Samsung/Anikethan | CR 1792 24.501 Rel-16 | Agreed Revision of C1-202149 |  |
|  |  |  | C1-202815 | Specify UE azaros for NOTIFICATION message for additional state/sub-states |  |  | Samsung/Anikethan | CR 2051 24.501 Rel-16 | Agreed Revision of C1-202145 |  |
|  |  |  | C1-202652 | Clarification on URSP in EPS |  |  | ZTE / Joy | CR 0073 24.526 Rel-16 | Agreed Revision of C1-202144 |  |
|  |  |  | C1-202752 | PS Data Off status report for non-3GPP access |  |  | LG Electronics, Ericsson / SangMin | CR 2102 24.501 Rel-16 | Agreed Revision of C1-202289 |  |
|  |  |  | C1-202868 | Correcting order in which connections/sessions are transferred if there is an emergency call |  |  | BlackBerry Uk Ltd. | CR 1782 24.501 Rel-16 | Agreed Revision of C1-202670 |  |
|  |  |  | C1-202873 | Clarification on use of operator-defined access categories |  |  | Qualcomm Incorporated, Ericsson, Nokia, Nokia Shanghai Bell / Lena | CR 1795 24.501 Rel-16 | Agreed Revision of C1-202100 |  |
|  |  |  | C1-202705 | Allowed SSC mode for association between an application and a PDU session |  |  | OPPO / Rae | CR 0075 24.526 Rel-16 | Agreed Revision of C1-202491 |  |
|  |  |  | C1-202706 | Handling of unallowed SSC mode |  |  | OPPO / Rae | CR 2183 24.501 Rel-16 | Agreed Revision of C1-202492 |  |
|  |  |  | C1-202890 | Additional condition to start T3540 |  |  | Samsung/Anikethan | CR 2050 24.501 Rel-16 | Agreed Revision of C1-202811 |  |
|  |  |  | C1-202874 | Additional QoS error handling related to mapped EBI |  |  | Qualcomm Incorporated / Lena | CR 2101 24.501 Rel-16 | Agreed Revision of C1-202285 |  |
|  |  |  | C1-202893 | Initial registration for initiating emergency PDU session |  |  | Huawei, HiSilicon / Cristina | CR 2121 24.501 Rel-16 | Agreed Revision of C1-202858 |  |
|  |  |  | C1-202783 | Editorial corrections |  |  | Ericsson / Mikael | CR 2074 24.501 Rel-16 | Agreed Revision of C1-202218 |  |
|  |  |  | C1-202820 | Unsupported 5QI values |  |  | MediaTek Inc. / JJ | CR 0686 27.007 Rel-16 | Agreed Revision of C1-202537 |  |
|  |  |  | C1-202897 | storage of counters for UE in PLMN |  |  | vivo | CR 2071 24.501 Rel-16 | Agreed Revision of C1-202713 |  |
|  |  |  | C1-202696 | UE behaviour when the UE receives the rejected NSSAI for the current RA in the registration reject message and the RA is not stored |  |  | SHARP | CR 2202 24.501 Rel-16 | Agreed Revision of C1-202528 |  |
|  |  |  | C1-202615 | Applicable URSP is not optional for a UE |  |  | Motorola Mobility, Lenovo | CR 2098 24.501 Rel-16 | Agreed Needs revision, release should be Rel-16 |  |
|  |  |  | C1-202801 | Corrections on the abnormal cases of registration procedure for initial registration |  |  | Huawei, HiSilicon/Lin | CR 1379 24.501 Rel-16 | Agreed Revision of C1-202476 |  |
|  |  |  | C1-202802 | Single-registration mode without N26 |  |  | Huawei, HiSilicon/Lin | CR 2182 24.501 Rel-16 | Agreed Revision of C1-202478 |  |
|  |  |  | C1-202803 | Single-registration mode without N26 |  |  | Huawei, HiSilicon/Lin | CR 3358 24.301 Rel-16 | Agreed Revision of C1-202479 |  |
|  |  |  | C1-202806 | Handling of mapped EPS bearer contexts |  |  | Huawei, HiSilicon/Lin | CR 2214 24.501 Rel-16 | Agreed Revision of C1-202593 |  |
|  |  |  | C1-202807 | Integrity check interworking in 5GMM-CONNECTED mode |  |  | Huawei, HiSilicon/Lin | CR 2215 24.501 Rel-16 | Agreed Revision of C1-202594 |  |
|  |  |  | C1-202808 | Correction on LADN DNN based congestion control |  |  | Huawei, HiSilicon/Lin | CR 2216 24.501 Rel-16 | Agreed Revision of C1-202595 |  |
|  |  |  | C1-202709 | NW triggered SR over N3GPP under MM congestion control |  |  | OPPO / Rae | CR 2104 24.501 Rel-16 | Agreed Revision of C1-202324 |  |
|  |  |  | C1-202931 | Attach request message for N1 mode |  |  | Nokia, Nokia Shanghai Bell, Ericsson | CR 3150 24.301 Rel-16 | Agreed Revision of C1-202391 |  |
|  |  |  | C1-202929 | Inclusion of ATTACH REQUEST message in REGISTRATION REQUEST message during initial registration when 5G-GUTI mapped from 4G-GUTI is used |  |  | Nokia, Nokia Shanghai Bell, Ericsson, Qualcomm Incorporated | CR 0793 24.501 Rel-16 | Agreed Revision of C1-202390 |  |
|  |  |  | C1-202932 | Paging with two valid 5G-GUTIs |  |  | Nokia, Nokia Shanghai Bell | CR 1841 24.501 Rel-16 | Agreed Revision of C1-202392 |  |
|  |  |  | C1-202852 | MICO in an SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2154 24.501 Rel-16 | Agreed Revision of C1-202410 Shifted from Vertical_LAN |  |
|  |  |  | C1-203787 | Handling of allowed NSSAI when the RA includes the TAI belonging to EPLMN |  |  | SHARP | CR 2198 24.501 Rel-16 | Revision of C1-202889 _________________________________________ Agreed Chairman requests revision in the next meeting to correct changes over changes. |  |
|  |  |  | C1-203046 | UDM support of communication with SOR-AF |  |  | Ericsson / Ivo | CR 0530 23.122 Rel-16 | Mariusz, Tue, 10:22 Challenging step 3a), proposed modification in 2nd paragraph Ivo, Tue, 13:44 Provides a rev Ban, Tue, 14:40 Commenting on the rev, new rev needed Ivo, Tue, 15:38 Providing a rev Ban, Tue, 18:49 Not agreeing with latest rev Ivo, Wed, 08.10 Does not agree with Ban Ban, Wed, 09:29 New proposal Ivo, Wed, 12:14 discussing Ivo, Wed, 22:17 Offers rev Mariusz, Thu, 14:22 commenting |  |
|  |  |  | C1-203052 | void - allocated by mistake |  |  | void | CR 0531 23.122 Rel-16 | Withdrawn |  |
|  |  |  | C1-203067 | SOR-AF in scope |  |  | Ericsson / Ivo | CR 0532 23.122 Rel-16 | Mariusz, Tue, 10:00 shouldn’t SP-AF and USIM be included as well? Ivo, Tue, 14:16 Provides rev Ban, Tue, 1850 SP-AF not in scope of 23.122 Ivo, Wed, 08:22 Can go either way regarding scope Ban, Wed, 09:35 Follow drafting rules, LyThanh, Wed, 10:33 Not in scope Ivo, Wed, 20:54 Drafting rules don’t provide guidance Ban, Thu, 06:38 On the scope Ivo, thu, 07:54 Aksing frederic Ivo, thu, 09:00 Based on explanation form Frederic SP-AF is out of scope, i.e. 3067 would not requires a change |  |
|  |  |  | C1-203070 | Incorrect set up of PDN type in inter-system change from S1 mode to N1 mode |  |  | Ericsson / Ivo | CR 2221 24.501 Rel-16 |  |  |
|  |  |  | C1-203091 | Mobile Terminated Voice Gap for MPS |  |  | Perspecta Labs Inc. | CR 2227 24.501 Rel-16 | Roozbeh, Tue, 19:11 Cover page and CR are not in synch Osamah, Tue, 19:26 Questions - Does it sound needed?! T3346 will be stopped at UE side for receiving paging. I thought network will stop its MM congestion timer as well before sending paging for MT services. - We do not have such text in TS 24.301 right?! Marko, Thu, 07:40 Wants to merge MTK tdoc C1-203374 into this one PeterM, Thu, 15:54 Rev |  |
|  |  |  | C1-203231 | Indication that the emergency services fallback attempt failed |  |  | Nokia, Nokia Shanghai Bell | CR 2142 24.501 Rel-16 | Revision of C1-202928 ---------------------------------------------- Was agreed Needs revision, missing tdoc number on cover Revision of C1-202382 |  |
|  |  |  | C1-203239 | Update of emergency number list using Configuration Update Command |  |  | Apple | CR 2248 24.501 Rel-16 | Ivo, Tue, 09:32 Not clear in which use case the existing indication of emergency numbers in registration accept is not sufficient. Vishnu, Tue, 10:16 CR is not needed, existing signaling can be used Sunhee, Tue, 14:03 Stage-2 requirement is needed to update the list by using UCU. Reinhard, Wed, 12:03 DT co-signs Ivo, Wed, 13:44 Describes what would be ok for him Chen, Wed, 15:02 So what I am asking for is the reason for change and consequences if not approved need to reflect what is broken or is being improved and how this is worth the risk of overloading the paging and signalling channels. |  |
|  |  |  | C1-203240 | Change of E-UTRAN UE Capability |  |  | Apple | CR 2249 24.501 Rel-16 | Behrouz, Tue, 09:24 addition of “or E-UTRAN” in Bullet “n” Is not needed as you have added a new bullet “zc” that covers it Lena, Tue, 17:45 CR is not needed Robert, Wed, 10:52 Explains the reationale Robert, Wed, 10:52 Explains to behrouz Behrouze, Thu, 07:34 Asking for clarification Robert, Thu, 10:54 Provides rev Rae, Thue, 11:11 Question Maoki, Thu, 12:45 Asking question Robert, thu 13:22 Explains to Rae Robert, 13:55 New rev to take Maoki comments on board Lena, Thu, 15:48 change of radio capability for E-UTRAN is included in the existing text Behourz, Thu, 15:56 Fine Robert, Thu, 17:10 Discussing with Lena |  |
|  |  |  | C1-203243 | Updating idle mode manual selection mode procedure receiving SoR indication |  |  | Apple | CR 0537 23.122 Rel-16 | Sunhee, Tue, 10:00 I think manual search and trigger is not needed. Current spec is enough Mariusz, Tue, 10:19 Ok in general, proposes rewording Vishnue, Tue, 10:29 CR is not needed, explains why John-Luc, Tue, 16:31 “display” requirements are handled by SA1. We agree that, if a requirement is needed, it should not be limited to Annex C SoR only. Yanchao, Tue, 16:43 According to stage-1, SOR shall not impact the manual network selection mode, then the SOR information shall not affect the list of PLMNs displayed for user selection in manual PLMN selection mode. Lena, Tue, 17:45 So clearly the MS should not trigger any PLMN selection when receiving SoR info while in manual PLMN selection mode, instead the UE should stay on the currently selected VPLMN. The received SoR info should take effect only once the MS switches back to automatic PLMN selection mode. |  |
|  |  |  | C1-203251 | PDU session release for an inactive UE with RAN paging failure |  |  | Nokia, Nokia Shanghai Bell | CR 1833 24.501 Rel-16 | Revision of C1-202394 Ivo, Tue, 09:32 - there is ongoing discussion between RAN3 and SA2 on this topic - S2-2003531 + S2-2003805 and we need to wait until it settles Lin, Wed, 05:49 Can not agree, nothing to be done in CT1 |  |
|  |  |  | C1-203274 | storing the PLMN identity in the forbidden PLMN list due to 5GMM cause #73 Serving network not authorized |  |  | vivo | CR 0538 23.122 Rel-16 |  |  |
|  |  |  | C1-203275 | Clarification of the forbidden PLMN list used for non-3GPP access |  |  | vivo | CR 2255 24.501 Rel-16 | Ivo, Tue, 09:32 is changes one occurences of "forbidden PLMN list" usage in relation to non-3GPP access but does not change all the other occurences (e.g. in 5.5.1.2.5). The existing text needs to be interpretted together with subclause 5.3.13A. Yanchao, Thu, 05:41 rev |  |
|  |  |  | C1-203276 | Re-enabling the N1 mode capability upon request from upper layers |  |  | vivo | CR 2256 24.501 Rel-16 |  |  |
|  |  |  | C1-203277 | Correction of re-enabling E-UTRA capability |  |  | vivo | CR 2257 24.501 Rel-16 |  |  |
|  |  |  | C1-203278 | native 5G-GUTI in Additional GUTI IE |  |  | vivo | CR 2258 24.501 Rel-16 | Behrouz, Tue, 09:25 Need to use stage-3 language, i.e better to change 5GC to “an AMF” Ani, Wed, 13:18 Commenting Yanchao, Thu, 09:03 Explaining to Ani Ani, Thu, 15:42 Not agreeing |  |
|  |  |  | C1-203279 | Clarification of the use of T3245 |  |  | vivo | CR 0539 23.122 Rel-16 |  |  |
|  |  |  | C1-203280 | Clarification of the cause of start of T3245 |  |  | vivo | CR 3218 24.008 Rel-16 | Behrouz, Tue, 09:24 Is work item correct? For 24.008 only PCO parameters when it comes to 24.008 part of it Yanchao, Wed, 05:19 Offers to use SAES Behrouz, Wed Use TEI16 |  |
|  |  |  | C1-203281 | Alignment of several IEI |  |  | vivo | CR 2259 24.501 Rel-16 | Behrouz, Tue, 09:25 New IE definition is wrong, IEs are type 1 Ivo, Tue, 09:32 - this is incorrect - for TV formatted type 1 IEs, the IEI is only in half of the octet Yanchao, Wed, 08:24 Will update the IEI Yanchao, Thu, 05:46 Rev Ivo, Thu, 13:28 ok |  |
|  |  |  | C1-203287 | stop T3346 before sending NAS message |  |  | vivo | CR 2264 24.501 Rel-16 | Behroz, Tue, 09:25 Challenges the need for the CR Amer, Tue, 18:05 agreed CR in C1-202709 covers this, CR not needed Yanchao, Wed, 06:23 Defending Amer, Wed, 15:00 OK, thanks for clarification Ani, Wed, 15:57 Not convinced, but if it goes forward, then cover page needs to be enhanced Behrouz, Wed, 17:09 Challenges the consequences if not approved Yanchao, Thursday, 09:27 rev |  |
|  |  |  | C1-203303 | Correction to handling of 5GSM timers in abnormal cases |  |  | MediaTek Inc. / Carlson | CR 2267 24.501 Rel-16 | Ivo, Tue, 09:32 Styles are broken Vishnu, Tue, 11:37 Fine with the CR, styles are broken Carlson, Thu, 0450 Provides rev |  |
|  |  |  | C1-203305 | Clarification for de-registration procedure initiation |  |  | MediaTek Inc. / Carlson | CR 2269 24.501 Rel-16 | Behourz, Tue, 09:25 Remove superfluous “a”, bullet d seems not needed Vishnu, Tue, 11:42 Editorials Roozbeh, Tue, 19:15 Formatting not correct Freder, tue, 19:21 Editorials Osama, Tue, 20:47 Technically not wrong, but challenging the need, editorial Carlson, Wed, 06:53 Provides a rev Behrouz, Thu, 07:07 Fine with the rev |  |
|  |  |  | C1-203306 | Clarification in state transition of 5GMM-DEREGISTERED from another 5GMM state |  |  | MediaTek Inc. / Carlson | CR 2270 24.501 Rel-16 |  |  |
|  |  |  | C1-203307 | Clarification of SMS over NAS supported bit in initial registration |  |  | MediaTek Inc. / Carlson | CR 2271 24.501 Rel-16 |  |  |
|  |  |  | C1-203308 | Clarification on missing subclause in 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION |  |  | MediaTek Inc. / Carlson | CR 2272 24.501 Rel-16 |  |  |
|  |  |  | C1-203309 | Clarification on missing subclause in 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE |  |  | MediaTek Inc. / Carlson | CR 2273 24.501 Rel-16 |  |  |
|  |  |  | C1-203310 | Clarification regarding update status in NR RAT |  |  | MediaTek Inc. / Carlson | CR 2274 24.501 Rel-16 |  |  |
|  |  |  | C1-203311 | Correction to paging timer stop in case of integrity check failure |  |  | MediaTek Inc. / Carlson | CR 2275 24.501 Rel-16 | Ivo, Tue, 09:32 Broken styles, UE should be registered for emergency services Roozbeh, Tue, 19:17 Formatting not correct Frederic, Tue, 19;25 styles Carlson, Wed, 09:19 Provides rev Ivo, Thu, 13:40 Co-sign Carlson, Thu, 14:34 rev |  |
|  |  |  | C1-203312 | Correction to Release of the N1 NAS signalling connection |  |  | MediaTek Inc. / Carlson | CR 2276 24.501 Rel-16 | Behrouz, Tue, 09:25 Comments on the new text and the table Sunhee, Tue, 10:01 Why is the explanation in parentheses needed for #31, #62? Kaj, Tue, 10:33 Not needed, explains why, parenthesis is something new, should not be started Vishnu, Tue, 12:05 Not needed Osama, Teu, 19:35 Not needed Carlson, Wed, 09:06 Defending Behrouz, Wed, 18:35 (if nobody else cares) Can live with the first change, not the table Osama, Wed, 18:50 Not needed |  |
|  |  |  | C1-203313 | Correction to spelling mistakes |  |  | MediaTek Inc. / Carlson | CR 2277 24.501 Rel-16 | Ivo, Tue, 09:32 “containing” -> “contains” CAT D Carlson, Thu, 04:57 Rev Ivo, Thu, 13:42 ok |  |
|  |  |  | C1-203325 | Correction on allowed NSSAI for UE not supporting NSSAA |  |  | OPPO / Rae | CR 2282 24.501 Rel-16 | Sunhee, Tue, 10:03 Wid should be eNS Kaj, Tue, 10:37 We don’t think the CR is needed or even correct Roozbeh, Tue, 19:18 Suggestion for rewording Rae, Wed, 04:10 Defending Roozbeh, Wed, 05:52 Offering wording Rae, Wed, 09:53 Provides wording Roozbeh, Wed, 15:13 FINE Kaj, Wed, 20:41 Withdraws his comment Sung, Wed, 22:33 Make alignment with C1-203582 Rae, Thu, 04:18 Provides rev |  |
|  |  |  | C1-203335 | Abnormal case handling for MO IMS registration related signalling |  |  | NTT DOCOMO INC. | CR 6421 24.229 Rel-16 | Osama, Tue, 20:18 For b) leave things for UE optional |  |
|  |  |  | C1-203351 | Adding the parameter "access technology" as input to SOR-AF |  |  | DOCOMO Communications Lab., Thales, China Mobile | CR 0515 23.122 Rel-16 | Revision of C1-202902 alternate proposal in C1-203547 Ivo, Tue, 09:32 - we see no need for the UDM providing the SOR-AF with the "access technology" as: - if the SOR-AF wants "access technology" where the UE is registering, due to its own decision, the SOR-AF can obtain the "access technology" where the UE is registering, based on the "access type" where the UE is registering. The "access type" where the UE is registering is provided to the SOR-AF already according to 23.122 baseline. - our preference is NOT to have several methods how to pass information with overlapping semantic ("access type", "access technology") between network entities. It is waste of development time and effort. - the UDM has no use for "access technology" where the UE is registering. Thus, determination of "access technology" where the UE is registering, if wanted by SOR-AF, is a SoR related task. We believe that SoR related tasks should be in the SOR-AF rather than in the UDM. The UDM should provide the data which are already available in the UDM, in the form which is available in the UDM Mariusz, Tue, 10:29 Supports the CR, some comments on the coding, not on the CR Ban, Tue, 11:34 Acks Mariusz, will need to provide rev Lin, Wed, 16:04 Some comments, wants to co-sign Ban, Wed, 17:08 Acks Lin Sung, Wed, 17:20 do not support adding neither access technology nor RAT type. Lin, Thu, 05:13 Support the proposal Ban, Thu, 14:44 rev |  |
|  |  |  | C1-203370 | Removal of duplicate words |  |  | MediaTek Inc. / Marko | CR 2289 24.501 Rel-16 |  |  |
|  |  |  | C1-203371 | Correction to 5GMM-REGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 2290 24.501 Rel-16 | Roozbeh, Tue, 19:49 Formatting Marko, Thu, 12:23 acks |  |
|  |  |  | C1-203374 | Correction to handling of NAS level mobility management congestion control |  |  | MediaTek Inc. / Marko | CR 2291 24.501 Rel-16 | Merged into C1-203091 Based on request from author, Thu, 07:34 Kaj, Tue, 10:40 We don’t think the CR makes it complete as there are probably more exceptions not covered by the CR Vishnu, Tue, 12:13 This one collides with C1-203091, 3091 is preferred Marko, Tue, 14:15 Explains to Kaj that this is complete, asks for a specific case that is missing Amer, Tue, 1806 Check ME box, seems no impact on UE Roozbeh ,Tue, 19:50 formatting |  |
|  |  |  | C1-203377 | Correction to handling of #3/#6/#7 |  |  | MediaTek Inc. / Marko | CR 2292 24.501 Rel-16 | Ani, Wed, 17:10 change is ok for cause #3 and #6. But for cause #7 the USIM is invalid only for a particular domain. Hence this sub-state would not hold good. |  |
|  |  |  | C1-203380 | Correction to handling of #9 |  |  | MediaTek Inc. / Marko | CR 2293 24.501 Rel-16 | Vishnu, Tue, 13:47 this CR is not needed as ….. |  |
|  |  |  | C1-203393 | Correction to 5GMM-DEREGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 2294 24.501 Rel-16 | Sunhee, Tue, 10:10 Requests change in the new text Ani, Tue, 14:30 Not needed Marko, Thu, 11:57 Explaining Arni, Thu, 15:23 Cover page would need to reflect the change |  |
|  |  |  | C1-203397 | Correction to handling of cause #31 for mobility registration update procedure |  |  | MediaTek Inc. / Marko | CR 2295 24.501 Rel-16 | Sunhee, Tue, 10:12 suggests rewording Vishnu, Tue, 14:11 Don’t agree with the CR Osamah, Tue, 20:46 Keep sub-state as for CC#27 |  |
|  |  |  | C1-203398 | Correction to subclause in Requested NSSAI |  |  | MediaTek Inc. / Marko | CR 2296 24.501 Rel-16 |  |  |
|  |  |  | C1-203399 | Clarification in usage of SIM terminology in 5GS services |  |  | MediaTek Inc. / Marko | CR 2297 24.501 Rel-16 |  |  |
|  |  |  | C1-203400 | Clarification of notification in 5GMM-REGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 2298 24.501 Rel-16 | Behrouz, Tue, 09:25 I believe you need to clarify that the NOTIFICATION message was received over Non-3GPP access. The way this is written now, one can interpret it the wrong way Kaj, Tue, 10:42 Not needed Marko, Thu, 12:14 Explaining why it is needed Behrouz Thu ,16:21 Asking for clarificaiton |  |
|  |  |  | C1-203466 | Not including NSSAI for emergency session for interworking without N26 interface |  |  | Qualcomm Incorporated | CR 2317 24.501 Rel-16 |  |  |
|  |  |  | C1-203470 | AMF not using 5GMM registration status in UE status IE |  |  | Qualcomm Incorporated | CR 2318 24.501 Rel-16 | Kaj, Tue, 10:44 We think the UE status is relevant to AMF in this case, this to not initiate the UE cancellation in MME via UDM. Osama, Tue, 17:28 To Kaj, Can you clarify your comment more? Why UE trying to register in 5G to AMF needs to tell AMF its 5GMM registration status as registered or not? If there is any, where in TS 24.501 this is defined?! Roozbeh, Tue, 19:43 Not convinced this is right Osama, Tue, 19:54 Answering Roozbeh Kaj, Wed, 20:55 Challenging the new note Osama, Wed, 21:24 Explaining the need for the Notes |  |
|  |  |  | C1-203471 | MME not using EMM registration status in UE status IE |  |  | Qualcomm Incorporated | CR 3402 24.301 Rel-16 | Kaj, Tue, 10:45 We think the UE status is relevant to MME in this case, this to not initiate the UE cancellation in AMF via UDM Roozbeh, Tue, 19:43 Not convinced this is right, no need for a note Kaj, Wed, 20:55 Challenging the new note Osama, Thu, 01:29 Explaining the NOTE |  |
|  |  |  | C1-203475 | Additional abnormal case handling for NOTIFICATION message |  |  | Samsung/Anikethan | CR 1791 24.501 Rel-16 | Withdrawn Revision of C1-202146 |  |
|  |  |  | C1-203477 | Analysis on PDU session type |  |  | Motorola Mobility, Lenovo | discussion 24.501 | Ivo, Tue, 09:27 - the UE is mandated to include the PDU session type in 24.501 6.4.1.2. Lena, Tue, 17:45 CT1 made a conscious decision to mandate inclusion of the PDU session type in the PDU session establishment request message via the agreement of C1-192652 and C1-192613. isagree with the analysis in C1-203477, and we still request C1-202615 to be updated. |  |
|  |  |  | C1-203478 | PDU session type |  |  | Motorola Mobility, Lenovo | CR 2320 24.501 Rel-16 | Ivo, Tue, 09:36 the UE is mandated to include the PDU session type in 24.501 6.4.1.2. Lena, Tue, 17:45 CT1 made a conscious decision to mandate inclusion of the PDU session type in the PDU session establishment request message via the agreement of C1-192652 and C1-192613. 3478 can’t be agreed, update 2615 is requested Roozbeh, Wed, 05:20 Asking from Ivo Joy, Wed, 09:45 Commenting the content Lena, Thu, 00:42 Explaining the comments Roozbeh, Thu, 03:28 Further discussion Vishnu, Thu, 09:35 cannot agree to C1-203478. Rae, Thu, 09:47 concerns |  |
|  |  |  | C1-203487 | Correction to default NSSAI conditions |  |  | Qualcomm Incorporated / Amer | CR 2161 24.501 Rel-16 | Revision of C1-202418 ----------------------------------- Was agreed Needs revision, missing clauses affected |  |
|  |  |  | C1-203489 | Discussion on standardized STT |  |  | Qualcomm Incorporated / Amer | discussion Rel-16 | Sunhee, Tue, 10:13 Where did the specification mention that there is no mapped SST when standard SST is used Rae, Wed, 05:42 Solution can be left to implementation Sunhee, Wed, 08:28 Explaining postion |  |
|  |  |  | C1-203490 | Handling od standardized SST - Alt.1 |  |  | Qualcomm Incorporated / Amer | CR 2324 24.501 Rel-16 | Frederic, Tue, 12:13 Cover sheet issues: missing tdoc and CR numbers. Kaj, Tue, 10:47 We don’t see any issue as standard SST in roaming scenarios for configured NSSAI and allowed NSSAI will be with both S-NSSAI for serving PLMN and mapped for HPLMN although it may be the same values. Cristina, Thu, 10:55 We prefer to let network solve this problem, as long as VPLMN provides mapped SST then no need such complicated change. - C1-203490---UE fill the mapped to the same value, the good thing is no need to impact network, but doesn’t solve the problem when the standardized services in the HPLMN use non-standardized SST value; |  |
|  |  |  | C1-203491 | Handling od standardized SST - Alt.2 |  |  | Qualcomm Incorporated / Amer | CR 2325 24.501 Rel-16 | Frederic, Tue, 12:13 Cover sheet issues: missing tdoc and CR numbers. Kaj, Tue, 10:47 We don’t see any issue as standard SST in roaming scenarios for configured NSSAI and allowed NSSAI will be with both S-NSSAI for serving PLMN and mapped for HPLMN although it may be the same values. Cristina, Thu, 10:55 We prefer to let network solve this problem, as long as VPLMN provides mapped SST then no need such complicated change. - C1-203491---newly defined “mapped standardized NSSAI” and a serials of related network operations, since already need to change the network side, why can’t simply request VPLMN must to provide mapped SST in this case |  |
|  |  |  | C1-203492 | Associating S-NSSAI-based congestion backoff timers with S-NSSAI when S-NSSAI is provided during PDU session establishment |  |  | Qualcomm Incorporated, SHARP, Nokia, Nokia Shanghai Bell / Amer | CR 2326 24.501 Rel-16 | Competing with C1-203354 Ivo, Tue, 09:27 Long list of comments Frederic, Tue, 13:22 Cover sheet issues: missing tdoc and CR numbers. |  |
|  |  |  | C1-203496 | T3346 triggers for 5GS |  |  | Ericsson / Mikael | CR 3225 24.008 Rel-16 |  |  |
|  |  |  | C1-203497 | NAS level mobility management congestion control in 5GS |  |  | Ericsson, MediaTek Inc. / Mikael | discussion Rel-16 | Behrouz, Tue, 09:25 agree with the analysis done in this DP and our preference is, as mentioned in the DP, to create a new Rel-17, CT1 only Amer, Tue, 18:16 support the solution in the agreed CR#2190, additional things can be done via TEI Mikael, 22:32 Long explanation, fail to understand why you want to pursue a CR that results in T3346 being access-specific (for which there is no stage 2 requirement) and not the CR that maintains T3346 access agnostic and adds allowing PLMN specific instances (following stage 2 requirements). |  |
|  |  |  | C1-203498 | T3346 handling when the UE is registered over both 3GPP and non-3GPP access |  |  | Ericsson, MediaTek Inc. / Mikael | CR 2329 24.501 Rel-16 | Behourz, Tue, 09:25 Prefers Rel-17 only Amer, Tue, 18:19 support the solution in the agreed CR#2190. So we think that the CR is not needed. |  |
|  |  |  | C1-203506 | Support for emergency services for roaming users as an input to update "Operator Controlled PLMN Selector with Access Technology" |  |  | Apple | CR 0551 23.122 Rel-16 | Ivo, Tue, 09:36 Seems not needed, explains Ban, Tue, 11:17 See no need for the CR Sunghoon, Tue, 16:46 No strong view on this proposal. However, the coversheet should be corrected – untick ME box, and tick CN box, as it has no UE impact. |  |
|  |  |  | C1-203509 | Clarification on handling of rejected NSSAI for the current registration area |  |  | SHARP | CR 2332 24.501 Rel-16 | Sunhee, Tue, 10:16 CR is not needed, explains why Sunghoon, Tue, 14:59 CR is not needed Yoko, Wed, 02:24 Answering to sunhee Sunhee, Wed, 09:33 Still needs clarficiation Yoko, Wed, 10:26 Explaining Sunghoon, Thu, 13:46 Already covered in C1-202696 |  |
|  |  |  | C1-203513 | Discarding a SECURITY MODE COMMAND message which fails integrity check |  |  | Nokia, Nokia Shanghai Bell | CR 2139 24.501 Rel-16 | Revision of C1-202379 Sunhee, Tue, 10:19 OK, but referring 33.102 seems better Mikael, Wed ,12:14 propose to bring this directly to SA3, and CT1 will align stage 3, if needed. Sung, Wed, 20:52 Answering the comments, fine to go to SA3, but wants to avoid being sent back Osama, Wed, 21:11 Answering Sung, needs to go to SA3 first Lin, Thu, 05:57 Directly to SA3 Sunhee, Thu, 11:45 Withdraws comment |  |
|  |  |  | C1-203521 | Correction on unclear texts regarding the CONFIGURATION UPDATE COMMAND message |  |  | SHARP | CR 2341 24.501 Rel-16 |  |  |
|  |  |  | C1-203530 | Handling of MCS data in various 5GMM states |  |  | Samsung | CR 1415 24.501 Rel-16 | Revision of C1-202376 Ivo, Tue, 09:36 Editorial, wants to co-sign Lena, Tue, 17:46 There are no SA1 requirements, no need for stage-3 Changes on changes Frederic, Wed, 16:23 Cover page |  |
|  |  |  | C1-203531 | Additional triggers for Service Request over non-3GPP access |  |  | Samsung/Anikethan | CR 2344 24.501 Rel-16 | Behrouz, Tue, 09:25 Requests that new bullet is merged with existing bullet f Amer, Tue, 18:29 Understands the issue, does not agree the solution Roozbehm Tue, 19:52 bullet f should already cover this. OR there is a difference that I did not see? Ani, Tue, 19;58 Answering Ani, Wed, 03:56 Answering to Roozbeh Roozbeh, Wed, 05:54 Fine Amer, Wed, 06:17 Offers a new approach Ani, Wed, 06:29 Defends his proposal against Amer Joy, Thu, 10:26 Same view as Amer |  |
|  |  |  | C1-203533 | Provisioning of DNS server security information to the UE |  |  | Samsung/Kundan | CR 3226 24.008 Rel-16 | Ivo, Tue, 09:27 - for interoperability, coding of content of the PCO parameter needs to be specified. However, 33.501 X.2 does not specify coding (it only gives examples of information to be transferred). - if a "credentials to authenticate the DNS server" as in 33.501 X.2 can be a certificate, then such certificate can be quite long and thus PCO parameter with the length of two octets would be needed. - "DNS server security information Indicator" -> "DNS server security information indicator" - wrong WI code Lena, Tue, 17:46 Supportive, but there are issues to be fixed Kundan, Wed, 20:34 Acks Ivo and Lena |  |
|  |  |  | C1-203534 | Provisioning of DNS server security information to the UE-25.401 |  |  | Samsung/Kundan | CR 2345 24.501 Rel-16 | Ivo, Tue, 09:36 Requesting changes Mariusz, Tue, 10:14 Misleading title 25.401 |  |
|  |  |  | C1-203535 | Provisioning of DNS server security information to the UE-23.401 |  |  | Samsung/Kundan | CR 3404 24.301 Rel-16 | Ivo, Tue, 09:27 Requests a number of changes |  |
|  |  |  | C1-203543 | Condition for setting the Selected EPS NAS algorithm IE to NULL |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2347 24.501 Rel-16 |  |  |
|  |  |  | C1-203547 | On the parameters provided to the SOR-AF from the UDM |  |  | Nokia, Nokia Shanghai Bell, Ericsson | CR 0552 23.122 Rel-16 | alternate proposal in C1-203351 Marizusz, Tue, 10:29 Prefers C1-203351 Sung, Wed, 22:50 Explaining to Mariusz Lin, Thu, 08:35 This is competing with C1-203351 and we would prefer C1-203351 Ivo, Thu, 11:23 explaining |  |
|  |  |  | C1-203548 | Correction in the AMF behaviour upon LADN information update |  |  | Nokia, Nokia Shanghai Bell | CR 2350 24.501 Rel-16 | Kaj, Tue, 10:50 Fine with the CR as such, but needs some improvement Yudai, Wed, 04:06 More changes Sung, Wed, 23:27 Provides rev Yudai, Thu, 03:53 Co-sign |  |
|  |  |  | C1-203549 | Discussion w.r.t. C1-203513 |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-16 | Sunhee, Tue, 10:20 SMC seems RAN2 scope Osama, Wed, 02:04 Detailed comments Mikael, Wed ,12:14 propose to bring this directly to SA3, and CT1 will align stage 3, if needed. Sung, Wed, 20:52 Answering the comments, fine to go to SA3, but wants to avoid being sent back Osama, Wed, 21:11 Answering Sung, needs to go to SA3 first Lin, Thu, 05:57 Directly to SA3 Sunhee, Thu, 11:45 Withdraws comment |  |
|  |  |  | C1-203550 | Timer_T3245_Behaviour leaf applicable in 5GS |  |  | Nokia, Nokia Shanghai Bell | CR 0050 24.368 Rel-16 |  |  |
|  |  |  | C1-203551 | PDU session release for an inactive UE with RAN paging failure |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-16 | Altenative to C1-203704 Ivo, Tue, 09:36 - there is ongoing discussion between RAN3 and SA2 on this topic - S2-2003531 + S2-2003805 and we need to wait until it settles Lin, thu, 08:38 Sees no CT1 work and no LS out to CT4 |  |
|  |  |  | C1-203552 | Initiation of ESFB by a UE in the state 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE |  |  | Nokia, Nokia Shanghai Bell | CR 2140 24.501 Rel-16 | Revision of C1-202380 Ivo, Tue, 09:36 Requesting a change of wording Marko, Wed, 12:02 No need for this CR Sung, Wed, 23:33 Explaining and providing rev Lin, Thu, 08:44 No problem with the direction, comments |  |
|  |  |  | C1-203553 | PDU session number control based on priority |  |  | Nokia, Nokia Shanghai Bell | CR 2351 24.501 Rel-16 | Postponed Requested by author Ivo, Tue, 09:36 seems theoretical issue only stage-2 for such feature is needed before stage-3 Amer, Tue, 18:32 multiple concerns with this CR: Roozbeh, Tue, 19:55 Why not use the entire one byte Rae, Wed, 06:03 Same as Amer |  |
|  |  |  | C1-203556 | PDU session number control based on priority |  |  | Nokia, Nokia Shanghai Bell | CR 0080 24.526 Rel-16 | Postponed Requested by author Ivo, Tue, 09:36 seems theoretical issue only, stage-2 for such feature is needed before stage-3 Roozbeh, Tue, 19:55 Why not use the entire one byte |  |
|  |  |  | C1-203582 | Unify terminology for default S-NSSAIs and subscribed S-NSSAIs marked as default |  |  | LG Electronics France / Sunhee Kim | CR 2352 24.501 Rel-16 | Frederic, Tue, 20:44 Wrong work item code |  |
|  |  |  | C1-203583 | Discussion on Selected EPS NAS algorithm delivery |  |  | MediaTek Inc. / Marko | discussion Rel-16 | Behrouz, Tue, 09:25 Agrees with the issue, sol 2 and 3 do not work, solution 1 is not even on the table? Mikael, Tue, 10:40 Agrees with the problem, however, not obivious to introduce a solution as the additional SMC is not frequesnt. Some involvement of SA2/SA3 might be needed for the proposed solutions Vishnu, Tue, 14:40 Does not agree there is a problem, does not agree with any of the CRs. Marko, Wed, 10:45 Explaining Behrouz, Thu, 07:24 Explaining the sol-1 will not be accepted by CT1 Mikael, Thu, 15:05 Further explaiing |  |
|  |  |  | C1-203584 | Delivery of selected EPS NAS security algorithms - solution alt#1 |  |  | MediaTek Inc. / Marko | CR 2353 24.501 Rel-16 | Vishnu, Tue, 14:45 Object the CR. Amer, Tue, 18:51 Which IEs are to be sent in cleartext was determined by SA3 in 33.501 sc. 6.4.6. CT1 cannot change this without SA3 approval. As such, we oppose this CR. |  |
|  |  |  | C1-203585 | Delivery of selected EPS NAS security algorithms - solution alt#2 |  |  | MediaTek Inc. / Marko | CR 2354 24.501 Rel-16 | Vishnu, Tue, 14:45 Object the CR. Amer, Tue, 18:36 Does not agree |  |
|  |  |  | C1-203586 | Delivery of selected EPS NAS security algorithms - solution alt#3 |  |  | MediaTek Inc. / Marko | CR 2355 24.501 Rel-16 | Vishnu, Tue, 14:45 Object the CR. Amer, Tue, 18:39 Do not agree |  |
|  |  |  | C1-203587 | Discussion on UL NAS COUNT used for AS SMC at radio bearer establishment |  |  | MediaTek Inc. / Marko | discussion Rel-16 | Mikael, Tue, 11:20 Even if there is a problem, ne need for discussion in CT1 first, company contribution to SA3 Marko, Thu, 12.48 Agrees with Mikael, some arguments |  |
|  |  |  | C1-203589 | MRU after SR for ESFB aborted |  |  | MediaTek Inc. / Marko | CR 2185 24.501 Rel-16 | Revision of C1-202682 Cristina, Thu, 05:47 I’m ok with the CR in general, only one comment … ----------------------------------------- Was agreed Revision of C1-202503 |  |
|  |  |  | C1-203592 | Correction to Handling of T3521 timer |  |  | MediaTek Inc. / Marko | CR 2193 24.501 Rel-16 | Revision of C1-202687 ----------------------------------------------- Was agreed Needs revision, missing clauses affected Revision of C1-202514 |  |
|  |  |  | C1-203593 | Correction to Service Reject with cause #28 |  |  | MediaTek Inc. / Marko | CR 2356 24.501 Rel-16 |  |  |
|  |  |  | C1-203594 | UE behaviour when receiving allowed NSSAI in CUC |  |  | MediaTek Inc. / Marko | CR 2357 24.501 Rel-16 | Ani, Tue, 10:49 Not convinced about the scenario Kaj, Tue, 10:52 Fine with most of the changes, first changes seems not needed Sunghoon, Tue, 15:56 First change not needed, second change needs to be improved |  |
|  |  |  | C1-203595 | Ciphering initial registration message with NULL algorithm |  |  | MediaTek Inc. / Marko | CR 2358 24.501 Rel-16 |  |  |
|  |  |  | C1-203597 | mobility registration type for 5G steering of roaming over control plane |  |  | LG Electronics France / Sunhee Kim | CR 0555 23.122 Rel-16 | Ivo, Tue, 09:36 - we see no need of this CR with explanation Marius, Tue, 09:57 also think that this CR is not needed. Ban, Tue, 11:02 CR is NOT NEEDED Sunhee, Tue, 12:48 Explaining Lena, Tue, 17:46 Commenting errors in the Cr Ivo, Wed, 22:50 believe that AMF communicating with UDM at time of mobility registration update would generate too much load on UDM. Also, it is not aligned with stage-2 in SA2. Sung, Thu, 03:20 Same as Ivo, load on UDM is critical Sunhee, Thu, 05:18 Provides a rev Ban, Thu, 08:17 CR is NOT NEEDED Ivo, Thu, 13:56 CR is not needed, explains why |  |
|  |  |  | C1-203600 | Clarification to the Secondary authorization and authentication by an DN-AAA |  |  | Samsung/Kundan | CR 2360 24.501 Rel-16 | Ivo, Tue, 09:36 - wrong WID -> should be SINE_5G - is there any stage-2 requirement indicate this? If not, why is it not possible for PDN connection to succeed? - stage-2 text - replace with a reference to a particular 5GSM cause Amer, Tue, 18:42 CR has several issues that make it unagreeable for us Kundan, Thu, 16:14 Explaining |  |
|  |  |  | C1-203605 | No available S-NSSAIs at handover with established emergency PDU session |  |  | Ericsson /kaj | discussion Rel-16 | Roozbeh, Tue, 20:01 Question for clarification Kaj, Wed, 14:01 Kaj, confirms that the disc should mention C1-203643. |  |
|  |  |  | C1-203606 | Clean up description of Cause #34 in TS 24.501 |  |  | Huawei, HiSilicon / Cristina | CR 2362 24.501 Rel-16 | Kaj, Tue, 11:06 We don’t think cause should be removed as the cause is a catch all and well aligned with TS 24.301 Cristina, Wed, 03:12 Explaining |  |
|  |  |  | C1-203627 | Semantic error check for duplicate QRI or QFI |  |  | Huawei, HiSilicon / Cristina | CR 2364 24.501 Rel-16 | Ivo, Tue, 09:36 Superfluous TAB Amer, Tue, 19:41 It would be a severe NW implementation error to include multiple rules with the same QFI in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST or ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message. So these error checks are not needed, althoug not incorrect. Cristina, Wed, 03:56 Defending Amer, Wed, 06:33 Will not object Cristina, Thu, 03:38 Wants to proceed |  |
|  |  |  | C1-203629 | Discussion on whether mobility registration updating type is needed for SOR procedure |  |  | LG Electronics France / Sunhee Kim | discussion Rel-16 | Withdrawn |  |
|  |  |  | C1-203631 | Missing QoS flow description parameters for GBR QoS flows in 5GSM and ESM coordination |  |  | Huawei, HiSilicon / Cristina | CR 2122 24.501 Rel-16 | Revision of C1-202861 |  |
|  |  |  | C1-203643 | No available S-NSSAIs and emergency PDU session at handover |  |  | Ericsson /kaj | CR 2088 24.501 Rel-16 | Revision of C1-202843 Ani, Wed, 10:06 Not needed Kaj, Thu, 11.57 Discussing with Ani Ani, Thu, 15:26 Discussing |  |
|  |  |  | C1-203667 | Delete rejected NSSAI if no need for re-registation |  |  | Huawei, HiSilicon / Cristina | CR 2373 24.501 Rel-16 | Ani, Tue, 10:36 Explanation -> CR is not needed Sunghoon, Tue, 15:45 CR seems not correct Cristina, Wed, 09:16 Defending Ani, Wed, 09:42 Explaining the problem, CR conflicts with C1-203738 Cristina, Wed, 10:16 Agrees with Ani, will provide an update Cristina, Wed, 10:50 Explaining to SUnghoone Ani, Wed, 11:28 Does not agree with the use case Cristina provides use case Ani, Wed Why is anything new needed?? Cristina, Ongoing Ani, Ongoing Sunghoon, Wed. 16:33 Explaining what is NOT correct Cristina, Thu, 05:56 Discussion with Ani Cristina, Thu, 06:25 Explaining to Sunghoon Ani, Thu, 09:08 Asking for a rev for futher with restricted changes Sunghoon, Thu, 14:13 which point there is signaling improvement or correction? Why don’t we just send DEREG with ‘re-regi required’ |  |
|  |  |  | C1-203669 | Delete rejected NSSAI if no need for re-registation |  |  | Huawei, HiSilicon / Cristina | CR 2374 24.501 Rel-16 | Withdrawn |  |
|  |  |  | C1-203671 | NAS message transmission failure indication with delayed TAI change |  |  | Huawei, HiSilicon / Cristina | CR 2375 24.501 Rel-16 | Kaj, Tue, 09:31 No CN changes, this is UE internal, should be more clear in the changes Roozbeh, Tue, 20:03 Numbering of the notes Osama, Tue, 20:47 Intention OK, suggestion for rewording Cristina, Wed, 04:34 Acks Roozbeh, Kaj, Osama, provides some wording Osama, Wed, 19:18 Provides wording proposal Cristina, Thu, 04:42 Fine with the NOTE from Osama, enhancing it Osama, Thu, 16:39 Cristina proposal is fine |  |
|  |  |  | C1-203696 | Domain descriptors in URSP |  |  | Huawei, HiSilicon/Lin | CR 0081 24.526 Rel-16 | Ivo, Tue, 09:36 - syntax and semantic of regular expression needs to be specified - it needs to be clear how the UE distinguishes FQDN from regular expression as one string can have different semantic as FQDN and as regular expression. Lena, Tue, 17:46 Aspects in the CR are unclear Roozbeh, Tue, 20:05 Too vague Joy, Wed, 12:24 Change is not complete Lin, Thu, 09:50 rev Joy, Thu, 11:09 Question for clarification on the rev Ivo, Thu, 14.11 Goes in right direction, additional CR to 24.501 is needed |  |
|  |  |  | C1-203697 | 5GS update status set for 5GC interworking |  |  | Huawei, HiSilicon/Lin | CR 3408 24.301 Rel-16 |  |  |
|  |  |  | C1-203698 | Correction on MME security handling for 5GC interworking in idle mode |  |  | Huawei, HiSilicon/Lin | CR 3409 24.301 Rel-16 |  |  |
|  |  |  | C1-203699 | No CSFB following emergency services fallback from 5GS |  |  | Huawei, HiSilicon/Lin | CR 3410 24.301 Rel-16 | JohnLuc, Tue, 18:42 Conflicts with 3407, prefers 3407, number of comments Roozbeh, Tue, 20:07 Cr is fine, re-format the bullet Lin, Wed, 04:16 Answers to John-Luc Sung, Wed, 21:32 Prefers 3699 over 3407 Ban, Thu, 05:29 NTT DOCOMO also prefers this CR (C1-203699) over C1-203407 Lin, Thu, 09:57 Rev Sung, Thu, 15:19 Fine |  |
|  |  |  | C1-203700 | Handling of multiple QoS flow descriptions |  |  | Huawei, HiSilicon/Lin | CR 2382 24.501 Rel-16 |  |  |
|  |  |  | C1-203701 | Single-registration mode without N26 for three Ies |  |  | Huawei, HiSilicon/Lin | CR 2383 24.501 Rel-16 | Sunghoon, Wed, 13:45 I don’t see real value of this CR with this UE impact Lin, thu, 11:16 Explaining Sunghoon, Thu, 15:50 Not agreeing |  |
|  |  |  | C1-203702 | Discussion on NAS COUNT handling for replay protection |  |  | Huawei, HiSilicon/Lin | discussion Rel-16 | Background for 3703 Mikael, Tue, 11:48 Agrees with the analysis, … o, I believe we need to make the proposed correction more flexible so that also NAS COUNT-Y are considered as replay, where Y is implementation dependent. Yang, Tue, 12:50 The main purpose of replay protection is to make sure that a recorded and replayed message will not be accepted by the receiving entity. The agreed CR in C1-202089 already fully achieves this. Osamah, Tue, 17:11 I have to say that we have concerns with previously agreed VDF CR in C1-202089 and this proposal in DP to avoid failing integrity check for receiving same NAS message with same NAS SQN. Here are our reasons: Prefers to correct 2089 Lin, Wed, 04:33 Asking Mikael for wording proposal Lin, Wed, 04:42 Explaining to Yang that 2089 creates problems, open to further work Lin, Wed, 04:48 Asking Osama for wording proposal Yang, Wed, 13:40 Providing comments to Osama Osama, Wed, 16:20 Provides wording |  |
|  |  |  | C1-203703 | Clarification of NAS COUNT handling in 5G |  |  | Huawei, HiSilicon/Lin | CR 2384 24.501 Rel-16 | This is an alternative to agreed CR from last meeting (C1-202089) Yang, Tue, 10:11 Explaining 2089, requests some changes Lin, Tue, 11:34 Explains the CR to Yang Yang, tue, 12:10 New proposal is not inline with GSMA FSAG LS Osama, Tue, 17:19 We have concern with this CR. Keeping it up to receiver implementation will put us back in same issue raised by research paper in this area. Please see detailed comments in DP C1-203702 Lin, Wed, 04:48 Explaining to Yang that agreed solution has issues Lin, Wed, 05:27 Asking form Osama for wording proposal Reinhard, Wed, 11:07 Requests clear procedure in the spec Mikael, Thu, 11:25 Agrees with the problems described in 3702 CT1 may not be the right group to take a decision here and we may want to request security experts in SA3 to assess whether the GSMA/C1-202089 proposed changes and identified drawbacks are acceptable. Lin, Thu, 11:50 Discussing with Osama, modifies Ossamas proposal Yang, Thu, 11:48 Need clear spec, proposal for an update of 2089, based on Lins wording Mikael, Thu, 16:16 The out of order aspect from Yang’s proposal is not agreeable Lin, Thu, 16:38 Not agreeing any CR ((C1-202089 and C1-203703 )and sending an LS to SA3 is fine Yang, Thu, 16:51 Provides proposal, without out of order NAS message Lin, Thu, 17:14 Not agreeing with proposal from Yang Osama, Thu, 17:18 Out of order happens, needs to be considered Mikael, THi, 17:27 Does not agree with Yang proposal Mikael, Thu, 17:34 …I do not say that an implementation should not be allowed to implement out-of-order handling. I am saying any requirement for NAS out-of-order handling is not acceptable. |  |
|  |  |  | C1-203704 | Discussion on CT1 required work for UE not reachable for PDU session release |  |  | Huawei, HiSilicon/Lin | discussion Rel-16 | Alternative to C1-203551 Ivo, tue, 09:36 there is ongoing discussion between RAN3 and SA2 on this topic - S2-2003531 + S2-2003805 and we need to wait until it settles Lin, Wed, 05:47 Does not see the link to s2 discussion, Sung, Wed, 23:23 Explains relation |  |
|  |  |  | C1-203736 | A new approach for registering and retrieving OS/App IDs |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-16 | Ivo, Tue, 09:36 - not clear how FQDN helps. - UUID are unique so uniqueness of OSID is ensured using existing mechanisms already. - OS vendor can provide a UUID just as well as reserving FQDN Lena, Tue, 17:59 Hinting at CT1 chairman at CT87, don’t spend time on this topic in Rel-16 Shahram(att), Tue, 19:05 Objects to any change on this topic in rel-16 Roozbeh, Tue, 20:11 Supports the idea, questions on the approach Lazaros, Wed, 17:56 Some explanations Lin, Thu, 08:57 Still not clear |  |
|  |  |  | C1-203737 | Discussion on prevention of loop scenario for 5GMM cause #62 |  |  | Huawei, HiSilicon / Vishnu | discussion 24.501 Rel-16 | Kaj, Tue, 12:31 Problem not easily to be solved, however, rare case, should be avoided by operators Vishnu, Wed, 14:17 Explaining Kaj, Thu, 07:30 Ongoing disc |  |
|  |  |  | C1-203738 | Prevention of loop scenario for 5GMM cause #62 |  |  | Huawei, HiSilicon / Vishnu | CR 2394 24.501 Rel-16 | Amer, Tue, 19:02 Change in sc. 5.5.1.2.2 is not needed, Changes in sc. 5.5.1.2.5 and sc. A.3 are not needed |  |
|  |  |  | C1-203739 | Handling of PDU session authentication |  |  | Samsung/Grace | CR 2395 24.501 Rel-16 | Ivo, Tue, 09:36 Shall in NOTE Amer, Tue, 19:05 Shall in note, how long is ban?, secondary auth with 3rd party out of scope for 3g spec Roozbehm, Tue, 20:14 Questions |  |
|  |  |  | C1-203741 | mobility registration type for 5G steering of roaming over control plane |  |  | LG Electronics France / Sunhee Kim | CR 2396 24.501 Rel-16 | Withdrawn |  |
|  |  |  | C1-203353 | Addition of 5GSM cause #59 |  |  | MediaTek Inc., Ericsson / JJ | CR 2205 24.501 Rel-16 | Revision of C1-202821 Sunhee, Tue, 10:07 Asking for clarification JJ, Tue, 10:16 Does not get Sunhee’s questions Sunhee, Weed, 06:56 Withdraws the comment, has a question JJ, Wed, 07:30 Answres Sunhee Sunhee, Wed, 07:56 FINE --------------------------------------- Was Agreed Revision of C1-202538 |  |
|  |  |  | C1-203354 | Correction of S-NSSAI based congestion control |  |  | MediaTek Inc., Huawei, HiSilicon. / JJ | CR 1738 24.501 Rel-16 | Revision of C1-198427 Competing with C1-203492 Ivo, Tue, 09:30 - Ericsson has preference for C1-203492 - conflicts with C1-203492 - deviates from Rel-15 principles - determining S-NSSAI associated with the timer is different in PDU session establishment procedure and in PDU session modification/release procedure - too confusing. - not possible to prevent PDU session establishments with no DNN by invoking the S-NSSA+DNN congestion control in a PDU session release of a PDU session established with UE providing no DNN Amer, Tue, 19:44 Listing the key issues of this proposal |  |
|  |  |  | C1-203355 | [Disc] S-NSSAI based congestion control |  |  | MediaTek Inc. / JJ | discussion | Related to CR in CR in C1-203354 Ivo, Tue, 09:30 - Ericsson has preference for C1-203492 |  |
|  |  |  | C1-203358 | Indicate 5GSM cause when initiating 5GSM procedure for error handling |  |  | MediaTek Inc. / JJ | CR 2283 24.501 Rel-16 | Amer, Tue, 19:50 UE sends error cause, even if it does not diagnose an errer. Cannot agree the CR JJ, Wed, 11:42 Explaining Amer, Thu, 02:29 FINE with the CR |  |
|  |  |  | C1-203359 | PDU session release upon receipt of PDU session status IE |  |  | MediaTek Inc. / JJ | CR 2284 24.501 Rel-16 | Amer, Tue, 19:52 Not needed JJ, Wed, 07:56 Needs clarification on Amers comment Vishnu, Wed, 09:04 CR has issue, prefers the current text JJ, Wed, 09:36 Explains to Vishnu Vishnu, Wed, 11:03 if we add an exception for the PDU session establishment procedure, we think it could work JJ, Wed, 11:33 Fine with vihnu’s proposal Vishnau, Wed ,12:03 Commenting JJ, Thu, 10:05 Rev Vishnu, Thu, 16:53 Co-sign |  |
|  |  |  | C1-203360 | PDU session handling when the S-NSSAI is not in the allowed NSSAI but the PSI is active in the PDU session status IE |  |  | MediaTek Inc. / JJ | CR 2285 24.501 Rel-16 | Postponed Requested by author Kaj, Tue 09:39 Two cases, with/without AMF change - Given this the UE does not have to act on allowed NSSAI to locally release PDU sessions at all, and shouldn't this to be consistent with the network handling. - The CR proposal adds unnecessary signaling JJ, Tue, 18:05 Discussion Roozbeh, Tue, 19:20 Shouldn’t there be an exception if the PDU session is an emergency PDU session? Amer, Tue, 19:57 CR is not needed JJ, Wed, 10:33 Discussing Kaj, Wed, 19:48 Ok with doing nothing for this case |  |
|  |  |  | C1-203362 | Stop back-off timer upon receipt of 5GSM #39 |  |  | MediaTek Inc. / JJ | CR 2287 24.501 Rel-16 |  |  |
|  |  |  | C1-203363 | Handling of S-NSSAI provided by the ePDG |  |  | MediaTek Inc. / JJ | CR 3384 24.301 Rel-16 | Ivo, Tue, 09:28 - there is no justification that the network always provides S-NSSAI during handover of the PDN connection from untrusted non-3GPP access connected to EPC to EPS. The quoted text is conditional on S-NSSAI being provided in (e)PCO. JJ, Tue, 18:24 Asking questions to Ivo Amer, Tue, 20:00 Additional comments JJ, Wed, 08:01 Will revise Ivo, Wed, 22:40 Commenting JJ, Thu, 11:24 New rev Ivo, Thu, 13:47 Co-sign |  |
|  |  |  | C1-203404 | Disable CIOT and PSM when transferring an emergency PDU session using stand-alone PDN connectivity request |  |  | BlackBerry UK Ltd. | CR 3399 24.301 Rel-16 | Amer, Tue, 20:06 CR is not needed John-Luc, Wed, 18:49 Provides a rev Amer, Thu, 17:25 Not needed |  |
|  |  |  | C1-203405 | Support for continuity of emergency session upon attach failure |  |  | BlackBerry UK Ltd. | CR 3400 24.301 Rel-16 | Ivo, Tue, 09:28 - "did include a PDN CONNECTIVITY REQUEST message with request type set to "handover of emergency bearer services" and the other PLMN is an equivalent PLMN." (+ other places) - this prevents usage of non-equivalent PLMNs of the shared cell. Those need to be tried too (possibly after the equivalent PLMNs were tried and failed) John-Luc, Wed, 00:05 Offers a rev Sunghoon, Wed, 09:02 Comments John-Luc, Wed, 15:47 New rev Ivo, Thu, 00:20 Rev DOES NOT address the comments John-Luc, Thu, 00:48 Does not agree with Ivo Sunghoon, Thu, 16:43 Commenting the rev |  |
|  |  |  | C1-203406 | Support for continuity of emergency session upon registration failure |  |  | BlackBerry UK Ltd. | CR 2299 24.501 Rel-16 | Ivo, Tue, 09:28 See above, - ESTABLISMENT -> ESTABLISHMENT Roozbeh, Tue, 19:41 May in note not allowed John-Luc, Wed, 00:05 Offers a rev Sunghoon, Wed, 09:11 Comments John-Luc, Wed, 17:34 Explaining to Sunghoon Ivo, Thu, 00:20 Rev DOES NOT address the comments Sunghoon, Thu, 16:47 Comments as for 3405 |  |
|  |  |  | C1-203407 | Clarify behavior when the UE needs to send a CSFB request due to EENL |  |  | BlackBerry UK Ltd. | CR 3401 24.301 Rel-16 | Lin, Thu, 05:37 This is collided with CR C1-203699 and we prefer C1-203699, justification why the CR is incorrect |  |
|  |  |  | C1-203423 | Updating the requirements of Rejected NSSAI in roaming scenarios |  |  | China Mobile, ZTE, Huawei, HiSilicon | CR 2301 24.501 Rel-16 | Kaj, Tue, 10:12 There is a problem, but CR seems to have backward comp issue, in addition some rewording Amer, Tue, 20:08 how is the UE is supposed to know if the rejected S-NSSAI is the S-NSSAI in the VPLMN (legacy interpretation) or in the HPLMN (new interpretation)? Xu, Wed, 10:32 Provides rev Xu, Wed, 10:53 Explaining to Amer Kaj, Wed, 20:24 Still has issue, explaining |  |
|  |  |  | C1-203746 | Transfer of PDN connection from untrusted non-3GPP access connected to EPC to 5GS |  |  | Ericsson /kaj | CR 2397 24.501 Rel-16 | Was not shown in previous version of agenda Roozbeh, Tue, 20:40 - PDN session type should be changed to PDU session type. - The first list is an optional list which gives option 3 which has mandatory steps. Perhaps it should be clarified with adding something like “if step 3 is chosen” then listing the mandatory steps. Kaj, Wed, 15:11 Explaining, will update |  |
|  |  |  | C1-203756 | MICO indication needs to be included without Network Slicing Subscription Change Indication in UCU. |  |  | Samsung Electronics Polska / Ricky | CR 2230 24.501 Rel-16 | Revision of C1-203131 Kaj, Tue, 10:24 We don’t see the CR needed, explains why Ricky, Tue, 11.17 In view of kaj’s comment see that there are other changes needed, asking for Kaj#s position Kaj, Wed, 08:25 Withdraws his comment |  |
|  |  |  | C1-203757 | Conditions for use of S-NSSAIs after receiving Rejected NSSAI |  |  | Samsung Electronics Polska / Ricky | CR 2231 24.501 Rel-16 | Revision of C1-203132 Sunghoon, Thu, 13:34 Cover sheet improvement |  |
|  |  |  | C1-203761 | UE stopping back-off timer when receiving PDU SESSION AUTHENTICATION COMMAND |  |  | Samsung Electronics Polska / Ricky | CR 2235 24.501 Rel-16 | Revision of C1-203136 Ivo, Tue, 09:26 shoulnd't the same be done also for 6.3.1.3 "PDU EAP result message transport procedure"? Ricky, Tue, 19:07 Answering Ivo Roozbeh, Tue, 19:14 Questions (subject line 3136) Ricky, tue, 19:20 Asking from Roozbeh whether his comment is on correct tdoc Roozbeh, Tue, 19:23 Comment withdrawn, to be against 3758 |  |
|  |  |  | C1-203555 | PLMN selection procedure for steering of UE in VPLMN |  |  | LG Electronics France / sunhee kim | CR 0553 23.122 Rel-16 | Shifted from 16.2.6 Work item on cover sheet needs to be corrected Ivo, Tue, 09:36 Work item code wrong, TAB missing Ly Thanh, Tue, 15:23 Comments Mariusz, Tue, 16:18 Not agreeing with the Cr John-Luc, Tue, 18:50 Work item wrong, comments on the procedure Sunhee, Wed, 05:17 Explaining to Ly Thanh Lin, Wed, 16:26 Seems already covered in spec Sung, Wed, 21:01 Agrees with Lin Frederic, Thu, 08:45 Cover sheet issues and editorials Sunhee, Thu, 06:09 New rev |  |
|  | 5Gprotoc16-non3GPP |  |  | Peter – Main |  |  |  |  | Stage-3 5GS NAS protocol development related to non-3GPP access |  |
|  |  |  | C1-202279 | Add handling for UE configured to use timer T3245 in 5GS for non-3GPP access |  |  | Qualcomm Incorporated | CR 0121 24.502 Rel-16 | Agreed |  |
|  |  |  | C1-202907 | Extending congestion notification to capture ePDG overload |  |  | Nokia, Nokia Shanghai Bell, Charter Communications | CR 0718 24.302 Rel-16 | Agreed Revision of C1-202578 |  |
|  |  |  | C1-202903 | Extending congestion notification to capture N3IWF or TNGF overload |  |  | Nokia, Nokia Shanghai Bell, Charter Communications | CR 0130 24.502 Rel-16 | Agreed Revision of C1-202579 |  |
|  |  |  | C1-202901 | Enable N3IWF to initiate TCP connection establishment upon failure |  |  | Nokia, Nokia Shanghai Bell | CR 0131 24.502 Rel-16 | Agreed Revision of C1-202580 |  |
|  |  |  | C1-203244 | Store the received S-NSSAI via ePDG in the configured NSSAI |  |  | ZTE / Joy | CR 2250 24.501 Rel-16 |  |  |
|  |  |  | C1-203458 | Correcting editorial errors |  |  | Motorola Mobility, Lenovo | CR 0138 24.502 Rel-16 | Ivo, Tue, 09:26 CR is CAT F Roozbeh, Wed, 22:45 Provides rev |  |
|  |  |  | C1-203459 | Resolution of editor's notes under clauses 7.3.4 and 7.3.5 |  |  | Huawei, HiSilicon /Christian | CR 0139 24.502 Rel-16 | Lazaros, Tue, 14:24 Has 5WWC as work item code, but is in protoc AI |  |
|  |  |  | C1-203461 | Resolution of editor's notes under clauses 7.5.5 and 7.5.6 |  |  | Huawei, HiSilicon /Christian | CR 0141 24.502 Rel-16 | Lazaros, Tue, 14:24 Has 5WWC as work item code, but is in protoc AI |  |
|  | ATSSS |  |  | Peter – Main |  |  |  |  | CT aspects of Access Traffic Steering, Switch and Splitting support in 5G system Is TS 24.193 sufficiently stable to be sent to CT#88 for approval? Show of hands, 16.04./17.04. Support for C1-202019 (Ericsson) 24 Support for C1-202266 (Apple) 14 |  |
|  |  |  | C1-202009 | EPS interworking of MA PDU session of 5G-RG when N26 is not supported |  |  | Ericsson / Ivo | CR 2027 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202650 | Editorial fix in 9.11.4 |  |  | Apple | CR 2169 24.501 Rel-16 | Agreed Revision of C1-202431 |  |
|  |  |  | C1-202701 | Applicability of PS data off to MA PDU |  |  | OPPO / Rae | CR 2042 24.501 Rel-16 | Agreed Revision of C1-202120 |  |
|  |  |  | C1-202816 | Handlings of MA PDU session when deregistration from an access |  |  | MediaTek Inc. / JJ | CR 2203 24.501 Rel-16 | Agreed Revision of C1-202531 |  |
|  |  |  | C1-203047 | Detecting and ignoring delayed PMFP messages |  |  | Ericsson / Ivo | discussion Rel-16 | Carlson, Tue, 12:41 Alt-2 is preferred Roozbeh, Tue, 18:11 one octet is too little. If there is no option than 4, then we prefer 4. |  |
|  |  |  | C1-203048 | Alternative 1 for detecting and ignoring delayed PMFP messages |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 | Carlson, Tue, 12:41 Alt-2 is preferred |  |
|  |  |  | C1-203049 | Alternative 2 for detecting and ignoring delayed PMFP messages |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 | Carlson, Tue, 12:41 Alt-2 is preferred |  |
|  |  |  | C1-203050 | MA PDU session parameters when the 5G-RG establishes a PDN connection as a user-plane resource of an MA PDU session to be established |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 |  |  |
|  |  |  | C1-203051 | EPS interworking of MA PDU session of 5G-RG |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 |  |  |
|  |  |  | C1-203071 | Session-AMBR and MA PDU session |  |  | Ericsson / Ivo | CR 2222 24.501 Rel-16 |  |  |
|  |  |  | C1-203074 | Introduction of ATSSS |  |  | ZTE / Joy, InterDigital | CR 2223 24.501 Rel-16 | Roozbeh, Tue, 18:43 Capitalization, “can” |  |
|  |  |  | C1-203075 | Resolve editor note in clause 5.2 |  |  | ZTE / Joy, InterDigital | pCR 24.193 Rel-16 |  |  |
|  |  |  | C1-203076 | Steering modes for GBR traffic |  |  | ZTE / Joy | pCR 24.193 Rel-16 |  |  |
|  |  |  | C1-203077 | IETF reference updates |  |  | ZTE / Joy | pCR 24.193 Rel-16 |  |  |
|  |  |  | C1-203081 | Editor's note on security of PMFP |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 | Roozbeh, Tue, 18:45 UP security mechanism is between the gNB and the UE. It does not protect all the way to the UPF. The N3 interface is separately protected by IPsec. like the NOTE BTW Ivo, Wed, 10:10 Provides a rev Roozbeh, Wed, 21.50 Fine with the rev |  |
|  |  |  | C1-203082 | Handling of unknown, unforeseen, and erroneous PMFP data |  |  | Ericsson / Ivo | pCR 24.193 Rel-16 | Roozbeh, Tue, 18:52 Many comments Joy, Wed, 04:15 Asking for a NOTE Ivo, Wed, 09:50 Offers rev |  |
|  |  |  | C1-203085 | "MA PDU request" when the UE has an MA PDU session established over one access and requests establishment of user plane resources over the other access |  |  | Ericsson / Ivo | CR 2224 24.501 Rel-16 |  |  |
|  |  |  | C1-203126 | EPS interworking of MA PDU session of 5G-RG when N26 is supported |  |  | Ericsson / Ivo | CR 2029 24.501 Rel-16 | Revision of C1-202695 ------------------------------------------- Was agreed Revision of C1-202031 |  |
|  |  |  | C1-203639 | Add the reference and the supported NFs of MA PDU session |  |  | China Telecom Corporation Ltd. | CR 2365 24.501 Rel-16 | Merged into C1-203639 Based on author’s request Frederic, Tue, 10:23 -missing clauses affected - missing CR# (should be 2365) Joy, Thu, 04:17 Should be merged into3639 |  |
|  |  |  | C1-203740 | Service Request |  |  | Samsung/Grace | pCR 24.193 Rel-16 | Roozbeh, Tue, 19:07 Note to be number, note cannot have mandatory wording, grammar incorected, question Sunghoon, Wed, 13:34 Scenario not clear, clarification needed |  |
|  | eNS |  |  | Peter – Main |  |  |  |  | CT aspects on enhancement of network slicing |  |
|  |  |  | C1-202134 | Stopping of T3346 after receiving the NSSA Command message |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2049 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202224 | T3540 is not started if the Registration Accept includes a pending NSSAI |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2075 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202241 | Fixing typo related to eNS |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2080 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202475 | Term on rejected NSSAI for the failed or revoked NSSAA |  |  | Huawei, HiSilicon/Lin | CR 2181 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202629 | Missing condition for inclusion of “NSSAA to be performed” indicatory |  |  | Samsung Electronics Polska | CR 2043 24.501 Rel-16 | Agreed Revision of C1-202121 |  |
|  |  |  | C1-202678 | Clarify that NSSAA can occur during periodic registration or mobility updating for NB-N1 mode UEs |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2079 24.501 Rel-16 | Agreed Revision of C1-202234 |  |
|  |  |  | C1-202827 | Exception to initiate the service request procedure during NSSAA when there is no allowed NSSAI |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2089 24.501 Rel-16 | Agreed Revision of C1-202257 |  |
|  |  |  | C1-202784 | Missing condition at registration reject due to no available slices |  |  | Ericsson /kaj | CR 2091 24.501 Rel-16 | Agreed Revision of C1-202261 |  |
|  |  |  | C1-202813 | S-NSSAI in rejected NSSAI for the failed or revoked NSSAA not to be requested |  |  | Ericsson /kaj | CR 1734 24.501 Rel-16 | Agreed Revision of C1-202247 |  |
|  |  |  | C1-202825 | Alignment of UE actions of rejected NSSAI for the failed or revoked NSSAA |  |  | Ericsson /kaj | CR 2084 24.501 Rel-16 | Agreed Revision of C1-202248 |  |
|  |  |  | C1-202872 | Update description on whether UE indicate supporting NSSAA |  |  | China Telecom Corporation Ltd. | CR 2039 24.501 Rel-16 | Agreed Revision of C1-202792 |  |
|  |  |  | C1-202778 | Pending NSSAI update for the configured NSSAI in the CUC message |  |  | China Telecom Corporation Ltd. | CR 2040 24.501 Rel-16 | Agreed Revision of C1-202113 |  |
|  |  |  | C1-202776 | Clarification on the rejected S-NSSAI included in requested NSSAI in registration procedure. |  |  | China Telecom Corporation Ltd. | CR 2053 24.501 Rel-16 | Agreed Revision of C1-202157 |  |
|  |  |  | C1-202774 | Clarification S-NSSAI status in AMF for NSSAA |  |  | China Telecom Corporation Ltd. | CR 2038 24.501 Rel-16 | Agreed Revision of C1-202111 |  |
|  |  |  | C1-202608 | AMF triggers PDU session release |  |  | Samsung Electronics Polska | CR 2044 24.501 Rel-16 | Agreed Revision of C1-202122 |  |
|  |  |  | C1-202871 | Inclusion of pending S-NSSAI(s) in the requested NSSAI |  |  | Huawei, HiSilicon, China Telecom/Lin | CR 2180 24.501 Rel-16 | Agreed Revision of C1-202800 Revision of C1-202473 |  |
|  |  |  | C1-202881 | Handling of Pending S-NSSAI |  |  | Samsung/Kundan | CR 2144 24.501 Rel-16 | Agreed Revision of C1-202385 |  |
|  |  |  | C1-203037 | S-NSSAIs always selected from allowed NSSAI by AMF |  |  | Ericsson /kaj | CR 2086 24.501 Rel-16 | Revision of C1-202252 Related C1-203596 Roozbeh, Tue, 20:59 Ok, some editorials Lin, Wed, 10:54 Same as Roozbeh Mahmoud, Wed, 19:01 Fundamental issue, explaining, can not agree the CR Amer, Thu, 04:13 Tends to agree with Mahmoud, if the CR gets agreed, then untick ME Kaj, Thu, 07:04 Explaining, does not see the problem from Mahmoud, |  |
|  |  |  | C1-203122 | Adding AAA-S via NSSAAF to support NSSAA |  |  | China Telecom Corporation Ltd. | CR 2228 24.501 Rel-16 | Frederic, Tue, 09:31 Cover sheet issues Amer, Thu, 04:14 Untick ME |  |
|  |  |  | C1-203228 | NSSAA in an SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2135 24.501 Rel-16 | Revision of C1-202833 -------------------------------------------------- Was agreed Needs revision, rev counter should be 1 Revision of C1-202374 Revision of C1-202374 |  |
|  |  |  | C1-203235 | Disabling of N1 capabilities when all requested S-NSSAIs subjected to NSSAA are rejected due to failure of NSSAA or when no slice is available for UE |  |  | Apple | CR 2244 24.501 Rel-16 | Sunhee, Tue, 09:48 This CR seems to prevent to move to different PLMN in 5G NW. So, I Prefer keeping current specification. Kundan, Tue, 11:55 Samsung supports the CR Rae, Tue, 12:23 Does not agree with the CR, Yanchao, Tue, 16:52 When to re-enable the N1 mode capability is missing in for those PLMN wherein the N1 mode capability is disabled due to no slice available Roozbeh, Tue, 21:42 We do not think this is a good idea to disable the N1 and fallback for the case which may never happen. We don’t think this should be standardized. Kaj, Wed, 09:47 Motiviation for Change in 4.6.2.2 not clear Sunhee, Wed, 10:48 Asking clarification from Kundan Krisztian, Thu, 03:06 Explaining to Sunhee Amer, Thu, 06:16 Not a good idea Kundan, Thu, 15:32 Asking Sunheed |  |
|  |  |  | C1-203236 | Reactivation of previously rejected S-NSSAI due to NSSAA failure |  |  | Apple | CR 2245 24.501 Rel-16 | Sunhee, Tue, 09:49 NSSAA is valid per PLMN. Why all PLMN is removed as in the CR (and across all PLMNs for N1 mode)? Ani, Tue, 09:53 CR is not needed Yanchao, Tue, 16:58 The first change is not needed; The 3rd change is not needed; Roozbeh, Tue, 21:59 Comments Kaj, Wed, 10:25 Not convinced about the use case Cristina, Wed, 11:24 SA2 first before any stage-3 |  |
|  |  |  | C1-203259 | Re-initiation of NSSAA – Proactive Solution |  |  | Samsung, Huawei, HiSilicon | CR 2253 24.501 Rel-16 | Same topic as in C1-203260 Sung, Wed, 23:51 Wants changes |  |
|  |  |  | C1-203260 | Re-initiation of NSSAA – Reactive solution |  |  | Samsung, Huawei, HiSilicon, China Mobile | CR 2254 24.501 Rel-16 | Same topic as in C1-203259 Sung, Tue, 19:19 Has a generic issue with this Mahmoud and Sung discuss how to structure this and whether to link it to some CAG aspects Sung, Tue, 23.30 Not clear why the AMF should wait with providing the pending NSSAI to the UE while the UE is in connected mode. If the UE is in idle mode, the AMF can update the UE with pending NSSAI during or right after the service request procedure Mahmoud, Wed, 00.18 Asking for clarification form Sung. Sung, Wed, 0141 Discussion Ongoing, not capture Mahmoud, Wed 04:19 Is there further comments Sung, Wed, 23:38 I disagree with having two solutions for one problem. |  |
|  |  |  | C1-203324 | Clarification on S-NSSAI deletion based on the rejected NSSAI due to NSSAA in the roaming case |  |  | OPPO / Rae | CR 2281 24.501 Rel-16 | Sunhee, Tue, 09:52 Keep existing spec Rae, Thu, 04:48 Defending Amer, Thu, 06:57 Grammar needs to be changed Rae, Thu, 08:37 rev |  |
|  |  |  | C1-203334 | Deleting Editors note regarding indefinite wait at the UE for NSSAA completion |  |  | ZTE / Shuang | CR 1912 24.501 Rel-16 | Revision of C1-202340 Roozbeh, Tue, 23:16 Fine with the content, some editorial |  |
|  |  |  | C1-203336 | Work Plan for eNS in CT1 |  |  | ZTE / Shuang | Work Plan Rel-16 |  |  |
|  |  |  | C1-203419 | Updating Rejected NSSAI IE for failed NSSAA case in roaming scenerios |  |  | China Mobile, Huawei, HiSilicon, Samsung, ZTE | CR 2108 24.501 Rel-16 | Revision of C1-202627 Roozbeh, 23:23 Some rewording, he does ont object Xu, Thu, 09:24 rev ------------------------------------- Was agreed Needs revision, rev counter should be 1 Revision of C1-202329 |  |
|  |  |  | C1-203420 | Updating requirements of NSSAA for roaming scenarios |  |  | China Mobile,ZTE, Samsung | CR 2059 24.501 Rel-16 | Revision of C1-202628 Roozbeh, Tue, 23.26 Clasue 4 not to contain any normative language Kaj, Wed, 10:18 Why is this needed at all Xu, Wed, 11:01 Will bring a rev Lin, Wed, 11:51 Proposal Lin, Wed, 11:55 Discussing Lin, Thu, 03:20 Wants to co-sign Xu, 14:10 Explaining Kaj, Thu, 14:49 Can live with it --------------------------------------- Was Agreed Revision of C1-202173 |  |
|  |  |  | C1-203421 | Clarifying the description for Network Slice-Specific Authorization Revocation |  |  | China Mobile, Motorola Mobility, Lenovo | CR 2058 24.501 Rel-16 | Revision of C1-202603 Kaj, Wed, 10:13 Wording changes needed Kaj, Thu, 14:54 Co-sign |  |
|  |  |  | C1-203422 | Updating the requirements of Rejected NSSAI for UE not supporting NSSAA feature in roaming scenerios |  |  | China Mobile, ZTE, Huawei, HiSilicon | CR 2300 24.501 Rel-16 | Roozbeh, Tue, 23:30 Rewording, double-check for overlap 3433 Rae, Wed, 08:41 Bullet a seems enough Kaj, Wed, 10.12 There is an issue, but is this backward comp? Amer, thu, 07:11 Same as Kaj, How is the UE is supposed to know if a rejected S-NSSAI is the S-NSSAI in the VPLMN or in the HPLMN? |  |
|  |  |  | C1-203424 | Abnormal case about missing EAP result for NSSAA |  |  | China Mobile | CR 2302 24.501 Rel-16 | Frederic, Tue, 10:13 Clauses affected Roozbeh, Tue, 23:45 Asking questions Xu, Wed, 17:07 Provides a rev, on cover sheet Amer, Thu, 07:24 CR is not needed |  |
|  |  |  | C1-203432 | UE behaviour when more than 8 S-NSSAIs received in pending NSSAI IE |  |  | Ericsson /kaj | CR 2309 24.501 Rel-16 | Ricky, Tue, 09:44 16 S-NSSAI to be reflected in the changes Kaj, Tue 12:23 While 16 was agreed in2871, this seems a mistake to Kaj, would have impact on 3705 and 3706 Atle, Tue, 14:19 This is in conflict with the Agreed CR in C1-202871. The reason for 16 S-NSSAIs is not explained that well in C1-202871, but we assume it is to align with other types of S-NSSAI and / or accommodate the extreme case of all configured NSSAIs across all access types We do not see any reason to challenge the agreement from CT1#123-e, hence we do not support this CR. Lin, Wed, 11:07 Supports Ricky Kaj, Wed, 15:27 not decoupled from C1-203706. Atle, Thu, 08:39 Either go with 8 or with 16, if we go back to 8 then we need to revise 2871 If we go with a 16, then we need a rev of 3432 Kaj, Thu, 11:34 If the meeting does not agree C1-203706, then the pending NSSAI could at most contain 8 S-NSSAIs as per my logic below. In line with allowed NSSAI. In that case we should revise C1-202871. If we agree C1-202871, then I will update my CR to 16 S-NSSAIs. Atle, Thu, 14.53 Fine, if the CR is revised as indicated by Kaj |  |
|  |  |  | C1-203433 | Pending NSSAI may contain serving PLMN and mapped HPLMN S-NSSAI values |  |  | Ericsson /kaj | CR 2310 24.501 Rel-16 | Withdrawn Based on request from author Ricky, Tue, 10:00 We do not believe that this CR is required because although …… Lin, Wed, 11:28 NOT NEEDED |  |
|  |  |  | C1-203434 | Status synchronization of S-NSSAIs pending NSSAA procedure |  |  | Ericsson /kaj | CR 2311 24.501 Rel-16 | Alternative to C1-303705 Sunhee, Tue, 09:56 NSSAA can be handled without pending NSSAI status from the UE. So, I think this CR is not needed. Rae, Wed, 08:43 Detailed comments Tsuyoshi, Wed, 0850 Clarification needed Atle, Wed, 11:03 we think C1-203706 is the way to solve this Lin, Wed, 11:36 we cannot see the reall issue here and the CR is not needed Kaj, Wed, 17:14 explainign Tsuyoshi, Thu, 07:23 Commenting Amer, Thu, 07:51 Agrees with Rae comments, solution needs to be optimized |  |
|  |  |  | C1-203507 | Correction related the pending NSSAI |  |  | SHARP | CR 2330 24.501 Rel-16 | Atle, Tue, 14:15 Supports, minor rewording, wants to co-sign Yokia, Wed, 09:39 Provides rev |  |
|  |  |  | C1-203508 | Pending NSSAI update for the new configured NSSAI in the UCU message |  |  | China Telecom, Samsung | CR 2331 24.501 Rel-16 | Roozbe, Tue, 23:50 Many editorials Kaj, Wed, 10:09 Several issues Amer, Thu, 08:02 Similar as Kaj, from UE perspective Shuzhen, Thu, 11:50 Answering |  |
|  |  |  | C1-203510 | Clarification on S-NSSAI(s) in URSP(NSSP) be added into the request NSSAI |  |  | China Telecom Corporation Ltd. | CR 2333 24.501 Rel-16 | Ani, Tue, 14:55 CR is not needed Roozbeh, Wed, 00:52 Valid CR, rewording Amer, Thu, 08:04 Same as Ani (not needed) |  |
|  |  |  | C1-203518 | A default S-NSSAI not subject to NSSAA |  |  | China Telecom Corporation Ltd. | CR 2339 24.501 Rel-16 | Sunhee, tue, 09:57 Commenting on Default S-NSSAI Kaj Wed, 10:03 Issues with the proposal Amer, Thu, 08:10 Untick ME |  |
|  |  |  | C1-203538 | Updating NSSAI status in AMF |  |  | NEC | CR 1990 24.501 Rel-16 | Revision of C1-202454 Kaj, Wed, 10:01 Not in favor of the CR Lin, Wed, 11:44 Support, some comments Tsuyoshi, Wed, 12:31 Clarification from Kaj Tsuyoshi, Thue, 06:37 Asking Kaj again |  |
|  |  |  | C1-203546 | PLMN selection procedure in case of NSSAA failure in roaming scenario |  |  | LG Electronics France | CR 2349 24.501 Rel-16 | Frederic, Tue, 10:19 Cover sheet issues Rae, Wed, 09:00 overkilled to prevent UE registers to the same PLMN when using “shall not” SUnhee, Wed, 10:06 Defending Lin, Wed, 16:19 Agrees with Rae Amer, Thu, 08:43 Would require 23.122 changes, which is not part of eNS slice aware plmn selecton not yet adopted |  |
|  |  |  | C1-203596 | Clarification on PDU session establishment without S-NSSAI indication |  |  | Nokia, Nokia Shanghai Bell | CR 2359 24.501 Rel-16 | Merged into C1-203037 Requested by author Related C1-203037 Lin, Wed, 16:49 collided with CR C1-203037 and prefer C1-203037 Mahmoud, Wed, 19:01 Fundamental issue, explaining, can not agree the CR |  |
|  |  |  | C1-203664 | Collision between CUC procedure (due to UDM change of slicing information) and ongoing NSSAA |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2370 24.501 Rel-16 | Kaj, Wed, 11:44 Not needed Mahmoud, Wed, 21:04 Explaining the need |  |
|  |  |  | C1-203675 | Discussion on NSSAA in roaming cases |  |  | BEIJING SAMSUNG TELECOM R&D | discussion Rel-16 | Related to C1-203434 |  |
|  |  |  | C1-203676 | NSSAA for UEs that roam across 5GS VPLMNs |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2378 24.501 Rel-16 | Kaj, Wed, 13:53 Asks for some changes Mahmoud, Wed, 23:50 Can take first on board, needs clarification for the second Kaj, Thu, 14:33 explains |  |
|  |  |  | C1-203705 | Inclusion of pending S-NSSAI(s) in the requested NSSAI |  |  | Huawei, HiSilicon, China Telecom, Samsung/Lin | CR 2385 24.501 Rel-16 | Alternative to C1-203434 Atle, Tue, 13:49 we do not agree with this CR. This CR is in conflict with the concept of Pending NSSAI. The handling of Pending NSSAI was most recently clarified in the SA2-Approved CR S2-2003475 at SA2#138e. Amer, Thu, 16:07 Wording needs to be improved |  |
|  |  |  | C1-203706 | Providing complete pending NSSAI for NSSAA |  |  | Huawei, HiSilicon, China Telecom, Samsung/Lin | CR 2386 24.501 Rel-16 | Related to C1-203760 Kaj, Wed, 09:55 Cover page problem, asking for clarification Atle, Wed, 10:25 Wants to cosign |  |
|  |  |  | C1-203707 | Abort NSSAA for S-NSSAI without in requested NSSAI |  |  | Huawei, HiSilicon, China Telecom, Samsung/Lin | CR 2387 24.501 Rel-16 | Releated to C1-203760 Roozbeh, Wed, 01:09 Difficult to understand Lin, Wed, 06:04 Offers rewording Kaj, Wed, 09:51 CR can only be agreed if also C1-203705 is agreed. Atle, Wed, 10:59 Can not agree, see 3705 |  |
|  |  |  | C1-203717 | Handling of rejected NSSAI when associated with 5GMM cause #62 |  |  | Samsung,Huawei,HiSilicon/Anikethan | CR 2052 24.501 Rel-16 | Revision of C1-202150 Roozbeh, Wed, 0115 mandating the network to send all the rejected S-NSSAIs which is redundant may not be a good idea. Ani, Wed, 08:48 Explaiing to Roozbeh |  |
|  |  |  | C1-203758 | Default S-NSSAI not subject to network slice-specific authentication and authorization |  |  | Samsung Electronics Polska / Ricky | CR 2232 24.501 Rel-16 | Revision of C1-203133 Roozbeh, Tue, 19:26 Comments Ricky, Tue, 20:41 Answering Roozbeh, Tue, 20:58 New question Ricky, Wed, 11:06 Explains to Roozbeh Roozbeh, wed, 21:42 Minor rewording Ricky, Wed, 22:27 explainig |  |
|  |  |  | C1-203759 | Performing network slice-specific re-authentication and re-authorisation |  |  | Samsung Electronics Polska / Ricky | CR 2233 24.501 Rel-16 | Revision of C1-203134 Roozbeh, Tue, 21:28 Why would paging play a role? Ricky, Wed, 00:37 Explaining to Roozbeh Kaj, Wed, 08:34 Fine, minor rewording Ricky, Wed, 17:12 Provides a rev Kaj, Wed, 20:30 Fine with the rev, co-sign |  |
|  |  |  | C1-203760 | Storage of pending NSSAI |  |  | Samsung Electronics Polska, Huawei, HiSilicon / Ricky | CR 2234 24.501 Rel-16 | Revision of C1-203135 Related to C1-20303706/07 Atle, Wed, 10:33 Co-sign Amer, Thu, 04:42 Question for clarification and Comment |  |
|  |  |  | C1-203762 | Pending NSSAI and equivalent PLMNs |  |  | Samsung Electronics Polska / Ricky | CR 2236 24.501 Rel-16 | Revision of C1-203138 Kaj, Wed, 08:38 What is the use case? Ricky, Wed, 11.36 explains Amer, Thu, 05:04 Question for clarification Ricky, Thu, 09:29 Explaining to amer Kaj, Thu, 13:46 Still commenting |  |
|  |  |  | C1-203763 | Emergency services during NSSAA that fails for all slices |  |  | Samsung Electronics Polska / Ricky | CR 2238 24.501 Rel-16 | Revision of C1-203140 Roozbhe, Tue, 21:36 Request to reformulate Ricky, Wed, 00:37 Explaining to Roozbeh why no reformulating is needed Roozbeh, Wed, 01:52 Fine Kaj, Wed, 08:49 UE has no allowed S-NSSAIs hence to move the UE to be emergency registered is not needed as the UE is restricted to access normal services Ricky, Wed, 10:51 Explains to Kaj Amer, Thu, 05:29 Proposal how to improve Ricky, Thu, 09:32 Acks Amer Kaj, Thu, 13:57 question |  |
|  |  |  | C1-203764 | NSSAA and NSSAI Inclusion Mode |  |  | Samsung Electronics Polska / Ricky | CR 2239 24.501 Rel-16 | Revision of C1-203141 Amer, Thu, 06:00 Comments Ricky, Thu, 09:23 Commenting to Amer Kaj, Thu, 14:01 this CR is dependent on the outcome of C1-203705. If C1-203705 is agreed, then it will impact this CR. Ricky, Thu, 14:07 Does not agree that it is dependant on 3705 |  |
|  |  |  | C1-203765 | Size of pending NSSAI in REGISTRATION ACCEPT message |  |  | Samsung Electronics Polska / Ricky | CR 2315 24.501 Rel-16 | Revision of C1-203456 |  |
|  | Vertical_LAN |  |  | Peter – Main |  |  |  |  | CT aspects of 5GS enhanced support of vertical and LAN services |  |
|  |  |  | C1-203092 | Work plan for Vertical_LAN |  |  | Nokia, Nokia Shanghai Bell | Work Plan Rel-16 |  |  |
|  |  |  |  |  |  |  |  |  | Stand-alone NPN |  |
|  |  |  | C1-202087 | Correction in UE upon receipt of 5GMM cause value #74 or #75 via a non-integrity protected NAS message |  |  | Nokia, Nokia Shanghai Bell | CR 2010 24.501 Rel-16 | Agreed Revision of C1-200970 |  |
|  |  |  | C1-202193 | update of the counter for SNPN |  |  | Vivo | CR 2064 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202194 | temporarily and permanently forbidden SNPNs lists per access type |  |  | Vivo | CR 2065 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202197 | 5GMM cause value #74 in an SNPN with a globally-unique SNPN identity |  |  | Vivo | CR 2068 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202393 | figures 1, 2a, 2b, 3 and table 2 not applicable in SNPN |  |  | Intel /Thomas | CR 0524 23.122 Rel-16 | Agreed |  |
|  |  |  | C1-202406 | Handling of a UE not allowed to access SNPN services via a PLMN by subscription with 5GMM cause value #72 |  |  | Nokia, Nokia Shanghai Bell | CR 2151 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202522 | Correct “theregistration” |  |  | MediaTek Inc. | CR 2196 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202777 | correction to network selection in case of multiple subscribed SNPNs |  |  | Intel /Thomas | CR 0528 23.122 Rel-16 | Agreed Revision of C1-202432 |  |
|  |  |  | C1-202710 | 5G GUTI of SNPN |  |  | Vivo | CR 2067 24.501 Rel-16 | Agreed Revision of C1-202196 |  |
|  |  |  | C1-202854 | UE in the 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE substate operating in SNPN access mode |  |  | Nokia, Nokia Shanghai Bell | CR 2157 24.501 Rel-16 | Agreed Revision of C1-202413 |  |
|  |  |  | C1-202859 | Management of forbidden SNPNs list upon receipt of a non-integrity protected reject message |  |  | Nokia, Nokia Shanghai Bell | CR 0511 23.122 Rel-16 | Agreed Revision of C1-202086 Ivo, Wed, 19:20 Wants a statement in the report, Ericsson sees a danger in C1-202086 (and its revision) enabling an attacker to temporarily prevent the UE from getting services from the selected SNPN by attacker sending a single fake reject message. |  |
|  |  |  | C1-202869 | 5GMM cause value #13 not supporting roaming for SNPN |  |  | Vivo | CR 2069 24.501 Rel-16 | Agreed Revision of C1-202712 |  |
|  |  |  | C1-202895 | storage of counters for UE in SNPN |  |  | Vivo | CR 2066 24.501 Rel-16 | Agreed Revision of C1-202711 Revision of C1-202195 |  |
|  |  |  | C1-202609 | Definition of registered SNPN |  |  | Intel /Thomas | CR 2060 24.501 Rel-16 | Agreed Revision of C1-202174 |  |
|  |  |  | C1-202799 | Non-3GPP access for PLMN and SNPN |  |  | Huawei, HiSilicon/Lin | CR 2177 24.501 Rel-16 | Agreed Revision of C1-202469 |  |
|  |  |  | C1-202857 | 3GPP PS data off in an SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2159 24.501 Rel-16 | Agreed Revision of C1-202415 |  |
|  |  |  | C1-202920 | Service area restrictions in an SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2153 24.501 Rel-16 | Agreed Revision of C1-202664 Revision of C1-202409 |  |
|  |  |  | C1-202923 | Miscellaneous clean-up for SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2152 24.501 Rel-16 | Agreed Revision of C1-202408 |  |
|  |  |  | C1-203087 | Human readable network name for SNPN (alternative to TS 23.122 CR 0527) |  |  | Ericsson / Ivo | CR 0533 23.122 Rel-16 | Conflicts with C1-203598 Joy, Tue, 09:42 … Introduction of supporting NITZ in SNPN into TS 22.042 should be done first Carlson, Tue, 12:45 -The "Core NW assigned Network Name in NITZ" should be better than "NG-RAN broadcasted HRNN in SIB", -NITZ can be cipher/integrity protected, SIB is not cipher/integrity protected, when both are available, better choose NITZ. Ban, Tue, 13:51 alternative solution to the one in C1-203598, NITZ requires SA1 first, DoCoMo thinks that SIB based is enough Ivo, Tue, 14:39 To Joy, Does not agree that NITZ is not part of SNPN Ivo, Tue, 14:44 Explaining to Carlson Ivo, Tue, 14:49 Explaining to Ban that 3087 provides additional methods over broadcast to save resources Carlson, Tue, 15:28 Arguing with Ivo Ivo, Tue, 15:52 Arguing with Carlson Lena, Wed, 02.19 Some issues with the CR, CT1 should liaise with SA1 on whether Annex A.3 in TS 22.101 applies to SNPNs, and if it does, which source network name has precedence. Sung, Wed, 02:59 Sa2 involvement needed Joy, Wed, 03:45 Not against NITZ, inform SA1 Ivo, Sung, Wed, xx Not agreein on SA2 responsibility Vishnu, Wed, 19:30 Cannot agree Kundan, Wed, 20:3 Questioning Vishnu’s comment Vishnu, Wed, 21:00 Explaining his position Ivo, Wed, 21:13 Discussing with Vishnu Kundan, Wed, 21:17 Supports the CR Vishnu, Wed, 22:07 Does not agree, should go to SA2 Sung, Wed, 23:57 Should go to SA2 Ivo, Thu, 00:56 Discussing with Sung and Vishnu Ban, Thu, 10:02 Need SA1 clarification Ivo, Thu, 11:35 Asking to send LS to SA1 |  |
|  |  |  | C1-203229 | NB-IoT not applicable for SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 2149 24.501 Rel-16 | Revision of C1-202853 ----------------------------------------- Was agreed Needs revision, missing tdoc number on cover sheet, wrong rev counter, should be 1 Revision of C1-202401 |  |
|  |  |  | C1-203230 | Routing indicator update in an SNPN |  |  | Nokia, Nokia Shanghai Bell, Ericsson | CR 2158 24.501 Rel-16 | Revision of C1-202856 ----------------------------------------------- Was agreed Needs revision, missing tdoc number on cover sheet Revision of C1-202414 |  |
|  |  |  | C1-203242 | Updates to SNPN selection |  |  | Apple | CR 0536 23.122 Rel-16 | Ivo, Tue, 09:26 Requests rewording, style of EN wrong Joy, Tue, 10:00 Clarification on the order of the SNPNs in "list temporarily forbidden SNPNs" and "list permanently forbidden SNPNs" should be stated as well Vishnu, Tue, 10:20 Better to leave this UE implementation specific CR is not needed. Carlson, Tue, 12:50 Comments on the cr Lena, Wed, 02:28 No stage-2, could live with a MAY Sung, Wed, 03:21 Same as Lena SangMin, Wed, 06:58 Leave it to implementation |  |
|  |  |  | C1-203255 | Introduction of SNPN-specific N1 mode attempt counters |  |  | Nokia, Nokia Shanghai Bell, Apple | CR 2011 24.501 Rel-16 | Revision of C1-202922 Lin, Thu, 04:09 we cannot agree this CR as way forward and prefer to go the revision of C1-203256. Marko, Thue, 10:27 Fail to see advantage Supporte 3256 |  |
|  |  |  | C1-203256 | Alternative to CR#2011: Re-enabling the N1 mode capability upon expiry of T3247 based on the SNPN-specific attempt counters |  |  | Nokia, Nokia Shanghai Bell | CR 2251 24.501 Rel-16 | Ivo, Tue, 09:26 - preference for the alternative in C1-203255+C1-203366 Lin, Thu, 04:14 Prefers this CRs, some comment Marko, Thue, 10:27 Support 3256 |  |
|  |  |  | C1-203257 | Alternative to CR#2151: Handling of a UE not allowed to access SNPN services via a PLMN by subscription with 5GMM cause value #72 |  |  | Nokia, Nokia Shanghai Bell | CR 2252 24.501 Rel-16 | Ivo, Tue, 09:26 - preference for the alternative in C1-203255+C1-203366 Lin, Thu, 04.44 Prefer to go this alternative than C1-202406 CR#2151 agreed in the last meeting. |  |
|  |  |  | C1-203258 | SNPN-specific N1 mode attempt counters |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-16 | Ivo, Tue, 09:26 - preference for Alt-1 since handling in SNPN is aligned with handling in PLMN. Lin, Thu, 04:45 Prefers Alt-2 |  |
|  |  |  | C1-203283 | operation of UE in SNPN access mode when timer T3247 expires |  |  | vivo | CR 2261 24.501 Rel-16 |  |  |
|  |  |  | C1-203284 | Reference correction for SNPN |  |  | vivo | CR 0540 23.122 Rel-16 |  |  |
|  |  |  | C1-203285 | Reference correction for SNPN |  |  | vivo | CR 2262 24.501 Rel-16 |  |  |
|  |  |  | C1-203320 | UE shall use the GUTI assigned by the same SNPN during registration |  |  | OPPO / Rae | CR 2278 24.501 Rel-16 | Ivo, Tue, 09:26 "when registered to an SNPN" -> "when registering or registered to an SNPN" as the statement applies also to initial registration Yanchao, Tue, 16:47 Prefers to add clarification to in registration procedures Lena, Wed, 02:30 Ok with the CR in principle but the wording needs improvement. Rae, Wed, 03:48 Explaining Sung, Wed, 04:16 Providing a proposal Lena, Thu, 00:03 Fine with Ivo’s proposal Rae, Thu, 03:44 Provides rev Ivo, Thu, 14:17 Wants rewoding |  |
|  |  |  | C1-203321 | Correct PLMN to SNPN in D.2.2.2 |  |  | OPPO / Rae | CR 2279 24.501 Rel-16 |  |  |
|  |  |  | C1-203366 | Correction of the handling of timer TG for SNPNs |  |  | Apple, Nokia, Nokia Shanghai Bell | CR 0514 23.122 Rel-16 | Revision of C1-202896 Lin, Thu, 04:11 Based the observations and proposal in the DP C1-203709, we cannot agree this CR as way forward and prefer to go C1-203367. Marko, Thue, 10:27 Fail to see advantage, do NOT support 3366 Supporte 3256 Robert, Thu, 14:06 Explaining the advantage |  |
|  |  |  | C1-203367 | Alternative to CR#0514: Correction of the handling of timer TG for SNPNs |  |  | Apple | CR 0542 23.122 Rel-16 | Ivo, Tue, 09:26 - preference for the alternative in C1-203255+C1-203366 Line, Thu, 04:49 Prefers this CR Marko, Thu, 10:35 Prefers this CR |  |
|  |  |  | C1-203441 | Clarification to SNPN manual selection. |  |  | Huawei, HiSilicon / Vishnu | CR 0549 23.122 Rel-16 |  |  |
|  |  |  | C1-203442 | Clarification to SNPN registration after SNPN selection. |  |  | Huawei, HiSilicon / Vishnu | CR 0550 23.122 Rel-16 | Ivo, Tue, 09:26 - there is no service which does not require registration and thus the UE needs to be capable of services which require registration Lena, Wed, 02:32 Not needed Sung, Wed, 03:20 Not needed Vishnu, Wed, 11:50 Explains Ivo, Wed, 23:00 Is there any SNPN service would NOT require registration Len, Thue, 01:04 Still think the CR is not needed Vishnu, Thu, 11:32 explaining |  |
|  |  |  | C1-203517 | RRC inactive for SNPN |  |  | vivo | CR 2338 24.501 Rel-16 | Ivo, Tue, 09:26 - the added text seems to be captured in the previous paragraph (unless we want to duplicate all the PLMN related requirements) Yanchao, Thu, 06:24 rev |  |
|  |  |  | C1-203520 | Clarification on the non-supported functions and procedures for SNPN |  |  | SHARP | CR 2340 24.501 Rel-16 | Ivo, Tue, 09:26 - "or PNI-NPN" - in PNI-NPN, the UE is registered to a PLMN. I suggest to remove "or PNI-NPN" Lena, Wed, 02:33 Rewording Kawasaki, Wed, 04:45 Provides rev Ivo, Wed, 23:02 Rev is fine Sung, Thu, 00:16 Some changes on the bulleting Lena, Thu, 00:28 Fine with the rev, and also with sungs suggestion, no strong pref Yudai, Thu, 04:29 New rev, based on Sung comment Lin, Thu, 04:53 Fine with rev3 |  |
|  |  |  | C1-203248 | Adding NID to PANI |  |  | Ericsson /Jörgen | CR 6420 24.229 Rel-16 | Lena, Wed, 02:35 Fine, but add some reference Sung, Wed, 18:35 Questioning the need |  |
|  |  |  | C1-203598 | Human readable network name for SNPN |  |  | Nokia, Nokia Shanghai Bell | CR 0527 23.122 Rel-16 | Revision of C1-202855 Ivo, Tue, 09:26 - this CR does not enable the UE to display network name to the UE when the HRNN is not broadcast, even thought 22.042 requires the UE to use network name provided in NITZ information at the earliest opportunity and 22.101 A.3 enables the UE to display stored network name. Sung, Wed, 00:14 This is aligned with stage-2 Lena, Wed, 02:38 Fine, but text needs to be updated Sung, Wed, 04:28 Provides rev Ban Wed 11:41 FINE Vishnu, Wed, 18:40 Some rewording, wants to co-sign Sung, Wed, 19:23 Provides rev Lena, Wed, 00:25 Fine with the rev SangMin, Thu, 04:20 Support this one, fine with the drfat Conflicts with C1-203087 ------------------------------------------ Was agreed Revision of C1-202407 |  |
|  |  |  | C1-203599 | 5GSM back-off mechanisms in an SNPN |  |  | Nokia, Nokia Shanghai Bell, Intel, Ericsson | CR 2156 24.501 Rel-16 | Revision of C1-202915 Carlson, Tue, 13:00 Ist the “not” necessary? Sung, Tue, 22:04 Providing rev Sung, Wed, 02:36 Providing rev Carlson, Wed, 05:52 fine --------------------------------------- Was agreed Revision of C1-202412 |  |
|  |  |  | C1-203602 | 3GPP PS data off configuration parameters for a UE operating in SNPN access mode |  |  | Nokia, Nokia Shanghai Bell | CR 0051 24.368 Rel-16 | Ivo, Tue, 09:25 - shouldn't all the parameters be related to SNPN identity? E.g. SM_RetryWaitTime could be different per SNPN too. Sung, Wed, 00:02 Offers some changes to ivo Lena, Wed, 02:43 Issue with the MOtree Ivo, Wed, 23:09 Asks that structure is made generic Sung, Thu, 00:31 Asks for clarification Lena, Thu, 01:33 Potential issue when using USIM file is that there is no concept of home SNPN Sung, THue, 03:16 New proposal to Lena Ivo, Thu, 14.26 Offers proposal for structure |  |
|  |  |  | C1-203640 | Editorial change to SNPN |  |  | Huawei, HiSilicon / Cristina | CR 2366 24.501 Rel-16 |  |  |
|  |  |  | C1-203641 | Update of the counters on receiving #27 in an SNPN |  |  | Samsung/Kundan | CR 2367 24.501 Rel-16 | Ivo, Tue, 09:25 need to be aligned or merged with C1-203255 Lena, Wed, 02:45 Rewording Lin, Thu, 05:03 CR can be merged into C1-203257 as no any change is needed for #27 in this case |  |
|  |  |  | C1-203665 | Management for SNPN access mode per access type |  |  | SHARP | CR 2371 24.501 Rel-16 | Lena, Wed, 02:47 New text confusing, at most a note Kawasaki, Wed, 07:06 Provides rev Len, Thu, 00:46 fine |  |
|  |  |  | C1-203709 | Discussion on SNPN-specific N1 mode attempt counter |  |  | Huawei, HiSilicon, MediaTek Inc./Lin | discussion Rel-16 | Ivo, Tue, 09:25 - prefer separate counters since #27 and #74/75 result into disabling for different times. Sung, Tue, 21.51 Three counters Lin, Wed, 06:18 Does not agree with Sung Sung, Wed, 16:04 Different counter is needed if different list is impacted. Both #11 and #73 impacts forbidden PLMN list. Osama, Wed, 20:29 In summary, we have slight preference to have separate attempt counter for N1 mode (Nokia+Apple proposal) to cover access related possible failure although it might sound is not that useful given that SNPN is only supported over one RAT=5G. |  |
|  |  |  | C1-203710 | SNPN services via a PLMN over 3GPP access |  |  | Huawei, HiSilicon/Lin | CR 2388 24.501 Rel-16 | Lena, Wed, 02:48 Current spec is clear enough, not good Lin, Wed, 06:34 Discussing with Lena Sung, Thu, 01:33 bullet h) in clause 4.14.2 is clear enough. Lin, Thu, 04:40 Providing rev Lena, Thu, 16:08 Not needed, can live with it, needs revision |  |
|  |  |  |  |  |  |  |  |  | Public network integrated NPN |  |
|  |  |  | C1-202008 | CAG-ID not provided to lower layers during NAS signalling connection establishment |  |  | Ericsson / Ivo | CR 1880 24.501 Rel-16 | Agreed Revision of C1-200937 |  |
|  |  |  | C1-202199 | Clarification of the cause of start of T3550 |  |  | Vivo | CR 2070 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202470 | No CAG in non-3GPP access |  |  | Huawei, HiSilicon/Lin | CR 2178 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202471 | Correction on 5GMM #27 for CAG |  |  | Huawei, HiSilicon/Lin | CR 2179 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202495 | Correction to Manual CAG selection procedure |  |  | Samsung/Kundan | CR 0529 23.122 Rel-16 | Agreed |  |
|  |  |  | C1-202840 | Providing configured HRNN for CAG selection |  |  | Ericsson / Ivo | CR 2009 24.501 Rel-16 | Agreed Needs revision, rev counter should be 2 Revision of C1-202015 |  |
|  |  |  | C1-202845 | Handling of HRNN information in a CAG cell |  |  | Huawei, HiSilicon / Vishnu | CR 0518 23.122 Rel-16 | Agreed Revision of C1-202256 |  |
|  |  |  | C1-202737 | Correction on no suitable cell |  |  | Vivo | CR 0517 23.122 Rel-16 | Agreed Revision of C1-202179 |  |
|  |  |  | C1-202886 | Provision of CAG information list in reject messages |  |  | Huawei, HiSilicon/ Vishnu | CR 2087 24.501 Rel-16 | Agreed Revision of C1-202253 |  |
|  |  |  | C1-202924 | CAG selection is optional in the manual network selection mode |  |  | Nokia, Nokia Shanghai Bell | CR 0526 23.122 Rel-16 | Agreed Revision of C1-202405 |  |
|  |  |  | C1-202912 | Selected CAG-ID from the NAS layer to the AS layer |  |  | Nokia, Nokia Shanghai Bell, vivo, Qualcomm Incorporated, Samsung, Huawei, HiSilicon | CR 0525 23.122 Rel-16 | Agreed Revision of C1-202397 |  |
|  |  |  | C1-202836 | Non-integrity protected REGISTRATION REJECT message including 5GMM cause #31 or #76 |  |  | Nokia, Nokia Shanghai Bell | CR 2134 24.501 Rel-16 | Agreed Revision of C1-202373 Shifted from 5G_CIoT |  |
|  |  |  | C1-203286 | Clarification on emergency services for UE not supporting CAG |  |  | vivo | CR 2263 24.501 Rel-16 | Ivo, Tue, 09:25 - if the cell is not barred, then the UE not supporting CAG will NOT see it as a CAG cell, but as a non-CAG cell Carlson, Tue, 12:54 Need to align wording Lena, Wed, 02:50 Not needed Sung, Wed, 03:41 Same as Ivo |  |
|  |  |  | C1-203300 | Discussion on UE consideration for "a CAG cell" and "not a CAG cell“ |  |  | MediaTek Inc. / Carlson | discussion Rel-16 | Ivo, Tue, 09:25 Same as for 3302 Vishnu, Tue, 10:56 Fails to see the two problems Lena, Wed, 02:52 Comments |  |
|  |  |  | C1-203301 | Correction on UE consideration for not a CAG cell |  |  | MediaTek Inc. / Carlson | CR 0541 23.122 Rel-16 | Ivo, Tue, 09:25 Same as for 3302 Vishnu, Tue, 11:13 Solution seems not correct, similar problem is addressed in 3437 Carlson, Tue, 11:31 Provides rev1 Sung, Tue, 20:24 no value of NOTE 7 but only harm Carlson, Wed, 04:57 Defending NOTE 7 Sung, Wed, 16:07 Not agreeing, if at all, needs to look different anduse 5Gprotoc wid Ivo, Wed, 14:32 Works for Ivo Vishnu, Thu, 17:07 commenting |  |
|  |  |  | C1-203302 | Correction on UE consideration for CAG cells |  |  | MediaTek Inc. / Carlson | CR 2266 24.501 Rel-16 | Ivo, Tue, 09:25 cell should be considered separately (a) per PLMN without CAG, (b) per PLMN+CAG and (c) per SNPN, Vishnu, Tue, 11.18 we don’t see the relevance of this CR. Sung, Tue, 20:00 CR is unclear |  |
|  |  |  | C1-203436 | Correction to CAG selection in automatic mode |  |  | Huawei, HiSilicon / Vishnu | CR 2312 24.501 Rel-16 | Withdrawn |  |
|  |  |  | C1-203437 | Correction to CAG selection in Automatic mode |  |  | Huawei, HiSilicon / Vishnu | CR 0545 23.122 Rel-16 | Ivo, Tue, 09:25 Requests some changes, also asks for a SA2 requirement Carlson, Tue, 11:49 Coments on the CR Yanchao, Tue, 16:13 Current text correct, Do no not not delete bullet 1 Xu, Tue, 16:42 Same thoughts as Carlson, comments on the CR Lena, Wed, 02:54 comments Vishnu, Thu, 10:53 Provides rev Carlson, Thu, 12:49 fine |  |
|  |  |  | C1-203438 | Indication to user about allowed CAG ID in manual selection |  |  | Huawei, HiSilicon / Vishnu | CR 0546 23.122 Rel-16 | merged into C1-203601 Lena, Wed, 02:54 Comments, overlaps with C1-203601, prefers 3601 |  |
|  |  |  | C1-203439 | Resolving editors note in Limited service condition on a CAG cell. |  |  | Huawei, HiSilicon / Vishnu | CR 0547 23.122 Rel-16 |  |  |
|  |  |  | C1-203440 | Removal of selected CAG-ID in automatic selection mode. |  |  | Huawei, HiSilicon / Vishnu | CR 0548 23.122 Rel-16 | Yanchao, Tue, 16:17 AS layer needs the selected CAG ID for cell selection, therefore the selection of CAG ID is needed in automatic mode. Lena, Wed, 02:58 Support the CR over C1-203603, header is wrong Vishnu, Wed, 11:34 Explaining to yanchao Vishnu, Thu, 11:49 Provides rev |  |
|  |  |  | C1-203443 | Handling of CAG information list in REGISTRATION ACCEPT messages |  |  | Huawei, HiSilicon / Vishnu | CR 2313 24.501 Rel-16 | Ivo, Tue, 09:25 - "current PLMN" -> "registered PLMN". Reason: UE is registered. - emergency PDU session should also be checked in a) 1) and a) 2) i) and b) 1) Yanchao, Tue, 16:22 -Why the UE enter the limited service state when the network accepts the registration request. -Does the “CAG Cell ” in bullet a) mean CAG only cell, if not, the UE still can get the normal service. Same comment applies to bullet a-2-ii）; -Why consider emergency PDU session in initiation registration, the UE has not established any emergency PDU session yet. Sung, Wed, 04:08 Asking Ivo Rae, Wed, 09:10 Asking question Ivo, Wed, 23:28 Withdraws second comment Vishnu, Thu, 16:44 rev |  |
|  |  |  | C1-203445 | Provision of CAG information list in SERVICE REJECT message. |  |  | Huawei, HiSilicon / Vishnu | CR 2314 24.501 Rel-16 | SangMin, Thu, 04:08 I don’t think that this CR is needed Kundan, Thu, 09:38 Explaining the need Vishnu, Thu, 12:04 explaining |  |
|  |  |  | C1-203532 | Sending CAG information list |  |  | Samsung/Kundan | CR 2128 24.501 Rel-16 | Revision of C1-202362 Ivo, Tue, 09:25 Summary of changes not aligned with the CR, long list of requested changes Frederic, Tue, 11:43 Missing clauses affected Yanchao, Tue, 16:34 1. How does the UE know whether the CAG information list is from the HPLMN or the VPLMN? 2. Bullet 5 is not clear, does the UE discard the whole CAG information list or part of the CAG information list? Sung, Tue, 18:50 we have sent an LS to SA2 on this matter, we should wait for their response. Lena, Wed, 03:05 CT1 should not agree this CR before having received a reply LS from SA2 |  |
|  |  |  | C1-203601 | Manual CAG selection |  |  | Nokia, Nokia Shanghai Bell, NTT DOCOMO, Ericsson, Huawei, HiSilicon | CR 0499 23.122 Rel-16 | Revision of C1-202862 Lena, Wed, 03:07 Prefers this over 3438 ------------------------------------------- Was Agreed Revision of C1-202398 |  |
|  |  |  | C1-203603 | CAG selection after automatic PLMN selection |  |  | Nokia, Nokia Shanghai Bell | CR 0556 23.122 Rel-16 | Withdrawn Requested by author, wed, 03:46 Ivo, Tue, 09:25 CR seems not needed, gives explanation Carlson, Tue, 12:00 Provides rewording Chenxu, Tue, 14:51 Asking for explanation and some comments Yanchao, Tue, 16:29 Requests changes Lena, Wed, 03:09 Not needed |  |
|  |  |  | C1-203604 | Rejection of non-emergency PDU session establishment with 5GMM cause #76 |  |  | Nokia, Nokia Shanghai Bell | CR 2361 24.501 Rel-16 | Ivo, Tue, 09:25 not clear why the AMF should wait with providing the CAG information to the UE while keeping the UE in 5GMM-CONNECTED on a cell not allowed by the new CAG information. |  |
|  |  |  | C1-203659 | No CAG ID in de-registration request |  |  | Huawei, HiSilicon / Cristina | CR 2368 24.501 Rel-16 | Lena, Wed, 03:14 typo Cristian, Wed, 04:26 Ack |  |
|  |  |  | C1-203691 | Sending CAG information list |  |  | Samsun/Kundan | CR 0522 23.122 Rel-16 | Revision of C1-202363 Ivo, Tue, 09:25 Summary of change no aligned with CR, list of requested changes Sung, Tue, 18:43 Wait for response from SA2 (we have sent LS) Lena, Wed, 03:23 CT1 should wait for SA2’s response |  |
|  |  |  | C1-203715 | Sending CAG information list -option 2 |  |  | Samsung/Kundan | CR 2389 24.501 Rel-16 | Ivo, Tue, 09:25 seems too complex, prefer C1-203691 and accepting the entire list when the UE is in the HPLMN, EHPLMN the or a PLMN equivalent to the HPLMN. Otherwise, only the entry of the VPLMN is used and updated. Sung, Tue, 18:42 we have sent an LS to SA2 on this matter, we should wait for their response. Lena, Wed, 03:23 CT1 should wait for SA2’s response Carlson, Wed, 05:03 Rewording Kundan, Wed, 20:28 Fine to wait for SA2 Kundan, Wed, 20:34 Explaining to Carlson Carslon, Thu, 05:25 Prefers C1-203691 |  |
|  |  |  | C1-203747 | Handling of CAG only configuration |  |  | Samsung/Kundan | CR 2363 24.501 Rel-16 | Revision of C1-203609 Ivo, Tue, 09:25 First sentence not needed, no justification for the second one Sung, Tue, 18:43 CR needs to be rejected, explanation why Lena, Wed, 03:12 disagrees |  |
|  |  |  |  |  |  |  |  |  | Time sensitive communication |  |
|  |  |  | C1-202192 | Abbreviation correction |  |  | Vivo | CR 0002 24.519 Rel-16 | Agreed |  |
|  |  |  | C1-202429 | IEEE Std 802.1Qbv-2016 rolled into IEEE Std 802.1Q-2018 |  |  | Nokia, Nokia Shanghai Bell | CR 0003 24.519 Rel-16 | Agreed |  |
|  |  |  | C1-202714 | Correction of the abnormal case in NW-TT-initiated Ethernet port management procedure |  |  | Vivo | CR 0001 24.519 Rel-16 | Agreed Revision of C1-202191 |  |
|  |  |  | C1-202860 | TSN working domain |  |  | Nokia, Nokia Shanghai Bell | CR 0002 24.535 Rel-16 | Agreed Revision of C1-202433 |  |
|  |  |  | C1-203607 | Assignment of timer numbers and IEIs |  |  | Nokia, Nokia Shanghai Bell | CR 0006 24.519 Rel-16 | Ivo, Tue, 09:25 IEs should be assigned with some logic, to enable the recipient to skip unknown IEs. It is proposed to use the logic specified in 24.007 Sung, Tue, 1817 Provides rev Ive, Wed, 23:37 Co-sign |  |
|  |  |  | C1-203642 | Spliting port management information into port- and bridge-specific information |  |  | Huawei, HiSilicon / Cristina | CR 0007 24.519 Rel-16 |  |  |
|  |  |  | C1-203663 | Correct the ETHERNET PORT MANAGEMENT NOTIFY ACK message name |  |  | Huawei, HiSilicon / Cristina | CR 0008 24.519 Rel-16 |  |  |
|  |  |  | C1-203425 | Updating definitions for Ethernet port management messages |  |  | China Mobile | CR 0005 24.519 Rel-16 |  |  |
|  |  |  | C1-203426 | Updating Port management information container IE |  |  | China Mobile | CR 2303 24.501 Rel-16 |  |  |
|  |  |  | C1-203792 | Introduction of Bridge management information |  |  | Intel, Nokia, Nokia Shanghai Bell / Thomas | CR 0004 24.519 Rel-16 | Revision of C1-203340 _________________________________________ Ivo, Tue, 09:25 - formal dependency on cover sheet is missing - IEs should be assigned with some logic, to enable the recipient to skip unknown IEs. It is proposed to use the logic specified in 24.007. Sung, Tue, 18:21 Provides the IEs definition Ivo, Wed, 23:56 Fine with the IEs definition |  |
|  | 5G_CioT |  |  | Peter – Main |  |  |  |  | CT aspects of Cellular IoT support and evolution for the 5G System |  |
|  |  |  | C1-202079 | Integrity protection data rate for UEs that don’t support N3 data transfer |  |  | Samsung | CR 2031 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202082 | Addition of Control Plane Service Request in the abnormal cases for service request procedure |  |  | InterDigital Communications | CR 2032 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202085 | Correcting a wrong reference |  |  | InterDigital Communications | CR 2035 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202176 | Correction of SGC |  |  | Vivo | CR 2062 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202367 | Correction on terminology for the Control plane CioT 5GS optimization |  |  | SHARP | CR 2130 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202419 | Corrections to CR#1907 |  |  | Qualcomm Incorporated / Amer | CR 2162 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202462 | Acknowledgement of truncated 5G-S-TMSI configuration |  |  | Huawei, HiSilicon/Lin | CR 2173 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202463 | NAS-MAC calculation for RRC connection reestablishment for NB-IoT CP optimisation |  |  | Huawei, HiSilicon/Lin | CR 2174 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202464 | Removal of Editor’s Note for CP congestion control |  |  | Huawei, HiSilicon/Lin | CR 2175 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202614 | QoS error checks for UEs in NB-N1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2145 24.501 Rel-16 | Agreed Revision of C1-202388 |  |
|  |  |  | C1-202626 | Retransmission of a CPSR message after integrity check failure at the AMF |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2150 24.501 Rel-16 | Agreed Revision of C1-202404 |  |
|  |  |  | C1-202662 | UE specific DRX for NB-S1 mode |  |  | Vodafone GmbH | CR 3353 24.301 Rel-16 | Agreed Revision of C1-202384 |  |
|  |  |  | C1-202674 | Correct handling of receiving EMM cause #31 in EPS |  |  | Qualcomm Incorporated | CR 3349 24.301 Rel-16 | Agreed Revision of C1-202270 |  |
|  |  |  | C1-202676 | Correct UE azarosi for receiving 5GMM cause #31 in 5GS |  |  | Qualcomm Incorporated | CR 2094 24.501 Rel-16 | Agreed Revision of C1-202271 |  |
|  |  |  | C1-202735 | Emergency PDU sesseion established after WUS negotiation |  |  | Vivo | CR 2063 24.501 Rel-16 | Agreed Revision of C1-202177 |  |
|  |  |  | C1-202699 | Handling of PDU session and PDN connection associated with Control plane only indication in case of N26 based interworking procedures |  |  | SHARP | CR 2132 24.501 Rel-16 | Agreed Revision of C1-202369 |  |
|  |  |  | C1-202779 | CioT user or small data container in CPSR message not forwarded |  |  | ZTE | CR 2114 24.501 Rel-16 | Agreed Revision of C1-202337 |  |
|  |  |  | C1-202782 | Clarification on the UE behaviour when receiving T3448 |  |  | ZTE | CR 2112 24.501 Rel-16 | Agreed Revision of C1-202335 |  |
|  |  |  | C1-202878 | Initial APN rate control parameters |  |  | Qualcomm Incorporated / Amer | CR 3216 24.008 Rel-16 | Agreed Revision of C1-202422 |  |
|  |  |  | C1-202880 | Signalling of EPS APN rate control parameters during PDU session establishment |  |  | Qualcomm Incorporated / Amer | CR 2164 24.501 Rel-16 | Agreed Revision of C1-202423 |  |
|  |  |  | C1-202692 | Correction to handling of T3447 timer |  |  | MediaTek Inc. | CR 2195 24.501 Rel-16 | Agreed Revision of C1-202521 |  |
|  |  |  | C1-202892 | Generic UE configuration update trigger for registration and EC Restriction change |  |  | Ericsson / Mikael | CR 2077 24.501 Rel-16 | Agreed Revision of C1-202230 |  |
|  |  |  | C1-202904 | Indication of change in the use of enhanced coverage |  |  | Samsung, InterDigital, Huawei, HiSilicon | CR 2030 24.501 Rel-16 | Agreed Revision of C1-202648 |  |
|  |  |  | C1-202866 | PDU session release due to CP only revocation |  |  | OPPO / Rae | CR 2107 24.501 Rel-16 | Agreed Revision of C1-202707 Revision of C1-202328 |  |
|  |  |  | C1-202795 | Enhancement on CPSR for CioT CP data transport |  |  | Huawei, HiSilicon, Vodafone, ZTE, China Mobile, China Telecom, CATT/Lin | CR 1701 24.501 Rel-16 | Agreed Revision of C1-202459 Revision of C1-200893 |  |
|  |  |  | C1-202796 | Correction on WUS assistance |  |  | Huawei, HiSilicon/Lin | CR 2176 24.501 Rel-16 | Agreed Revision of C1-202465 |  |
|  |  |  | C1-202926 | DRX parameters for NB-IoT |  |  | InterDigital Communications | CR 2034 24.501 Rel-16 | Agreed Revision of C1-202865 Revision of C1-202671 |  |
|  |  |  | C1-203089 | Correct Service Gap Control |  |  | CATT | CR 2225 24.501 Rel-16 | Merged into in C1-203431 Requested by author , overlaps with CR in C1-203431 Kaj, Tue, 15:10 - Wrong title I would say - ME is impacted - Baseline should be 16.4.1 - Not sure I fully understand the Note “Service Gap Control does not apply to exception reporting for NB-IoT”. Please elaborate more on this? - The CR overlaps with C1-203431 which I prefer as the base for a potential merge if other companies agrees with the main proposal of both CRs Amer, Tue, 20:23 QCOM prefers C1-203431 Lin, Wed, 10:51 Prefers 3431, merge 3089 to 3431 |  |
|  |  |  | C1-203090 | Add Enhanced Coverage Restriction information |  |  | CATT | CR 2226 24.501 Rel-16 | Not pursued Requested by author, Tue, 18:12 Asked to note this Behrouz, Tue, 09:25 New IE, but this has to be defined in a message first Mikael, Tue, 09:38 Not needed, covered by C1-202892 Chenxi, Tue, 10:30 Explaining why the CRis needed, but needs a rev1 to address some open aspects Mikael, Tue, 10:58 Disagrees, the CR is NOT NEEDED Mahmoud, Tue, 17:43 CR is not needed Behrouz, Tue, 20:08 Further discussin |  |
|  |  |  | C1-203282 | maintenance of T3517 |  |  | vivo | CR 2260 24.501 Rel-16 |  |  |
|  |  |  | C1-203289 | Condition under which the UE shall enter 5GMM-IDLE mode when user plane CIoT 5GS optimization is used |  |  | InterDigital Communications | CR 2265 24.501 Rel-16 | Lin, Tue, 14:01 Some minor rewording Behrouz, Tue, 21:29 Acks Lin |  |
|  |  |  | C1-203299 | Avoid repeated redirection for CIOT |  |  | OPPO / Rae | CR 2106 24.501 Rel-16 | Revision of C1-202734 Marko, Wed, 10:55 Not a good solution Rae, Thu, 03:58 Explaining to Marko Mikael, Thu, 09:40 If at all then a note |  |
|  |  |  | C1-203322 | Correction on reference to CN selection |  |  | OPPO / Rae | CR 2280 24.501 Rel-16 | Yanchao, Tue, 17:25 Current reference is correct Rae, Wed, 03:34 Defending the Cr Mikeal, Wed, 08:25 Current text seems correct, same view as Yanchao Rae, Wed, 10:36 Explain to Mikael Mikael, Thu, 08:57 discussing |  |
|  |  |  | C1-203323 | Correct Service Gap Control |  |  | CATT | CR 3373 24.301 Rel-16 | Revision of C1-203088 Kaj, Tue, 15:23 - No such stage 2 requirements for Service Gap control in EPS as for 5GS to not start timer for some exceptions Many errors in the CR, in case it goes forward Chenxi, Tue, 18:02 Rev2 Behrouz, Tue, 20:54 To Kaj, the work item code can’t be SAES Lin, Wed, 10:52 please try to align with the changes in C1-203431 for 5G as far as possible Chenxi, Wed, 11:25 Provides rev Kaj, Thu, 08:28 Still has concerns, justification is not good enough for the change Behrouz, Thu, 08:52 Spec number is wrong Chenxi, Thu, 10:28 Discussing with Kaj Chenxi, Thu, 11.13 rev |  |
|  |  |  | C1-203337 | Connection Resumption for Notification |  |  | ZTE, vivo | CR 2113 24.501 Rel-16 | Revision of C1-202775 -------------------------------------------- Was agreed Revision of C1-202336 |  |
|  |  |  | C1-203403 | Correction to +CNMPSD for NR |  |  | BlackBerry UK Ltd. | CR 0693 27.007 Rel-16 |  |  |
|  |  |  | C1-203418 | Adding a new abnormal case on the network side for CPSR |  |  | China Mobile, InterDigital | CR 2056 24.501 Rel-16 | Revision of C1-202749 ------------------------------------ Was agreed Needs revision Rev counter should be 2 Revision of C1-202169 |  |
|  |  |  | C1-203427 | De-registration request and CPSR collision case in the NW |  |  | Ericsson /kaj | CR 2304 24.501 Rel-16 |  |  |
|  |  |  | C1-203428 | Additional stop condition for timer T3580 |  |  | Ericsson /kaj | CR 2305 24.501 Rel-16 | Lin, Tue, 13:43 not a 5G_CIOT CR but should be 5GProtoc16 CR. Hence the cover page should be updated and move to the correct agenda Kaj, Wed, 17:31 Wants to keep it 5G CIoT |  |
|  |  |  | C1-203429 | Abonormal cases on UE side and the CPSR message |  |  | Ericsson /kaj | CR 2306 24.501 Rel-16 | partially overlaps with in C1-203282 (T3517 aspect) Lin, Tue, 13:46 how can a CPSR message be sent in connected mode Frederic, Wed, 16:29 Spec number on cover page wrong Kaj, Thu, 11:45 Acks, will update |  |
|  |  |  | C1-203430 | Service gap control: Alignment of NW and UE behaviour for timer T3447 |  |  | Ericsson /kaj | CR 2307 24.501 Rel-16 |  |  |
|  |  |  | C1-203431 | Service gap control: Exceptions to start of timer T3447 |  |  | Ericsson /kaj | CR 2308 24.501 Rel-16 | overlaps with CR in C1-203089 |  |
|  |  |  | C1-203462 | Corrections for Enhanced Coverage in 5GS for CIoT |  |  | Intel, Huawei, HiSilicon / Vivek | CR 2316 24.501 Rel-16 |  |  |
|  |  |  | C1-203476 | Remove redundant check for UE's support of CP CIoT optimization |  |  | Samsung/Anikethan | CR 2319 24.501 Rel-16 | Lin, Tue, 14:19 Does not believe changes are needed, but if Ani wants to go forward, then rewording Yanchao, Tue, 17:22 Some changes for AMF Mikael, Wed, 07:14 Explaining to Yanchao why the CR is ok Ani, Wed, 09:52 Discussion ongoing Yanchao, Wed, 11:25 Sees the problem, still requires rewording Mikael, Thu, 09:1 Supports the wording from ANi |  |
|  |  |  | C1-203483 | Truncated 5G-S-TMSI for eMTC UE |  |  | Qualcomm Incorporated / Amer | CR 2322 24.501 Rel-16 | Behrouz, Tue, 09:25 Section 8.2.7.37 does not show any changes Lin, Tue, 12:20 Fundamental comments, how could this work well? Frederic, Tue, 12:13 Cover sheet issue, CR# missing. |  |
|  |  |  | C1-203484 | Disucssion on Ethernet Header Compression |  |  | Qualcomm Incorporated / Amer | discussion Rel-16 | Lin, Tue, 13:36 what proposed by this DP was already there in the current spec. Kaj, Tue, 15:44 - Not obvious to me how the UE can be in the best position to decide EHC or not, I assume you mean the application should know at least. Why not NW Mahmoud, Tue, 21:09 We don’t see why Ethernet header compression at the 5GSM layer should be different from that of IPHC. Hence, we don’t support the proposal and we prefer to keep a consistent UE behavior for both IP and Ethernet HC. Yanchao, Wed, 11:30 Agree with the proposal 1 |  |
|  |  |  | C1-203485 | Ethernet header compression for CP CIoT – 5GMM aspects |  |  | Qualcomm Incorporated, Ericsson / Amer | CR 2165 24.501 Rel-16 | Revision of C1-202882 Lin, Tue, 13:37 Overlaps with C1-203462 --------------------------------------------- Was Agreed Needs revision, missing spec number on cover sheet Revision of C1-202425 |  |
|  |  |  | C1-203486 | Ethernet header compression for CP CIoT – 5GSM aspects |  |  | Qualcomm Incorporated / Amer | CR 2323 24.501 Rel-16 | Behrouz, Tue, 09:25 Some editorials Frederic, Tue, 12:09 Cover sheet issues: spec number and CR number missing Lin, Tue, 13:40 Not a new CR, so rev counter should be incremented Kaj, Tue, 15:47 - NW could control this, the UE shall include EHC IE at PDU session establishment if all conditions are fulfilled. Can be controlled by DN and/or slice if to enable compression or not. Also in-line with IP HC negotiation. - In that case, Ethernet header compression configuration IE should support that the core network could enable o disable ether header compression. Mahmoud, Tue, 21:09 We don’t see why Ethernet header compression at the 5GSM layer should be different from that of IPHC. Hence, we don’t support the proposal and we prefer to keep a consistent UE behavior for both IP and Ethernet HC. Yanchoa, Wed, 11:32 support |  |
|  |  |  | C1-203493 | Updates to non-allowed area restrictions |  |  | Ericsson, Samsung / Mikael | CR 2327 24.501 Rel-16 | Lin, Tue, 13:58 No agreement in SA2, SA2 has different proposals, should wait Mikael, Tue, 14:07 Agrees that this needs to be aligned with SA2, let´s keep this on hold for the time being and see how SA2 progresses |  |
|  |  |  | C1-203494 | Addition of MO parameter for allowing exception data in non-allowed area |  |  | Ericsson, Samsung | CR 0049 24.368 Rel-16 | Lin, Tue, 13:58 No agreement in SA2, SA2 has different proposals, should wait Mikael, Tue, 14:07 Agrees that this needs to be aligned with SA2, let´s keep this on hold for the time being and see how SA2 progresses Yanchao, Tue, 17:21 MO figures needs to be updated |  |
|  |  |  | C1-203511 | Adding DRX parameters for NB-IoT in the Registration procedure |  |  | SHARP | CR 2334 24.501 Rel-16 | Behrouz, Tue, 09:25 CR is NOT needed as the changes were already introduced in the last meeting by C1-202926 Lin, Tue, 14:18 Same as Behrouz Yanchao, Tue, 17:19 Is this overlapping with 2926 Yoko, Wed, 05:57 Rev, only difference form 2926 is left Behourz, Thu, 01:35 Not convinced Mikael, Thu, 09:29 Same as Behrouze |  |
|  |  |  | C1-203088 | Correct Service Gap Control |  |  | CATT | CR 3373 24.301 Rel-16 |  |  |
|  |  |  | C1-203536 | Add missing case check for CPSR in 5GMM-CONNECTED mode |  |  | Samsung/Anikethan | CR 2346 24.501 Rel-16 | Lin, Tue, 14:43 CR in general correct, shold be F, styles are wrong Ani, Wed, 06:15 Provides a rev |  |
|  |  |  | C1-203662 | Sending the EPS bearer context status IE in TAU after mobility from N1 mode with local bearer deactivation |  |  | BEIJING SAMSUNG TELECOM R&D | CR 3405 24.301 Rel-16 | Lin, Wed, 09:51 Provides rewording |  |
|  |  |  | C1-203666 | Redirection of UE from N1 mode to S1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2372 24.501 Rel-16 | Yanchao, Tue, 17:10 Why do we need redirection in SR Mahmoud, Tue, 18:01 Explains to Yanchao Mikael, Wed, 07:24 Fine with general intention, SR is fine, but preferable to trigger registration initiation Amer, Wed, 07:30 we are OK with adding the SERVICE REQUEST to the redirection feature. We do not agree with adding deregistration procedure to the redirection feature without stage 2 agreement in place. Lin, Wed, 10:03 There is no stage-2, but fine to go on as this allows more flexibility. More changes needed Behrouz, Wed, 23:01 I am supportive of this CR (SR for redirection). Mikael, Thu, 09:28 Therefore I still prefer to leave deregistation un-touched and go for the UCU trigger alternative |  |
|  |  |  | C1-203668 | Redirection of UE from S1 mode to N1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 3406 24.301 Rel-16 | Lin, Wed, 10:06 Same as for 3666 Behrouz, Wed, 23:01 I am supportive of this CR (SR for redirection). |  |
|  |  |  | C1-203672 | PDU session transfer between 3GPP and non-3GPP when CP CIoT 5GS optimization is being used |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2376 24.501 Rel-16 | Lin, Wed, 10:14 Fine in general, comments Yanchao, Wed, 11:49 Concur with Lin Kaj, Thu, 10:39 Same comment as Lin 2nd |  |
|  |  |  | C1-203673 | PDU session transfer between 3GPP and non-3GPP when UP CIoT 5GS optimization is being used |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2377 24.501 Rel-16 | Lin, Wed, 10:21 No problem, some comments Kaj, Thu, 10:42 This CR is dependent on another CR from you in C1-203516 which I have partly questioned and that impacts this CR. This proposal cannot be agreed as is until issues with C1-203516 is sorted out. |  |
|  |  |  | C1-203692 | Correction on CIoT small data container IE |  |  | Huawei, HiSilicon/Lin | CR 2379 24.501 Rel-16 |  |  |
|  |  |  | C1-203693 | Maximum length of Unstructured data via the control plane |  |  | Huawei, HiSilicon/Lin | CR 2380 24.501 Rel-16 |  |  |
|  |  |  | C1-203694 | Missing LCS/LPP container content in Payload container IE |  |  | Huawei, HiSilicon/Lin | CR 2381 24.501 Rel-16 |  |  |
|  |  |  | C1-203777 | Establishment of UP resources for NB-IoT based on number of supported DRBs |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2337 24.501 Rel-16 | Revision of C1-203516 Yanchao, Wed, 11:39 fine _________________________________________ Lin, Tue 14:30 Some rewording, wants to co-sign Yanchao, Tue, 17:17 New bullet b) has an error John-Luc, Tue, 19:07 Some comments, wants to co-sign Kaj, Thu, 10:11 I have still concerns with why the network have to know about this UE limitation. - Comment made under the subject line “…revised to….” |  |
|  |  |  | C1-203783 | Multiple DRB support for UEs in NB-N1 mode |  |  | Samsung, Huawei, HiSilicon, InterDigital | CR 2336 24.501 Rel-16 | Revision of C1-203515 _________________________________________ Frederic, Tue, 10:22 Missing clauses affected Kaj, Tue, 16:03 Wants to understand the problem, consequences if not approved is not good enough Mahmoud, Tue, 22:36 Offers an update of the cover page to Kaj Kaj, Thu, 10:06 Arguing against - Comment made under the subject line “…revised to….” |  |
|  |  |  | C1-203784 | IP header compression after inter-system change from S1 mode to N1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2342 24.501 Rel-16 | Revision of C1-203526 _________________________________________ Kaj, Tue, 16:22 Fine with the CR but should be more clear that it only applies for single registration with N26. Mahmoud, Thu, 01:57 Agrees, provides rev Behourz, Thu, 06:16 Co-sign Kaj, Thu, 10:16 FINE |  |
|  |  |  | C1-203785 | IP header compression after inter-system change from N1 mode to S1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 3403 24.301 Rel-16 | Revision of C1-203529 _________________________________________ Lin, Tue, 14:33 Fine some rewording Kaj, Tue, 16:24 - Fine with the CR but should be more clear if added that this only applies for single registration with N26. Mahmoud, Thu, 0218 Provides rev Behourz, Thu, 06:16 Co-sign Kaj, Thu, 10:19 Co-sign |  |
|  |  |  | C1-203786 | No dedicated EPS bearer for interworking from WB-N1 to NB-S1 mode |  |  | BEIJING SAMSUNG TELECOM R&D | CR 2369 24.501 Rel-16 | Revision of C1-203661 _________________________________________ Lin, Tue, 14:48 Fine in principle, one case seems missing Amer, Wed, 07:13 Condition for bullet 1 and bullet 2 are the same Mahmoud, Wed, Provides a rev |  |
|  | 5WWC |  |  | Peter – Main |  |  |  |  | CT aspects on wireless and wireline convergence for the 5G system architecture |  |
|  |  |  | C1-202168 | ANDSP is not supported by 5G-RG and W-AGF |  |  | ZTE / Joy | CR 2055 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202694 | Secondary authentication and W-AGF acting on behalf of N5GC |  |  | Ericsson / Ivo | CR 2028 24.501 Rel-16 | Agreed Revision of C1-202018 |  |
|  |  |  | C1-202653 | Error type on failure of reserving QoS resources over non-3GPP access |  |  | ZTE / Joy | CR 0126 24.502 Rel-16 | Agreed Revision of C1-202293 |  |
|  |  |  | C1-202612 | Inclusion of requested NSSAI in AN parameters |  |  | Motorola Mobility, Lenovo | CR 0122 24.502 Rel-16 | Agreed Revision of C1-202284 |  |
|  |  |  | C1-202636 | Removal of editor’s notes |  |  | Motorola Mobility, Lenovo | CR 0123 24.502 Rel-16 | Agreed Revision of C1-202290 |  |
|  |  |  | C1-203064 | void |  |  | void | CR 2217 24.501 Rel-16 | Withdrawn |  |
|  |  |  | C1-203065 | Primary authentication of an N5GC device |  |  | Ericsson / Ivo | CR 2218 24.501 Rel-16 | Roozbeh, Tue, 1818 list of comments Ivo, Wed, 10:57 Provides rev Roozbeh, Wed, 21:57 Comments Ivo, Thu, 00:41 Offers new wording Roozbeh, Thu, 02:19 Fine with rev Ivo, Thu, 08:54 Updated rev |  |
|  |  |  | C1-203066 | Stop of enforcement of wireline access service area restrictions and forbidden wireline access area |  |  | Ericsson / Ivo | CR 2219 24.501 Rel-16 |  |  |
|  |  |  | C1-203068 | IPv6 configuration for W-AGF acting on behalf of FN-RG |  |  | Ericsson / Ivo | CR 2220 24.501 Rel-16 | Roozbeh, Tue, 18:40 Coments Ivo, Wed, 09:04 Explaining, rev Sunghoon, Wed, 14:50 First change not correct Ivo, Wed, 21:06 Provides a rev |  |
|  |  |  | C1-203222 | Removal of requirement to transfer emergency PDU session first when transferring PDU sessions from non-3GPP access connected to the 5GC, to EPS |  |  | Qualcomm Incorporated / Lena | CR 2240 24.501 Rel-16 | Ivo, Tue, 09:25 transferring the emergency PDU session first seems reasonable stage-3 handling as some networks support only limited amount of PDN connections and emergency PDU session should be one of those transferred ones John-Luc, Tue, 23.49 Long explanation, CR is not needed |  |
|  |  |  | C1-203446 | Work plan for the CT1 part of 5WWC |  |  | Huawei, HiSilicon /Christian | discussion Rel-16 |  |  |
|  |  |  | C1-203449 | Access network parameters |  |  | Motorola Mobility, Lenovo | CR 0134 24.502 Rel-16 | Ivo, Tue, 09:25 why do we need two different AN-parameters? it should be sufficient to have only one AN-parameter based on 5GS mobile identity IE and distinguish 5G-GUTI or SUCI based on type of identity field of the 5GS mobile identity Roozbeh, Wed, 04:45 Provides rev Joy, Wed, 08:49 Why is AN parameter type completely rewritten? Ivo, Wed, 23:40 Co-sign Roozbeh, Thu, 00:42 Rev Joy, Thu, 05:16 There are too many changes |  |
|  |  |  | C1-203451 | Correction of TNGF procedure |  |  | Motorola Mobility, Lenovo | CR 0135 24.502 Rel-16 | Ivo, Tue, 09:25 - 7.3A.2.3 last sentence - the link layer protocol is terminated in TNAP (not TNFG). Please remove "towards the TNGF". - TNGF IPv4 contact info and TNGF IPv6 contact info need to be removed from Table 9.3.2.2.3-3, Roozbeh, Thu, 00:38 Provides rev |  |
|  |  |  | C1-203454 | Correcting reference |  |  | Motorola Mobility, Lenovo | CR 0136 24.502 Rel-16 | Ivo, Tue, 09:25 details of N3IWF - AMF interface are out of scope of TS 24.502 Roozbeh, Wed, 02:39 Asking for clarification Ivo, Wed, 23:45 Explains his comment Roozbeh, Wed, 23:50 explaining |  |
|  |  |  | C1-203460 | N5CW device registration and IP assignment |  |  | Motorola Mobility, Lenovo | CR 0140 24.502 Rel-16 | Ivo, Tue, 09:25 - details of N3IWF - AMF interface are out of scope of TS 24.502 - 5GMM and 5GSM messages are out of scope of TS 24.502, if needed then 24.501 Roozbeh, Wed, 02:23 Asking for clarification from Ivo Sunghoon, Wed, 08:15 CAT B, need rewording Roozbeh, Wed, 19:40 This is CAT F, long explanation, accepts that some parts need to go to 24.501 Provides rev Ivo, Wed, 23:56 Clarifying his comment |  |
|  |  |  | C1-203468 | Resolution of editor's note under clause 7.3A.4.2 |  |  | Huawei, HiSilicon /Christian | CR 0142 24.502 Rel-16 | Roozbeh, Tue, 18:54 support |  |
|  |  |  | C1-203479 | Removal of TMBR |  |  | Huawei, HiSilicon /Christian | CR 2321 24.501 Rel-16 |  |  |
|  |  |  | C1-203730 | N5GC NAS aspects |  |  | Nokia, Nokia Shanghai Bell, Charter Communications, CableLabs | CR 2390 24.501 Rel-16 | Ivo, Tue, 09:25 - 5.3.2 - new statement contradicts the existing statement as "the MAC address usage restriction indication set to "no restrictions". " implies PEI of "MAC address" type of identity while the new sentences implies PEI of "EUI-64" type of identity - 5.4.2.3 - superfluous "or" at the end of bullet a) Roozbeh, Tue, 18:56 Comments |  |
|  |  |  | C1-203731 | N5GC service area restrictions |  |  | Nokia, Nokia Shanghai Bell, CableLabs | CR 2391 24.501 Rel-16 | Roozbeh, tue, 18:58 CR is fine, but why parenthesis |  |
|  |  |  | C1-203732 | SUPI/SUCI of N5GC devices |  |  | Nokia, Nokia Shanghai Bell, Charter Communications, CableLabs | CR 0143 24.502 Rel-16 | Roozbeh, Wed, 15:17 commenting |  |
|  |  |  | C1-203733 | Adding Multicast support for 5G-RG and 5GLAN |  |  | Nokia, Nokia Shanghai Bell, Charter Communications, CableLabs | CR 2392 24.501 Rel-16 | Ivo, Tue, 09:25 - there is no description of what the IPTV service *is*, except that the IPTV service uses IP multicast which is based on usage of user data packets (and not NAS) and thus not in scope of 24.501. - STB does not seem to use NAS procedures and thus is not in scope of 24.501 - most of the procedures are not related to NAS (IP multicast messages are IP based and thus are sent as user plane packets) and thus are not in scope of 24.501 - Establishment of " a PDU session of IPv4, IPv6 or IPv4v6 PDU session type with the corresponding DNN and S-NSSAI" for "IP multicast service" is the same as for "IP unicast service", so this is already captured in PDU session establishment with "IPv4", "IPv6" or "IPv4v6" PDU session type. No need to add details on IP unicast, multicast, or anycast, as those are details of IP. Roozbeh, Tue, 19:02 No mandatory wording in clause 4, several comments |  |
|  |  |  | C1-203734 | RG SUCI generation |  |  | Nokia, Nokia Shanghai Bell | CR 2393 24.501 Rel-16 | Ivo, Tue, 09:25 CR seems not needed Roozbeh, Tue, 19:04 NOTE in 33.501, now a may in 24.501 -> is this corrected? |  |
|  |  |  | C1-203735 | URSP fix for RGs |  |  | Nokia, Nokia Shanghai Bell | CR 0082 24.526 Rel-16 | Ivo, Tue, 09:24 Justification of the CR is incorrect Roozbeh, Tue, 19:05 New editorials |  |
|  | PARLOS |  |  | Lena – Breakout |  |  |  |  | CT aspects of System enhancements for Provision of Access to Restricted Local Operator Services by Unauthenticated UEs 100% |  |
|  |  |  | C1-202601 | Miscellaneous editorial corrections |  |  | Samsung Electronics Polska | CR 3340 24.301 Rel-16 | Agreed Revision of C1-202126 |  |
|  |  |  | C1-202879 | Clarify UE behaviour for reject cause #9 and #10 received when attached for RLOS |  |  | Samsung/Anikethan | CR 3342 24.301 Rel-16 | Agreed Revision of C1-202147 |  |
|  |  |  | C1-203373 | Correction to implementation of CR #3338 |  |  | MediaTek Inc. / Marko | CR 3386 24.301 Rel-16 |  |  |
|  |  |  | C1-203376 | Correction to paging timer stop in case of RLOS |  |  | MediaTek Inc. / Marko | CR 3388 24.301 Rel-16 |  |  |
|  |  |  | C1-203394 | Clarification in Initiation of Location Registration for periodic timer expiry in 5U2 NOT UPDATED |  |  | MediaTek Inc. / Marko | CR 0543 23.122 Rel-16 |  |  |
|  | 5G_eLCS (CT4) |  |  | Peter – Main |  |  |  |  | CT aspects of Enhancement to the 5GC LoCation Services |  |
|  |  |  | C1-202548 | Adding Location Privacy Setting operation |  |  | CATT | CR 0001 24.571 Rel-16 | Agreed |  |
|  |  |  | C1-203125 | Resolve EN for Ciphering Key data IE regarding positioning SIBs |  |  | Qualcomm Incorporated / Sunghoon | CR 2229 24.501 Rel-16 | Lin, Thu, 11:01 comments |  |
|  |  |  | C1-203364 | Sending location services data in a SERVICE ACCEPT for MO Control Plane CIoT 5GS optimization |  |  | Qualcomm Incorporated | CR 2288 24.501 Rel-16 | Mikael, Tue, 13:35 So I propose to either update the CRs to follow existing principles in EPS and inform SA2 to align. Or to provide concerns and request SA2 to modify stage 2 as a first step. My preference is the first alternative as CT1 needs to take primary responsibility for the protocol design Scott, Tue, 18:22 Comments Sunghoon, Wed, 12:54 Answers to Scott Sunghoon, Wed, 13:22 Discussing with Mikael Mikael, Wed, 23:14 Discussing/explaining Scott, Thu, 09:21 Discussing with Sunghoon Lin, Thu, 10:30 Same as Mikael, this is not good, no work for CT1 Sunghoon, Thu, 17:38 Rev Sunghoon, Thu, 17:51 Will provide new rev |  |
|  |  |  | C1-203365 | UE initiated Event Reporting Procedure for Low Power Event Reporting |  |  | Qualcomm Incorporated | CR 0002 24.571 Rel-16 | Mikael, Tue, 13:35 So I propose to either update the CRs to follow existing principles in EPS and inform SA2 to align. Or to provide concerns and request SA2 to modify stage 2 as a first step. My preference is the first alternative as CT1 needs to take primary responsibility for the protocol design Scott, Tue, 18:26 Comments Sunghoon, Wed, 12:54 Answers to Scottt Scott, Wed, 18:51 Commenting Lin, Thu, 10:42 Why not update existing 5.2.2.4 |  |
|  |  |  | C1-203635 | New AT command supporting for 5G Location Services |  |  | CATT | CR 0694 27.007 Rel-16 | Frederic, Tue, 10:08 Typo on cover sheet Atle, Tue, 12:17 The intention of this new AT-command is not clear to me Sunghoon, Tue, 14:46 Validity time period should be added Scott, Thu, 03:07 Validity timer not needed Scott, Thu, 11:41 Explaining to Atle |  |
|  |  |  | C1-203636 | Removing the ENs for the enhancement to 5G Location Serivces |  |  | CATT | CR 0695 27.007 Rel-16 | Atel, Tue, 12:17 As the Editor’s Notes are from Rel-15, Move this CR to 15.1.3 (WI 5GS_Ph1-CT) and submit the CRs in Rel-15 with Rel-16 mirrors. Scott, Thu, 10:39 Acks Atle, |  |
|  | V2XAPP |  |  | Lena – Breakout |  |  |  |  | CT aspects of V2XAPP Is TS 24.486 sufficiently stable to be sent to CT#88 for approval |  |
|  |  |  | C1-203342 | Work plan for the CT1 part of V2XAPP |  |  | Huawei, HiSilicon /Christian | discussion Rel-16 |  |  |
|  |  |  | C1-203343 | Latest reference version of draft TS 24.486 |  |  | Huawei, HiSilicon /Christian | draft TS 24.486 Rel-16 |  |  |
|  |  |  | C1-203345 | Introduction of commands for VAE layer configuration clause |  |  | Huawei, HiSilicon /Christian | CR 0690 27.007 Rel-16 |  |  |
|  |  |  | C1-203347 | Introduction of +CVAECFG; AT command for VAE layer configuration |  |  | Huawei, HiSilicon /Christian | CR 0691 27.007 Rel-16 |  |  |
|  |  |  | C1-203348 | Introduction of +CVAEREG; AT command for VAE layer registration |  |  | Huawei, HiSilicon /Christian | CR 0692 27.007 Rel-16 |  |  |
|  |  |  | C1-203349 | Network monitoring procedure; V2X UE subscription for network monitoring information |  |  | Huawei, HiSilicon /Christian | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203350 | Structure and data semantics for V2X UE subscription for network monitoring information |  |  | Huawei, HiSilicon /Christian | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203448 | Miscellaneous corrections |  |  | Huawei, HiSilicon /Christian | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203450 | Correction to the application level location tracking procedure |  |  | Huawei, HiSilicon /Christian | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203452 | Resolution of the editor's note under clause 6.2.3 |  |  | Huawei, HiSilicon /Christian | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203568 | File distribution procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203569 | On-network dynamic group creation procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203570 | Structure and data semantics for on-network dynamic group creation procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203571 | On-network dynamic group notification procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203572 | Structure and data semantics for on-network dynamic group notification procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203573 | Notifications for network monitoring information procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203574 | Structure and data semantics for notifications for network monitoring information procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203575 | Update the root element of the VAE xml body |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203576 | XML scheme for V2X UE registration procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203577 | IANA registration template for VAE |  |  | Huawei, HiSilicon / Chen | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203621 | VAE specific extension of UE configuration |  |  | Samsung / Sapan | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203622 | Update to service-discovery-data element |  |  | Samsung / Sapan | pCR 24.486 Rel-16 |  |  |
|  |  |  | C1-203623 | Clarification on initial VAE server address |  |  | Samsung / Sapan | pCR 24.486 Rel-16 |  |  |
|  | eV2XARC |  |  | Lena – Breakout |  |  |  |  | CT aspects of eV2XARC |  |
|  |  |  | C1-202022 | Incorrect reference |  |  | Ericsson / Ivo | CR 0001 24.587 Rel-16 | Agreed |  |
|  |  |  | C1-202165 | Update to the V2X policies regarding RAN parameters |  |  | LG Electronics / SangMin | CR 0003 24.588 Rel-16 | Agreed |  |
|  |  |  | C1-202438 | Resolution of editor's note under 5.2.3 |  |  | Huawei, HiSilicon /Christian | CR 0031 24.587 Rel-16 | Agreed |  |
|  |  |  | C1-202439 | Resolution of editor's note under 6.1.2.5.2 |  |  | Huawei, HiSilicon /Christian | CR 0032 24.587 Rel-16 | Agreed |  |
|  |  |  | C1-202453 | Miscellaneous corrections |  |  | Huawei, HiSilicon /Christian | CR 0033 24.587 Rel-16 | Agreed |  |
|  |  |  | C1-202639 | Add the missing figure for UE-requested V2X policy provisioning procedure |  |  | OPPO / Rae | CR 0007 24.587 Rel-16 | Agreed Revision of C1-202115 |  |
|  |  |  | C1-202704 | Non-standadized QoS characteristics over PC5-S |  |  | OPPO / Rae | CR 0009 24.587 Rel-16 | Agreed Revision of C1-202117 |  |
|  |  |  | C1-202731 | Correction for the IP address configuration IE in the DIRECT LINK ESTABLISHMENT ACCEPT message |  |  | Huawei, HiSilicon / Chen | CR 0026 24.587 Rel-16 | Agreed Revision of C1-202317 . |  |
|  |  |  | C1-202732 | Correction for the link local IPv6 address IE in the DIRECT LINK ESTABLISHMENT ACCEPT message |  |  | Huawei, HiSilicon / Chen | CR 0027 24.587 Rel-16 | Agreed Revision of C1-202318 |  |
|  |  |  | C1-202739 | Handling of link modification accept |  |  | vivo | CR 0014 24.587 Rel-16 | Agreed Revision of C1-202182 |  |
|  |  |  | C1-202741 | Updates to link release procedure |  |  | vivo | CR 0016 24.587 Rel-16 | Agreed Revision of C1-202184 |  |
|  |  |  | C1-202742 | Correction of the timers of link identifier update procedure |  |  | vivo | CR 0017 24.587 Rel-16 | Agreed Revision of C1-202185 |  |
|  |  |  | C1-202744 | Handling of link identifier update not accept |  |  | vivo | CR 0019 24.587 Rel-16 | Agreed Revision of C1-202187 |  |
|  |  |  | C1-202748 | Introducing V2X communications over NR PC5 in EPC |  |  | LG Electronics / SangMin | CR 0024 24.386 Rel-16 | Agreed Revision of C1-202160 |  |
|  |  |  | C1-202757 | Indicating support of V2X over NR-PC5 |  |  | LG Electronics / SangMin | CR 3344 24.301 Rel-16 | Agreed Revision of C1-202162 |  |
|  |  |  | C1-202758 | Clarifications on configuration parameters for the PC5 QoS profile |  |  | LG Electronics / SangMin | CR 0012 24.587 Rel-16 | Agreed Revision of C1-202163 |  |
|  |  |  | C1-202760 | Clarifications on the V2X policies regarding QoS |  |  | LG Electronics / SangMin | CR 0002 24.588 Rel-16 | Agreed Revision of C1-202164 |  |
|  |  |  | C1-202768 | Resolution of editor's note under 6.1.2.3.6 |  |  | Huawei, HiSilicon /Christian | CR 0034 24.587 Rel-16 | Agreed |  |
|  |  |  | C1-202769 | Resolution of the editor's note under 6.1.2.5.7.2 |  |  | Huawei, HiSilicon /Christian | CR 0035 24.587 Rel-16 | Agreed Revision of C1-202456 |  |
|  |  |  | C1-202780 | T3540 for service request for V2X communications |  |  | ZTE | CR 2111 24.501 Rel-16 | Agreed Revision of C1-202333 |  |
|  |  |  | C1-202842 | Correction on conditions to initiate a PC5 unciast link establishment procedure |  |  | Huawei, HiSilicon / Vishnu | CR 0036 24.587 Rel-16 | Agreed Revision of C1-202457 |  |
|  |  |  | C1-202844 | Packet filter for PC5 QoS flows |  |  | Huawei, HiSilicon / Vishnu | CR 0037 24.587 Rel-16 | Agreed Revision of C1-202485 |  |
|  |  |  | C1-202867 | Remove FFS on GFBR and MFBR for UL and DL |  |  | OPPO / Rae | CR 0010 24.587 Rel-16 | Agreed Revision of C1-202703 |  |
|  |  |  | C1-202908 | Handling of link establishment accept |  |  | vivo | CR 0013 24.587 Rel-16 | Agreed Revision of C1-202738 Revision of C1-202181 |  |
|  |  |  | C1-202913 | ENs resolving in modification pocedure |  |  | vivo | CR 0015 24.587 Rel-16 | Agreed Revision of C1-202909 Revision of C1-202898 |  |
|  |  |  | C1-202919 | Maximum number of NR PC5 unicast links for a UE |  |  | Huawei, HiSilicon / Vishnu | CR 0029 24.587 Rel-16 | Agreed Revision of C1-202848 Revision of C1-202427 |  |
|  |  |  | C1-203053 | Correction of configuration of PC5 RAT selection and Tx profiles |  |  | Ericsson / Ivo | CR 0039 24.587 Rel-16 |  |  |
|  |  |  | C1-203054 | Correction of coding of configuration of PC5 RAT selection and Tx profiles |  |  | Ericsson / Ivo | CR 0004 24.588 Rel-16 |  |  |
|  |  |  | C1-203055 | Correction of configuration of default mode of communication |  |  | Ericsson / Ivo | CR 0040 24.587 Rel-16 |  |  |
|  |  |  | C1-203056 | Correction of coding of configuration of default mode of communication |  |  | Ericsson / Ivo | CR 0005 24.588 Rel-16 |  |  |
|  |  |  | C1-203057 | Correction of PC5 RAT names |  |  | Ericsson / Ivo | CR 0041 24.587 Rel-16 |  |  |
|  |  |  | C1-203058 | Correction of coding of PC5 RAT names |  |  | Ericsson / Ivo | CR 0006 24.588 Rel-16 |  |  |
|  |  |  | C1-203059 | Correction of PC5 QoS mapping configuration |  |  | Ericsson / Ivo | CR 0042 24.587 Rel-16 |  |  |
|  |  |  | C1-203060 | Correction of coding of PC5 QoS mapping configuration |  |  | Ericsson / Ivo | CR 0007 24.588 Rel-16 |  |  |
|  |  |  | C1-203061 | Correction in coding of PC5 QoS profile |  |  | Ericsson / Ivo | CR 0008 24.588 Rel-16 |  |  |
|  |  |  | C1-203062 | Correction of served by E-UTRAN |  |  | Ericsson / Ivo | CR 0043 24.587 Rel-16 |  |  |
|  |  |  | C1-203063 | Correction of coding of validity timers |  |  | Ericsson / Ivo | CR 0009 24.588 Rel-16 |  |  |
|  |  |  | C1-203083 | Editor's note on security of V2X over Uu |  |  | Ericsson / Ivo | CR 0044 24.587 Rel-16 |  |  |
|  |  |  | C1-203084 | Editor's note on PDU session establishment for V2X over Uu |  |  | Ericsson / Ivo | CR 0045 24.587 Rel-16 |  |  |
|  |  |  | C1-203117 | NR PC5 unicast security policy provisioning |  |  | Qualcomm Incorporated, Ericsson | CR 0001 24.588 Rel-16 | Revision of C1-202106 |  |
|  |  |  | C1-203118 | Adding general subclause on security of PC5 signalling messages |  |  | Qualcomm Incorporated / Sunghoon | CR 0005 24.587 Rel-16 | Revision of C1-202877 --------------------------------------- Was Agreed Revision of C1-202108 |  |
|  |  |  | C1-203119 | NR PC5 unicast security policy provisioning |  |  | Qualcomm Incorporated / Sunghoon | CR 0003 24.587 Rel-16 | Revision of C1-202105 |  |
|  |  |  | C1-203120 | PC5 unicast link security establishment |  |  | Qualcomm Incorporated / Sunghoon | CR 0002 24.587 Rel-16 | Revision of C1-202875 ---------------------------------------- Was agreed Revision of C1-202104 |  |
|  |  |  | C1-203123 | Destination Layer 2 ID derivation from the group identifier |  |  | Qualcomm Incorporated / Sunghoon | CR 0046 24.587 Rel-16 |  |  |
|  |  |  | C1-203124 | PC5 unicast link re-keying procedure |  |  | Qualcomm Incorporated / Sunghoon | CR 0004 24.587 Rel-16 | Revision of C1-202876 ----------------------------------------------- Was agreed Revision of C1-202107 |  |
|  |  |  | C1-203127 | Additional transport over Uu for V2X messages of V2X services identified by V2X service identifiers |  |  | Ericsson / Ivo | CR 0023 24.386 Rel-16 | Revision of C1-202838 ---------------------------------- Was Agreed Revision of C1-202010 |  |
|  |  |  | C1-203128 | Configuration parameters for additional transport over Uu for V2X messages of V2X services identified by V2X service identifiers |  |  | Ericsson / Ivo | CR 0020 24.385 Rel-16 | Revision of C1-202839 ---------------------------------------- Was agreed Revision of C1-202011 |  |
|  |  |  | C1-203137 | On usage of MSB/LSB vs MSBs/LSBs |  |  | InterDigital Communications | discussion | Withdrawn |  |
|  |  |  | C1-203142 | Adding new definitions to 24.587 |  |  | InterDigital Communications | CR 0047 24.587 Rel-16 |  |  |
|  |  |  | C1-203217 | Modification of the Link Release procedure |  |  | InterDigital Communications | CR 0048 24.587 Rel-16 |  |  |
|  |  |  | C1-203218 | Inclusion of Target User Info |  |  | InterDigital Communications | CR 0049 24.587 Rel-16 |  |  |
|  |  |  | C1-203219 | Defining new parameters needed for the Link Identifier Update procedure |  |  | InterDigital Communications | CR 0028 24.587 Rel-16 | Revision of C1-202930 -------------------------------------- Was agreed Revision of C1-202870 |  |
|  |  |  | C1-203265 | Encoding of link modification reject message |  |  | vivo | CR 0050 24.587 Rel-16 |  |  |
|  |  |  | C1-203266 | Alignment of the name of cause#5 |  |  | vivo | CR 0051 24.587 Rel-16 |  |  |
|  |  |  | C1-203267 | Handling of link release procedure |  |  | vivo | CR 0052 24.587 Rel-16 |  |  |
|  |  |  | C1-203268 | Handling of PC5 unicast link ID update accept |  |  | vivo | CR 0053 24.587 Rel-16 |  |  |
|  |  |  | C1-203269 | Handling of communication mode |  |  | vivo | CR 0054 24.587 Rel-16 |  |  |
|  |  |  | C1-203270 | Handling of PC5 unicast QoS flow match and establishment |  |  | vivo, Huawei, HiSilicon, Ericsson | CR 0020 24.587 Rel-16 | Revision of C1-202745 ------------------------------------------ Was Agreed Revision of C1-202188 |  |
|  |  |  | C1-203271 | Handling of PC5 broadcast QoS flow match and establishment |  |  | vivo, Ericsson | CR 0021 24.587 Rel-16 | Revision of C1-202914 ----------------------------------------------- Was Agreed Revision of C1-202910 Revision of C1-202900 Revision of C1-202899 Revision of C1-202746 Revision of C1-202189 |  |
|  |  |  | C1-203272 | Encoding of link identifier update messages and parameters |  |  | vivo, InterDigital, CATT | CR 0018 24.587 Rel-16 | Revision of C1-202743 -------------------------------------- Was agreed Revision of C1-202186 |  |
|  |  |  | C1-203273 | Destination L2 ID for groupcast |  |  | vivo | CR 0055 24.587 Rel-16 |  |  |
|  |  |  | C1-203290 | Correction to the privacy timer |  |  | Huawei, HiSilicon /Christian | CR 0024 24.587 Rel-16 | Revision of C1-202767 -------------------------------- Was agreed Revision of C1-202226 |  |
|  |  |  | C1-203291 | Timer values for timers of the PC5 unicast link management procedures |  |  | Huawei, HiSilicon /Christian | CR 0023 24.587 Rel-16 | Revision of C1-202773 -------------------------------------- Was agreed Revision of C1-202598 Revision of C1-202225 |  |
|  |  |  | C1-203295 | Single privacy timer of L2ID for unicast |  |  | ASUSTeK | CR 0056 24.587 Rel-16 |  |  |
|  |  |  | C1-203296 | Definition of UE aborting the PC5 unicast link identifier update procedure |  |  | ASUSTeK | CR 0057 24.587 Rel-16 |  |  |
|  |  |  | C1-203297 | Definition of UE aborting the PC5 unicast link modification procedure |  |  | ASUSTeK | CR 0058 24.587 Rel-16 |  |  |
|  |  |  | C1-203298 | Corrections to unicast mode communication |  |  | ASUSTeK | CR 0059 24.587 Rel-16 |  |  |
|  |  |  | C1-203326 | Group size and menber ID from application layer for groupcast |  |  | OPPO / Rae | CR 0011 24.587 Rel-16 | Revision of C1-202708 ------------------------------------ Was agreed Revision of C1-202119 |  |
|  |  |  | C1-203327 | Change the term service authorisation provisioning |  |  | OPPO / Rae | CR 0060 24.587 Rel-16 |  |  |
|  |  |  | C1-203328 | Abnormal case of link release including Knrp ID |  |  | OPPO / Rae | CR 0061 24.587 Rel-16 |  |  |
|  |  |  | C1-203329 | Remove IP address for privacy timer |  |  | OPPO / Rae | CR 0010 24.588 Rel-16 |  |  |
|  |  |  | C1-203402 | On the usage of MSB/LSB vs. MSBs/LSBs |  |  | InterDigital Communications | discussion Rel-16 |  |  |
|  |  |  | C1-203447 | Work plan for the CT1 part of eV2XARC |  |  | Huawei, HiSilicon /Christian | discussion Rel-16 |  |  |
|  |  |  | C1-203453 | Clarification on the relation between a pair of Layer-2 IDs and a PC5 unicast link |  |  | Huawei, HiSilicon / Vishnu | CR 0062 24.587 Rel-16 |  |  |
|  |  |  | C1-203457 | Addition of function for converting the group identifier to the destination Layer-2 ID |  |  | Huawei, HiSilicon / Vishnu | CR 0063 24.587 Rel-16 |  |  |
|  |  |  | C1-203480 | Upates to link modification procedure |  |  | Huawei, HiSilicon / Vishnu | CR 0064 24.587 Rel-16 |  |  |
|  |  |  | C1-203481 | Updates to NR PC5 unicast link release procedure |  |  | Huawei, HiSilicon / Vishnu | CR 0065 24.587 Rel-16 |  |  |
|  |  |  | C1-203539 | DDF update for V2X over NR-PC5 |  |  | LG Electonics, Huawei, HiSilicon / SangMin | CR 0022 24.385 Rel-16 |  |  |
|  |  |  | C1-203540 | Mapping between V2X Service ID and PFI for a PC5 unicast link establishment |  |  | LG Electronics / SangMin | CR 0066 24.587 Rel-16 |  |  |
|  |  |  | C1-203541 | Updating PC5 unicast link modification procedure |  |  | LG Electronics / SangMin | CR 0067 24.587 Rel-16 |  |  |
|  |  |  | C1-203542 | V2X MO update for V2X over NR PC5 |  |  | LG Electronics, Huawei, HiSilicon / SangMin | CR 0021 24.385 Rel-16 | Revision of C1-202756 ------------------------------------------ Was agreed Revision of C1-202161 |  |
|  |  |  | C1-203554 | Resolution of the editor's note on exact semantic and length of validity timer field |  |  | Huawei, HiSilicon /Christian | CR 0011 24.588 Rel-16 |  |  |
|  |  |  | C1-203578 | Adding the missing clause affected in the cover sheet |  |  | Huawei, HiSilicon / Chen | CR 0025 24.587 Rel-16 | Revision of C1-202730 ---------------------------------------- Was agreed Needs revision, missing clauses afftected Revision of C1-202316 |  |
|  |  |  | C1-203634 | Adding the new V2X message family |  |  | CATT | CR 0068 24.587 Rel-16 | Sunghoon, Tue, 14:28 QCOM wants to co-sign |  |
|  |  |  | C1-203748 | security handling |  |  | Samsung/Grace | CR 0012 24.588 Rel-16 | Revision of C1-203744 |  |
|  | RACS (CT4 lead) |  |  | Peter – Main |  |  |  |  | CT aspects of optimizations on UE radio capability signalling 100% |  |
|  |  |  | C1-202693 | RACS parameters in generic UE configuration procedure |  |  | Ericsson / Mikael | CR 2078 24.501 Rel-16 | Agreed Revision of C1-202233 |  |
|  |  |  | C1-203223 | Discussion on registration/TAU procedures to signal UE radio capability ID triggered by move to ePLMN |  |  | Qualcomm Incorporated / Lena | discussion Rel-16 | Sung, Thu, 17:58 if QC does not consider the solution is not good enough, it should have been raised back then. It is not good to provide solutions from multiple WGs to the same problem. |  |
|  |  |  | C1-203224 | Avoiding too frequent registration procedures due to signalling of UE radio capability ID |  |  | Qualcomm Incorporated / Lena | CR 2241 24.501 Rel-16 | Rae, Thu, 05:41 Alternative proposal |  |
|  |  |  | C1-203225 | Avoiding too frequent tracking area updating procedures due to signalling of UE radio capability ID |  |  | Qualcomm Incorporated / Lena | CR 3374 24.301 Rel-16 |  |  |
|  |  |  | C1-203495 | Correction of RACS ID deletion via UCU |  |  | Ericsson / Mikael | CR 2328 24.501 Rel-16 | Frederic, Tue, 10:15 Clauses affected missing Lena, Wed, 03:41 Editorials Yanchao, Wed, 10:43 editorials |  |
|  |  |  | C1-203708 | Correction on UE radio capability ID availability IE name |  |  | Huawei, HiSilicon/Lin | CR 3411 24.301 Rel-16 |  |  |
|  | 5G_SRVCC (CT4 lead) |  |  | Peter – Main |  |  |  |  | CT aspects of single radio voice continuity from 5GS to 3G 100% |  |
|  |  |  | C1-202638 | Initial Registration after 5G-SRVCC |  |  | ZTE, China Unicom | CR 2115 24.501 Rel-16 | Agreed Revision of C1-202529 |  |
|  | xBDT (CT3 lead) |  |  | Peter – Main |  |  |  |  | CT aspects on 5GS Transfer of Policies for Background Data 100% |  |
|  | IAB-CT (CT4 lead) |  |  | Peter – Main |  |  |  |  | CT aspects of support for integrated access and backhaul (IAB) CT1 no longer affected by this work item |  |
|  |  |  | C1-203226 | Unified access control is not applicable to a UE operating as IAB-node |  |  | Qualcomm Incorporated / Lena | CR 2242 24.501 Rel-16 | Lena, Thu, 00:19 No need to wait with this CR for RAN2, it is fully aligned Sung, Thu, 01:07 No problem with progressing this CR Comments SangMin, Thu, 11:33 Issue with 3226 and 3512, undefined terminology |  |
|  |  |  | C1-203512 | UAC and IAB-MT |  |  | Nokia, Nokia Shanghai Bell | CR 2335 24.501 Rel-16 | Lena, Wed, 03:45 Overlaps with 3226, wait for RAN2 conclusion Sung, Wed, 03:55 Resume 3512 and 3226 SangMin, Thu, 11:33 Issue with 3226 and 3512, undefined terminology |  |
|  | 5GS_OTAF (CT4 lead) |  |  | Peter – Main |  |  |  |  | 5GS Enhanced support of OTA mechanism for UICC configuration parameter update |  |
|  |  |  | C1-203557 | SP-AF services |  |  | Nokia, Nokia Shanghai Bell | CR 0554 23.122 Rel-16 | Ban, Wed, 10.11 Additional change needed Sung, Wed, 21:06 Additional change from Ban already covered in CR from last meeting Ban, Thu, 05:43 fine |  |
|  | 5G_URLLC (CT4 lead) |  |  | Peter – Main |  |  |  |  | CT aspects of CT Aspects of 5G URLLC |  |
|  | SEAL |  |  | Lena – Breakout |  |  |  |  | CT aspects of Service Enabler Architecture Layer for Verticals Is TS 24.548 sufficiently stable to be sent to CT#88 for approval? |  |
|  |  |  | C1-202137 | Updates to User Authentication Client (SIM-C) procedure |  |  | Intel / Vivek | CR 0001 24.547 Rel-16 | Agreed |  |
|  |  |  | C1-202138 | Updates to User Authentication Server (SIM-S) procedure |  |  | Intel / Vivek | CR 0002 24.547 Rel-16 | Agreed |  |
|  |  |  | C1-202319 | IANA registration template of SEAL location management |  |  | Huawei, HiSilicon / Chen | CR 0001 24.545 Rel-16 | Agreed |  |
|  |  |  | C1-202320 | Removal of editor’s note on MIME types |  |  | Huawei, HiSilicon / Chen | CR 0002 24.545 Rel-16 | Agreed |  |
|  |  |  | C1-202321 | Resolution of editor's note on application unique ID |  |  | Huawei, HiSilicon / Chen | CR 0003 24.545 Rel-16 | Agreed |  |
|  |  |  | C1-202322 | Structure and data semantics for query list of users based on location procedure |  |  | Huawei, HiSilicon / Chen | CR 0004 24.545 Rel-16 | Agreed |  |
|  |  |  | C1-202447 | SIP based subscribe/notify procedures for SEAL group management |  |  | Samsung / Sapan | CR 0001 24.544 Rel-16 | Agreed |  |
|  |  |  | C1-202449 | Indication from SGM-S to SGM-C about group join required |  |  | Samsung / Sapan | CR 0003 24.544 Rel-16 | Agreed |  |
|  |  |  | C1-202450 | SIP based subscribe/notify procedures for configuration management |  |  | Samsung / Sapan | CR 0001 24.546 Rel-16 | Agreed |  |
|  |  |  | C1-202809 | Removal of Editor’s notes |  |  | Samsung / Sapan | CR 0002 24.544 Rel-16 | Agreed Revision of C1-202448 |  |
|  |  |  | C1-202810 | Removal of Editor’s notes. |  |  | Samsung / Sapan | CR 0002 24.546 Rel-16 | Agreed Revision of C1-202451 |  |
|  |  |  | C1-203435 | Latest reference version of draft TS 24.548 |  |  | Huawei, HiSilicon /Christian | draft TS 24.548 Rel-16 |  |  |
|  |  |  | C1-203444 | Correction of references |  |  | Huawei, HiSilicon /Christian | CR 0013 24.545 Rel-16 |  |  |
|  |  |  | C1-203465 | Updates to Token Exchange Client (SIM-C) procedure |  |  | Intel / Vivek | CR 0003 24.547 Rel-16 | Revision of C1-202828 ------------------------------------- Was Agreed Revision of C1-202139 |  |
|  |  |  | C1-203467 | Updates to Token Exchange Server (SIM-S) procedure |  |  | Intel / Vivek | CR 0004 24.547 Rel-16 | Revision of C1-202829 -------------------------------------- Was Agreed Revision of C1-202140 |  |
|  |  |  | C1-203558 | Removal of the SIP based procedure for MBMS bearer announcement over MBMS bearer procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203559 | Removal of the SIP based procedure for MBMS bearer quality detection procedure |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203560 | IANA registration template for VALInfo of SEAL network resourcement management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203561 | IANA registration template for UnicastInfo of SEAL network resourcement management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203562 | IANA registration template for MBMSInfo of SEAL network resourcement management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203563 | XML schema of VALInfo for SEAL network resource management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203564 | XML schema of UnicastInfo for SEAL network resource management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203565 | XML schema of MBMSInfo for SEAL network resource management |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203566 | Wrong implementation under request for modification of unicast resources procedure with SIP core |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203567 | Miscellaneous editorial corrections |  |  | Huawei, HiSilicon / Chen | pCR 24.548 Rel-16 |  |  |
|  |  |  | C1-203579 | Resolution of the editor’s note on access token |  |  | Huawei, HiSilicon / Chen | CR 0014 24.545 Rel-16 |  |  |
|  |  |  | C1-203580 | XML scheme for SEAL location management |  |  | Huawei, HiSilicon / Chen | CR 0005 24.545 Rel-16 | Revision of C1-202733 ------------------------------------------------- Was agreed Needs revision, rev counter should be 1 Revision of C1-202323 |  |
|  |  |  | C1-203581 | Updates to SIP based procedure for location information subscription procedure |  |  | Huawei, HiSilicon / Chen | CR 0015 24.545 Rel-16 | Competes with C1-203624 |  |
|  |  |  | C1-203615 | Corrections in HTTP request URI |  |  | Samsung / Sapan | CR 0004 24.544 Rel-16 |  |  |
|  |  |  | C1-203616 | IANA registration for SEAL group document |  |  | Samsung / Sapan | CR 0005 24.544 Rel-16 |  |  |
|  |  |  | C1-203617 | Adding VAL user id in subscription parameter |  |  | Samsung / Sapan | CR 0006 24.544 Rel-16 |  |  |
|  |  |  | C1-203618 | Corrections in HTTP request URI |  |  | Samsung / Sapan | CR 0003 24.546 Rel-16 |  |  |
|  |  |  | C1-203619 | IANA registration for VAL user profile and UE configuration document |  |  | Samsung / Sapan | CR 0004 24.546 Rel-16 |  |  |
|  |  |  | C1-203620 | Using proper element names in VAL UE Configuration |  |  | Samsung / Sapan | CR 0005 24.546 Rel-16 |  |  |
|  |  |  | C1-203624 | SIP based subscription procedures |  |  | Samsung / Sapan | CR 0016 24.545 Rel-16 | Competes with C1-203581 |  |
|  |  |  | C1-203625 | Adding required XML elements for subscription |  |  | Samsung / Sapan | CR 0017 24.545 Rel-16 |  |  |
|  |  |  | C1-203626 | Timers used in location management |  |  | Samsung / Sapan | CR 0018 24.545 Rel-16 |  |  |
|  | Other Rel-16 non-IMS issues |  |  | Peter – Main |  |  |  |  | Other Rel-16 non-IMS topics |  |
|  |  |  | C1-202083 | Correction of certain erroneous Information Element Identifiers |  |  | InterDigital Communications | CR 2033 24.501 Rel-16 | Agreed |  |
|  |  |  | C1-202148 | SMS timer extension for the MS using CP CioT 5GS optimization |  |  | NTT DOCOMO | CR 0066 24.011 Rel-16 | Agreed |  |
|  |  |  | C1-202273 | Remove invalid cases in error handling for TFT operation |  |  | Qualcomm Incorporated | CR 3214 24.008 Rel-16 | Agreed |  |
|  |  |  | C1-202274 | Remove invalid cases in error handling for TFT operation in EPS |  |  | Qualcomm Incorporated | CR 3350 24.301 Rel-16 | Agreed |  |
|  |  |  | C1-202467 | WUS assistance for TAU |  |  | Huawei, HiSilicon/Lin | CR 3356 24.301 Rel-16 | Agreed |  |
|  |  |  | C1-202512 | Correction to Handling of T3321 timer |  |  | MediaTek Inc. | CR 3217 24.008 Rel-16 | Agreed |  |
|  |  |  | C1-202700 | RPDU transfer for 5GS using Control Plane CioT Optimization |  |  | NTT DOCOMO INC. | CR 0067 24.011 Rel-16 | Agreed Revision of C1-202217 |  |
|  |  |  | C1-202781 | Clarification on the UE behaviour when receiving T3448 |  |  | ZTE | CR 3351 24.301 Rel-16 | Agreed Revision of C1-202334 |  |
|  |  |  | C1-202736 | Emergency PDN connection established after WUS negotiation |  |  | Vivo | CR 3345 24.301 Rel-16 | Agreed Revision of C1-202178 |  |
|  |  |  | C1-202691 | Correction to handling of T3447 timer |  |  | MediaTek Inc. | CR 3370 24.301 Rel-16 | Agreed Revision of C1-202520 |  |
|  |  |  | C1-202906 | Clarification for the use of enhanced coverage in EPS |  |  | Samsung, Huawei, HiSilicon, InterDigital | CR 3339 24.301 Rel-16 | Agreed Revision of C1-202645 |  |
|  |  |  | C1-202822 | New AT command for linking packet filters +CGLNKPF |  |  | MediaTek Inc. / JJ | CR 0687 27.007 Rel-16 | Agreed Revision of C1-202539 |  |
|  |  |  | C1-202804 | Correction on retry restriction for ESM#66 |  |  | Huawei, HiSilicon/Lin | CR 3363 24.301 Rel-16 | Agreed Revision of C1-202484 |  |
|  |  |  | C1-202798 | Retry restriction for NB-IoT UEs due to out of tariff package |  |  | Huawei, HiSilicon/Lin | CR 3357 24.301 Rel-16 | Agreed Revision of C1-202468 |  |
|  |  |  | C1-202797 | WUS assistance for emergency |  |  | Huawei, HiSilicon/Lin | CR 3355 24.301 Rel-16 | Agreed Revision of C1-202466 |  |
|  |  |  | C1-202823 | New AT command for deleting packet filters +CGDELPF |  |  | MediaTek Inc. / JJ | CR 0688 27.007 Rel-16 | Agreed Revision of C1-202540 |  |
|  |  |  | C1-202814 | handling of ePWS message |  |  | Samsung/ Kyungjoo Grace Suh | CR 0217 23.041 Rel-16 | Agreed Revision of C1-202563 Shifted from ePWS |  |
|  |  |  | C1-203107 | TA change during Authentication procedure in EMM-CONNECTED mode |  |  | Apple | CR 3347 24.301 Rel-16 | Revision of C1-202850 Osamah, Tue, 19:19 Cr has issues, offers possible way forward Lin, Thu, 08:59 fine |  |
|  |  |  | C1-203108 | TA change during Authentication procedure in 5GMM-CONNECTED mode |  |  | Apple | CR 2092 24.501 Rel-16 | Revision of C1-202851 Osamah, Tue, 19:19 Cr has issues, offers possible way forward Lin, Thu, 08:59 fine |  |
|  |  |  | C1-203129 | Segmentation in query port numbers procedure |  |  | Ericsson / Ivo | CR 0017 24.250 Rel-16 | Revision of C1-194182 Osama, Thu, 00:01 Comments Lin, Thu, 09:04 Comments Ivo, Thu, 10.10 explaining Ivo, Thu, 10:11 Further explaining Vivek, Thu, 10:20 Prefers different approach Ivo, Thu, 12:01 Discussing with Vivek |  |
|  |  |  | C1-203139 | Corecting the incorrect mode of the UE |  |  | InterDigital Communications | CR 2237 24.501 Rel-16 |  |  |
|  |  |  | C1-203232 | Forbidden PLMN list for emergency service |  |  | Apple | CR 3375 24.301 Rel-16 | Ivo, Tue, 09:33 why should the network reject emergency service while accepting non-emergency services Sunghoon, Tue, 13:45 currently no "forbidden PLMN list for emergency service" in 23.122 this should not be CAT F, if you want a new concept, then SA1 is needed first Ban, Tue, 14:55 Clarify the benefits Not keen on the concept, especiall for EPS Marko, Wed, 11:04 Change is not justified Chen, Wed, 11:49 OPPO too have concerns with introducing this new Forbidden PLMN list for emergency services. Reinhard, Wed ,12:07 Do not agree to introduce a new list |  |
|  |  |  | C1-203233 | Forbidden PLMN list for emergency service |  |  | Apple | CR 2243 24.501 Rel-16 | Ivo, Tue, 09:33 why should the network reject emergency service while accepting non-emergency services Sunghoon, Wed, 09:20 currently no "forbidden PLMN list for emergency service" in 23.122 this should not be CAT F, if you want a new concept, then SA1 is needed first Marko, Wed, 11:04 Change is not justified |  |
|  |  |  | C1-203234 | Forbidden PLMN list for emergency service |  |  | Apple | CR 0534 23.122 Rel-16 | Ivo, Tue, 09:33 why should the network reject emergency service while accepting non-emergency services Sunghoon, Wed, 09:18 I think SA1 should look at this to decide whether such a new list is useful as this is not in scope for CT1 to decide. Marko, Wed, 11:04 Change is not justified |  |
|  |  |  | C1-203304 | Correction to Handling of DNN based congestion control |  |  | MediaTek Inc. / Carlson | CR 2268 24.501 Rel-16 | Ivo, Tue, 09:33 New statement does not cover all cases Amer, Tue, 20:58 Stage-3 does not have to repeat everything form satge-2, however, not opposing Carlson, Wed, 06:37 Explaining the CR Ivo, Wed, 12:54 Does not agreed |  |
|  |  |  | C1-203314 | Correction to Handling of APN based congestion control |  |  | MediaTek Inc. / Carlson | CR 3376 24.301 Rel-16 | Ivo, Tue, 09:32 24.301 8.3.20.3 prevents inclusion of APN in PDN connectivity request for emergency PDU session Carlson, Thu, 04:26 Provides rev |  |
|  |  |  | C1-203372 | Correction to EMM-REGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 3385 24.301 Rel-16 |  |  |
|  |  |  | C1-203375 | Correction to handling of NAS level mobility management congestion control |  |  | MediaTek Inc. / Marko | CR 3387 24.301 Rel-16 |  |  |
|  |  |  | C1-203378 | Correction to handling of #3/#6/#7 |  |  | MediaTek Inc. / Marko | CR 3389 24.301 Rel-16 | Ani, Wed, 17:10 change is ok for cause #3 and #6. But for cause #7 the USIM is invalid only for a particular domain. Hence this sub-state would not hold good. Marko, Thu, 12:35 explaining |  |
|  |  |  | C1-203379 | Correction to handling of #3/#6/#7 |  |  | MediaTek Inc. / Marko | CR 3219 24.008 Rel-16 | Ani, Wed, 17:10 change is ok for cause #3 and #6. But for cause #7 the USIM is invalid only for a particular domain. Hence this sub-state would not hold good. |  |
|  |  |  | C1-203381 | Correction to handling of #9 |  |  | MediaTek Inc. / Marko | CR 3390 24.301 Rel-16 |  |  |
|  |  |  | C1-203382 | Correction to handling of #9 |  |  | MediaTek Inc. / Marko | CR 3220 24.008 Rel-16 |  |  |
|  |  |  | C1-203383 | Correction to handling of #12/#13/#15 in EMM SERVICE procedure |  |  | MediaTek Inc. / Marko | CR 3391 24.301 Rel-16 | Osama, Wed, 00:50 Referred text in 008 is gprs specific |  |
|  |  |  | C1-203384 | Correction to handling of paging in GMM-REG.ATTEMPTING-TO-UPDATE |  |  | MediaTek Inc. / Marko | CR 3221 24.008 Rel-16 | Osama, Wed, 00.50 Idea looks ok, some rewording |  |
|  |  |  | C1-203385 | Correction to handling of paging in MM IDLE ATTEMPTING TO UPDATE state |  |  | MediaTek Inc. / Marko | CR 3222 24.008 Rel-16 | Behrouz, Tue, 09:57 Asking for clarification Marko, Thu ,11:35 Explaining Behrouz, Thu, 16:13 Will not object |  |
|  |  |  | C1-203386 | Correction to Handling of #42 |  |  | MediaTek Inc. / Marko | CR 3392 24.301 Rel-16 | Osama, Wed, 00.50 Detailed comments |  |
|  |  |  | C1-203387 | Correction to handling of abnormal cases of Network initiated detach procedure |  |  | MediaTek Inc. / Marko | CR 3393 24.301 Rel-16 | Kaj, Tue, 09:32 Fine with the changes, update consequences if not approved Marko, Thu, 11:37 Acks Kaj |  |
|  |  |  | C1-203388 | Correction to Handling of paging in EMM-REGISTERED.ATTEMPTING-TO-UPDATE-MM |  |  | MediaTek Inc. / Marko | CR 3394 24.301 Rel-16 |  |  |
|  |  |  | C1-203389 | Correction to EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH |  |  | MediaTek Inc. / Marko | CR 3395 24.301 Rel-16 | Ani, Tue, 16:18 Cr is not needed Marko, Thu, 12:35 Explaining Ani, Thu, 15:11 Not agreeing |  |
|  |  |  | C1-203390 | Correction to GMM-DEREGISTERED.ATTEMPTING-TO-ATTACH |  |  | MediaTek Inc. / Marko | CR 3223 24.008 Rel-16 | Ani, Tue, 16:28 Proposed change is not correct |  |
|  |  |  | C1-203391 | Correction to EMM-DEREGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 3396 24.301 Rel-16 | Ivo, Tue, 09:34 Update does not reflect summary of changes, some parts missing Ani, Tue, 15:55 Fine with the first change, second change is not needed. Marko, Thu, 11:44 Explaining Ani, Thu, 15:33 Cr is not complete |  |
|  |  |  | C1-203392 | Correction to GMM-DEREGISTERED.NORMAL-SERVICE |  |  | MediaTek Inc. / Marko | CR 3224 24.008 Rel-16 | Ivo, Tue, 09:33 Updates does not reflect summary of changes, some parts are missing Ani, Tue, 16:07 Ok with first change, second change not needed |  |
|  |  |  | C1-203395 | Clarification of cause #35 in limited service state |  |  | MediaTek Inc. / Marko | CR 0544 23.122 Rel-16 | Osama, Wed, 00.50 Asking for clarification |  |
|  |  |  | C1-203401 | Correction to spelling mistakes |  |  | MediaTek Inc. / Marko | CR 3398 24.301 Rel-16 |  |  |
|  |  |  | C1-203463 | Add handling for parameter set to “value is not used” in EPS |  |  | Qualcomm Incorporated, Ericsson | CR 3348 24.301 Rel-16 | Revision of C1-202633 |  |
|  |  |  | C1-203464 | Allow lower layer to change RRC establishment cause during voice EPS fallback |  |  | Qualcomm Incorporated, Ericsson | CR 3316 24.301 Rel-16 | Revision of C1-202830 Lin, Tue, 12:12 Wants an update of the cover page Osama, Tue, 16:40 Does not agree with Lin Marko, Wed, 11:42 issues Osama, Wed, 16:10 Defending against Marko |  |
|  |  |  | C1-203590 | Reset of PLMN-specific attempt counter |  |  | MediaTek Inc. / Marko | CR 3364 24.301 Rel-16 | Revision of C1-202685 --------------------------------------------- Was agreed Revision of C1-202511 |  |
|  |  |  | C1-203591 | Correction to Handling of T3421 timer |  |  | MediaTek Inc. / Marko | CR 3365 24.301 Rel-16 | Revision of C1-202686 ---------------------------------------- Was agreed Needs revision, missing clauses affted Revision of C1-202513 |  |
|  |  |  | C1-203670 | QoE measurement control |  |  | Ericsson /Jörgen | CR 0696 27.007 Rel-16 | Lena, Wed, 03:50 Requests in RAN2/RAN3 to postpone this to Rel17, CT1 should not agree to this CR in Rel-16 if the decision in RAN2/RAN3 is to postpone this to Rel-17. Atle, Wed, 12:19 Many comments |  |
|  |  |  | C1-203695 | EN resolution on WUS |  |  | Huawei, HiSilicon/Lin | CR 3407 24.301 Rel-16 |  |  |
|  |  |  | C1-203711 | Geo-fencing check for no stored "warning message" matched |  |  | Huawei, HiSilicon/Lin | CR 0220 23.041 Rel-16 | PeterSanders, Tue, 10:23 Cover page: It is not the modem layer where this happens but it happens at the application layer. Some editorials Lena, Wed, 03:57 Fnctional split inside UE out of scope Added text is not needed Lin, Wed, 08:20 Takes PeteS coment on board Lin, Wed, 08:20 Explains to Lea the added text Lena, Thu, 00:51 Not agreeing with Lin PeterS, Thu, 09:59 Defends Lin |  |
|  |  |  | C1-203712 | Type 3 IE is not recommended to be used as an optional IE |  |  | Huawei, HiSilicon/Lin | CR 0129 24.007 Rel-16 | Ivo, Tue, 09:32 Agrees with intention, suggests different wording Behrouz, Tue, 10:02 Support, two editorials Lin, Wed, 09:27 Provides rev Ivo, Wed, 12:58 Nearly ok, co-sign Behourz, Thu, 07:18 Cr looks great, co-sig |  |
|  |  |  | C1-203713 | No retry in 2G/3G/5G for PDN type related ESM causes |  |  | Huawei, HiSilicon/Lin | CR 3412 24.301 Rel-16 |  |  |
|  |  |  | C1-203714 | No retry in 4G for PDP type related SM causes |  |  | Huawei, HiSilicon/Lin | CR 3227 24.008 Rel-16 |  |  |
|  |  |  | C1-203344 | Enhancement in UE handling when error number #65 is received from network. |  |  | MediaTek Beijing Inc. | CR 3383 24.301 Rel-16 | Shifted from IMSProtoc16, work item code needs to be corrected Ivo, Tue, 09:48 - "stop any retry while connected to the same PLMN" - not clear what this means. - seems to be captured in 6.5.1.4A already Osamah, Tue, 18:29 Not needed Mike, Tue, 20:16 Concerns Rohit, Wed, 05:55 Answering Ivo and Mike Ivo, Wed, 13:02 Not agreeing Rohit, Thu, 09:05 Further explanation Providing ref |  |
|  | Wis for IMS |  |  | Jörgen – Breakout |  |  |  |  |  |  |
|  | MCCI_CT |  |  |  |  |  |  |  | Mission Critical Communication Interworking with Land Mobile Radio Systems 100% |  |
|  |  |  | C1-202610 | Editorial corrections |  |  | Sepura Ltd, Hytera Communications Corp | CR 0001 29.582 Rel-16 | Agreed Revision of C1-202286 _________________________________________ |  |
|  | MCProtoc16 |  |  | Jörgen – Breakout |  |  |  |  | Protocol enhancements for Mission Critical Services for Rel-16 100% |  |
|  |  |  | C1-202555 | Corrections to step reference in terminating controlling function |  |  | Samsung | CR 0560 24.379 Rel-16 | Agreed |  |
|  |  |  | C1-202556 | Corrections to step reference in create a group regroup using preconfigured group |  |  | Samsung | CR 0561 24.379 Rel-16 | Agreed |  |
|  |  |  | C1-202557 | Corrected the client origination procedure subclause text of 11.1.6.2.1.1 |  |  | Samsung | CR 0562 24.379 Rel-16 | Agreed |  |
|  |  |  | C1-202558 | Allow an emergency and immenit peril calls during max simultaneous sessions |  |  | Samsung | CR 0563 24.379 Rel-16 | Agreed |  |
|  |  |  | C1-202630 | Check regroup ID |  |  | FirstNet / Mike | CR 0553 24.379 Rel-16 | Agreed Revision of C1-202220 |  |
|  |  |  | C1-202631 | Clarification of 11.1.6.2.1.2 |  |  | FirstNet / Mike | CR 0554 24.379 Rel-16 | Agreed Revision of C1-202221 |  |
|  |  |  | C1-202632 | Update affiliation definition to support preconfigured regroups |  |  | FirstNet / Mike | CR 0555 24.379 Rel-16 | Agreed Revision of C1-202222 _________________________________________ |  |
|  |  |  | C1-202656 | Check for MCPTT ID bindng and validity period of existing binding |  |  | Samsung | CR 0557 24.379 Rel-16 | Agreed Revision of C1-202552 _________________________________________ . |  |
|  |  |  | C1-202657 | Corrections to location sharing during call setup |  |  | Samsung | CR 0558 24.379 Rel-16 | Agreed Revision of C1-202553 _________________________________________ |  |
|  |  |  | C1-202658 | Corrections to current talker location in ambient call |  |  | Samsung | CR 0559 24.379 Rel-16 | Agreed Revision of C1-202554 _________________________________________ |  |
|  |  |  | C1-202660 | Talker location sharing in remote ambient call |  |  | Samsung | CR 0231 24.380 Rel-16 | Agreed Revision of C1-202560 _________________________________________ . |  |
|  |  |  | C1-203078 | CR Cancellation of a Private Call (without Floor Control) prior the setup |  |  | BDBOS, Airbus | CR 0565 24.379 Rel-16 |  |  |
|  |  |  | C1-203143 | 4.12 Improve NOTE |  |  | FirstNet / Mike | CR 0574 24.379 Rel-16 |  |  |
|  |  |  | C1-203144 | 6.2.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0575 24.379 Rel-16 |  |  |
|  |  |  | C1-203145 | 6.2.8.1.6 Correct reference to group document |  |  | FirstNet / Mike | CR 0576 24.379 Rel-16 |  |  |
|  |  |  | C1-203146 | 6.3.2.1.8.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0577 24.379 Rel-16 |  |  |
|  |  |  | C1-203147 | 6.3.3.1.13.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0578 24.379 Rel-16 |  |  |
|  |  |  | C1-203148 | 6.3.3.1.13.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0579 24.379 Rel-16 |  |  |
|  |  |  | C1-203149 | 6.3.3.1.13.5 Correct reference to group document |  |  | FirstNet / Mike | CR 0580 24.379 Rel-16 |  |  |
|  |  |  | C1-203150 | 6.3.3.1.13.7 Correct reference to group document |  |  | FirstNet / Mike | CR 0581 24.379 Rel-16 |  |  |
|  |  |  | C1-203151 | 6.3.3.3 Correct reference to group document |  |  | FirstNet / Mike | CR 0582 24.379 Rel-16 |  |  |
|  |  |  | C1-203152 | 6.3.3.5.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0583 24.379 Rel-16 |  |  |
|  |  |  | C1-203153 | 6.3.3.5.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0584 24.379 Rel-16 |  |  |
|  |  |  | C1-203154 | 6.3.4.1.4 Correct reference to group document |  |  | FirstNet / Mike | CR 0585 24.379 Rel-16 |  |  |
|  |  |  | C1-203155 | 6.3.5.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0586 24.379 Rel-16 |  |  |
|  |  |  | C1-203156 | 6.3.5.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0587 24.379 Rel-16 |  |  |
|  |  |  | C1-203157 | 6.3.5.3 Correct reference to group document |  |  | FirstNet / Mike | CR 0588 24.379 Rel-16 |  |  |
|  |  |  | C1-203158 | 6.3.5.4 Correct reference to group document |  |  | FirstNet / Mike | CR 0589 24.379 Rel-16 |  |  |
|  |  |  | C1-203159 | 6.3.5.5 Correct reference to group document |  |  | FirstNet / Mike | CR 0590 24.379 Rel-16 |  |  |
|  |  |  | C1-203160 | 10.1.1.4.1.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0591 24.379 Rel-16 |  |  |
|  |  |  | C1-203161 | 10.1.1.4.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0592 24.379 Rel-16 |  |  |
|  |  |  | C1-203162 | 10.1.1.4.5.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0593 24.379 Rel-16 |  |  |
|  |  |  | C1-203163 | 10.1.1.5.2.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0594 24.379 Rel-16 |  |  |
|  |  |  | C1-203164 | 10.1.1.5.4 Correct reference to group document |  |  | FirstNet / Mike | CR 0595 24.379 Rel-16 |  |  |
|  |  |  | C1-203165 | 10.1.1.5.5 Correct reference to group document |  |  | FirstNet / Mike | CR 0596 24.379 Rel-16 |  |  |
|  |  |  | C1-203166 | 10.1.2.4.1.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0597 24.379 Rel-16 |  |  |
|  |  |  | C1-203167 | 10.1.2.5.1.8 Correct reference to group document |  |  | FirstNet / Mike | CR 0598 24.379 Rel-16 |  |  |
|  |  |  | C1-203168 | 10.1.3.4.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0599 24.379 Rel-16 |  |  |
|  |  |  | C1-203169 | 10.1.3.4.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0600 24.379 Rel-16 |  |  |
|  |  |  | C1-203170 | 10.1.3.5.1 Correct reference to group document |  |  | FirstNet / Mike | CR 0601 24.379 Rel-16 |  |  |
|  |  |  | C1-203171 | 10.1.3.5.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0602 24.379 Rel-16 |  |  |
|  |  |  | C1-203172 | 10.2.1.1.2 Correct reference to group document |  |  | FirstNet / Mike | CR 0603 24.379 Rel-16 |  |  |
|  |  |  | C1-203176 | Client SIP INVITE request descriptions |  |  | FirstNet / Mike | CR 0604 24.379 Rel-16 | Withdrawn CR number was reserved against incorrect spec. New Tdoc number is C1-203773, CR#0179 for 24.282 |  |
|  |  |  | C1-203773 | Client SIP INVITE request descriptions |  |  | FirstNet / Mike | CR 0179 24.262 Rel-16 |  |  |
|  |  |  | C1-203179 | Correct mcdata-calling-user-identity |  |  | FirstNet / Mike | CR 0136 24.282 Rel-16 |  |  |
|  |  |  | C1-203182 | Correct warning message and number in 10.1.1.4.2 |  |  | FirstNet / Mike | CR 0608 24.379 Rel-16 |  |  |
|  |  |  | C1-203183 | Editorial correction - 6.3.6.1 |  |  | FirstNet / Mike | CR 0137 24.282 Rel-16 |  |  |
|  |  |  | C1-203184 | Error correction - 10.2.5.4.4 |  |  | FirstNet / Mike | CR 0138 24.282 Rel-16 |  |  |
|  |  |  | C1-203185 | Error correction - 13.2.1.1 |  |  | FirstNet / Mike | CR 0139 24.282 Rel-16 |  |  |
|  |  |  | C1-203214 | Remove duplicate RFC 3856 reference |  |  | FirstNet / Mike | CR 0165 24.282 Rel-16 |  |  |
|  |  |  | C1-203246 | Implement missing reference number |  |  | Ericsson /Jörgen | CR 0167 24.282 Rel-16 |  |  |
|  |  |  | C1-203247 | Correction of node formats |  |  | Ericsson /Jörgen | CR 0076 24.483 Rel-16 |  |  |
|  |  |  | C1-203250 | Correction of warning text in Connect message |  |  | Ericsson /Jörgen | CR 0236 24.380 Rel-16 |  |  |
|  |  |  | C1-203648 | Align the NOTE related to multitalker or dual floor for receive RTP event in any state |  |  | Samsung | CR 0239 24.380 Rel-16 |  |  |
|  |  |  | C1-203649 | Handle Floor taken message in ‘Pending request’ state of floor participant state m/c |  |  | Samsung | CR 0240 24.380 Rel-16 |  |  |
|  |  |  | C1-203650 | Include the missing events in floor participant state m/c |  |  | Samsung | CR 0241 24.380 Rel-16 |  |  |
|  |  |  | C1-203651 | Authentication of the MIKEY-SAKKE I_Message validation in pre-established session |  |  | Samsung | CR 0230 24.380 Rel-16 | Revision of C1-202659 |  |
|  |  |  | C1-203652 | Corrections in 6.3.5.2.2 and 6.3.5.3.3 |  |  | Samsung | CR 0069 24.581 Rel-16 |  |  |
|  |  |  | C1-203653 | Corrections in 6.3.5.4.2 |  |  | Samsung | CR 0070 24.581 Rel-16 |  |  |
|  |  |  | C1-203654 | Corrections in 6.3.6.3.6 |  |  | Samsung | CR 0071 24.581 Rel-16 |  |  |
|  |  |  | C1-203655 | Incorrect counter Cx upper limit check |  |  | Samsung | CR 0072 24.581 Rel-16 |  |  |
|  |  |  | C1-203656 | New instance creation and release for basic / general reception control state m/c. |  |  | Samsung | CR 0073 24.581 Rel-16 |  |  |
|  |  |  | C1-203658 | Authorisation validation for first-to-answer call origination requesting user using pre-established session |  |  | Samsung | CR 0556 24.379 Rel-16 | Revision of C1-202834 -------------------------------------------- Was agreed Needs revision, missing tdoc number Revision of C1-202655 _________________________________________ Revision of C1-202551 |  |
|  | MuD |  |  | Jörgen – Breakout |  |  |  |  | Multi-device and multi-identity 100% |  |
|  |  |  | C1-202494 | Text for empty headings |  |  | Ericsson /Jörgen | CR 0001 24.174 Rel-16 | Agreed |  |
|  |  |  | C1-202586 | Reference update for PASSporT Extension for Diverted Calls |  |  | Orange / Mariusz | CR 0002 24.174 Rel-16 | Agreed |  |
|  | IMSProtoc16 |  |  | Jörgen – Breakout |  |  |  |  | IMS Stage-3 IETF Protocol Alignment for Rel-16 100% |  |
|  | MCSMI_CT |  |  | Jörgen – Breakout |  |  |  |  | Mission Critical system migration and interconnection |  |
|  | eMCData2 |  |  | Jörgen – Breakout |  |  |  |  | CT aspects of Enhancements to Functional architecture and information flows for Mission Critical Data |  |
|  |  |  | C1-202637 | Deposit an object |  |  | AT&T | CR 0118 24.282 Rel-16 | Agreed Revision of C1-202023 _________________________________________ . |  |
|  |  |  | C1-202640 | Create a subscription to notifications |  |  | AT&T | CR 0119 24.282 Rel-16 | Agreed Revision of C1-202024 _________________________________________ |  |
|  |  |  | C1-202641 | Delete a subscription to notifications |  |  | AT&T | CR 0120 24.282 Rel-16 | Agreed Revision of C1-202025 _________________________________________ . |  |
|  |  |  | C1-202643 | Update a subscription to notifications |  |  | AT&T | CR 0121 24.282 Rel-16 | Agreed Revision of C1-202026 _________________________________________ . |  |
|  |  |  | C1-202646 | Synchronization notification |  |  | AT&T | CR 0122 24.282 Rel-16 | Agreed Revision of C1-202027 _________________________________________ |  |
|  |  |  | C1-202647 | Search-based Synchronization |  |  | AT&T | CR 0123 24.282 Rel-16 | Agreed Revision of C1-202028 . |  |
|  |  |  | C1-202649 | List folder |  |  | AT&T | CR 0124 24.282 Rel-16 | Agreed Revision of C1-202029 _________________________________________ |  |
|  |  |  | C1-202677 | Typo fixes |  |  | AT&T | CR 0125 24.282 Rel-16 | Agreed Revision of C1-202030 _________________________________________ |  |
|  |  |  | C1-202794 | Fix minor issues in MCData pre-etsblished session |  |  | Samsung / Sapan | CR 0131 24.282 Rel-16 | Agreed Revision of C1-202452 |  |
|  |  |  | C1-203294 | Resolving EN for identifying user between MCData Server and MCData message store |  |  | AT&T | CR 0168 24.282 Rel-16 |  |  |
|  |  |  | C1-203504 | Signalling plane support in MCData for user plane SDS using MBMS |  |  | AT&T / Val | CR 0170 24.282 Rel-16 |  |  |
|  |  |  | C1-203505 | Media plane control in MCData for user plane SDS using MBMS |  |  | AT&T / Val | CR 0011 24.582 Rel-16 |  |  |
|  |  |  | C1-203519 | Configuration of resource priority for MCData emergency |  |  | AT&T / Val | CR 0137 24.484 Rel-16 | Revision of C1-202750 ------------------------------------------- Was Agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202386 |  |
|  |  |  | C1-203522 | Auxiliary procedures in support of Emergency Alerts for MCData |  |  | AT&T / Val | CR 0130 24.282 Rel-16 | Revision of C1-202751 ----------------------------------------- Was Agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202288 _________________________________________ |  |
|  |  |  | C1-203523 | Handling of MCData Emergency Alerts at the MCData controlling server |  |  | AT&T / Val | CR 0129 24.282 Rel-16 | Revision of C1-202754 -------------------------------------- Was agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202287 |  |
|  |  |  | C1-203524 | Handling of MCData Emergency Alerts at the MCData participating servers |  |  | AT&T / Val | CR 0128 24.282 Rel-16 | Revision of C1-202755 ------------------------------------- Was Agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202281 |  |
|  |  |  | C1-203525 | Emergency Alerts for MCData – client procedures |  |  | AT&T / Val | CR 0127 24.282 Rel-16 | Revision of C1-202761 ------------------------------------------ Was agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202262 |  |
|  |  |  | C1-203527 | Support for MCData emergency alert and communications |  |  | AT&T / Val | CR 0126 24.282 Rel-16 | Revision of C1-202771 ---------------------------------------- Was agreed Needs revision, tdoc number format error on cover in first line Revision of C1-202260 |  |
|  |  |  | C1-203645 | Editor’s note for hostname of MCData message store is addressed |  |  | Samsung | CR 0125 24.282 Rel-16 | Revision of C1-202677 Incorrectly, as 2677 is a document from ATT |  |
|  |  |  | C1-203646 | Included the MessageStoreHostname element |  |  | Samsung | CR 0077 24.483 Rel-16 |  |  |
|  |  |  | C1-203647 | Included the MessageStoreHostname element |  |  | Samsung | CR 0141 24.484 Rel-16 |  |  |
|  |  |  | C1-203657 | Corrections to file upload-download procedure as per stage 2 architecture changes |  |  | Samsung | CR 0133 24.282 Rel-16 | Revision of C1-202835 ----------------------------------------- Was agreed Needs revision, missing tdoc number on cover sheet Revision of C1-202654 _________________________________________ Revision of C1-202550 |  |
|  | E2E_DELAY (CT4) |  |  | Jörgen – Breakout |  |  |  |  | CT Aspects of Media Handling for RAN Delay Budget Reporting in MTSI 100% |  |
|  | VBCLTE (CT3 lead) |  |  | Jörgen – Breakout |  |  |  |  | Volume Based Charging Aspects for VoLTE CT |  |
|  | ISAT-MO-WITHDRAW |  |  | Jörgen – Breakout |  |  |  |  | Withdrawal of TS 24.323 from Rel-11, Rel-12, Rel-13 No CRs needed, listed for the sake of completeness 100% |  |
|  | MONASTERY2 |  |  | Jörgen – Breakout |  |  |  |  | Mobile Communication System for Railways Phase 2 |  |
|  |  |  | C1-202883 | IPConnectivity extension to include IP Information |  |  | Kontron Transportation France | CR 0067 24.483 Rel-16 | Agreed Revision of C1-202496 _________________________________________ . |  |
|  |  |  | C1-202884 | IPConnectivity extension to include IP Information |  |  | Kontron Transportation France | CR 0138 24.484 Rel-16 | Agreed Revision of C1-202497 _________________________________________ . |  |
|  |  |  | C1-202885 | IPConnectivity extension to include IP Information |  |  | Kontron Transportation France | CR 0132 24.282 Rel-16 | Agreed Revision of C1-202498 _________________________________________ |  |
|  |  |  | C1-203173 | Add functional alias status definitions |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0134 24.282 Rel-16 |  |  |
|  |  |  | C1-203174 | Add functional alias to clause 4.6 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0135 24.282 Rel-16 |  |  |
|  |  |  | C1-203175 | Add PointCoordinate, Speed, Heading nodes in 5.2.48W |  |  | FirstNet / Mike | CR 0072 24.483 Rel-16 |  |  |
|  |  |  | C1-203177 | Correct capitalisation in ListOfFunctionalaliases |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0073 24.483 Rel-16 |  |  |
|  |  |  | C1-203178 | Correct editorial in FA status definition |  |  | FirstNet / Mike | CR 0605 24.379 Rel-16 |  |  |
|  |  |  | C1-203180 | Correct spelling of functional alias |  |  | FirstNet / Mike | CR 0606 24.379 Rel-16 |  |  |
|  |  |  | C1-203181 | Correct subclause references in 9A |  |  | FirstNet / Mike | CR 0607 24.379 Rel-16 |  |  |
|  |  |  | C1-203186 | Functional Alias - 5.2 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0140 24.282 Rel-16 |  |  |
|  |  |  | C1-203187 | Functional Alias - 5.3 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0141 24.282 Rel-16 |  |  |
|  |  |  | C1-203188 | Functional alias - 9.2.1.2 |  |  | FirstNet / Mike | CR 0142 24.282 Rel-16 |  |  |
|  |  |  | C1-203189 | Functional alias - 9.2.2.2.1 |  |  | FirstNet / Mike | CR 0143 24.282 Rel-16 |  |  |
|  |  |  | C1-203190 | Functional alias - 9.2.2.3.1 |  |  | FirstNet / Mike | CR 0144 24.282 Rel-16 |  |  |
|  |  |  | C1-203191 | Functional alias - 9.2.3.2.3 |  |  | FirstNet / Mike | CR 0145 24.282 Rel-16 |  |  |
|  |  |  | C1-203192 | Functional alias - 9.2.3.3.3 |  |  | FirstNet / Mike | CR 0146 24.282 Rel-16 |  |  |
|  |  |  | C1-203193 | Functional alias - 9.2.4.2.3 |  |  | FirstNet / Mike | CR 0147 24.282 Rel-16 |  |  |
|  |  |  | C1-203194 | Functional alias - 9.2.4.3.3 |  |  | FirstNet / Mike | CR 0148 24.282 Rel-16 |  |  |
|  |  |  | C1-203195 | Functional alias - 9.2.5.1.1 |  |  | FirstNet / Mike | CR 0149 24.282 Rel-16 |  |  |
|  |  |  | C1-203196 | Functional alias - 9.2.5.2.1.1 |  |  | FirstNet / Mike | CR 0150 24.282 Rel-16 |  |  |
|  |  |  | C1-203197 | Functional alias - 9.2.5.3.1.1 |  |  | FirstNet / Mike | CR 0151 24.282 Rel-16 |  |  |
|  |  |  | C1-203198 | Functional alias - 10.2.4.2.1 |  |  | FirstNet / Mike | CR 0152 24.282 Rel-16 |  |  |
|  |  |  | C1-203199 | Functional alias - 10.2.4.3.1 |  |  | FirstNet / Mike | CR 0153 24.282 Rel-16 |  |  |
|  |  |  | C1-203200 | Functional alias - 10.2.5.2.3 |  |  | FirstNet / Mike | CR 0154 24.282 Rel-16 |  |  |
|  |  |  | C1-203201 | Functional alias - 10.2.5.2.4 |  |  | FirstNet / Mike | CR 0155 24.282 Rel-16 |  |  |
|  |  |  | C1-203202 | Functional alias - 10.2.5.3.3 |  |  | FirstNet / Mike | CR 0156 24.282 Rel-16 |  |  |
|  |  |  | C1-203203 | Functional alias - 16.2.1.1 |  |  | FirstNet / Mike | CR 0157 24.282 Rel-16 |  |  |
|  |  |  | C1-203204 | Functional alias - 16.2.1.2 |  |  | FirstNet / Mike | CR 0158 24.282 Rel-16 |  |  |
|  |  |  | C1-203205 | Functional alias - 20.2.1 |  |  | FirstNet / Mike | CR 0159 24.282 Rel-16 |  |  |
|  |  |  | C1-203206 | Functional alias - 20.2.2 |  |  | FirstNet / Mike | CR 0160 24.282 Rel-16 |  |  |
|  |  |  | C1-203207 | Functional alias - affiliation procedures in 8.2 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0161 24.282 Rel-16 |  |  |
|  |  |  | C1-203208 | Functional alias - Coding 22.3 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0162 24.282 Rel-16 |  |  |
|  |  |  | C1-203209 | Functional Alias - MCData Client procedures 22.2.1 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0163 24.282 Rel-16 |  |  |
|  |  |  | C1-203210 | Functional Alias - MCData Server procedures 22.2.2 |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0164 24.282 Rel-16 |  |  |
|  |  |  | C1-203211 | Functional alias in MCData user profile |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0140 24.484 Rel-16 |  |  |
|  |  |  | C1-203212 | MCData (de)affiliation by location criteria MOs |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0074 24.483 Rel-16 |  |  |
|  |  |  | C1-203213 | MCData Functional Alias by location criteria |  |  | FirstNet, Nokia, Nokia Shanghai Bell / Mike | CR 0075 24.483 Rel-16 |  |  |
|  |  |  | C1-203215 | Schema error - FA Coding 9A.3.1.2 |  |  | FirstNet / Mike | CR 0609 24.379 Rel-16 |  |  |
|  |  |  | C1-203216 | Update MCData Overview clause 4.1 |  |  | FirstNet / Mike | CR 0166 24.282 Rel-16 |  |  |
|  |  |  | C1-203332 | Corrections in IP Connectivity SDP offer/answer generation |  |  | Kontron Transportation France | CR 0169 24.282 Rel-16 |  |  |
|  |  |  | C1-203718 | Work plan for the CT1 part of MONASTERY2 |  |  | Nokia, Nokia Shanghai Bell | discussion Rel-16 |  |  |
|  |  |  | C1-203719 | Limiting the number of emergency group calls accepted based on calling FA |  |  | Nokia, Nokia Shanghai Bell | CR 0616 24.379 Rel-16 |  |  |
|  |  |  | C1-203720 | Resolution of called functional alias in first-to-answer calls |  |  | Nokia, Nokia Shanghai Bell | CR 0617 24.379 Rel-16 |  |  |
|  |  |  | C1-203721 | Update service configuration to support limiting the number of authorized clients per MCPTT/MCData user |  |  | Nokia, Nokia Shanghai Bell | CR 0142 24.484 Rel-16 |  |  |
|  |  |  | C1-203722 | Update service authorization procedures to support limiting the number of authorized clients per MCData user |  |  | Nokia, Nokia Shanghai Bell | CR 0177 24.282 Rel-16 |  |  |
|  |  |  | C1-203723 | Restricting incoming MCData communications MO |  |  | Nokia, Nokia Shanghai Bell | CR 0078 24.483 Rel-16 |  |  |
|  |  |  | C1-203724 | Restricting incoming MCData communications- control |  |  | Nokia, Nokia Shanghai Bell | CR 0178 24.282 Rel-16 |  |  |
|  |  |  | C1-203725 | Restricting incoming MCData communications- user profile |  |  | Nokia, Nokia Shanghai Bell | CR 0143 24.484 Rel-16 |  |  |
|  |  |  | C1-203726 | MO Corrections |  |  | Nokia, Nokia Shanghai Bell | CR 0079 24.483 Rel-16 | Withdrawn Document not uploaded on |  |
|  |  |  | C1-203727 | Corrections on the structure of MCPTT user profile |  |  | Nokia, Nokia Shanghai Bell | CR 0144 24.484 Rel-16 |  |  |
|  | eIMS5G_SBA |  |  | Jörgen – Breakout |  |  |  |  | CT aspects of SBA interactions between IMS and 5GC |  |
|  |  |  | C1-202066 | No impact from SBA on main body |  |  | Nokia, Nokia Shanghai Bell, Ericsson | CR 6408 24.229 Rel-16 | Agreed Revision of C1-200353 |  |
|  | enh2MCPTT-CT |  |  | Jörgen – Breakout |  |  |  |  | Enhancements for Mission Critical Push-to-Talk CT aspects 100% |  |
|  | eIMSVideo |  |  | Jörgen – Breakout |  |  |  |  | Video enhancement of IMS CAT/CRS/announcement services |  |
|  |  |  | C1-202817 | Restrictions of providing video announcement |  |  | China Telecom,Huawei,China Unicom,HiSilicon / Michelle | CR 0076 24.628 Rel-16 | Agreed Revision of C1-202356 _________________________________________ . |  |
|  |  |  | C1-202863 | Use preconditions for CRS when terminating UE supports precondition |  |  | Huawei,China Telecom,China Unicom,HiSilicon /Hongxia | CR 0063 24.183 Rel-16 | Agreed Revision of C1-202605 _________________________________________ Revision of C1-202156 _________________________________________ |  |
|  |  |  | C1-202891 | Use preconditions for CAT when originating UE supports precondition |  |  | Huawei,China Telecom,China Unicom,HiSilicon /Hongxia | CR 0119 24.182 Rel-16 | Agreed Revision of C1-202604 _________________________________________ Revision of C1-202155 _________________________________________ |  |
|  |  |  | C1-203249 | Preconditions correction for forking model |  |  | Ericsson /Jörgen | CR 0120 24.182 Rel-16 |  |  |
|  | Other Rel-16 IMS & MC issues |  |  | Jörgen – Breakout |  |  |  |  | Other Rel-16 IMS topics |  |
|  |  |  | C1-202072 | Correction in CRS interactions with CDIV |  |  | Orange / Mariusz | CR 0062 24.183 Rel-16 | Agreed |  |
|  |  |  | C1-202080 | UE must not render local tones in case of call is being forwarded or call is queued |  |  | Qualcomm Incorporated | CR 0075 24.628 Rel-16 | Agreed |  |
|  |  |  | C1-202081 | NG eCall support over NR connected to the 5GC |  |  | Qualcomm Incorporated | CR 6414 24.229 Rel-16 | Agreed |  |
|  |  |  | C1-202759 | Correction in IMS_Registration_handling policy about how UE should deregister |  |  | MediaTek Inc. | CR 6404 24.229 Rel-16 | Agreed |  |
|  |  |  | C1-202837 | SRVCC from E-UTRAN to GERAN/UTRAN when IMS voice call is initiated in 5GS |  |  | Ericsson / Ivo | CR 1298 24.237 Rel-16 | Agreed Revision of C1-202133 |  |
|  |  |  | C1-202917 | Editorial clean-up |  |  | Ericsson /Jörgen | CR 0064 24.183 Rel-16 | Agreed Revision of C1-202785 _________________________________________ Revision of C1-202488 . |  |
|  |  |  | C1-203038 | NG eCall support over NR connected to the 5GC |  |  | Qualcomm Incorporated | CR 6414 24.229 Rel-16 | Revision of C1-202081 |  |
|  |  |  | C1-203086 | Support of "a=3gpp-qos-hint" SDP attribute for MTSI data channels |  |  | Ericsson / Nevenka | CR 6418 24.229 Rel-16 |  |  |
|  |  |  | C1-203093 | IMS call restoration on UE |  |  | Huawei, HiSilicon | CR 6419 24.229 Rel-16 |  |  |
|  |  |  | C1-203408 | SRVCC from E-UTRAN to GERAN/UTRAN when IMS voice call is initiated in 5GS and support of scenario where the SCC AS sends a request to the HSS to retrieve the SRVCC data for the UE using SBA |  |  | BlackBerry UK Ltd. | CR 1300 24.237 Rel-16 |  |  |
|  |  |  | C1-203469 | EPS fallback |  |  | Motorola Mobility, Lenovo | CR 6422 24.229 Rel-16 |  |  |
|  |  |  | C1-203472 | Registration and Authentication |  |  | Motorola Mobility, Lenovo | CR 0143 24.173 Rel-16 |  |  |
|  |  |  | C1-203745 | Correction of data type for verification signing |  |  | Ericsson /Jörgen | CR 6423 24.229 Rel-16 |  |  |
|  | Release 17 work items |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Work Item Descriptions |  |  | Peter - Main |  |  |  |  | New and revised Work Item Descritpions |  |
|  |  |  | C1-203069 | Stage-3 5GS NAS protocol development 17 |  |  | Ericsson / Ivo | WID new Rel-17 | Mariusz, Tue, 09:49 Support Sung, Tue, 14:12 Add nokia, NokiaShanghaiBell Christian, Tue, 19:25 Support |  |
|  |  |  | C1-203079 | New WID on Stage 3 of Multimedia Priority Service (MPS) Phase 2 |  |  | Perspecta Labs Inc. | WID new Rel-17 | PeterM, Tue, 20:14 Provides rev, based on Ct3 comments Lena, Wed, 04:01 Support the WID, but would like it to be postponed to August, due to status in SA2 Jörgen confcall, vidoa part is undetermined, media actions are open PeterM, Thu, 00:48 on “media aspects are open”, we don’t envision any changes to the conferencing itself, the media nor media description. We only plan to modify the RPH in an UPDATE or reINVITE to cause the IMS session to be upgraded. Will a note such as this in the objectives clause suffice? Jörgen, Thu, 09:46 Comments, provides a rev |  |
|  |  |  | C1-203094 | eIMS5G_nonSBA SID |  |  | Huawei, HiSilicon | SID new Rel-17 | Mariusz, Tue, 09:45 Wants to co-sign, who is rapporteur Bill, in confcall some comments form CT3 and CT4, additional TRs Sung, Wed, 16:24 Support the wid SangMin, Thu, 04:23 Requests “the work in CT WGs does not include any normative work with architectural impact” in the objective Bill, Thu, 14:42 rev |  |
|  |  |  | C1-203113 | New on Enhancement for the 5G Control Plane Steering of Roaming for UE in CONNECTED mode |  |  | DOCOMO Communications Lab. | WID new Rel-17 | Ivo, Tue, 09.24 - objective CT1 4) - stage-1 does not require sending of the parameters in SoR information container - it can e.g. be provided to the UE using UPU + UPU might be easier to extend that CP-SoR. it is proposed to reduce the objective to reflect stage-1. Christian, Tue, 19:25 Fine with WID, wants to improve description Ban, Wed, 07:16 Provides rev Ivo, Wed, 13:11 Commenting the rev Ban, Thu, 08:51 Rev LyThanh, Thu, 14:30 Potential impacts on USIM Sung, Thu, 15:39 support |  |
|  |  |  | C1-203220 | New WID on Stage-3 SAE Protocol Development |  |  | InterDigital Communications | WID new Rel-17 | Mariusz, Tue, 09:53 Support Sung, Tue, 14:12 Add nokia, NokiaShanghaiBell Christian, Tue, 19:25 Support Behourz, Tue, 22:06 Acks Mariusz |  |
|  |  |  | C1-203293 | CT aspects of Enhancements to Functional architecture and information flows for Mission Critical Data |  |  | AT&T / Val | WID new Rel-17 | Val, Wed, 04:51 Provides rev Dom support, requests changes to justification |  |
|  |  |  | C1-203331 | CT aspects of 5G ProSe |  |  | OPPO / Rae | WID new Rel-17 | Ivo, Tue, 09:23 Too early for stage-3 WID, in SA2 this is a SID Behrouz, Tue, 10:05 Too early, given status in SA2 Rae, Tue, 10:15 Acks that this is early, idea is to get comments and co-signers Sunhee, Tue, 13:39 Too early Christian, Tue, 19:25 Support the work in SA, however, too early for CT Rae, Wed, 04.17 Acks that this is for info and collect comments |  |
|  |  |  | C1-203514 | Multi-device and multi-identity enhancements |  |  | vivo Mobile Communication, Ericsson, China Mobile | WID new Rel-17 | Behrouz, Tue, 10:08 Objectives need to be formulated clearer Mariusz, Tue, 10:35 What is the expected outcome of the WID? Adrian, Tue, 18:44 Provides rev Christian, Tue, 19:25 Support Jörgen, Wed, 10:20 Not happy with the rev yet Mariusz Support |  |
|  |  |  | C1-203729 | New WID on IMS Stage-3 IETF Protocol Alignment |  |  | Nokia, Nokia Shanghai Bell | WID new Rel-17 | Mariusz, Tue, 09:53 Support Christian, Tue, 19:25 Support |  |
|  |  |  | C1-203644 | Protocol enhancements for Mission Critical Services |  |  | Ericsson /Jörgen | WID new | Shifted from 16.1 |  |
|  | CRs and Discussion Documents related to new or revised Work Items |  |  | Peter - Main |  |  |  |  | CRs and Disc papers related to new Work Items |  |
|  |  |  | C1-203292 | Impacts of eV2XAPP to CT WGs |  |  | Huawei, HiSilicon /Christian | discussion Rel-17 |  |  |
|  |  |  | C1-203330 | Discussion paper on CT aspects of 5G_ProSe |  |  | OPPO / Rae | discussion Rel-17 |  |  |
|  |  |  | C1-203716 | Impacts of UASAPP to CT WGs |  |  | Huawei, HiSilicon/Lin | discussion Rel-17 | Atle, Tue, 12:39 No work on KI#2 in SA2 Lin, Wed, 09:32 Agrees with atle |  |
|  | Status of other Work Items |  |  | Peter - Main |  |  |  |  | Status information on other relevant Rel-17 Work Items |  |
|  | Release 17 documents for information |  |  | Peter - Main |  |  |  |  | Miscellaneous documents provided for information |  |
|  |  |  | C1-203368 | Discussion paper on 5MBS work item |  |  | Huawei | discussion Rel-17 |  |  |
|  |  |  | C1-203369 | Technical feasibility of Solution #13 in 3GPP TR 23.737 |  |  | Nokia, Nokia Shanghai Bell, Thales | discussion Rel-17 | Mariusz, Tue, 12:00 Comments on the DISC Sung, Tue, 14:18 Explaining to Mariusz |  |
|  | Output Liaison Statements |  | Tdoc | Title |  |  | Prepared by | To/CC | Result & comment |  |
|  |  |  | C1-203114 | [Draft] Reply to LS on PLMN selection solutions for satellite access |  |  | THALES | LS out Rel-17 | Withdrawn |  |
|  |  |  | C1-203221 | Reply LS on support of eCall over NR |  |  | Qualcomm Incorporated / Lena | LS out Rel-16 | No comments |  |
|  |  |  | C1-203252 | LS on PDU session release for UE in RRC INACTIVE state with NG-RAN paging failure |  |  | Nokia Shanghai Bell | LS out Rel-16 | Revision of C1-202849 Ivo, Tue, 09:23 - there is ongoing discussion between RAN3 and SA2 on this topic - S2-2003531 + S2-2003805 and we need to wait until it settles Lin, Wed, 05:54 Does not agree that an LS needs to be sent Why is CT1 involved at all Sung, Wed, 20:22 Explaining why CT1 should send an ls, however, ok to wait for SA2 |  |
|  |  |  | C1-203264 | [Draft] LS on Unicode based pictogram for 3GPP ePWS work |  |  | SyncTechno Inc. | LS out Rel-16 | Withdrawn |  |
|  |  |  | C1-203288 | [draft]LS on link identifier update |  |  | vivo | LS out Rel-16 | Behrouz, Tue, 10:13 Not in favor of sending LS to SA2, some rewording if something is set CT1 to follow SA3 Sunghoon, Tue, 14:53 Supports to send the LS, fine with wording proposal form Behrouz Christian, Tue, 20:01 Do NOT support sending the LS Yanchao, Wed, 10:11 SA2 and SA3 are in conflict, we need clarification Sunghoon, Wed, 14.00 Supports the LS Mahmoud, Wed, conf call This is SA3, and they have concluded Yanchao, Wed, 17:24 S2-2003194, which proposed to mandate the target UE to update its ID, was discussed and noted in SA2#138e. So it is clear that it is optional in SA2 specification. Besides I cannot understand why the target UE has to change its ID, especially considering the case when the target UE has no privacy configuration. Rae, Thu, 04:37 I think the issue has been discussed in SA2 last meeting and the result is NOTED. So it is not appropriate to ignore this. Since the misalign happens between two stage 2 groups, I think it is better to send the LS to ask them. Behrouz, Thu, 06:47 Does not agree with Rae, Yanchao, Sunghoon Yanchao, Thu, 10:17 SA2 say “MAY” Sunghoo, Thu, 17:05 Explaining |  |
|  |  |  | C1-203121 | Reply LS on AAA-S via NSSAAF to support NSSAA |  |  | China Telecom Corporation Ltd. | LS out Rel-16 | Shifted from 16.2.6 |  |
|  |  |  | C1-203346 | LS on secure that a UE does not wait indefinitely for completion of NSSAA procedure |  |  | ZTE / Shuang | LS out Rel-16 |  |  |
|  |  |  | C1-203352 | Reply LS on 5G Steering of Roaming |  |  | DOCOMO Communications Lab. | LS out Rel-16 | Related CR in C1-203351 Ivo, Tue, 09:23 we do not see a need of "access technology" providing by the UDM. If the SOR-AF needs this information, the SOR-AF can determine it from the access type Mariusz, Tue, 10:31 In general ok with the LS, some remarks Sung, Wed, 20:14 We do not support adding neither access technology nor RAT type |  |
|  |  |  | C1-203417 | LS on mandate to provide "any PLMN" entry in the non-3GPP access node selection information |  |  | BlackBerry UK Ltd. | LS out | related to C1-203416 (DISC) and C1-203412 - C1-203413 (CRs) Ivo, Tue, 09:23 - contradicts 23.501 which states "The list of PLMNs shall include the HPLMN and shall include an "any PLMN" entry, which matches any PLMN the UE is connected to except the HPLMN.". - this is architecture issue which needs to be discussed in SA2 rather than in CT1. if BlackBerry sees this as important issue, BlackBerry needs to bring a company contribution to SA2. Amer, Tue, 22:09 , I agree with Ivo’s comment on the need for SA2 discussion based on an SA2 contribution. According to my colleagues in SA2, the argument for optional “any PLMN” entry was presented in SA2 and not agreed. So we do not agree to sending this LS. John-Luc, Wed, 23:59 rev Andrew, Thu, 12:07 LI not working is a FASMO, but is open for a possible way forward |  |
|  |  |  | C1-203473 | Reply LS on the applicability of 5G NAS protocol for 5G-RG and FN-RG (LIAISE-397) |  |  | Huawei, HiSilicon /Christian | LS out Rel-16 | Ivo, Tue, 09:23 we do not have any CRs adding wireline access specific clauses to TS 24.502, 24.526, 24.193. Thus, the last sentence of overall description should be removed. Can live with the LS Christian, Tue, 16:56 Open for some modification, but does not want to remove the para Lazaros, Wed, 11:24 Supports the LS, requires changes Chrstian, Wed, 13:15 Asks Lazaros for proposal Roozbeh, Some concerns about promises |  |
|  |  |  | C1-203474 | Reply LS on status of 5WWC work |  |  | Huawei, HiSilicon /Christian | LS out Rel-16 | Ivo, Tue, 09:23 - LS does not seem to be needed as: - in 23.316, RG-TMBR is a parameter in N2 message. Thus, CT1 should not comment on RG-TMBR. - BBF in their LS C1-203010 did not ask about session-TMBR and thus it is not clear why CT1 should inform BBR about session-TMBR state. Christian, Tue, 16:39 Explaining why LS is needed Lazaros, Wed, 11:06 No ned for the LS Christian, conf call, Can live to not send the LS |  |
|  |  |  | C1-203482 | LS on Early UE capoability retrieval |  |  | Qualcomm Incorporated / Amer | LS out Rel-16 | Shifted from 16.2.8 |  |  |
|  |  |  | C1-203503 | Reply LS on IANA assigned values for mission critical |  |  | Ericsson /Jörgen | LS out |  |  |
|  |  |  | C1-203537 | Reply to LS on Updated User Plane Integrity Protection advice |  |  | Samsung/Kundan | LS out Rel-16 | Ivo, Tue, 09:22 in S3-201487/C1-203753, SA3 asked CT1 "to review this information and update their specifications from release 16 accordingly". Agreement on related CT1 CRs should be sufficient, LS seems unnecessary. Christian, Tue, 20.18 CT1 needs to wait for the reply from SA2 first before sending any reply |  |
|  |  |  | C1-203588 | LS on NAS uplink COUNT used for AS SMC at radio bearer establishment |  |  | MediaTek Inc. / Marko | LS out Rel-16 | Mikael, Wed, 22:48 I do not believe an LS from CT1 is needed but the issue should be directly brought to SA3 for discussion and possible correction. Marko, Thu, 12.52 Agrees to go to SA 3 directly. |  |
|  |  |  | C1-203674 | Reply LS on QoE Measurement Collection |  |  | Ericsson /Jörgen | LS out |  |  |
|  | EARLY |  | C1-203791 | Reply to LS on PLMN selection solutions for satellite access |  |  | THALES | LS out Rel-17 | Revision of C1-203790 _________________________________________ Revision of C1-203115 _________________________________________ LyThan, Tue, 14:19 Requested for early LSout Christian, Tue, 20:22 Support to send, but needs to be revised, in remit of CT1, welcome SA2 input Amer, Tue, 21:25 would prefer to briefly respond that CT1 would perform the design of the PLMN selection procedure for satellite access and any SA2 input is valuable in this process. Sung, Christian and amer comments valid Chen, send a high level answer, ct1 is responsible LyThan, Wed, 17:21 Proposal Sung, Wed, 20:23 Proposal looks good |  |
|  | Late and misplaced documents |  | Tdoc | Title Prioritization of documents within this category will be done during the meeting. Some tdocs are left in the main agenda item, although they are late (e.g. papers reporting IETF progress, which are usually more up to date the later they are submitted) |  |  | Source | Tdoc info | Result & comments Late documents and documents which were submitted with erroneous or incomplete information |  |
|  |  |  | C1-203032 | void |  |  | RAN2 | LS in | Withdrawn |  |
|  |  |  | C1-203033 | Void |  |  | RAN2 | LS in | Withdrawn |  |
|  | A.O.B. |  | Tdoc | Title |  |  | Source | Tdoc info | Result & comments |  |
|  | Closing Friday by 16:00 at the latest |  |  | Did you mark your attendance to this meeting? |  |  |  |  | Any meeting document which is not mentioned in this report or with no recorded decision shall be interpreted as "reserved", i.e. not defined and shall be ignored if received |  |
|  |  |  |  | Last upload of revisions: Tuesday 9th June 2020 16:00 CEST Last comments: Wednesday 10th June 2020 16:00 CEST Chairman Report of the meeting: Thursday 11th June 2020 |  |  |  |  |  |  |