GPP TSG-CT WG1 Meeting #124-e	C1-203950

Electronic meeting, 2-10 June 2020	(was C1-203345)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0690 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of commands for VAE layer configuration clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-06-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | VAE layer functionality has been introduced in Rel-16 under V2XAPP work. The V2XAPP work item identifies the need of introducing new AT commandsfor a TE to control MT supporting VAE layer. Stage 2 requirements can be found in TS 23.286 and 23.285 while the stage 3 under CT1 responsibility can be found in TS 24.486. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause for commands for VAE layer configuration is added to the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to use AT commands for a TE to control MT supporting VAE layer. Stage 2 requirements on VAE layer functionality will not be complete within Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 16 (new), 16.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 22.002: "Bearer Services (BS) supported by a GSM Public Land Mobile Network (PLMN)".

[2]	3GPP TS 22.003: "Teleservices supported by a GSM Public Land Mobile Network (PLMN)".

[3]	3GPP TS 22.081: "Line identification supplementary services - Stage 1".

[4]	3GPP TS 22.082: "Call Forwarding (CF) supplementary services - Stage 1".

[5]	3GPP TS 22.083: "Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 1".

[6]	3GPP TS 22.088: "Call Barring (CB) supplementary services - Stage 1".

[7]	3GPP TS 23.003: "Numbering, addressing and identification".

[8]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols-Stage 3".

[9]	GSM MoU SE.13, GSM MoU Permanent Reference Document SE.13: "GSM Mobile Network Codes and Names".

[10]	ITU-T Recommendation E.212: "Identification plan for land mobile stations".

[11]	ITU-T Recommendation T.31: "Asynchronous facsimile DCE control, service class 1".

[12]	ITU-T Recommendation T.32: "Asynchronous facsimile DCE control, service class 2".

[13]	ITU-T Recommendation T.50: "International Reference Alphabet (IRA) (Formerly International Alphabet No. 5 or IA5) - Information technology - 7-bit coded character set for information exchange".

[14]	ITU-T Recommendation V.250: "Serial asynchronous automatic dialling and control".

[15]	TIA IS-99: "Data Services Option Standard for Wideband Spread Spectrum Digital Cellular System".

[16]	TIA IS-135: "800 MHz Cellular Systems, TDMA Services, Async Data and Fax".

[17]	PCCA STD-101 Data Transmission Systems and Equipment: "Serial Asynchronous Automatic Dialling and Control for Character Mode DCE on Wireless Data Services".

[18]	3GPP TS 24.022: "Radio Link Protocol (RLP) for data and telematic services on the Mobile Station - Base Station System (MS - BSS) interface and the Base Station System - Mobile-services Switching Centre (BSS - MSC) interface".

[19]	3GPP TS 22.030: "Man Machine Interface (MMI) of the Mobile Station (MS)".

[20]	3GPP TS 45.008: "Radio subsystem link control".

[21]	3GPP TS 22.085: "Closed User Group (CUG) supplementary services - Stage 1".

[22]	3GPP TS 22.084: "MultiParty (MPTY) supplementary services - Stage 1".

[23]	3GPP TS 22.090: "Unstructured Supplementary Service Data (USSD) - Stage 1".

[24]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[25]	3GPP TS 23.038: "Alphabet and language specific information".

[26]	3GPP TS 22.024: "Description of Charge Advice Information (CAI)".

[27]	3GPP TS 22.086: "Advice of Charge (AoC) supplementary services - Stage 1".

[28]	3GPP TS 51.011: "Specification of the Subscriber Identity Module - Mobile Equipment (SIM-ME) interface".

[29]	3GPP TS 22.034: "High Speed Circuit Switched Data (HSCSD) - Stage 1".

[30]	3GPP TS 22.091: "Explicit Call Transfer (ECT) supplementary service - Stage 1".

[31]	3GPP TS 22.072: "Call Deflection (CD) supplementary service - Stage 1".

[32]	ISO/IEC 10646: "Universal Multiple-Octet Coded Character Set (UCS)"; UCS2, 16 bit coding.

[33]	3GPP TS 22.022: "Personalization of GSM Mobile Equipment (ME) Mobile functionality specification".

[34]	3GPP TS 27.060: "General requirements on Mobile Stations (MS) supporting General Packet Radio Bearer Service (GPRS)".

[35]	Void.

[36]	CCITT Recommendation V.120: "Support by an ISDN of data terminal equipment with V-Series type interfaces with provision for statistical multiplexing".

[37]	Void.

[38]	3GPP TS 45.005: "Radio transmission and reception".

[39]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting GPRS and Packet Data Networks (PDN)".

[40]	3GPP TS 23.081: "Line identification supplementary services - Stage 2".

[41]	3GPP TS 27.001: "General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS)".

[42]	3GPP TS 29.007: "General requirements on interworking between the Public Land Mobile Network (PLMN) and the Integrated Services Digital Network (ISDN) or Public Switched Telephone Network (PSTN)".

[43]	Infrared Data Association; Specification of Ir Mobile Communications (IrMC).

[44]	IrDA Object Exchange Protocol.

[45]	3GPP TS 27.010: "Terminal Equipment to User Equipment (TE-UE) multiplexer protocol User Equipment (UE)".

[46]	3GPP TS 23.107: "Quality of Service, Concept and Architecture".

[47]	3GPP TS 23.060: "General Packet Radio Service (GPRS) Service description; Stage 2".

[48]	Void.

[49]	3GPP TS 43.068: "Voice Group Call service (VGCS) - Stage 2".

[50]	3GPP TS 43.069: "Voice Broadcast Service (VBS) - Stage 2".

[51]	Void.

[52]	3GPP TS 44.068: "Voice Group Call service (VGCS) - Stage 3".

[53]	3GPP TS 44.069: "Voice Broadcast Service (VBS) - Stage 3".

[54]	3GPP TS 22.067: "enhanced Multi-Level Precedence and Pre-emption service (eMLPP) - Stage 1".

[55]	3GPP TS 42.068: "Voice Group Call service (VGCS) - Stage 1".

[56]	3GPP TS 42.069: "Voice Broadcast Service (VBS) - Stage 1".

[57]	Void.

[58]	3GPP TS 22.087: "User-to-User Signalling (UUS) - Stage 1".

[59]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) Application".

[60]	ETSI TS 102 221 "Smart Cards; UICC-Terminal interface; Physical and logical characteristics (Release 1999)".

[61]	3GPP TS 44.065: "Mobile Station (MS) – Serving GPRS Support Node (SGSN); Subnetwork Dependent Convergence Protocol (SNDCP)".

[62]	3GPP TS 25.323: "Packet Data Convergence Protocol (PDCP)".

[63]	3GPP TS 23.227 "Applications and User interaction in the UE-Principles and specific requirements", Release 5.

[64]	Void.

[65]	3GPP TS 31.101: "UICC-Terminal Interface; Physical and Logical Characteristics."

[66]	ETSI TS 102 310: "Smart Cards; Extensible Authentication Protocol support in the UICC".

[67]	Void.

[68]	RFC 3748: "Extensible Authentication Protocol (EAP)".

[69]	RFC 3629: "UTF-8, a transformation format of ISO 10646".

[70]	3GPP TS 44.318: "Generic Access (GA) to the A/Gb interface; Mobile GA interface layer 3 specification".

[71]	3GPP TS 44.060: "General Packet Radio Service (GPRS); Mobile Station (MS) - Base Station System (BSS) interface; Radio Link Control/Medium Access Control (RLC/MAC) protocol".

[72]	3GPP TS 25.308: "High Speed Downlink Packet Access (HSDPA): Overall Description; Stage 2".

[73]	3GPP TS 25.319: "Enhanced Uplink; Overall Description; Stage 2".

[74]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[75]	3GPP TS 24.216: "Communication Continuity Management Object (MO)".

[76]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[77]	3GPP TS 25.305 "User Equipment (UE) positioning in Universal Terrestrial Radio Access Network (UTRAN); Stage 2".

[78]	IEC 61162: "Maritime navigation and radio communication equipment and systems – Digital interfaces".

[79]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC), Radio Resource LCS Protocol (RRLP)".

[80]	3GPP TS 49.031: "Base Station System Application Part, LCS Extension (BSSAP-LE)".

[81]	Void.

[82]	3GPP TS 23.401: "GPRS enhancements for E-UTRAN access".

[83]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS)".

[84]	Void.

[85]	3GPP TS 23.203: "Policy and charging control architecture".

[86]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[87]	3GPP TS 24.173: "IMS multimedia telephony communication service and supplementary services; Stage 3".

[88]	RFC 4291: "IP Version 6 Addressing Architecture".

[89]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP)".

[90]	3GPP TS 23.221: "Architectural requirements".

[91]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity".

[92]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM) Application Toolkit (USAT)".

[93]	3GPP TS 22.096: "Name identification supplementary services - Stage 1".

[94]	3GPP TS 23.096: "Name identification supplementary services - Stage 2".

[95]	3GPP TS 25.133: "Requirements for support of radio resource management (FDD)".

[96]	3GPP TS 25.123: "Requirements for support of radio resource management (TDD)".

[97]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[98]	3GPP TS 31.102: "Characteristics of the Universal Subscriber Identity Module (USIM) application".

[99]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[100]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[101]	3GPP TS 24.341: "Support of SMS over IP networks".

[102]	3GPP TS 24.167: "3GPP IMS Management Object (MO); Stage 3".

[103]	IETF STD 5: "Internet Protocol".

[104]	IETF STD 51: "The Point-to-Point Protocol (PPP)".

[105]	RFC 1144: "Compressing TCP/IP Headers for Low-Speed Serial Links".

[106]	RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[107]	RFC 2507: "IP Header Compression".

[108]	RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[109]	3GPP TS 24.080: "Mobile radio interface Layer 3 supplementary service specification; Formats and coding".

[110]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[111]	RFC 3261: "SIP: Session Initiation Protocol".

[112]	RFC 3966: "The tel URI for Telephone Numbers".

[113]	RFC 3969: "The Internet Assigned Number Authority (IANA) Uniform Resource Identifier (URI) Parameter Registryfor the Session Initiation Protocol (SIP)".

[114]	RFC 5341: "The Internet Assigned Number Authority (IANA) tel Uniform Resource Identifier (URI) Parameter Registry".

[115]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[116]	RFC 2141: "URN Syntax".

[117]	RFC 3406: "Uniform Resource Names (URN) Namespace Definition Mechanisms".

[118]	RFC 5031: "A Uniform Resource Name (URN) for Emergency and Other Well-Known Services".

[119]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[120]	3GPP TS 24.608: "Terminating Identification Presentation (TIP) and Terminating Identification Restriction (TIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[121]	3GPP TS 24.654: "Closed User Group (CUG) using IP Multimedia (IM) Core Network (CN) subsystem, Protocol Specification".

[122]	RFC 4715: "The Integrated Services Digital Network (ISDN) Subaddress Encoding Type for tel URI".

[123]	3GPP TS 22.093: "Completion of Calls to Busy Subscriber (CCBS); Service description, Stage 1".

[124]	3GPP TS 22.094: "Follow Me service description; Stage 1".

[125]	3GPP TS 22.097: "Multiple Subscriber Profile (MSP) Phase 2; Service description; Stage 1".

[126]	3GPP TS 22.135: "Multicall; Service description; Stage 1".

[127]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol specification".

[128]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) Customized Ringing Signal (CRS); Protocol specification".

[129]	3GPP TS 24.239: "Flexible Alerting (FA) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[130]	3GPP TS 24.259: "Personal Network Management (PNM)".

[131]	3GPP TS 24.390: "Unstructured Supplementary Service Data (USSD) using IP Multimedia (IM) Core Network (CN) subsystem IMS".

[132]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[133]	3GPP TS 24.605: "Conference (CONF) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[134]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[135]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[136]	3GPP TS 24.611: "Anonymous Communication Rejection (ACR) and Communication Barring (CB) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[137]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol Specification".

[138]	3GPP TS 24.616: "Malicious Communication Identification (MCID) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[139]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[140]	3GPP TS 24.642: "Completion of Communications to Busy Subscriber (CCBS) and Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[141]	3GPP TS 24.647: "Advice Of Charge (AOC) using IP Multimedia (IM) Core Network (CN) subsystem".

[142]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)".

[143]	3GPP TS 25.102: "Multiplexing and channel coding (TDD)".

[144]	3GPP TS 25.212: "Multiplexing and channel coding (FDD)".

[145]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[146]	3GPP TS 45.001: "Physical layer on the radio path; General description".

[147]	3GPP TS 22.101: "Service aspects; Service principles".

[148]	3GPP TS 24.090: "Unstructured Supplementary Service Data (USSD); Stage 3".

[149]	3GPP TS 23.682: "Architecture Enhancements to facilitate communications with Packet Data Networks and Applications".

[150]	3GPP TS 36.443: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); M2 Application Protocol (M2AP)".

[151]	Wi-Fi Alliance: "Hotspot 2.0 (Release 2) Technical Specification, version 1.0.0", 2014-08-08.

[152]	IEEE Std 802.11™-2012: "Information Technology- Telecommunications and information exchange between systems-Local and metropolitan area networks-Specific requirements-Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[153]	3GPP TS 24.312: "Access Network Discovery and Selection Function (ANDSF) Management Object (MO)".

[154]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[155]	RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[156]	3GPP TS 44.018: "GSM/EDGE Radio Resource Control (RRC) protocol".

[157]	CEN EN 15722:2015 (April 2015): "Intelligent transport systems - ESafety - ECall minimum set of data".

[158]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[159]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[160]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[161]	3GPP TS 24.501: "Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[162]	3GPP TS 37.340: "Evolved Universal Terresterial Radio Access (E-UTRA) and NR; Multi-Connectivity; Stage 2".

[163]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[164]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[165]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[166]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[167]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer; Measurements".

[168]	3GPP TS 24.250: "Protocol for Reliable Data Service between UE and SCEF; Stage 3".

[169]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[170]	3GPP TS 22.011: "Service accessibility".

[171]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[r24486]	3GPP TS 24.486: "Vehicle-to-Everything (V2X) Application Enabler (VAE) layer; Protocol aspects; Stage 3".

* * * Next Change * * * *

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

5GCN	5G Core Network

5GS	5G System

AT	ATtention; this two-character abbreviation is always used to start a command line to be sent from TE to TA

ASCI	Advanced Speech Call Items, including VGCS, VBS and eMLPP

BCD	Binary Coded Decimal

BL	Bandwidth reduced Low complexity

CBR	Channel Busy Ratio

CSG	Closed Subscriber Group

eMLPP	Enhanced Multi-Level Precedence and Pre-emption Service

ETSI	European Telecommunications Standards Institute

FTM	Frame Tunnelling Mode (refer 3GPP TS 27.001 [41] and 3GPP TS 29.007 [42])

HSCSD	High Speed Circuit Switched Data

IMEI	International Mobile station Equipment Identity

IRA	International Reference Alphabet (ITU-T Recommendation T.50 [13])

IrDA	Infrared Data Association

ISO	International Standards Organization

ITU-T	International Telecommunication Union - Telecommunications Standardization Sector

ME	Mobile Equipment

MMTEL	Multimedia Telephony

MoU	Memorandum of Understanding (GSM operator joint)

MT	Mobile Termination

MTU	Maximum Transfer Unit

NB-IoT	NarrowBand Internet of Things

NG-RAN	Next Generation Radio Access Network

NSLPI	NAS Signalling Low Priority Indication

PCCA	Portable Computer and Communications Association

PTT	Push to Talk

RDI	Restricted Digital Information

RLP	Radio Link Protocol

SIM	Subscriber Identity Module

TA	Terminal Adaptor, e.g. a GSM data card (equal to DCE; Data Circuit terminating Equipment)

TE	Terminal Equipment, e.g. a computer (equal to DTE; Data Terminal Equipment)

TIA	Telecommunications Industry Association

UDI	Unrestricted Digital Information

UE	User Equipment

UICC	Universal Integrated Circuit Card

USAT	USIM Application Toolkit

USIM	Universal Subscriber Identity Module

VAE	V2X Application Enabler

VBS	Voice Broadcast Service

VGCS	Voice Group Call Service

* * * Next Change * * * *

# 16	Commands for VAE layer configuration

## 16.1	General

This clause defines AT commands that a TE can use to control MT supporting VAE layer. The VAE layer supports V2X communication over LTE-Uu as defined in 3GPP TS 24.486 [r24486]

A comprehensive set of VAE layer-specific commands is defined to provide the flexibility needed by the more complex MT. The commands are designed to be expandable to accommodate new VAE layer feature options and interface protocols, merely by defining new values for many of the parameters. The commands use the extended information and error message capabilities described in the present specification.

The +CVAEACT command activate or deactivate the VAE layer support in the MT by performing the V2X service discovery procedure according to 3GPP TS 24.486 [r24486] subclause 6.6. Once the VAE layer is enabled, then the MT can request the registration of V2X service identifier(s) by performing the V2X UE registration procedure according to 3GPP TS 24.486 [r24486] subclause 6.2 using the +CVAEREG command.

Editor's note [WI: V2XAPP, CR#690]:	It needs to be defined the AT command syntax and semantics of +CVAEACT command and +CVAEREG command.

* * * End of Change * * * *