3GPP TSG-CT WG1 Meeting #125-e	C1-20XXXX

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6435 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS behavior for EPS fallback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | TS 23.502, TS 23.228 andTS 29.514 enhance the signalling to enable the P-CSCF to optimize the call session setup which was originally initiated in NR. The P-CSCF is therefore made aware once the IMS packet transmission is disabled in the NR and once the IMS packet transmission is reenabled in the EPS. This enables the P-CSCF to improve the IMS session establishment. TS 29.513 elaborates on this enhancement by P-CSCF subscribing to the EPS fallback and access network information to avoid loss of SIP signalling and to perform resource allocation in prior to forwarding the SIP message containing SDP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR implements the procedures for the P-CSCF to subscibe to event notifications defined in TS 29.514 to improve the IMS session establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The functionality that the P-CSCF may choose to perform the resource allocation at the time of EPS fallback prior to forward SIP signalling when the interworking is supported without N26 interface, is not implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, U.3.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------- next Change ----------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[XXX]	3GPP TR 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

----------------------------- next Change ----------------------------------

U.3.2.X EPS fallback

While establishing an IMS session upon authorizing the resources necessary for this IMS session over NR, a P-CSCF may get the information about EPS fallback and the access network information as specified in 3GPP TS 29.514 [273]. A SIP message is transmitted towards the targeted UE in this case after a successful handover from NR to EPS with established voice bearer as specified in 3GPP TS 29.513 [XXX].

----------------------------- end of Change ----------------------------------