3GPP TSG-CT WG1 Meeting #125-e	C1-20qwer

Electronic meeting, 20-28 August 2020 	(was C1-205150)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.484 | CR | 0153 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on configurations documents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONASTERY2 |  |  |  |  |  | Date: |  |  | 2020-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | In the service configuration document a list of MCPTT IDs appears in the structure without any mention of its type. In addition the functional alias lists are missing from the MCData service profile configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)Update the structure to State that the mcptt-user-list is a list element. 2) Update MCData service profile configuration to include FA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear structure of the configuration file and FAs cannot be supported missing elements from MCData configurations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.1, 10.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.4.2.1	Structure

The service configuration document structure is specified in this subclause.

The <service configuration> document:

1)	shall include a "domain" attribute;

2)	may include a <common> element;

3)	may include an <on-network> element;

4)	may include an <off-network> element; and

5)	may include any other attribute for the purposes of extensibility.

The <common> element:

1)	may include a <min-length-alias> element;

2)	may contain a <broadcast-group> element containing:

a)	a <num-levels-group-hierarchy> element; and

b)	a <num-levels-user-hierarchy> element;

The <on-network> element:

1)	may contain a <emergency-call> element containing:

a)	a <private-cancel-timeout> element; and

b)	a <group-time-limit> element.

2)	may contain a <private-call> element containing:

a)	a <hang-time> element;

b)	a <max-duration-with-floor-control> element; and

c)	a <max-duration-without-floor-control> element;

3)	may contain a <num-levels-hierarchy> element;

4)	may contain a <transmit-time> element containing:

a)	a <time-limit> element; and

b)	a <time-warning> element;

5)	may contain a <hang-time-warning> element;

6)	may contain a <floor-control-queue> element containing:

a)	a <depth> element; and

b)	a <max-user-request-time> element; and

7)	shall contain a <fc-timers-counters> element containing:

a)	a <T1-end-of-rtp-media> element;

b)	a <T3-stop-talking-grace> element;

c)	a <T7-floor-idle> element;

d)	a <T8-floor-revoke> element;

e)	a <T11-end-of-RTP-dual> element;

f)	a <T12-stop-talking-dual> element;

g)	a <T15-conversation> element;

h)	a <T16-map-group-to-bearer> element;

i)	a <T17-unmap-group-to-bearer> element;

j)	a <T20-floor-granted> element;

k)	a <T55-connect> element;

l)	a<T56-disconnect> element;

m)	a <C7-floor-idle> element;

n)	a <C17-unmap-group-to-bearer> element;

o)	a <C20-floor-granted> element;

p)	a <C55-connect> element; and

q)	a <C56-disconnect> element;

8)	may contain a <signalling-protection> element containing:

a)	a <confidentiality-protection> element; and

b)	an <integrity-protection> element;

9)	shall include one <emergency-resource-priority> element containing:

a)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

b)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20];

10)	shall include one <imminent-peril-resource-priority> element containing:

a)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

b)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20];

11)	shall include one <normal-resource-priority> element containing:

a)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

b)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20]; and

12)	may contain a <protection-between-mcptt-servers> element containing:

a)	an <allow-signalling-protection> element; and

b)	an <allow-floor-control-protection> element; and

13)	may contain an <anyExt> element containing:

a)	a <functional-alias-list> element containing:

i)	a <functional-alias> element;

ii)	a <max-simultaneous-activations> element;

iii)	an <allow-takeover> element;

iv)	an <mcptt-user-list> element; and

v)	may contain an <anyExt> element containing a <functional-alias-priority> element; and

b)	a <max-simultaneous-authorizations> element.

The <off-network> element:

1)	may contain a <emergency-call> element containing:

a)	a <private-cancel-timeout> element; and

b)	a <group-time-limit> element.

2)	may contain a <private-call> element containing:

a)	a <hang-time> element; and

b)	a <max-duration-with-floor-control> element;

3)	may contain a <num-levels-hierarchy> element;

4)	may contain a <transmit-time> element containing:

a)	a <time-limit> element; and

b)	a <time-warning> element.

5)	may contain a <hang-time-warning> element;

6)	may contain a <default-prose-per-packet-priority> element; and

7)	may contain a <allow-log-metadata> element.

*** Next change ***

#### 10.4.2.1	Structure

The MCData service configuration document structure is specified in this subclause.

The <service configuration> document:

1)	shall include a "domain" attribute;

2)	may include a <common> element;

3)	may include an <on-network> element;

4)	may include an <off-network> element; and

5)	may include any other attribute for the purposes of extensibility.

The <common> element:

1)	may include a <tx-and-rx-control> element containing:

a)	a <time-temp-data-waiting> element.

The <on-network> element:

1)	may include a <tx-and-rx-control> element containing:

a)	a <max-data-size-sds-bytes> element;

b)	a <max-payload-size-sds-cplane-bytes> element;

c)	a <max-data-size-fd-bytes> element;

d)	a <max-data-size-auto-recv-bytes> element;

e)	a <default-file-availability> element; and

f)	a <max-file-availability> element.

2)	may contain a <signalling-protection> element containing:

a)	a <confidentiality-protection> element; and

b)	an <integrity-protection> element;

3)	may contain a <protection-between-mcdata-servers> element containing:

a)	an <allow-signalling-protection> element;

Editor's Note: Mechanisms for signaling protection and media protection are yet to be agreed by SA3. It is expected that configuration for security protection will need to be added. P2P signalling protection would cover signalling content in XML (e.g. group-id) and MCData signalling content. Media protection is E2E between clients.

4)	may contain an <anyExt> element containing:

a)	one <emergency-resource-priority> element containing:

i)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

ii)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20];

b)	one <imminent-peril-resource-priority> element containing:

i)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

ii)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20];

c)	one <normal-resource-priority> element containing:

i)	one <resource-priority-namespace> string element containing a namespace defined in IETF RFC 8101 [20]; and

ii)	one <resource-priority-priority> string element element containing a priority level in the range specified in IETF RFC 8101 [20];

d)	a <max-simultaneous-authorizations> element; and

e)	a <functional-alias-list> element containing:

i)	a <functional-alias> element;

ii)	a <max-simultaneous-activations> element;

iii)	an <allow-takeover> element;

iv)	an <mcdata-user-list> element; and

v)	may contain an <anyExt> element containing a <functional-alias-priority> element.

The <off-network> element:

1)	may contain a <default-prose-per-packet-priority> element containing:

a)	an <mcdata-one-to-one-call-signalling> element; and

b)	an <mcdata-one-to-one-call-media> element.

#### 10.4.2.3	XML Schema

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema attributeFormDefault="unqualified" elementFormDefault="qualified"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

targetNamespace="urn:3gpp:ns:mcdataServiceConfig:1.0"

xmlns:mcdatasc="urn:3gpp:ns:mcdataServiceConfig:1.0">

<!-- the root element -->

<xs:element name="service-configuration-info" type="mcdatasc:service-configuration-info-Type"/>

<!-- the root type -->

<!-- this is refined with one or more sub-types -->

<xs:complexType name="service-configuration-info-Type">

<xs:sequence>

<xs:element name="service-configuration-params" type="mcdatasc:service-configuration-params-Type" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<!-- definition of the service-configuration-params-Type subtype-->

<xs:complexType name="service-configuration-params-Type">

<xs:sequence>

<xs:element name="common" type="mcdatasc:commonType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="on-network" type="mcdatasc:on-networkType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="off-network" type="mcdatasc:off-networkType" minOccurs="0" maxOccurs="unbounded"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="domain" type="xs:anyURI" use="required"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="commonType">

<xs:sequence>

<xs:element name="tx-and-rx-control" type="mcdatasc:common-tx-and-rx-controlType" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="on-networkType">

<xs:sequence>

<xs:element name="tx-and-rx-control" type="mcdatasc:on-network-tx-and-rx-controlType" minOccurs="0"/>

<xs:element name="signalling-protection" type="mcdatasc:signalling-protectionType" minOccurs="0"/>

<xs:element name="protection-between-mcdata-servers" type="mcdatasc:server-protectionType" minOccurs="0"/>

<xs:element name="file-availability" type="mcdatasc:on-network-file-availabilityType"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:element name="emergency-resource-priority" type="mcdatasc:resource-priorityType"/>

<xs:element name="imminent-peril-resource-priority" type="mcdatasc:resource-priorityType"/>

<xs:element name="normal-resource-priority" type="mcdatasc:resource-priorityType"/>

<xs:complexType name="off-networkType">

<xs:sequence>

<xs:element name="default-prose-per-packet-priority" type="mcdatasc:default-prose-per-packet-priorityType" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="default-prose-per-packet-priorityType">

<xs:sequence>

<xs:element name="mcdata-one-to-one-call-signalling" type="xs:unsignedShort" minOccurs="0"/>

<xs:element name="mcdata-one-to-one-call-media" type="xs:unsignedShort" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="common-tx-and-rx-controlType">

<xs:sequence>

<xs:element name="time-temp-data-waiting" type="xs:duration" minOccurs="0"/>

<xs:element name="time-periodic-announcement" type="xs:duration" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="signalling-protectionType">

<xs:sequence>

<xs:element name="confidentiality-protection" type="xs:boolean" minOccurs="0" default="true"/>

<xs:element name="integrity-protection" type="xs:boolean" minOccurs="0" default="true"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="server-protectionType">

<xs:sequence>

<xs:element name="allow-signalling-protection" type="xs:boolean" minOccurs="0" default="true"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="on-network-tx-and-rx-controlType">

<xs:sequence>

<xs:element name="max-data-size-sds-bytes" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="max-payload-size-sds-cplane-bytes" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="max-data-size-fd-bytes" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="max-data-size-auto-recv-bytes" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="on-network-file-availabilityType">

<xs:sequence>

<xs:element name="default-file-availability" type="xs:unsignedInt"/>

<xs:element name="max-file-availability" type="xs:unsignedInt" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="resource-priorityType">

<xs:sequence>

<xs:element name="resource-priority-namespace" type="xs:string"/>

<xs:element name="resource-priority-priority" type="xs:string"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:element name="functional-alias-list" type="mcdatasc:functional-aliasType"/>

<!-- definition of the functional-aliasType subtype-->

<xs:complexType name="functional-aliasType">

<xs:sequence>

<xs:element name="functional-alias" type="xs:anyURI"/>

<xs:element name="max-simultaneous-activations" type="xs:positiveInteger"/>

<xs:element name="allow-takeover" type="xs:boolean"/>

<xs:element name="mcdata-user-list" type="mcdatasc:ListEntryType"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:element name="functional-alias-priority" type="xs:positiveInteger"/>

<xs:element name="max-simultaneous-authorizations" type="xs:positiveInteger"/>

<xs:complexType name="ListEntryType">

<xs:choice minOccurs="0" maxOccurs="unbounded">

<xs:element name="entry" type="mcdatasc:EntryType"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:choice>

<xs:attribute ref="xml:lang"/>

<xs:attributeGroup ref="mcdatasc:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:complexType name="EntryType">

<xs:sequence>

<xs:element name="uri-entry" type="xs:anyURI"/>

<xs:element name="display-name" type="mcdatasc:DisplayNameElementType" minOccurs="0"/>

<xs:element name="anyExt" type="mcdatasc:anyExtType" minOccurs="0"/>

<xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attributeGroup ref="mcdatasc:IndexType"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:attributeGroup name="IndexType">

<xs:attribute name="index" type="xs:token"/>

</xs:attributeGroup>

<xs:complexType name="DisplayNameElementType">

<xs:simpleContent>

<xs:extension base="xs:string">

<xs:attribute ref="xml:lang"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="anyExtType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

</xs:complexType>

</xs:schema>

*** Next change ***

#### 10.4.2.7	Data Semantics

The "domain" attribute of the <service-configuration-params> element contains the domain name of the mission critical organization.

The <common> element contains service configuration data common to both on and off network service.

The <on-network> element contains service configuration data for on-network service only.

The <off-network> element contains service configuration data for off-network service only.

In the <common> element:

1)	the <time-temp-data-waiting> element of the <tx-and-rx-control> element contains the time limit for the temporarily stored data that is waiting to be delivered to a receiving user which corresponds to the "TimeTempDataWaiting" element as specified in subclause 11.2.7 of 3GPP TS 24.483 [4]; and

2)	the <time-periodic-announcement> element of the <tx-and-rx-control> element contains the timer for the periodic announcement which contains a list of available recently invited data group communications which corresponds to the "TimePeriodicAnnouncement" element as specified in subclause 11.2.8 of 3GPP TS 24.483 [4].

In the <on-network> element:

1)	the <max-data-size-sds-bytes> element of the <tx-and-rx-control> element contains the maximum data that the originating client can send in an SDS message;

2)	the <max-payload-size-sds-cplane-bytes> element of the <tx-and-rx-control> element contains the maximum payload data that the originating client can send in an SDS message over C-plane;

3)	the <max-data-size-fd-bytes> element of the <tx-and-rx-control> element contains the maximum data that the originating client can send in an FD message;

4)	the <max-data-size-auto-recv-bytes> element of the <tx-and-rx-control> element contains the maximum data that the server can send to the terminating client without requesting the user to indicate a present need for the data;

5)	the <default-file-availability> element of the <file-availability> element contains the default time for which a file is available on the server for download, if a explicit time period is not requested by the originating client;

6)	the <max-file-availability> element of the <file-availability> element contains the maximum time for which a file can be made available on the server for download.

7)	the <confidentiality-protection> element of the <signalling-protection> element contains a boolean indicating whether confidentiality protection of MCData signalling is enabled or disabled between the MCData client and MCData server;

8)	the <integrity-protection> element of the <signalling-protection> element contains a boolean indicating whether integrity protection of MCData signalling is enabled or disabled between the MCData client and MCData server;

9)	The <emergency-resource-priority> element of the <anyExt> element is of type "resource-priorityType" and indicates how a Resource-Priority header field is to be populated for MCData emergency communications;

10)	The <imminent-peril-resource-priority> element of the <anyExt> element is of type "resource-priorityType" and indicates how a Resource-Priority header field is to be populated for MCData Imminent Peril communications;

11)	The <normal-resource-priority> element of the <anyExt> element is of type "resource-priorityType" and indicates how a Resource-Priority header field is to be populated when downgrading to normal priority from an MCData emergency communication or MCData imminent peril communication;

12)	the <allow-signalling-protection> element of the <protection-between-mcdata-servers> element contains a boolean indicating whether protection of MCData signalling is enabled between MCData servers;

13)	the <max-simultaneous-authorizations> element of the <anyExt> element is of type "positiveInteger" and indicates the maximum allowed number of simultaneous service authorizations for an MCData user;

NOTE 1:	The default values of the <confidentiality-protection> element, the <integrity-protection> element, the <allow-signalling-protection> element and the <allow-floor-control-protection> element are "true".

14)	the <functional-alias> element of the <functional-alias-list> element is of type "anyURI" and contains the identity of a functional alias;

15)	the <max-simultaneous-activations> element of the <functional-alias> element of the <functional-alias-list> element is of type "positiveInteger" and contains the allowed number of concurrent activations that are allowed for the functional alias contained in the corresponding <functional-alias> element;

16)	the <allow-takeover> element of the <functional-alias-list> element is of type "boolean" and indicates whether take over by another MCData user is allowed for a currently activated functional alias contained in the corresponding <functional-alias> element;

17)	the <entry> element of the <mcdata-user-list> element of the <functional-alias-list> element is of type "entryType" and contains the MCData ID of an MCData user that is allowed to activate the functional alias contained in the corresponding <functional-alias> element;

18)	the <functional-alias-priority> element of the <functional-alias-list> element is of type "positiveInteger" and indicates the relative priority level of the functional alias contained in the corresponding <functional-alias> element; and

NOTE 2:	The usage of this parameter by the MCData server is up to implementation.

19)	the <max-simultaneous-authorizations> element of the <anyExt> element is of type "positiveInteger" and indicates the maximum allowed number of simultaneous service authorizations for an MCData user.

In the <off-network> element:

1)	the <default-prose-per-packet-priority> element contains priority values for off-network calls, for each of the following constituent elements:

a)	<mcdata-one-to-one-call-signalling> element, which corresponds to the "MCDataOneToOneSignalling" element as specified in subclause 11.2.11 of 3GPP TS 24.483 [4]; and

b)	<mcdata-one-to-one-call-media> element, which corresponds to the "MCDataOneToOneMedia" element as specified in subclause 11.2.12 of 3GPP TS 24.483 [4].

*** End of changes ***