3GPP TSG-CT WG1 Meeting #125-e	C1-20xxxx

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | xxxx | rev | x | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The requirements of Rejected NSSAI for unknown cause value |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,ZTE, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc16 |  |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | F |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  | The Rejected NSSAI IE contains multiple rejected S-NSSAIs, and each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. Considering that: - An early version UE may not know a cause value defined in late version. - The cause value indicates the associated S-NSSAI belonging to VPLMN or HPLMN. - It’s not proper for UE to store a rejected S-NSSAI with an unknown cause, although the S-NSSAI is rejected by the network for some reason. - Each rejected S-NSSAI can be decoded independently. - It’s not proper for UE to ignore a whole Rejected NSSAI IE due to a reserved cause value within in one of the rejected S-NSSAIs, since other rejected S-NSSAIs can be decoded normally. It is suggested that the UE shall not ignore the whole IE but skip the Rejected S-NSSAI with the reserved value. That is, UE can keep other rejected S-NSSAIs with a known cause value. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | Specify that UE ignores the rejected S-NSSAI with an unknown cause value, and keeps other rejected S-NSSAIs in the Rejected NSSAI IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - UE may store a rejected S-NSSAI without knowing it belongs to VPLMN or HPLMN. - UE may ignore a whole Rejected NSSAI IE due to an unknown cause value, which could impact other rejected S-NSSAIs decoded normally. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 9.11.3.46 |  |  |  |  |  |  |  |  |
|  |  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

***************************** NEXT CHANGE *************************************

9.11.3.46	Rejected NSSAI

The purpose of the Rejected NSSAI information element is to identify a collection of rejected S-NSSAIs.

The Rejected NSSAI information element is coded as shown in figure 9.11.3.46.1, figure 9.11.3.46.2 and table 9.11.3.46.1.

The Rejected NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 42 octets.

NOTE:	The number of rejected S-NSSAI(s) cannot exceed eight.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rejected NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Rejected NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.46.1: Rejected NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of rejected S-NSSAI |  |  |  | Cause value |  |  |  | octet 1 |
| SST |  |  |  |  |  |  |  | octet 2 |
| SD |  |  |  |  |  |  |  | octet 3* octet 5* |

Figure 9.11.3.46.2: Rejected S-NSSAI

Table 9.11.3.46.1: Rejected NSSAI information element

| Value part of the Rejected NSSAI information element (octet 3 to v) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Rejected NSSAI information element consists of one or more rejected S-NSSAIs. Each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. The length of each rejected S-NSSAI can be determined by the 'length of rejected S-NSSAI' field in the first octet of the rejected S-NSSAI. |  |  |  |  |  |
| The UE shall store the complete list received. If more than 8 rejected S-NSSAIs are included in this information element, the UE shall store the first 8 rejected S-NSSAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Rejected S-NSSAI: |  |  |  |  |  |
| Cause value (octet 1) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 |  | S-NSSAI not available in the current PLMN or SNPN |
| 0 | 0 | 0 | 1 |  | S-NSSAI not available in the current registration area |
| 0 | 0 | 1 | 0 |  | S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization. |
| All other values are reserved. The UE shall ignore the rejected S-NSSAI associated with a reserved cause value. |  |  |  |  |  |
| Slice/service type (SST) (octet 2) |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. (NOTE 2) |  |  |  |  |  |
| Slice differentiator (SD) (octet 3 to octet 5) |  |  |  |  |  |
| This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. (NOTE 3) |  |  |  |  |  |
| NOTE 1: If octet 3 is included, then octet 4 and octet 5 shall be included. NOTE 2: If the Cause value is “S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization”, this field shall contain the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. NOTE 3: If the Cause value is “S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization”, this field shall contain the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN. |  |  |  |  |  |

***************************** END of CHANGE *********************************