3GPP TSG-CT WG1 Meeting #125-e	C1-20wxyz

Electronic meeting, 20-28 August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.379 | CR | CR# | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial – SIP URI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17 |  |  |  |  |  | Date: |  |  | 20 August 2020 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The TS contains one occurrence of "SIP-URI". It should be "SIP URI". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the occurrence to "SIP URI". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorially incorrect specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### * * * * * FIRST CHANGE * * * * *

##### 6.6.2.1.5	Sending a SIP INVITE request on receipt of SIP 3xx response

This clause is referenced from other procedures.

Upon:

1)	having sent a SIP INVITE request to the controlling MCPTT function; and

2)	having received a SIP 302 (Moved Temporarily) response from the controlling MCPTT function sent to the SIP INVITE request in step 1) with:

a)	a Contact header field containing a SIP URI; and

b)	an application/vnd.3gpp.mcptt-info+xml MIME body with an <mcptt-request-uri> element;

the IWF performing the participating role:

1)	shall generate a SIP INVITE request with the Request-URI set to the contents of the Contact header field of the SIP 302 (Moved Temporarily) response;

2)	shall include in the SIP INVITE request all Accept-Contact header fields and all Reject-Contact header fields, with their feature tags and their corresponding values along with parameters according to rules and procedures of IETF RFC 3841 [5] included in the original SIP INVITE request sent to the controlling MCPTT function;

3)	should include the Session-Expires header field according to IETF RFC 4028 [6]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

4)	shall include the option tag "timer" in the Supported header field;

5)	shall set the P-Asserted-Identity header field of the outgoing SIP INVITE request to the contents of the P-Asserted-Identity header field of the original SIP INVITE request sent to the controlling MCPTT function;

6)	shall include the g.3gpp.mcptt media feature tag into the Contact header field of the outgoing SIP INVITE request;

7)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the outgoing SIP INVITE request;

8)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [3]), into the P-Asserted-Service header field of the outgoing SIP INVITE request;

9)	shall copy the contents of the application/vnd.3gpp.mcptt-info+xml MIME body of the original SIP INVITE request sent to the controlling MCPTT function into the outgoing SIP INVITE request;

10)	shall copy the contents of the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body received in the SIP 302 (Moved Temporarily) response, to the <mcptt-request-uri> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the outgoing SIP INVITE request;

11)	shall set the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the MCPTT ID of the calling user that was determined by the IWF performing the participating role; and

12)	if the <session-type> element is received in the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP 3xx response, shall set the <session-type> element of the application/vnd.3gpp.mcptt-info+xml MIME body of the SIP INVITE request to the value of the <session-type> element received in the SIP 3xx response.

##### * * * * * END CHANGES * * * * *