3GPP TSG-CT WG1 Meeting #125-e	C1-205351

Electronic meeting, 20-28 August 2020	(was C1-205182)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 2552 | rev | 2 | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Type of the N5GC indication information element |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, InterDigital, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-08-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | A type 2 was introduced to TS 24.501 when this is not good: (1) Firstly, it is inconsistent with the description of the information element in the specification, quote of clause 5.5.1.2.2: If the W-AGF acting on behalf of an N5GC device initiates an initial registration as specified in 3GPP TS 23.316 [6D], the W-AGF acting on behalf of the N5GC device shall include the N5GC indication IE with the N5GC device indication bit set to "N5GC device registration is requested" in the REGISTRATION REQUEST message. The above description implies that the use of type 2 for the N5GC indication is as matter of fact not possible; (2) IEIs of type 2 do not exist in the specification and neither in NAS for EPS (TS 24.301), and this is because of the fact that they can create trouble as already discussed during CT1#123-e (see discussion on C1-202083 on the 3GPP_TSG_CT_WG1_E_MAIN@LIST.ETSI.ORG mailing list for further details). For example, note that once type 2 IEIs are introduced in a message new type 1 IEI in that message will be not be possible any longer, e.g. if a type 2 IEI “A1” is introduced in a message, then a type 1 IEI “A-“ would not be possible any longer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Type 1 is proposed instead for the N5GC indication information element. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not appropriate type for the N5GC indication information element remains in the specification. Inconsistency within the specification on the N5GC indication information element remains which leads to problems for implementation in both the UE and the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.6.1, 9.11.3.72 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.6.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the AMF. See table 8.2.6.1.1.

Message type:	REGISTRATION REQUEST

Significance:	dual

Direction:		UE to network

Table 8.2.6.1.1: REGISTRATION REQUEST message content

| IEI |  | Information Element |  | Type/Reference |  | Presence |  | Format |  | Length |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Extended protocol discriminator |  | Extended Protocol discriminator 9.2 |  | M |  | V |  | 1 |  |
|  |  | Security header type |  | Security header type 9.3 |  | M |  | V |  | 1/2 |  |
|  |  | Spare half octet |  | Spare half octet 9.5 |  | M |  | V |  | 1/2 |  |
|  |  | Registration request message identity |  | Message type 9.7 |  | M |  | V |  | 1 |  |
|  |  | 5GS registration type |  | 5GS registration type 9.11.3.7 |  | M |  | V |  | 1/2 |  |
|  |  | ngKSI |  | NAS key set identifier 9.11.3.32 |  | M |  | V |  | 1/2 |  |
|  |  | 5GS mobile identity |  | 5GS mobile identity 9.11.3.4 |  | M |  | LV-E |  | 6-n |  |
| C- |  | Non-current native NAS key set identifier |  | NAS key set identifier 9.11.3.32 |  | O |  | TV |  | 1 |  |
| 10 |  | 5GMM capability |  | 5GMM capability 9.11.3.1 |  | O |  | TLV |  | 3-15 |  |
| 2E |  | UE security capability |  | UE security capability 9.11.3.54 |  | O |  | TLV |  | 4-10 |  |
| 2F |  | Requested NSSAI |  | NSSAI 9.11.3.37 |  | O |  | TLV |  | 4-74 |  |
| 52 |  | Last visited registered TAI |  | 5GS tracking area identity 9.11.3.8 |  | O |  | TV |  | 7 |  |
| 17 |  | S1 UE network capability |  | S1 UE network capability 9.11.3.48 |  | O |  | TLV |  | 4-15 |  |
| 40 |  | Uplink data status |  | Uplink data status 9.11.3.57 |  | O |  | TLV |  | 4-34 |  |
| 50 |  | PDU session status |  | PDU session status 9.11.3.44 |  | O |  | TLV |  | 4-34 |  |
| B- |  | MICO indication |  | MICO indication 9.11.3.31 |  | O |  | TV |  | 1 |  |
| 2B |  | UE status |  | UE status 9.11.3.56 |  | O |  | TLV |  | 3 |  |
| 77 |  | Additional GUTI |  | 5GS mobile identity 9.11.3.4 |  | O |  | TLV-E |  | 14 |  |
| 25 |  | Allowed PDU session status |  | Allowed PDU session status 9.11.3.13 |  | O |  | TLV |  | 4-34 |  |
| 18 |  | UE's usage setting |  | UE's usage setting 9.11.3.55 |  | O |  | TLV |  | 3 |  |
| 51 |  | Requested DRX parameters |  | 5GS DRX parameters 9.11.3.2A |  | O |  | TLV |  | 3 |  |
| 70 |  | EPS NAS message container |  | EPS NAS message container 9.11.3.24 |  | O |  | TLV-E |  | 4-n |  |
| 74 |  | LADN indication |  | LADN indication 9.11.3.29 |  | O |  | TLV-E |  | 3-811 |  |
| 8- |  | Payload container type |  | Payload container type 9.11.3.40 |  | O |  | TV |  | 1 |  |
| 7B |  | Payload container |  | Payload container 9.11.3.39 |  | O |  | TLV-E |  | 4-65538 |  |
| 9- |  | Network slicing indication |  | Network slicing indication 9.11.3.36 |  | O |  | TV |  | 1 |  |
| 53 |  | 5GS update type |  | 5GS update type 9.11.3.9A |  | O |  | TLV |  | 3 |  |
| 41 |  | Mobile station classmark 2 |  | Mobile station classmark 2 9.11.3.31C |  | O |  | TLV |  | 5 |  |
| 42 |  | Supported codecs |  | Supported codec list 9.11.3.51A |  | O |  | TLV |  | 5-n |  |
| 71 |  | NAS message container |  | NAS message container 9.11.3.33 |  | O |  | TLV-E |  | 4-n |  |
| 60 |  | EPS bearer context status |  | EPS bearer context status 9.11.3.23A |  | O |  | TLV |  | 4 |  |
| 6E |  | Requested extended DRX parameters |  | Extended DRX parameters 9.11.3.26A |  | O |  | TLV |  | 3 |  |
| 6A |  | T3324 value |  | GPRS timer 3 9.11.2.5 |  | O |  | TLV |  | 3 |  |
| 67 |  | UE radio capability ID |  | UE radio capability ID 9.11.3.68 |  | O |  | TLV |  | 3-n |  |
| 35 |  | Requested mapped NSSAI |  | Mapped NSSAI 9.11.3.31B |  | O |  | TLV |  | 3-42 |  |
| 48 |  | Additional information requested |  | Additional information requested 9.11.3.12A |  | O |  | TLV |  | 3 |  |
| 1A |  | Requested WUS assistance information |  | WUS assistance information 9.11.3.71 |  | O |  | TLV |  | 3-n |  |
| A- |  | N5GC indication |  | N5GC indication 9.11.3.72 |  | O |  | T |  | 1 |  |
| 30 |  | Requested NB-N1 mode DRX parameters |  | NB-N1 mode DRX parameters 9.11.3.73 |  | O |  | TLV |  | 3 |  |

* * * Next Change * * * *

#### 9.11.3.72	N5GC indication

The purpose of the N5GC indication information element is to indicate to the network that the registration request by the W-AGF is on behalf of an N5GC device.

The N5GC indication information element is coded as shown in figure 9.11.3.72.1.

The N5GC indication is a type 1 information element.

| 8 | 7 | 6 | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N5GC indication IEI |  |  |  |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | N5GCREG |  | octet 1 |  |

Figure 9.11.3.72.1: N5GC indication

Table 9.11.3.72.1: N5GC indication

| N5GC device indication bit (N5GCREG) (octet 1, bit 1) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | N5GC device registration is not requested |
| 1 | N5GC device registration is requested |
| Bits 2 to 4 are spare and shall be coded as zero, |  |

* * * End of Change * * * *