3GPP TSG-CT WG1 Meeting #126-e	C1-20xxxx

Electronic meeting, 15-23 October 2020												was C1-205871

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.587 | CR | 0120 | rev | 1 | Current version: | 16.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of validity timer for V2X policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | There is a validity timer for the validity of the configuration parameters for V2X communication over PC5 and Uu, if the validity timer for a V2X policy expires, the UE shall initiate the UE-requested V2X policy provisioning procedure, so far, - There is no timer number for this validity timer; and - There is no related description of how to handle this validity timer during the UE-requested V2X policy provisioning procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Have a timer number for the validity timer of V2X policy. - Add the UE handling behaviors for the validity timer of V2X policy during UE-requested V2X policy provisioning procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no timer number defination for the validity timer of V2X policy. How to handle the validity timer of V2X policy is still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1, 5.3.2.3, 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

5.3.2.1	General

The UE-requested V2X policy provisioning procedure enables the UE to request V2X policy from the PCF in the following cases:

a)	if the T5xyz for a V2X policy expires over PC5;

b)	if the T5abc for a V2X policy expires over Uu; or

b)	if there are no valid configuration parameters, e.g., for the current area, or due to abnormal situation.

The UE shall follow the principles of PTI handling for UE policy delivery service procedures defined in 3GPP TS 24.501 [6] clause D.1.2.

* * * Second Change * * * *

5.3.2.3	UE-requested V2X policy provisioning procedure accepted by the network

Upon receipt of and accepting the UE POLICY PROVISIONING REQUEST message, the PCF shall create a MANAGE UE POLICY COMMAND message and shall behave as described in clause D.2.1 of 3GPP TS 24.501 [6].

Upon receipt of the MANAGE UE POLICY COMMAND message with the same PTI as included in the UE POLICY PROVISIONING REQUEST message, the UE shall stop timer T5010 and handles the MANAGE UE POLICY COMMAND message as specified in clause D.2.1 of 3GPP TS 24.501 [6]. The UE shall stop the timer(s) T5xyz and T5abc, if it is running, and start the timer(s) T5xyz and T5abc with the value in the V2X policy (V2XP) included in the MANAGE UE POLICY COMMAND message.

* * * Next Change * * * *

10.2	Timers of provisioning of parameters for V2X configuration procedures

Timers of provisioning of parameters for V2X configuration are shown in table 10.2.1.

Table 10.2.1: Timers of provisioning of parameters for V2X configuration – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |
| --- | --- | --- | --- | --- |
| T5010 | 16s | Transmission of UE POLICY PROVISIONING REQUEST message | MANAGE UE POLICY COMMAND or UE POLICY PROVISIONING REJECT message received | Retransmission of UE POLICY PROVISIONING REQUEST message |
| T5xyz | TBD | Receiption of MANAGE UE POLICY COMMAND message with a timer value for T5xyz in the V2X policy (V2XP) over PC5 | Upon receiving the MANAGE UE POLICY COMMAND message with the V2XP over PC5 | Initiate the UE-requested V2X policy provisioning procedure |
| T5abc | TBD | Receiption of MANAGE UE POLICY COMMAND message with a timer value T5abc in the V2X policy (V2XP) over Uu | Upon receiving the MANAGE UE POLICY COMMAND message with the V2XP over Uu | Initiate the UE-requested V2X policy provisioning procedure |

* * * End of Change * * * *