3GPP TSG-CT WG1 Meeting #127bis-e	C1-210401

- Electronic meeting; 25-29 January 2021									Revision of C1-210132

Source:	China Mobile

Title:	Solution to KI#1-Slice selection by IMS subscription

Spec:	3GPP TR 23.700-10 v0.2.0

Agenda item:	17.3.3

Document for:	Agreement

1. Introduction

Key issue #1 Network Slicing and IMS addresses two scenarios and the the following aspects:

a)	What information the UE needs to be able to select an appropriate slice.

b)	How the IMS network selects an appropriate slice.

c)	How the IMS network handles unavailability of a slice.

d)	When the IMS FEs and UE are in different PLMNs (i.e. roaming), how the two networks interact.

e)	Whether the service offered by the IMS network is aware of the 5GC slice information.

f)	If additional information is needed in the IMS signalling between the IMS network and the UE

g)	Any changes to the P-CSCF discovery procedures due to 5GC slicing.

h)	If any additional information is needed for the UE and IMS network to identify services supported by the IMS network.

i)	IMS backward compatibility to Rel-16.

j)	How an appropriate slice for a user initiating or upgrading to Multimedia Priority Service (MPS) is selected.

This proposal attempts to provide a solution for Key issue #1 a) and b) on Scenario 1(i.e. Each 5GC network slice is associated with a separate and distinct IMS network).

2. Reason for Change

To provide a solution for Key issue #1 a) and b) on Scenario 1.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-10 v0.2.0.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DNN	Data Network Name

IMPI	IM Private Identity

ISIM	IM Services Identity Module

MCC	Mobile Country Code

MNC	Mobile Network Code

NSSAI	Network Slice Selection Assistance Information

S-NSSAI	Single NSSAI

SUPI	Subscription Permanent Identifier

URSP	UE Route Selection Policy

USIM	Universal Subscriber Identity Module

* * * Next Change * * * *

## 6.X Solution #X: Selecting an appropriate slice by IMS subscription

### 6.X.1	Description

This solution addresses Key Issue #1 for scenario 1.

In scenario 1, the IMS traffic of the UE to the separate IMS networks is routed to different 5G network slices. For the separate IMS networks, the UE has corresponding IMS subscriptions which can be differentiated by the information retrieved from ISIM, derived from USIM, configured in the ME or provided by IMS applications.

NOTE:	IMPI may be considered to differentiate the IMS subscriptions. In this case, multiple IMPIs need to be configured instead of only being derived from USIM by the SUPI (e.g. "<SUPI>@ims.mnc<MNC>.mcc<MCC>.3gppnetwork.org").

Case#1: The information of IMS subscriptions can be provided by IMS applications

In this case, when the URSP is used to provide the UE with NSSP to route the IMS services to the separate IMS networks via appropriate slices, IMS applications need to provide the corresponding information of IMS subscriptions which matches the URSP rule.

Case#2: The information of IMS subscriptions retrieved from ISIM, derived from USIM or configured in the ME

In this case, when the URSP is used to provide the UE with NSSP to route the IMS services to the separate IMS networks via appropriate slices, while the separate IMS networks deployed in same DNN, defining the information of IMS subscriptions as a Traffic Descriptor needs to be considered.

The examples of URSP rules are as follows:

URSP rule 1:

-	Traffic Descriptor:

`-	Service Type: IMS.signalling

-	[new Type for IMS subscription]: IMPI x

-	Route Selection Descriptor:

-	Network Slice Selection: S-NSSAI x (dedicated slice for SIP message of IMS network 1).

-	DNN selection: DNN1.

URSP rule 2:

-	Traffic Descriptor:

`-	Service Type: IMS.media.Audio

-	[new Type for IMS subscription]: IMPI y

-	Route Selection Descriptor:

-	Network Slice Selection: S-NSSAI y (dedicated slice for IMS audio media of IMS network 2).

-	DNN selection: DNN1.

- NOTE:	In the scenario of combined case#1 and #2, both requirements need to be considered.

### 6.X.2	Impacts on existing nodes and functions

The PCF and the UE need to support the extended URSP in the case#2.

NOTE:The impact to PCF is expected and is not in the scope of this TR, but in CT3 responsibility and is FFS in CT3.

### 6.X.3	Solution Evaluation

Editor's note:	This clause provides an evaluation of the solution.