3GPP TSG-CT WG1 Meeting #127bis-e	C1-210180

Electronic meeting, 25-29 January 2021

Source:	vivo

Title:	MINT: Updates to the KI#2

Spec:	3GPP TR 24.811

Agenda item:	17.2.9

Document for:	Agreement

1. Reason for Change

When a Disaster Condition applies to a particular PLMN or PLMNs, the PLMN(s) without Disaster Condition in the same country shall be notified that Disaster Condition applies to PLMN(s) or Disaster Condition no longer applies. Now the questions in the KI#2 only consider the corresponding solutions of how to deliver the information on the Disaster Condition to the PLMNs without Disaster Condition. As for what kind of information should be delivered to the PLMNs without Disaster Condition, it is still not involved.

2. Proposal

It is proposed to add the following question in KI#2.

-	What kind of information should be delivered to the PLMNs without Disaster Condition.

* * * Change * * *

## 5.2	Key Issue #2: Notification of applicability on Disaster Condition to PLMNs without Disaster Condition

### 5.2.1	Description

According to 3GPP TS 22.261 [3] subclause 6.31.2.2:

Subject to regulatory requirements or operator’s policy, the 3GPP system shall support a PLMN operator to be made aware of the failure or recovery of other PLMN(s) in the same country when the Disaster Condition is applies, or when the Disaster Condition is not applicable.

The 3GPP system shall support means for a PLMN operator to be aware of the area where Disaster Condition applies.

When a Disaster Condition applies to a particular PLMN or PLMNs, the PLMN(s) without Disaster Condition in the same country shall be notified that Disaster Condition applies to PLMN(s) or Disaster Condition no longer applies. Also as per service requirement quoted above, the PLMN(s) in the same country can be made aware of the area where Disaster Condition applies.

The following questions are expected to be studied within this Key Issue:

-	How to deliver the information on the Disaster Condition to the PLMNs without Disaster Condition;

-	Who or which entity decides the Disaster Condition;

-	How to provide information on the area where Disaster Condition applies; and

-	What kind of information should be delivered to the PLMNs without Disaster Condition other than what is mentioned above.

NOTE:	The interactions between this Key Issue and the Key Issue #8 (Prevention of signalling overload by returning UEs in PLMN previously with Disaster Condition) need to be considered, since preventing UEs from overloading a PLMN previously with Disaster Condition could have an impact on how the PLMNs without Disaster Condition are notified that the Disaster Condition no longer applies.