3GPP TSG-CT WG1 Meeting #127-e	C1-20xxxx

Electronic meeting, 13-20 November 2020											   was C1-207260

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0049 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to V2X USD provisioning procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | V2XAPP |  |  |  |  |  | Date: |  |  | 2020-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As C1-206618 was agreed in CT1 meeting #126e, the name of the elements were updated in the XML schema for V2X USD provisioning procedure. The related procedure therefore needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the name of elements of V2X USD provisioning procedure; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | XML schema is not aligned with procedures and with Stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2, 7.2.3, 8.3, 8.5, 9.2.3, 9.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 7.2.2	Client procedure

Upon receiving an HTTP POST request message containing:

a)	an Accept header field set to "application/vnd.3gpp.vae-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

c)	an application/vnd.3gpp.vae-usd-announcement-info+xml MIME body with a <V2X-USD-announcement-info> element;

the VAE-C:

a)	shall store the received V2X USD information; and

b)	if the SEAL layer (see 3GPP TS 24.548 [13]) indicates that the V2X USD information was sent by unicast, the VAE-C shall send an acknowledgement of the V2X USD information to the VAE-S.

* * * Next Change * * * *

### 7.2.3	Server procedure

For each VAE-C that the VAE-S is sending a V2X USD announcement to, the VAE-S shall generate an HTTP POST request message request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an "application/vnd.3gpp.vae-info+xml" MIME body with a <V2X-USD-announcement-info> element associated with the MBMS bearer used to send V2X messages in the <VAE-info> root element which:

1)	shall include a <V2X-UE-id> element set to the identity of the V2X UE; and

2)	shall include a <V2X-USD-configuration-data> element set to the V2X USD configuration data as specified in 3GPP TS 23.285 [21] which:

i)	shall include a <TMGI> element set to a TMGI value;

ii)

shall include one or more MBMS service area IDs in <mbms-service-area-id> elements in the <mbms-service-areas> element;

iii)

if multiple carriers are supported, shall include the frequency to be used in the <frequency> element; and

iv)

shall include a <V2X-mbms-sdp> element set to the SDP configuration information applicable to MBMS bearer to use for sending V2X messages; and

d)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 2616 [19].

* * * Next Change * * * *

## 8.3	Structure

The VAE document shall conform to the XML schema described in clause 8.4.

The <VAE-info> element shall be the root element of the VAE document.

The <VAE-info> element shall include at least one of the followings:

a)	an <identity> element;

b)	a <registration-info> element;

c)	a <de-registration-info> element;

d)	a <location-tracking-info> element;

e)	a <message-info> element;

f)	a <service-discovery-info> element;

g)	a <local-service-info> element;

h)	an <V2X-USD-announcement-info> element;

i)	a <PC5-parameters-request> element;

j)	a <V2X-app-requirement-notification> element;

k)	a <layer2-group-id-mapping> element;

l)	an <id-list-notification> element;

m)	a <configure-dynamic-group-notification> element;

n)	a <subscription-request> element;

o)	a <subscription-response> element; or

p)	a <network-monitoring-info-notification> element.

The <identity> element shall include a <V2X-UE-id> child element.

The <service-discovery-info> element shall include:

a)	an <identity> element; or

b)	a <result> element and may include a <service-discovery-data> element.

The <service-discovery-data> element shall include a <V2X-service-mapping-list> element.

The <V2X-service-mapping-list> element shall include one or more <V2X-service-map> elements. Each <V2X-service-map> element shall include following elements:

a)	one or more <V2X-service-id> element(s); and

b)	a <V2X-AS-address> element.

The <registration-info> element shall include at least one of the followings:

a)	a <V2X-UE-id> element and one or more <V2X-service-ID> element(s); or

b)	a <result> element.

The <service> element shall include a <V2X-service-id> or a <V2X-MSG-type> child element.

The <de-registration-info> element shall include the followings:

a)	a <V2X-UE-id> element; and

b)	one or more <V2X-service-id> element(s).

The <location-tracking-info> element shall include either:

a)	the following elements:

-	an <identity> element shall include a <V2X-UE-id> element;

-	a <geographical-identifier> element shall include a <geo-id> element; and

-	an <operation> element; or

b)	the following elements:

-	a <result> element; and

-	an <operation> element.

The <geographical-identifier> element shall include one or more <geo-id> elements.

The <message-info> element shall include at least one of the followings:

a)	an <identity> element shall include a <V2X-UE-id> element;

b)	a <group> element shall include a <V2X-group-id>;

c)	a <payload> element;

d)	a <service> element shall include a <V2X-service-id>;

e)	a <geographical-identifier> element shall include a <geo-id> element;

f)	a <message-reception-ind> element;

g)	<message-reception-uri>; or

h)	a <result> element.

The <group> element shall include a <V2X-group-id> child element.

The <local-service-info> element shall include one of the following:

a)	an <identity> element and a <geographical-identifier> element; or

b)	a <result> element and optionally a <local-service-info-content> element.

The <V2X-USD-announcement-info> element shall include the followings:

a)	a <V2X-UE-id> element; and

b)	a <V2X-USD-configuration-data> element which shall include the followings:

1)	a <TMGI> element;

2)

a <mbms-service-areas> element;

3)

a <frequency> element; and

4)

a <V2X-mbms-sdp> element.

The <PC5-parameters-request> element shall include the followings:

a)	a <expiration-time> element;

b)	a <plmn-list> element which shall include one or more <plmn-id> elements;

c)	an <authorized-when-not-served-by-E-UTRAN> element;

d)	a <radio-parameters-list> element which shall include the following elements:

1)	a <radio-parameters-content> element;

2)	a <geographical-area> element which shall include:

i)	a <polygon-area> element; or

ii)	an <ellipsoid-arc-area> element; and

3)	a <operator-managed> element;

e)	a <V2X-service-ids-list > element which shall include the following elements:

1)	a <V2X-service-id> element; or

2)	a <layer-2-id> element.

The <layer2-group-id-mapping> element shall include the followings:

a)	a <dynamic-group-info> element which shall include the following elements:

1)	a <dynamic-group-id> element;

2)	a <group-definition> element; and

3)	a <group-leader-id> element; and

b)	a <prose-layer2-group-id> element.

The <id-list-notification> element shall include the followings:

a)	a <dynamic-group-id> element;

b)	one or more <group-member-id> element(s), each of which shall include the followings:

1)	an <identity> element shall include a <V2X-UE-id> element; and

2)	a <group-scope> element.

The <configure-dynamic-group-notification> element shall include the followings:

a)	a <dynamic-group-id> element; and

b)	one or more <group-member-id> element(s), each of which shall include the followings:

1)	an <identity> element shall include a <V2X-UE-id> element; and

2)	a <group-scope> element.

The <subscription-info> element shall include either:

a)	the following elements:

1)	an <identity> element;

2)	a <subscription-events> element which shall include one or more <event> elements; and

3)	a <triggering-criteria> element; or

b)	the following elements:

1)	an <identity> element; and

2)	a <result> element.

The <triggering-criteria> element shall include at least one of the following elements:

1)	a <cell-change> element shall include one of the following sub-elements:

i)	an <any-cell-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-cell> element shall include a <trigger-id> element; or

iii)	an <exit-specific-cell> element include a <trigger-id> element;

2)	a <tracking-area-change> element shall include one of the following sub-elements:

i)	an <any-tracking-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-tracking-area> element shall include a <trigger-id> element; or

iii)	an <exit-specific-trackin-area> element shall include a <trigger-id> element;

3)	a <plmn-change> element shall include one of the following sub-elements:

i)	an <any-plmn-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-plmn>element shall include a <trigger-id> element; or

iii)	an <exit-specific-plmn> element shall include a <trigger-id> element;

4)	an <mbms-sa-change> element shall include one of the following sub-elements:

i)	an <any-mbms-sa-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbms-sa> element shall include a <trigger-id> element; or

iii)	an <exit-specific-mbms-sa> element shall include a <trigger-id> element;

5)	an <mbsfn-area-change> element shall include one of the following sub-elements:

i)	an <any-mbsfn-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-mbsfn-area> element shall include a <trigger-id> element; or

iii)	an <exit-specific-mbsfn-area> element shall include a <trigger-id> element;

6)	a <periodic-report> element shall include a <trigger-id> element;

7)	a <travelled-distance> element shall include a <trigger-id> element;

8)	a <vertical-application-event> element shall include one of the following sub-elements:

i)	an <initial-log-on> element shall include a <trigger-id> element;

ii)	a <location-configuration-received> element shall include a <trigger-id> element; or

iii)	an <any-other-event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers a request for a location report. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

9)	a <geographical-area-change> element shall include one of the following sub-elements:

i)	an <any-area-change> element shall include a <trigger-id> element;

ii)	an <enter-specific-area> element shall include the following sub-element:

A)	a <geographical-area> element shall include the following two sub-elements:

I)	a <polygon-area> element shall include a <trigger-id> element; or

II)	an <ellipsoid-arc-area> element shall include a <trigger-id> element;

iii)	an <exit-specific-area-type> element shall include a <trigger-id> element;

The <notification-info> element shall include the followings:

a)	a <V2X-ue-id> element; and

b)	a <network-monitoring-info> element, which shall include one or more <trigger-id> elements and may include:

1)	an <uplink-quality-level> element;

2)	a <congestion-level> element;

3)	a <overload-level> element;

4)	a <geographical-area> element which shall include at least one of the followings:

i)	a <cell-area> element; or

ii)	a <tracking-area> element;

5)	a <time-validity> element; or

6)	an <MBMS-level> element which may include:

i)	an <MBMS-coverage-level> element; or

ii)	an <MBMS-bearer-level-event> element.

* * * Next Change * * * *

## 8.5	Data semantics

The <VAE-info> element is the root element of the XML document. The <VAE-info> element contains the <identity>, <registration-info>, <de-registration-info>, <location-tracking.info>, <message-info>, <service-discovery-info>, <local-service-info>, <V2X-USD-announcement-info>, <PC5-parameters-request>, <V2X-app-requirement-notification>, <layer2-group-id-mapping>, <id-list-notification>, <configure-dynamic-group-notification>, <subscription-request>, <subscription-response> and <network-monitoring-info-notification> sub-elements.

<identity> is a mandatory element used to include the identity of a VAL client. The <identity> element contains a <V2X-UE-id> attribute that contains the identity of the VAL client.

<registration-info> element contains the following elements:

a)	<V2X-UE-id>, an element contains the identity of the V2X UE; and

b)	one or more <V2X-service-id> elements. Each <V2X-service-id> element contains the V2X service ID which the V2X UE is interested in receiving (e.g. PSID or ITS AID of ETSI ITS DENM, ETSI ITS CAM); or

c)	<result>, an element which indicates a value either "success" or "fail".

<de-registration-info> element contains the following elements:

a)	<V2X-UE-id>, an element contains the identity of the V2X UE; and

b)	one or more <V2X-service-id> elements. Each <V2X-service-id> element contains the V2X service ID which the V2X UE is no longer interested in receiving (e.g. PSID or ITS AID of ETSI ITS DENM, ETSI ITS CAM).

<service-discovery-info> is a mandatory element used to include the V2X service discovery response information. The <service-discovery-info> element contains either:

a)	an <identity> sub-element; or

b)	a <result> sub-element and an optional <service-discovery-data> sub-element.

The <service-discovery-data> is an optional element shall include a <V2X-service-mapping-list> element which shall include one or more <V2X-service-map> elements.

The <V2X-service-map> element shall include following attributes:

1)	one or more <V2X-service-id> attributes that each contains a V2X service identifier as specified in ETSI TS 102 965 [18] and ISO TS 17419 [20]; and

2)	a <V2X-AS-address> attribute that contains a V2X application server address as specified in 3GPP TS 23.285 [21].

<geographical-identifier>, an optional element specifying one or more geographical area identifiers. This element consists of one or more <geo-id> elements. The <geo-id> element contains a geographical area identity representing a geographical area.

<operation> is a mandatory element which indicates a value either "subscribe" or "unsubscribe".

<group> is an optional element used to include the identity of a VAL group. The <group> element contains a <V2X-group-id> attribute that contains the group identity of a set of VAL clients according to the VAL service.

<payload> is an optional element used to include the payload of the V2X message as specified in ETSI TS 102 965 [18].

<message-reception-ind> is an optional element used to indicate that a reception report is required to be sent.

<message-reception-uri> is an optional element to indicate the destination URI of a requested reception report, and includes a URI as specified in IETF RFC 2616 [19].

<local-service-info-content> is an optional element: V2X server USD information, V2X application server address information and V2X USD information.

<V2X-USD-announcement-info> is an element used to describe the V2X USD information that V2X UE received from the VAE server which contains the <V2X-UE-id> and <V2X-USD-configuration-data> sub-elements.

<V2X-USD-configuration-data> element is a mandatory element set to the V2X USD configuration data as specified in 3GPP TS 23.285 [21] which contains the <TMGI>, <mbms-service-areas>, <frequency> and <V2X-mbms-sdp> sub-elements.

<TMGI> is a mandatory element encoded as specified in 3GPP TS 24.008 [6] excluding the Temporary mobile group identity IEI and the length of Temporary mobile group identity IE contents.

<mbms-service-areas> is a mandatory element which contains one or more <mbms-service-area-id> elements. Each <mbms-service-area-id> contains a MBMS SAI, encoded as specified in 3GPP TS 23.003 [2].

<frequency> is an optional element encoded as specified in 3GPP TS 29.468 [15].

<V2X-mbms-sdp> is mandatory element which contains SDP configuration information encoded as specified in 3GPP TS 24.386 [8] clause 7.2.2.

<expiration-timer> is a mandatory element encoded as specified in 3GPP TS 24.385 [7] clause 5.5.2.

<plmn-id> is a mandatory element encoded as specified in 3GPP TS 23.003 [2].

<authorized-when-not-served-by-E-UTRAN> is a mandatory element encoded as specified in 3GPP TS 24.385 [7] clause 5.5.8.

<radio-parameters-content> is a mandatory element encoded as specified in3GPP TS 36.331 [17] clause 9 for the SL-V2X-Preconfiguration.

<geographical-area> is a mandatory element specifying a geographical area and has the following sub-elements:

a)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 of 3GPP TS 23.032 [3]; and

b)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 of 3GPP TS 23.032 [3].

<operator-managed> is a mandatory element encoded as specified in 3GPP TS 24.385 [7] clause 5.5.19.

<layer-2-id> is a mandatory element encoded as the DestinationLayer2ID specified in 3GPP TS 36.300 [16].

<V2X-app-requirement-notification> element contains a string set to either "success" or "failure" used to indicate success or failure of the network resource adaptation corresponding to the V2X application requirement.

<layer2-group-id-mapping> element contains the following elements:

a)	<dynamic-group-info> element; and

b)	<prose-layer2-group-id>, an element contains the identity of the ProSe Layer-2 Group.

<dynamic-group-info> element contains the following elements:

a)	<dynamic-group-id>, an element contains the identity of the dynamic group;

b)	<group-definition>, an element containing dynamic group definition information; and

c)	<group-leader-id>, an element contains the identity of the group leader.

<id-list-notification> element contains the following sub-elements:

a)	<dynamic-group-id>, an element set to the identity of the dynamic group; and

b)	one or more <group-member-id> element(s), each <group-member-id> element contains the following sub-elements:

1)	<identity> element shall include a <V2X-UE-id> element, an element set to the identity of the joined or left V2X UE; and

2)	<group-scope>, an element that has the value "joined" or "left". The value "joined" means that the V2X UE joined the group. The value "left" means that the V2X UE left the group.

<configure-dynamic-group-notification> element contains the following sub-elements:

a)	<dynamic-group-id>, an element set to the identity of the dynamic group; and

b)	one or more <group-member-id> element(s), each <group-member-id> element contains the following sub-elements:

1)	<identity> element shall include a <V2X-UE-id> element, an element set to the identity of the joined or left V2X UE; and

2)	<group-scope>, an element that has the value "joined" or "left". The value "joined" means that the V2X UE joined the group. The value "left" means that the V2X UE left the group.

<subscription-request> is an optional element which contains the <identity>, <subscription-events> and <triggering-criteria> sub-elements.

<subscription-events> is a mandatory element which contains one or more <events> sub-elements.

<event> element contains a string set to either "uplink degradation" or "congestion" or "overload" or "coverage".

<triggering-criteria>, a mandatory element which contains at least one of the following sub-elements:

a)	<cell-change>, an optional element specifying what cell changes trigger the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-cell-change>, an optional element. The presence of this element specifies that any cell change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-cell>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-cell>, an optional element specifying an NCGI which when exited triggers the VAE-S to send monitoring reports to the VAE-C coded as specified in clause 19.6A in 3GPP TS 23.003 [2]. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

b)	<tracking-area-change>, an optional element specifying what tracking area changes trigger the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-tracking-area-change>, an optional element. The presence of this element specifies that any tracking area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when entered triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] which when exited triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

c)	<plmn-change>, an optional element specifying what PLMN changes trigger the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-plmn-change>, an optional element. The presence of this element specifies that any PLMN change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when entered triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-plmn>, an optional element specifying a PLMN id (MCC+MNC) coded as specified in 3GPP TS 23.003 [2] which when exited triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

d)	<mbms-sa-change>, an optional element specifying what MBMS changes trigger the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-mbms-sa-change>, an optional element. The presence of this element specifies that any MBMS SA change is a trigger for the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-mbms-sa>, an optional element specifying an MBMS service area id which when entered triggers the VAE-S to send monitoring reports to the VAE-C. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-mbms-sa>, an optional element specifying an MBMS service area id which when exited triggers the VAE-S to send monitoring reports to the VAE-C. The MBMS service area id is coded as specified in clause 15.3 in 3GPP TS 23.003 [2] for service area identifier (SAI). This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

e)	<mbsfn-area-change>, an optional element specifying what MBSFN changes trigger a request for the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-mbsfn-area-change>, an optional element. The presence of this element specifies that any MBSFN area change is a trigger for the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-mbsfn-area>, an optional element specifying an MBSFN area which when entered triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<exit-specific-mbsfn-area>, an optional element specifying an MBSFN area which when exited triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

f)	<periodic-report>, an optional element specifying that periodic request for the VAE-S to send monitoring reports to the VAE-C shall be sent. The value in seconds specifies the reporting interval. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

g)	<travelled-distance>, an optional element specifying that the travelled distance shall trigger a request for the VAE-S to send monitoring reports to the VAE-C. The value in metres specified the travelled distance. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

h)	<vertical-application-event>, an optional element specifying what application signalling events triggers the VAE-S to send monitoring reports to the VAE-C. The <vertical-application-event> element has the following sub-elements:

1)	<initial-log-on>, an optional element specifying that an initial log on triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<location-configuration-received>, an optional element specifying that a received location configuration triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string; and

3)	<any-other- event>, an optional element specifying that any other application signalling event than initial-log-on and location-configuration-received triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

i)	<geographical-area-change>, an optional element specifying what geographical are changes trigger the VAE-S to send monitoring reports to the VAE-C. This element consists of the following sub-elements:

1)	<any-area-change>, an optional element. The presence of this element specifies that any geographical area change is a trigger. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string;

2)	<enter-specific-area>, an optional element specifying a geographical area which when entered triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string. The <enter-specific-area> element has the following sub-elements:

i)	<geographical-area>, an optional element containing a <trigger-id> attribute and the following two sub-elements:

A)	<polygon-area>, an optional element specifying the area as a polygon specified in clause 5.2 in 3GPP TS 23.032 [3]; and

B)	<ellipsoid-arc-area>, an optional element specifying the area as an ellipsoid arc specified in clause 5.7 in 3GPP TS 23.032 [3]; and

3)	<exit-specific-area-type>, an optional element specifying a geographical area which when exited triggers the VAE-S to send monitoring reports to the VAE-C. This element contains a mandatory <trigger-id> attribute that shall be set to a unique string.

<subscription-response> is an optional element which contains the <identity> and <result> sub-elements.

The <notification-info> element contains the following sub-elements:

a)	<VAL-UE-id>, an element contains the identity of the V2X UE who subscribes the network monitoring information;

b)	<network-monitoring-info>, an element contains one or more <trigger-id> attributes that identifies the triggering criteria that resulted in the VAE-S sending the monitoring report to the VAE-C. In addition, the <network-monitoring-info> contains the following sub-elements:

1)	<uplink-quality-level>, an optional element contains an integer used to indicate the uplink quality level;

2)	<congestion-level>, an optional element contains an integer used to indicate the congestion level;

3)	<overload-level>, an optional element contains an integer used to indicate the overload level;

4)	<geographical-area>, an optional element contains the following elements:

i)	<cell-area>, an optional element specifying an NCGI which when entered triggers a request for alocation report coded as specified in clause 19.6A in 3GPP TS 23.003 [2] for which the monitoring applies;

ii)	<tracking-area>, an optional element specifying a tracking area identity coded as specified in clause 19.4.2.3 in 3GPP TS 23.003 [2] for which the monitoring applies;

5)	<time-validity>, an optional element specifies the period for which the monitoring applies; and

6)	<MBMS-level>, an optional element contains the following elements:

i)	<MBMS-coverage-level>, an optional element contains an integer used to indicate the MBMS coverage level; or

ii)	<MBMS-bearer-level-event>, an optional element contains an integer used to indicate the MBMS bearer level events.

* * * Next Change * * * *

### 9.2.3	Structure

The VAE client UE configuration document structure is described in clause 7.2 of 3GPP TS 24.546 [11] with the VAE specific clarifications specified in this clause.

The <on-network> element of the <seal-UE-configuration> element specified in clause 7.2 of 3GPP TS 24.546 [11]:

a)	shall include a <VAE-server-ip> element;

b)	shall include a <VAE-server-transport-port> element;

c)	may include an <V2X-USD-announcement-info> element as specified in clause 8; and

d)	may include a <geo-id> element as specified in clause 8.

* * * Next Change * * * *

### 9.2.5	Data semantics

The <VAL-UE-id> element in <seal-UE-configuration> element is V2X UE ID.

The <VAL-Service-id> element in <seal-UE-configuration> element is V2X service ID.

The <VAE-server-ip> element in <on-network> element of <seal-UE-configuration> element is IP address information of the initial VAE server serving the VAE client.

The <VAE-server-transport-port> element in <on-network> element of <seal-UE-configuration> element is port information of the initial VAE server serving the VAE client.

The <V2X-USD-announcement-info> element contains V2X server USD as specified in clause 8.

The <geo-id> element contains GEO ID identity information as specified in clause 8.

* * * End of Change * * * *