3GPP TSG-CT WG1 Meeting #128bis-e	C1-210765

Elbonia, 25 February – 5 March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.526 | CR | 0109 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct N3AN node selection due to permitted absence of "any PLMN" entry |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc16-non3GPP |  |  |  |  |  | Date: |  |  | 2021-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The CR allows inclusion of the "any PLMN" entry in the non-3GPP access node selection information to be optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Permit the "any PLMN" entry in the non-3GPP access node selection information to be optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between stage 2 and stage 3. Stage 3 depends on abnormal procedures for handling the absence of the "any PLMN" entry while stage 2 have permitted absence of the "any PLMN" entry going forward. Inconsistent UE behavior between Rel-16 and future releases (incl. Rel-17). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR 23.501 CR 2402 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also TS 24.502 CR#0184 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Simplified change as proposed by Amer. Improved consequences if not approved, to address one of Lazaros’ concerns. |  |  |  |  |  |  |  |  |

*** First change ***

#### 5.3.3.2	N3AN node selection information

The content of N3AN node selection information contains a sequence of the N3AN node selection information entries. Each N3AN node selection information entry contains a PLMN ID and information for the PLMN ID. The content of N3AN node selection information contains at least an N3AN node selection information entry with information for the HPLMN.

NOTE:	If N3AN node selection information does not contain

an N3AN node selection information entry with information for the HPLMN,

the N3AN node selection information is handled as a syntactically incorrect IE according to 3GPP TS 24.501 [11].

The content is encoded according to figure 5.3.3.2.1, figure 5.3.3.2.2 and table 5.3.3.2.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N3AN node selection information entry 1 |  |  |  |  |  |  |  | octet x+5 |
|  |  |  |  |  |  |  |  | octet y |
| N3AN node selection information entry 2 |  |  |  |  |  |  |  | octet y+1 octet t |
| … |  |  |  |  |  |  |  |  |
| N3AN node selection information entry n |  |  |  |  |  |  |  | octet u octet v |

Figure 5.3.3.2.1: Content of N3AN node selection information

| 8 | 7 | 6 |  | 5 |  | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of N3AN node selection information entry |  |  |  |  |  |  |  |  |  | octet x+5 |
| MCC digit 2 |  |  |  |  |  | MCC digit 1 |  |  |  | octet x+6 |
| MNC digit 3 |  |  |  |  |  | MCC digit 3 |  |  |  | octet x+7 |
| MNC digit 2 |  |  |  |  |  | MNC digit 1 |  |  |  | octet x+8 |
| FQDN format |  |  | Preference |  | Priority |  |  |  |  | octet x+9 |

Figure 5.3.3.2.2: N3AN node selection information entry

Table 5.3.3.2.1: N3AN node selection information

| Length of N3AN node selection information entry (octet x+5) contains length of subsequent fields in the N3AN node selection information entry. |  |  |
| --- | --- | --- |
| PLMN ID (octet x+6 to x+8) field shall be set to zero if it indicates "any_PLMN". Otherwise, |  |  |
| MCC, Mobile country code (octet x+6, and bits 4 to 1 of octet x+7) |  |  |
| The MCC field is encoded as in ITU-T Recommendation E.212 [10], annex A. |  |  |
| MNC, Mobile network code (bits 8 to 5 of octet x+7, and octet x+8) |  |  |
| The encoding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, MNC digit 3 shall be encoded as "1111". |  |  |
| Priority (bits 5 to 1 of octet x+9) indicates the preference order given to N3AN nodes of a PLMN. The lower value indicates higher priority. If the PLMN is the UE's HPLMN or the PLMN ID indicates "any_PLMN", this priority filed shall be ignored. |  |  |
| Preference (bit 6 of octet x+9) indicates which N3AN node type is preferred in this PLMN and is encoded as follows. |  |  |
| 6 |  |  |
| 0 | N3IWF is preferred |  |
| 1 | ePDG is preferred |  |
| FQDN format (bits 8 to 7 of octet x+9) indicates format to be used when the FQDN is constructed by the UE. This field is encoded as follows. |  |  |
| 8 | 7 |  |
| 0 | 0 | Operator identifier based ePDG FQDN format or operator identifier based N3IWF FQDN. |
| 0 | 1 | Tracking/location area identity based ePDG FQDN format or tracking area identity based N3IWF FQDN format. |
| All other values are reserved. |  |  |