3GPP TSG-CT WG1 Meeting #128bis-e	C1-210775

Electronic meeting, 25 February – 5 March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6515 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transfer of existing, active emergency session not successful |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BlackBerry UK Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMSProtoc17 |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | While upon failure to initiate an emergency call the UE may retry on almost any another access, the same flexibility does not apply to active, ongoing emergency calls. The specification is not clear about UE behavior when transfer of an active, ongoing emergency session is not successful. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE, upon learning that the emergency session transfer has failed, shall not automatically try to re-establish a new emergency call. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon being informed an emergency session transfer has failed, the UE shall not automatically initiate a new emergency session. UE permitted to terminate emergency session without user’s consent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.2.2.6.X (NEW), U.2.2.6.Y (NEW), W.2.2.6.Z (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Added “stand-alone” to Annex L. |  |  |  |  |  |  |  |  |

*** First change ***

#### L.2.2.6.X	Abnormal cases

If the UE receives, from the lower layers, a notification that a UE requested PDN connectivity procedure for initiating an emergency session using a stand-alone PDN CONNECTIVITY REQUEST message with request type set to "emergency" has failed (see 3GPP TS 24.301 [8J]), the UE shall make another emergency session setup attempt following domain selection as specified in 3GPP TS 23.167 [4B].

NOTE:	Domain selection as specified in 3GPP TS 23.167 [4B] does not preclude the use of CS domain. When a CS system based on 3GPP TS 24.008 [8] is to be used, clause B.5 applies.

If the UE has an emergency session and receives, from the lower layers, a notification that a UE requested PDN connectivity procedure using a stand-alone PDN CONNECTIVITY REQUEST message associated with the existing emergency session has failed (see 3GPP TS 24.501), the UE shall not perform domain selection as specified in 3GPP TS 23.167 [4B].

*** Next change ***

#### U.2.2.6.Y	Abnormal cases

If the UE receives, from the lower layers, a notification that a UE requested PDU session establishment procedure for initiating an emergency session using a PDU SESSION ESTABLISHMENT REQUEST message with request type set to "initial emergency request" has failed (see 3GPP TS 24.501 [258]), the UE shall make another emergency session setup attempt following domain selection as specified in 3GPP TS 23.167 [4B].

NOTE:	Domain selection as specified in 3GPP TS 23.167 [4B] does not preclude the use of CS domain. When a CS system based on 3GPP TS 24.008 [8] is to be used, clause B.5 applies.

If the UE has an emergency session and receives, from the lower layers, a notification that a UE requested PDU session establishment procedure associated with the existing emergency session has failed (see 3GPP TS 24.501), the UE shall not perform domain selection as specified in 3GPP TS 23.167 [4B].

*** Next change ***

#### W.2.2.6.Z	Abnormal cases

If the UE receives, from the lower layers, a notification that a UE requested PDU session establishment procedure for initiating an emergency session using a PDU SESSION ESTABLISHMENT REQUEST message with request type set to "initial emergency request" has failed (see 3GPP TS 24.501 [258]), the UE shall make another emergency session setup attempt following domain selection as specified in 3GPP TS 23.167 [4B].

NOTE:	Domain selection as specified in 3GPP TS 23.167 [4B] does not preclude the use of CS domain. When a CS system based on 3GPP TS 24.008 [8] is to be used, clause B.5 applies.

If the UE has an emergency session and receives, from the lower layers, a notification that a UE requested PDU session establishment procedure associated with the existing emergency call has failed (see 3GPP TS 24.501), the UE shall not perform domain selection as specified in 3GPP TS 23.167 [4B].