3GPP TSG-CT WG1 Meeting #128-e	C1-2XXX

Electronic meeting, 25 February – 5 March 2021	was C1-210965

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 3050 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MMTEL Voice and MMTEL Video in non-3GPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17-non3GPP |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the following text quoted from M 2.1.1 of TS 24.173 shows, the MMTEL layer may provide the “handover of ongoing MMTEL voice call from non-3GPP access” or “handover of ongoing MMTEL video call from non-3GPP access” relevant information to NAS layer. The ongoing MMTEL voice/video call can be handover from non-3GPP access, that also means the MMTEL voice/video call was performed over non-3GPP access. However there is no corresponding establishment cause for MMTEL voice call and MMTEL video call over non-3GPP access |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the establishment causes for MMTEL voice call and MMTEL video call over non-3GPP access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of establishment causes for MMTEL voice call and MMTEL video call over non-3GPP access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** start of change*****

4.7.2.2	Establishment cause for non-3GPP access

When establishment of an N1 NAS signalling connection over non-3GPP access is initiated, the UE shall:

a)	determine one or more access identities to be associated with the establishment of the N1 NAS signalling connection as specified in subclause 4.5.2 and table 4.5.2.1;

b)	select the establishment cause for non-3GPP access from the determined one or more access identities and the event which triggered initiation of the N1 NAS signalling connection over non-3GPP access by checking the rules specified in table 4.7.2.2.1; and

c)	provide the selected establishment cause for non-3GPP access to the lower layers.

If the access attempt matches more than one rule, the establishment cause for non-3GPP access of the lowest rule number shall be used.

Table 4.7.2.2.1: Mapping table for determination of establishment cause for non-3GPP access

| Rule # | Access identities | Type of access attempt | Requirements to be met | Establishment cause for non-3GPP access |
| --- | --- | --- | --- | --- |
| 1 | 1 | Any | Any | mps-PriorityAccess |
| 2 | 2 | Any | Any | mcs-PriorityAccess |
| 3 | 11, 15 | Any | Any | highPriorityAccess |
| 4 | 12,13,14, | Any | Any | highPriorityAccess |
| 5 | 0 | Emergency | UE is attempting access for an emergency session (NOTE 1) | emergency |
|  |  | UE NAS initiated 5GMM specific procedures | Access attempt is for MO signalling | mo-Signalling |
|  |  | UE NAS initiated 5GMM connection management procedures or 5GMM NAS transport procedure | Access attempt is for MO data | mo-Data |
|  |  | MO SMS over NAS | Access attempt is for MO SMS over NAS | mo-SMS |
|  |  | MO MMTel voice call | Access attempt is for MO MMTel voice call or for NAS signalling connection recovery during ongoing MO MMTel voice call | mo-VoiceCall |
|  |  | MO MMTel video call | Access attempt is for MO MMTel video call or for NAS signalling connection recovery during ongoing MO MMTel video call | mo-Videocall |
| NOTE 1: This includes 5GMM specific procedures while the service is ongoing and 5GMM connection management procedures required to establish a PDU session with request type = "initial emergency request" or "existing emergency PDU session", or to re-establish user-plane resources for such a PDU session. NOTE 2: See subclause 4.5.2, table 4.5.2.1 for use of the access identities of 0, 1, 2, and 11-15. |  |  |  |  |

***** start of change*****