3GPP TSG-CT WG1 Meeting #129-e	C1-21xxxx

# Electronic meeting; 19–23 April 2021										Revision of C1-212059

Source:	China Mobile

Title:	Evaluation of Solutions for KI#7

Spec:	3GPP TR 24.821 v1.0.0

Agenda item:	17.2.4

Document for:	Agreement

1. Introduction

In TR 24.821, Key issue #7 focuses on PLMN selection for emergency call.

This pCR provides an evaluation of the above solutions for this key issue.

2. Reason for Change

An evaluation of solutions for KI#7 is provided.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 24.821 v1.0.0.

* * * First Change * * * *

7.7	Evaluation of Solutions for Key Issue #7

Key issue #7 " Emergency calls " identifies the following issue in subclause 5.7:

-	What modifications in automatic PLMN selection mode are necessary for a UE using satellite access unable to obtain normal service from a PLMN?

There are two candidate solutions for this key issue. The evalution of these solutions is as follows.

Table 7.7-1  Evaluation of the solutions to KI#7

|  | Solution Summary | Evaluation |
| --- | --- | --- |
| Solution 16 | When a UE needs to camp on an acceptable cell with "satellite NG-RAN" access technology, it checks whether a candidate cell belongs to a PLMN allowed in the country where the UE is physically located, according to the solution to Key Issues #1 and #2. | Both solutions cover the UE behavior of camping on an acceptable cell with the consideration of the PLMN ID, according to the solution to KI#1 and KI#2. They are compatible in the situations the UE is unable to find a suitable cell to camp on or receiving a REGISTRATION REJECT with the cause "PLMN not allowed to operate at the present UE location". The two solutions are not compatible in the following situations: - USIM not inserted; - Receiving a REGISTRATION REJECT with a cause other than "PLMN not allowed to operate at the present UE location". |
| Solution 18 | When a UE needs to camp on an acceptable cell with "satellite NG-RAN" access technology, it distinguishes the situations: - When unable to find a suitable cell to camp on or receiving a REGISTRATION REJECT with the cause "PLMN not allowed to operate at the present UE location", it may camp on an acceptable cell according to the solution to Key Issues #1 and #2; - When receiving a REGISTRATION REJECT with a cause other than "PLMN not allowed to operate at the present UE location", it may camp on an acceptable cell of the selected PLMN; - When the USIM is not inserted, it may camp on an acceptable cell irrespective of the PLMN identity, without modifying the behaviour. |  |

Questions for discussion:

- Should the UE with USIM not inserted behave the same as the UE with USIM inserted when camping on an acceptable cell with "satellite NG-RAN" access technology?

- Should the reject causes other than "PLMN not allowed to operate at the present UE location" be considered to help the UE to camp on an acceptable cell of a PLMN allowed to operate in the UE location?

* * * Next Change * * * *