3GPP TSG-CT WG1 Meeting #129-e	C1-212119

Electronic meeting, 19-23 April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 3106 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New cause value for rejected NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-4-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 4.2.11.2 of TS 23.502 v17.1.0, a new rejection cause for rejected NSSAI is defined, which is rejected S-NSSAI due to maximum number of UEs reached. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It proposes to add a new rejection cause for rejected NSSAI, which is rejected S-NSSAI due to maximum number of UEs reached, to keep alignment with stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.3.75 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.11.3.75	Extended rejected NSSAI

The purpose of the Extended rejected NSSAI information element is to identify a collection of rejected S-NSSAIs if UE supports extended rejected NSSAI.

The Extended rejected NSSAI information element is coded as shown in figure 9.11.3.75.1, figure 9.11.3.75.2 and table 9.11.3.75.1.

The Extended rejected NSSAI is a type 4 information element with a minimum length of 4 octets and a maximum length of 74 octets.

NOTE:	The number of rejected S-NSSAI(s) cannot exceed eight.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended rejected NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Extended rejected NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| Rejected S-NSSAI 1 |  |  |  |  |  |  |  | octet 3 octet m |
| Rejected S-NSSAI 2 |  |  |  |  |  |  |  | octet m+1* octet n* |
| … |  |  |  |  |  |  |  | octet n+1* octet u* |
| Rejected S-NSSAI n |  |  |  |  |  |  |  | octet u+1* octet v* |

Figure 9.11.3.75.1: Extended rejected NSSAI information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of rejected S-NSSAI |  |  |  | Cause value |  |  |  | octet 3 |
| SST |  |  |  |  |  |  |  | octet 4 |
| SD |  |  |  |  |  |  |  | octet 5* octet 7* |
| Mapped HPLMN SST |  |  |  |  |  |  |  | octet 8* |
| Mapped HPLMN SD |  |  |  |  |  |  |  | octet 9* octet 11* |

Figure 9.11.3.75.2: Rejected S-NSSAI

Table 9.11.3.75.1: Extended rejected NSSAI information element

| Value part of the Extended rejected NSSAI information element (octet 3 to v) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The value part of the Extended rejected NSSAI information element consists of one or more rejected S-NSSAIs. Each rejected S-NSSAI consists of one S-NSSAI and an associated cause value. Each rejected S-NSSAI also includes the mapped HPLMN S-NSSAI if available The length of each rejected S-NSSAI can be determined by the 'length of rejected S-NSSAI' field in the first octet of the rejected S-NSSAI. |  |  |  |  |  |
| The UE shall store the complete list received. If more than 8 rejected S-NSSAIs are included in this information element, the UE shall store the first 8 rejected S-NSSAIs and ignore the remaining octets of the information element. |  |  |  |  |  |
| Rejected S-NSSAI: |  |  |  |  |  |
| Cause value (octet 3) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 |  | S-NSSAI not available in the current PLMN or SNPN |
| 0 | 0 | 0 | 1 |  | S-NSSAI not available in the current registration area |
| 0 | 0 | 1 | 0 |  | S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization. |
| 0 | 0 | 1 | 1 |  | S-NSSAI not available due to maximum number of UEs reached |
| All other values are reserved. |  |  |  |  |  |
| Slice/service type (SST) (octet 4) |  |  |  |  |  |
| This field contains the 8 bit SST value. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. (NOTE 5) |  |  |  |  |  |
| Slice differentiator (SD) (octet 5 to octet 7) |  |  |  |  |  |
| This field contains the 24 bit SD value. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. (NOTE 6) If the SST encoded in octet 4 is not associated with a valid SD value, and the sender needs to include a mapped HPLMN SST (octet 8) and a mapped HPLMN SD (octets 9 to 11), then the sender shall set the SD value (octets 5 to 7) to "no SD value associated with the SST". mapped HPLMN Slice/service type (SST) (octet 8) This field contains the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SST value is mapped. The coding of the SST value part is defined in 3GPP TS 23.003 [4]. mapped HPLMN Slice differentiator (SD) (octet 9 to octet 11) This field contains the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN to which the SD value is mapped. The coding of the SD value part is defined in 3GPP TS 23.003 [4]. |  |  |  |  |  |
| NOTE 1: Octet 3 and octet 4 shall always be included. NOTE 2: If the octet 5 is included, then octet 6 and octet 7 shall be included. NOTE 3: If the octet 8 is included, then octets 9, 10, and 11 may be included. NOTE 4: If the octet 9 is included, then octet 10 and octet 11 shall be included. NOTE 5: If the Cause value is “S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization”, this field shall contain the 8 bit SST value of an S-NSSAI in the S-NSSAI(s) of the HPLMN and octets 8, 9, 10, and 11 shall not be included. NOTE 6: If the Cause value is “S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization”, this field shall contain the 24 bit SD value of an S-NSSAI in the S-NSSAI(s) of the HPLMN and octets 8, 9, 10, and 11 shall not be included. |  |  |  |  |  |

* * * End of Change * * * *