3GPP TSG-CT WG1 Meeting #129-e	C1-212346

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0072 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Switching modes of operations for V2V communications procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification needs to define the stage 3 details of the switching modes of operations for V2V communications procedure defined in 3GPP TS 23.286 clause 9.8.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the switching modes of operations for V2V communications procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The switching modes of operations for V2V communications procedure is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.X	Switching modes of operations for V2V communications procedure

#### 6.X.1	Client procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with an <communication-status-info > element;

the VAE-C shall generate an HTTP 200(OK) response message according to procedures specified in IETF RFC 2616 [19]. In the HTTP 200(OK) response, the VAE-C:

a)	shall set the Request-URI to the URI included in the received HTTP response for the V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <communication-status-info> element included in the <VAE-info> root element which:

1)	shall include a <V2X-UE-id> element set to the identity of the V2X UE;

2)	shall include a <V2V-communication-mode> element indicating which V2V communication mode supported by the V2X UE;

3)	may include a <V2X-service-id> element corresponding to the communication status;

4)	may include a <cell-info> element indicating the cell information of which the V2X UE is located; and

5)	may include a <communication-link-status-info> element indicating the communication status of the V2X UE; and

d)	shall send the HTTP 200(OK) response towards the VAE-S according to IETF RFC 2616 [19].

#### 6.X.2	Server procedure

In oder to provide the assistance for V2V communication mode switching, the VAE-S may have acquired the application requirement from the V2X application specific server and may generate an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <communication-status-info> element in the <VAE-info> root element which:

1)	shall include a <V2X-UE-id> element set to the identity of the V2X UE; and

2)	may include a <V2X-service-id> element set to the identity of the V2X service being requested; and

d)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 2616 [19].

Based on the reception of the network monitoring information from the 3GPP network or the communication status information from the <communication-link-status-info> element of an HTTP 200(OK) response, the VAE-S may generate an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

a)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <V2V-communication-assistance-info> element in the <VAE-info> root element which:

1)	shall include a <V2X-UE-id> element set to the identity of the V2X UE;

2)	may include a <V2X-service-id> element set to the identity of the V2X service corresponding to the recommendation information; and

3)	shall include a <V2V-communication-assistance> element indicating the assistance information for V2V communication mode switching to the V2X UE; and

d)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 2616 [19].

* * * End of Change * * * *