3GPP TSG-CT WG1 Meeting #129-e	C1-212349

Electronic meeting, 19-23 April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0075 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAE client initiated on network dynamic group information update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification needs to define the stage 3 details of the VAE client initiated on network dynamic group information update procedure defined in 3GPP TS 23.286 clause 9.12.6.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the VAE client initiated on network dynamic group information update procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VAE client initiated on network dynamic group information update procedure is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.8.X	VAE client initiated on network dynamic group information update procedure

#### 6.8.X.1	Client procedure

In oder to update dynamic group information of an on-network V2X dynamic group, the VAE-C shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-C:

a)	shall set the Request-URI to the URI included in the <message-reception-uri> element in the received HTTP POST request message for the V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <dynamic-group-info-update-info> element included in the <VAE-info> root element which:

1)	shall include a <dynamic-group-info> element which shall include at least one of the followings;

i)	a <dynamic-group-id> element set to the identity of the dynamic group;

ii)	a <group-definition> element set to information about the V2X group; and

iii)	a <group-leader-id> element set to the identity of the new group leader; and

2)	shall include an <endpoint-info> element set to the end point information to which response has to be sent;

d)	shall send the HTTP POST request towards the VAE-S according to IETF RFC 2616 [19].

#### 6.8.X.2	Server procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info +xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with an <dynamic-group-info-update-info> element in the <VAE-info> root element;

the VAE-S:

a)	shall check for the V2X user authorization to update the group information, and

1)	if the authorization fails or if the updated group information is not valid, respond with a HTTP 403 (Forbidden) response to the HTTP POST request and skip rest of the steps;

b)	if the update in group information requires consent from other group member(s), shall obtain the consent from the user as specified in clause 6.8.XXX;

c)	shall generate an HTTP 200 (OK) response according to IETF RFC 2616 [19]. In the HTTP 200 (OK) response message, the VAE-S:

1)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

2)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <dynamic-group-info-update-info> element in the <VAE-info> root element which:

i)	shall include a <result> child element set to the value "success" or "failure" indicating success or failure of the Dynamic group information update request;

Editor’s note: Suggestions for the updated group information is FFS.

d)	shall send the HTTP 200 (OK) response towards the VAE-C; and

e)	shall update the group document as specified in clause 6.2.5.1 of TS 24.544 [9].

* * * End of Change * * * *