3GPP TSG-CT WG1 Meeting #130-e	C1-21xxxx

Electronic meeting, 20-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.486 | CR | 0091 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PC5 Provisioning in multi-operator V2X scenarios procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The specification needs to define the stage 3 details of the PC5 Provisioning in multi-operator V2X scenarios procedure defined in 3GPP TS 23.286 clause 9.15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the PC5 Provisioning in multi-operator V2X scenarios procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PC5 Provisioning in multi-operator V2X scenarios procedure is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.X	PC5 Provisioning in multi-operator V2X scenarios procedure

6.X.1	Client procedure

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <PC5-provisioning-status-info> element;

the VAE-C shall generate an HTTP 200(OK) response message according to procedures specified in IETF RFC 2616 [19]. In the HTTP 200(OK) response, the VAE-C:

a)	shall set the Request-URI to the URI included in the received HTTP response for the V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <PC5-provisioning-status-info> element included in the <VAE-info> root element which:

1)	shall include a <result> element set to the value "success" or "failure" indicating success or failure of the PC5 provisioning status request; and

2)	shall include a <PC5-policy-status-report> corresponding to the PC5 policy status request; and

d)	shall send the HTTP 200(OK) response towards the VAE-S according to IETF RFC 2616 [19].

Upon receiving an HTTP POST request message containing:

a)	a Content-Type header field set to "application/vnd.3gpp.vae-info+xml"; and

b)	an application/vnd.3gpp.vae-info+xml MIME body with a <multi-operator-PC5-provisioning-info> element;

the VAE-C shall generate an HTTP 200(OK) response message according to procedures specified in IETF RFC 2616 [19]. In the HTTP 200(OK) response, the VAE-C:

a)	shall set the Request-URI to the URI included in the received HTTP response for the V2X service discovery procedure (see clause 6.6);

b)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

c)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <multi-operator-PC5-provisioning-info> element included in the <VAE-info> root element which:

1)	shall include a <result> element set to the value "success" or "failure" indicating success or failure of the multi-operator V2X provisioning request; and

d)	shall send the HTTP 200(OK) response towards the VAE-S according to IETF RFC 2616 [19].

6.X.2	Server procedure

Upon receiving a V2X PC5 provisioning requirement from the V2X application specific server, the VAE-S:

a) may generate an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

1)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

3)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <PC5-provisioning-status-info> element in the <VAE-info> root element which:

A)	shall include a <VAE-server-id> element set to the identity of the VAE server which is requester of the PC5 parameters status;

B)	shall include a <V2X-service-id> element set to the identity of the V2X service for which the VAE server's request corresponds to; and

C)	may include a <PC5-provisioning-status-report-configuration> element set to the configuration of the VAE-client reporting related to the PC5 Policy/Parameters status, and optionally PC5 events like PC5 unavailability, PQI load/congestion info; and

4)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 2616 [19];

NOTE: if step a) is performed, the VAE-S shall wait until the response of step a) from the VAE-C and then perform step b).

b)	shall determine the updated PC5 provisioning policies to be jointly used across the V2X-UEs within the multi-operator V2X service and shall generate an HTTP POST request according to procedures specified in IETF RFC 2616 [19]. In the HTTP POST request, the VAE-S:

1)	shall set the Request-URI to the URI corresponding to the identity of the V2X UE;

2)	shall include a Content-Type header field set to "application/vnd.3gpp.vae-info+xml";

3)	shall include an application/vnd.3gpp.vae-info+xml MIME body with a <multi-operator-PC5-provisioning-info> element in the <VAE-info> root element which:

A)	shall include a <VAE-server-id> element set to the identity of the VAE server which is requester of the Multi-operator V2X provisioning request;

B)	shall include a <V2X-service-id> element set to the identity of the V2X service for which the VAE server's request corresponds to;

C)	shall include a <PC5-provisioning-policies> element set to the common provisioning parameters to be used by the V2X-UEs within the V2X service; and

D)	may include a <time-validity> element set to the time window for which the V2X provisioning applies; and

4)	shall send the HTTP POST request towards the VAE-C according to IETF RFC 2616 [19].

* * * End of Change * * * *