3GPP TSG-CT WG1 Meeting #131-e	C1-21XXXX

# E-meeting, 19-27 August 2021

Source:	Lenovo, Motorola Mobility

Title:	Network slice capability management procedures

Spec:	3GPP TS 24.549

Agenda item:	17.2.23

Document for:	Agreement

1. Introduction

WID for network slice capability management has been agreed and this CR is adding new needed features for 3GPP TS 24.549.

2. Reason for Change

Adding network slice capability management procedures. Plus related references and related abbreviations.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.549.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows;".

[Y]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[Z]	3GPP TS 24.547: "Identity management - Service Enabler Architecture Layer for Verticals (SEAL); Protocol specification;".

[W]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[r6750]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[r7159]	IETF RFC 7159: "The JavaScript Object Notation (JSON) Data Interchange Format".

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GCN	5G Core Network

DNN	Data Network Name

HTTP	Hypertext Transfer Protocol

PCF	Policy Control Function

SEAL	Service Enabler Architecture Layer

SNSCM-C	SEAL Network Slice Capability Management Client

SNSCM-S	SEAL Network Slice Capability Management Server

S-NSSAI	Single Network Slice Selection Assistance Information

URSP	UE Route Selection Policy

VAL	Vertical Application Layer

* * * Next Change * * * *

## 6.1	General

The network slice capability management procedures is a SEAL service providing capabilities for network slice re-mapping from one VAL application to one or more other VAL applications, 3GPP TS 23.434 [X]. The network server entity, providing the functionality for the network slice re-mapping, acts as an AF communicating with 5GCN to provide guidance to update and modify the S-NSSAIs and the DNNs of the route selection descriptors of the URSP rules, 3GPP TS 24.526 [Y], for one or more application traffics per UE.

* * * Next Change * * * *

## 6.2	On-network procedures

### 6.2.1	General

#### 6.2.1.1	Authenticated identity in HTTP request

Upon receiving an HTTP POST request from SNSCM-C, the SNSCM-S shall authenticate the identity of the sender of the HTTP POST request is authorized as specified in 3GPP TS 24.547 [Z], and if authentication is successful, the SNSCM-S shall use the identity of one or more VAL UEs of the HTTP POST request as authenticated identities.

### 6.2.2	Event triggered network slice adaptation

#### 6.2.2.1	General

These subclauses describes the procedures on the client and server side when a request for network slice adaptation is sent by the client to the server. The network slice adaptation request may be sent by a VAL server for the adaptation of the network slice to the VAL application. The network slice adaptation request may be sent by the SNSCM-C acting as application client requesting for a new slice enablement.

NOTE:	The interaction between VAL server and SNSCM-S is out of scope of this specification.

#### 6.2.2.2	Client procedure

##### 6.2.2.2.1	SNSCM client event triggered

In order to request for network slice adaptation, the SNSCM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 7231 [Y]. In the HTTP POST request message, the SNSCM-C:

a)	shall set the Request-URI to the URI identifying the SNSCM-C appended with VAL service identity and the value "/UE-triggered-slice-adaptation";

b)	shall include a Host header field with public user identity of SNSCM-C; and

c)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [r6750]; and

c)	shall include the parameters specified in annex A serialized into a JavaScript Object Notation (JSON) structure as specified in IETF RFC 7159 [r7159].

#### 6.2.2.3	Server procedure

Upon receipt an HTTP POST request from the SNSCM-C for network slice adaptation, the SNSCM-S shall determine the identity of the sender as specified in clause 6.2.1.1 to confirm whether the sender is authorized or not. If:

a)	the sender is not an authorized user, the SNSCM-S shall respond with an HTTP 403 (Forbidden) response and avoid the rest of steps; or

b)	the sender is an authorized user, the SNSCM-S:

i)	shall attempt to update the network slice for one or more VAL UEs for the VAL service by using the parameters from the HTTP POST request message which are described in table A.1.2-1 of annex A;

ii)	shall send the updated network slice and any new DNN to the PCF, if the update is successful, 3GPP TS 23.434 [X]; and

iii)	shall send an HTTP 200 response message containing the successful or failure status of the requested network slice adaptation to the SNSCM-C.

* * * Next Change * * * *

## 6.3	Off-network procedures

The off-network procedures are out of scope of the present document in this release of the specification.

* * * Next Change * * * *

Annex <A> (normative):
Parameters for different operations

# A.1	Event triggered network slice adaptation

## A.1.1	General

The information in this annex provides a normative description for the parameters which are used by the SNSCM-C to trigger a network slice adaptation for a VAL application by sending to the SNSCM-S.

## A.1.2	Client side parameters

The SNSCM-C use the parameters shown in table A.1.2-1 to trigger network slice adaptation for a VAL application.

Table A.1.2-1: Client side parameters for network slice adaptation trigger

| Parameter | Description |
| --- | --- |
| VAL UE List | REQUIRED. Represents a space-separated list of VAL UE Ids within the VAL service, for which the network slice adaptation trigger applies. |
| VAL service ID | REQUIRED. The VAL service ID of the VAL application |
| Requested S-NSSAI | REQUIRED. The new S-NSSAI which is requested |
| Requested DNN | OPTIONAL. The new DNN which is requested |
| Slice adaptation cause | OPTIONAL. Indicates the cause for the slice adaptation. |

* * * End of Change * * * *