3GPP TSG-CT WG1 Meeting #132-e	C1-215659

# E-meeting, 11-15 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0256 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Delete notification channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Specifying the procedure for deleting a notification channel. The delete request is sent from the Message notification client to the MCData notification server (see procedure in figure 7.13.3.17.3-3 as well as subclauses 7.13.3.17.3, 7.13.3.1.43 & 7.13.3.1.44 in TS 23.282). |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure is defined to delete a notification channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Message store client will not have a standard mechanism to manage and stop receiving notifications from MCData message store as defined in TS 23.282. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.2.Z (NEW), 21.2.Z.1 (NEW), 21.2.Z.2 (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

### 21.2.Z	Delete notification channel

#### 21.2.Z.1	Message notification client procedures

To delete a notification channel, the Message notification client, acting as an HTTP client shall follow the procedure described in subclause 6.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [xx] with the following clarification(s):

1)	shall generate an HTTP DELETE request as specified in subclause 6.2.6 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [xx] with following the clarifications:

a)	shall set the Host header field to a hostname identifying the MCData Notification server;

b)	shall include a valid MCData access token in the Authorization header; and

c)	shall send the HTTP DELETE request towards the MCData Notification server.

Upon receipt of a HTTP response, the Message notification client should follow the procedure as described in subclause 6.2.2 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [xx].

NOTE:	Upon successful deletion of the notification channel, the Message store client is expected to remove the notification subscription in the MCData Message store function as described in subclause 21.2.13A.

#### 21.2.Z.2	MCData Notification server procedures

Upon receipt of the HTTP DELETE request from the client, as per subclause 21.2.Z.1, with the Request-URI identifying a resource in the MCData Notification server, the MCData Notification server acting as an HTTP server:

1)	shall validate the MCData access token (with "Bearer" authentication scheme) received in the Authorization header of the request as specified in 3GPP TS 24.482 [24];

2)	if validation is successful then

a)	shall process the HTTP DELETE request by following the procedures described in subclause 6.2.6 of OMA-TS-REST_NetAPI_NotificationChannel-V1_0-20200319-C [xx]; and

3)	shall generate and send an HTTP response towards the Message notification client indicating the result of the operation.

* * * * * * * END CHANGES * * * * * * *