(3GPP TSG-CT WG1 Meeting #133-e	C1-216646

Electronic meeting, 11-19 November 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.483 | CR | 0138 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G Data Network Config parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | FirstNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCover5GS |  |  |  |  |  | Date: |  |  | 11 November 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The work in TS 23.289 v17.0.0 specifies that "The DNN shall be made available to the MC service UE either via UE (pre)configuration or via initial UE configuration on a per HPLMN and optionally also per VPLMN basis." |  |  |  |  |  |  |  |  |
| Summary of change: |  | Data network (DN) information is configured for each DN in the HPLMN and VPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not have access to the needed DNs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (all new) 8.2.27A, 8.2.27B, 8.2.27C, 8.2.27D, 8.2.27E, 8.2.39A, 8.2.39B, 8.2.39C, 8.2.39D, 8.2.39E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*  *  *  *  *  FIRST CHANGE  *  *  *  *  *

### 8.2.27A	/<x>/OnNetwork/HPLMN/Service/DNinfo

Table 8.2.27A.1: /<x>/OnNetwork/HPLMN/Service/DNinfo

| OnNetwork/HPLMN/Service/DNinfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the connection to a Data Network in the HPLMN. |  |  |  |  |

### 8.2.27B	/<x>/OnNetwork/HPLMN/Service/DNinfo/<x>

Table 8.2.27B.1: /<x>/OnNetwork/HPLMN/Service/DNinfo/<x>

| OnNetwork/HPLMN/Service/DNinfo/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per HPLMN basis. |  |  |  |  |

### 8.2.27C	/<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNN

Table 8.2.27C.1: /<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNN

| OnNetwork/HPLMN/Service/DNinfo/<x>/DNN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The DNN parameter specifies the Data Network Name of a data network that the UE can attach to in the HPLMN. |  |  |  |  |

-	Value: A character string.

### 8.2.27D	/<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNAAAServer

Table 8.2.27D.1: /<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNAAAServer

| OnNetwork/HPLMN/Service/DNAAAserver/<x>/DNAAAServer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | URI | Get, Replace |  |
|  | The DNAAAserver parameter specifies the URI to reach the AAA server for the data network in the HPLMN. |  |  |  |  |

-	Value: A valid URI.

### 8.2.27E	/<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNPDUSessionType

Table 8.2.27E.1: /<x>/OnNetwork/HPLMN/Service/DNinfo/<x>/DNPDUSessionType

| OnNetwork/HPLMN/Service/DNinfo/<x>/DNPDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The DNPDUSessionType parameter specifies the type of connection required for the data network. |  |  |  |  |

-	Values: "IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured".

*  *  *  *  *  NEXT CHANGE  *  *  *  *  *

### 8.2.39A	/<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo

Table 8.2.39A.1: /<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo

| OnNetwork/HPLMN/VPLMN/Service/DNinfo |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This interior node contains the configuration parameters for establishment of the connection to a Data Network in the VPLMN. |  |  |  |  |

### 8.2.39B	/<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>

Table 8.2.39B.1: /<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>

| OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | OneOrMore | node | Get, Replace |  |
|  | This run-time node acts as a placeholder for each reference to the connectivity parameters on a per VPLMN basis. |  |  |  |  |

### 8.2.39C	/<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNN

Table 8.2.39C.1: /<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNN

| OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The DNN parameter specifies the Data Network Name of a data network that the UE can attach to in the VPLMN. |  |  |  |  |

-	Value: A character string.

### 8.2.39D	/<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNAAAServer

Table 8.2.39D.1: /<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNAAAServer

| OnNetwork/HPLMN/VPLMN/Service/DNAAAserver/<x>/DNAAAServer |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | URI | Get, Replace |  |
|  | The DNAAAserver parameter specifies the URI to reach the AAA server for the data network in the VPLMN. |  |  |  |  |

-	Value: A valid URI.

### 8.2.39E	/<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNPDUSessionType

Table 8.2.39E.1: /<x>/OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNPDUSessionType

| OnNetwork/HPLMN/VPLMN/Service/DNinfo/<x>/DNPDUSessionType |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Status | Occurrence | Format | Min. Access Types |  |
|  | Required | One | chr | Get, Replace |  |
|  | The DNPDUSessionType parameter specifies the type of connection required for the data network. |  |  |  |  |

-	Values: "IPv4", "IPv6", "IPv4v6", "Ethernet", or "Unstructured".

*  *  *  *  *  END CHANGES  *  *  *  *  *