3GPP TSG-CT WG1 Meeting #133-e	C1-21aabb

# E-meeting, 11-19 November 2021                                                              was C1-216639

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 3616 | rev | 2 | Current version: | 17.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General subclause for NTN IoT in EPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_SAT_ARCH_EPS |  |  |  |  |  | Date: |  |  | 2021-11-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Satellite access to EPS needs to be introduced in TS 24.301. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subclause added to introduce NTN IoT in EPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.xx (new), 4.xx.1 (new), 4.xx.2 (new), 4.xx.2.1 (new), 4.xx.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.xx	Satellite access for CIoT

### 4.xx.1	General

UE and network may support satellite access in WB-S1 mode or NB-S1 mode with control plane CIoT EPS optimization. Support for E-UTRAN and NB-IoT satellite access is specified in TS 36.300 [20].

Editor’s Note: Definitions and abbreviations to be used for satellite access in WB-S1 mode and NB-S1 mode are FFS.

### 4.xx.2	Access determination and restrictions

The MME can determine the UE is accessing the network using a satellite access and the network can enforce mobility restriction for the UE using the satellite access in NB-S1 mode or WB-S1 mode as specified in 3GPP TS 23.401 [cc].

### 4.xx.2.1	Handling of EMM cause #78 and network's indication of country of UE location

The UE may receive the EMM cause #78 from the network in ATTACH REJECT message, TRACKING AREA UPDATE REJECT message or DETACH REQUEST message.

The EMM cause #78 is only applicable for a UE accessing a PLMN using a satellite access.

When the UE receives EMM cause #78 "PLMN not allowed to operate at the present UE location", in addition to the steps specified in subclauses 5.5.1.2.5, 5.5.3.2.5 and 5.6.1.5, the UE should:

1)	if the UE geographic location is available with acceptable accuracy, store the current UE geographical location for the PLMN;

2)	start an implementation-specific timer for the PLMN.

Editor's note: Further UE actions are FFS.

#### 4.xx.3	Extended NAS timers for satellite access

Editor's note:	Extended NAS timers for satellite access are FFS.