3GPP TSG-CT WG1 Meeting #134-e	C1-221199

# E-meeting, 17-25 February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.174 | CR | 0033 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MO-MMTEL indications towards the lower layer for call pull |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MuDTran |  |  |  |  |  | Date: |  |  | 2022-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to C1-221192, the CR implements the alternative where an access attempt occurred due to a call pull is categorized to an MO MMTEL call. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The IMS client provides indications related to MO-MMTEL call to the NAS layer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how an access attempt occurred due to a call pull is categorized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 4.5.3.1.2	Call pull handling

The UE may pull a session from another federated UE by performing the following steps:

Editor's Note: [WI MuDTran, CR#0030]:	It is FFS whether any procedure is needed for the to-be-pulled UE to authorize the call pull.

a)	the UE learns about the session to pull by subscribing to the dialog event package, specified in RFC 4235 [16];

b)	if:

1)	only audio or only real-time text or only both audio and real-time text are offered in the call to be pulled and no other originating multimedia telephony communication session initiated with offering only audio or only real-time text or only both audio and real-time text exists, then the UE sends the MO-MMTEL-voice-started indication to the non-access stratum; or

2)	video is offered in the call to be pulled and no other originating multimedia telephony communication session initiated with offering video exists, then the UE sends the MO-MMTEL-video-started indication to the non-access stratum.

The non-access stratum provides the barring result. If the barring result is:

1)	"not-barred", the UE continues the rest of the steps below; or

2)	"barred", the UE rejects the call pull and skips the rest of steps below;

c)	the UE sends an INVITE request towards the far end populated as follows:

1)	the Request-URI is set to the uri in the <target> element of the <remote> element;

2)	a Replaces header field containing the dialog identifiers contained as attributes in the <dialog> element of the <dialog-info> element; and

3)	any other header field or body as determined by the service logic; and

d)	when the UE receives a response to the INVITE request above the UE follows general procedures in TS 24.229 [3].

When the pulled call ends (e.g., a response to a BYE or a failure response to the initial INVITE request is transferred, the call is transferred to a different UE):

-	if the UE sent the MO-MMTEL-voice-started indication upon initiation of the call pull and no other originating multimedia telephony communication session initiated with offering only audio or only real-time text or only both audio and real-time text exists, the UE sends the MO-MMTEL-voice-ended to the non-access stratum; or

-	if the UE sent the MO-MMTEL-video-started indication upon initiation of the call pull and no other originating multimedia telephony communication session initiated with offering video exists, the UE sends the MO-MMTEL-video-ended indication to the non-access stratum.

Annex X (normative):
IP-Connectivity Access Network specific concepts when using EPS to access IM CN subsystem

# X.1	Scope

The present annex defines IP-CAN specific requirements for a multimedia telephony communication service and associated supplementary services in the IP Multimedia (IM) Core Network (CN) subsystem, where the IP-CAN is Evolved Packet System (EPS).

# X.2	EPS aspects when connected to the IM CN subsystem

## ALT 1: X.2.1	Procedures at the UE

### X.2.1.1	Service specific access control

#### X.2.1.1.1	General

This clause specifies service specific access control in addition to procedures as specified in annex J of 3GPP TS 24.173 [xx].

#### X.2.1.1.2	Call pull specific procedures

When the UE decides to pull a call from another federeted UE, the UE provides a "call-pull-initiated" indication to the lower layers.

## ALT 2: X.2.1	Procedures at the UE

### X.2.1.1	Service specific access control

#### X.2.1.1.1	General

This clause specifies service specific access control in addition to procedures as specified in annex J of 3GPP TS 24.173 [xx].

#### X.2.1.1.2	Call pull specific procedures

When the UE decides to pull a call from another federeted UE:

a)	if:

1)	only audio or only real-time text or only both audio and real-time text are offered in the call to be pulled and no other originating multimedia telephony communication session initiated with offering only audio or only real-time text or only both audio and real-time text exists, then the UE sends the MO-MMTEL-voice-started indication to the non-access stratum; or

2)	video is offered in the call to be pulled and no other originating multimedia telephony communication session initiated with offering video exists, then the UE sends the MO-MMTEL-video-started indication to the non-access stratum.

The non-access stratum provides the barring result. If the barring result is:

1)	"not-barred", the UE continues the call pull handling; or

2)	"barred", the UE rejects the call pull and stops the call pull procedure.

When the pulled call ends (e.g., a response to a BYE or a failure response to the initial INVITE request is transferred, the call is transferred to a different UE):

-	if the UE sent the MO-MMTEL-voice-started indication upon initiation of the call pull and no other originating multimedia telephony communication session initiated with offering only audio or only real-time text or only both audio and real-time text exists, the UE sends the MO-MMTEL-voice-ended to the non-access stratum; or

-	if the UE sent the MO-MMTEL-video-started indication upon initiation of the call pull and no other originating multimedia telephony communication session initiated with offering video exists, the UE sends the MO-MMTEL-video-ended indication to the non-access stratum.