3GPP TSG-CT WG1 Meeting #134-e	C1-221883

# E-Meeting, 17th – 25th February 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6553 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of in-call access update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The in-call access update procedure and naming is slightly ambiguous as it can be interpreted as only applying to 4G/5G change. The intent is however to be able to use this procedure also for VPLMN change as indicated by the presence of the PVNI header field.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note that the applicability is for both. Add clarifying text that the AS uses the procedure when it is interested in updates for PLMN or 4G/5G change.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impression that the in-call access update procedure only applies to 4G/5G change remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.14, 5.7.1.27.2, 5.7.1.27.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: Updated the Note in 5.2.14, clarified text in the AS sections. Removed the temporary Editor's Note existing in the first revision. Updated the cover page text accordingly. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 5.2.14	In-call access update

If the P-CSCF supports the in-call access update procedure, has received the g.3gpp.in-call-access-update feature-capability indicator during session set-up, and determines that the UE has changed location, then the P-CSCF shall generate a MESSAGE request populated as follows:

NOTE: This procedure can be used for both reporting change of IP-CAN and reporting change of PLMN.

1)	the Request URI set to the stored value of the g.3gpp.in-call-access-update feature-capability indicator;

2)	a From header field set to the FQDN of the P-CSCF sending the request;

3)	a To header field, set to the same value as the Request-URI;

4)	a P-Asserted-Identity header field set to the default public user identity of the served user;

5)	a P-Charging-Vector header field with the "icid-value" header field parameter populated with the ICID value used for the dialog related to the access change and a type 1 "orig-ioi" header field parameter. The P-CSCF shall set the type 1 "orig-ioi" header field parameter to a value that identifies the sending network of the request. The P-CSCF shall not include the type 1 "term-ioi" header field parameter;

6)	a P-Access-Network-Info header field including network-provided location information;

7)	a P-Visited-Network-ID header field; and

8)	if the MESSAGE request is to be sent via the S-CSCF, a Route header field with the S-CSCF address as received in the Service-Route header field during registration.

* * * Next Change * * * *

##### 5.7.1.27.2	Originating procedures

If the AS supports in-call access update procedures and the AS is interested in receiving updates of changes of IP-CAN or PLMN the AS shall include in responses to an initial INVITE request a g.3gpp.in-call-access-update feature-capability indicator with a value set to a public service identity resolving to the AS.

* * * Next Change * * * *

##### 5.7.1.27.3	Terminating procedures

If the AS supports in-call access update procedures and the AS is interested in receiving updates of changes of IP-CAN or PLMN the AS shall include in the INVITE request a g.3gpp.in-call-access-update feature-capability indicator with a value set to a public service identity resolving to the AS.

* * * End of Changes * * * *