3GPP TSG-CT WG1 Meeting #135-e	C1-222673

# E-Meeting, 6th – 12th April 2022

# Title:	Emergency services and UE rejected with "PLMN not allowed to operate in the country of the UE’s location"

# Response to:	- -

# Release:	Rel-17

# Work Item:	5GSAT_ARCH-CT

Source:	CT1

To:	SA1, RAN2

Cc:	SA2, SA3LI

Contact person:	Chen-Ho CHIN

chin.chenho@oppo.com

+32 477 413371

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

In a previous LS (in S1-211319/ C1-213551), SA1 provided the following guidance with respect to support of emergency services when network has rejected a UE with "PLMN not allowed to operate in the country of the UE’s location" – CT1 has assigned reject cause #78:-

[generally] that the PLMN is unable to provide support the various requirements related to emergency services, notably routing to a suitable PSAP

but

However, in border areas, PLMNs and/or PSAPs may provide some support for emergency calls from across a border ………in case the UE itself has determined that it is in a country the PLMN is not allowed to operate in, the UE may attempt to obtain emergency services

Thus these SA1 guidance has a linkage to whether the UE is able to determine its location, e.g. UE determines it is in border areas.

So in discussing the stage 3 support for emergency services where the UE does and does not know (or can or cannot ascertain) its location, CT1 would like to progress with:-

- If a UE is rejected with #78 for a normal registration or registration update or when requesting access for resources/services when normally registered, the UE is allowed to attempt emergency registration to that same rejecting NW in attempt to obtain emergency services, if no other NW can be found;

- If the UE is rejected with #78 for an emergency registration then UE shall not re-attempt to obtain emergency services from that PLMN until the validity of reject cause #78 expires, even if no other NW can be found.

In coming to the above, CT1 took into consideration

- the UE might not be able to determine its location (for whatever reason) or with sufficient accuracy;

- even if UE could determine its location, that determination should not be (fully) trusted;

- emergency services could be attempted even in a forbidden PLMN, if no other PLMNs could be found,

- there can be many locations where the NTN rejecting the UE with #78, is the only (mobile or otherwise) network around;

- the network has the ultimate right to refuse to provide emergency services, i.e reject the attempt for emergency registration, or rejecting request for resources for emergency services.

Would SA1 have any issues if CT1 progress with CRs for 1) and 2)?

2. Actions:

To SA1

ACTION1: 	CT1 kindly ask SA1 to indicate if SA1 is agreeable or have problems if CT1 progress with 1) and 2).

ACTION2: 	If SA1 is agreeable to 1) and 2), SA1 to kindly consider if a stage 1 change is needed to align with stage 3.

To RAN2

ACTION: 	Should SA1 find CT1's suggestions agreeable, CT1 kindly then request RAN2 to take that into account and make the necessary updates to TS 38.304 and any other RAN2 TS if appropriate.

3. Date of Next CT1 Meetings:

CT1#136e	12th - 20th May 2022	electronic meeting