3GPP TSG-CT WG1 Meeting #135-e	C1-222572

# E-Meeting, 6th – 12th April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 4143 | rev | 1 | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Secondary authentication via L3 relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-3-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 6.3.3.3.4 of TS 33.503, the PDU session authentication and authorization procedure for the L3 remote UE is defined. The exsiting PDU SESSION AUTHENTICATION COMMAND message and PDU SESSION AUTHENTICATION COMPLETE message are updated to include the remote user ID and exchange the EAP messages for L3 remote UE between the L3 relay UE and SMF. Since in the remote UE report procedure, the remote user ID has not been defined, still in this CR, the definition of Remote user ID IE is left to FFS. Diferrent from the exisitng PDU session authentication, instead of PDU SESSION AUTHENTICATION RESULT/REJECT message, the REMOTE UE REPORT RESPONSE message is used to inform the result to the L3 relay UE and send the EAP-success/failure to L3 remote UE. The stage 3 implementation is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PDU session authentication for remote UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing stage 2 requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1, 6.3.1.2.1, 6.3.1.2.2, 6.6.2.3, 8.3.4.1, 8.3.4.a(new), 8.3.5.1, 8.3.5.b(new), 8.3.20.1, 8.3.20.c(new), 8.3.20.d(new), 8.3.20.e(new), 9.11.4.z(new) and 9.11.4.f(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.3.1.1	General

The purpose of the PDU session authentication and authorization procedure is to enable the DN:

a)	to authenticate the upper layers of the UE, when establishing the PDU session;

b)	to authorize the upper layers of the UE, when establishing the PDU session;

c)	both of the above;

d)	to re-authenticate the upper layers of the UE after establishment of the PDU session; or

e)	to authenticate the upper layers of the 5G ProSe layer-3 remote UE, upon receipt of the REMOTE UE REPORT message.

The PDU session authentication and authorization procedure can be performed only during or after the UE-requested PDU session procedure establishing a non-emergency PDU session. The PDU session authentication and authorization procedure shall not be performed during or after the UE-requested PDU session establishment procedure establishing an emergency PDU session.

The upper layers store the association between a DNN and corresponding credentials, if any, for the PDU session authentication and authorization.

The network authenticates the UE using the Extensible Authentication Protocol (EAP) as specified in IETF RFC 3748 [34].

EAP has defined four types of EAP messages:

a)	an EAP-request message;

b)	an EAP-response message;

c)	an EAP-success message; and

d)	an EAP-failure message.

The EAP-request message is transported from the network to the UE using the PDU SESSION AUTHENTICATION COMMAND message of the PDU EAP message reliable transport procedure.

The EAP-response message to the EAP-request message is transported from the UE to the network using the PDU SESSION AUTHENTICATION COMPLETE message of the PDU EAP message reliable transport procedure.

If the PDU session authentication and authorization procedure is performed during the UE-requested PDU session establishment procedure:

a)	and the DN authentication of the UE completes successfully, the EAP-success message is transported from the network to the UE as part of the UE-requested PDU session establishment procedure in the PDU SESSION ESTABLISHMENT ACCEPT message.

b)	and the DN authentication of the UE completes unsuccessfully, the EAP-failure message is transported from the network to the UE as part of the UE-requested PDU session establishment procedure in the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU session authentication and authorization procedure is performed after the UE-requested PDU session establishment procedure:

a)	and the DN authentication of the UE completes successfully, the EAP-success message is transported from the network to the UE using the PDU SESSION AUTHENTICATION RESULT message of the PDU EAP result message transport procedure.

b)	and the DN authentication of the UE completes unsuccessfully, the EAP-failure message is transported from the network to the UE using the PDU SESSION RELEASE COMMAND message of the network-requested PDU session release procedure.

There can be several rounds of exchange of an EAP-request message and a related EAP-response message for the DN to complete the authentication and authorization of the request for a PDU session (see example in figure 6.3.1.1).

The SMF shall set the authenticator retransmission timer specified in IETF RFC 3748 [34] subclause 4.3 to infinite value.

NOTE:	The PDU session authentication and authorization procedure provides a reliable transport of EAP messages and therefore retransmissions at the EAP layer of the SMF do not occur.

Figure 6.3.1.1: PDU session authentication and authorization procedure

* * * Next Change * * * *

6.3.1.2.1	PDU EAP message reliable transport procedure initiation

In order to initiate the PDU EAP message reliable transport procedure, the SMF shall create a PDU SESSION AUTHENTICATION COMMAND message.

The SMF shall set the PTI IE of the PDU SESSION AUTHENTICATION COMMAND message to "No procedure transaction identity assigned".

The SMF shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message to the EAP-request message provided by the DN or generated locally.

The SMF shall set the Remote user ID IE of the PDU SESSION AUTHENTICATION COMMAND message to the remote user identify if received in the Remote UE context connected IE in the REMOTE UE REPORT message.

The SMF shall send the PDU SESSION AUTHENTICATION COMMAND message, and the SMF shall start timer T3590 (see example in figure 6.3.1.1).

Upon receipt of the PDU SESSION AUTHENTICATION COMMAND message, if the UE provided a DNN during the PDU session establishment, the UE shall stop timer T3396, if it is running for the DNN provided by the UE. If the UE did not provide a DNN during the PDU session establishment, the UE shall stop the timer T3396 associated with no DNN if it is running.

Upon receipt of the PDU SESSION AUTHENTICATION COMMAND message, if the UE provided an S-NSSAI and a DNN during the PDU session establishment, the UE shall stop timer T3584, if it is running for the [S-NSSAI of the PDU session, DNN] combination. If the UE provided a DNN but did not provide an S-NSSAI during the PDU session establishment, the UE shall stop timer T3584, if it is running for the same [no S-NSSAI, DNN] combination provided by the UE. If the UE provided an S-NSSAI but did not provide a DNN during the PDU session establishment, the UE shall stop timer T3584, if it is running for the same [S-NSSAI, no DNN] combination provided by the UE. If the UE provided neither a DNN nor an S-NSSAI during the PDU session establishment, the UE shall stop timer T3584, if it is running for the same [no S-NSSAI, no DNN] combination provided by the UE. The timer T3584 to be stopped includes the timer T3584 applied for all the PLMNs, if running, and the timer T3584 applied for the registered PLMN, if running.

Upon receipt of the PDU SESSION AUTHENTICATION COMMAND message, if the UE provided an S-NSSAI during the PDU session establishment, the UE shall stop timer T3585, if it is running for the S-NSSAI of the PDU session. If the UE did not provide an S-NSSAI during the PDU session establishment, the UE shall stop the timer T3585 associated with no S-NSSAI if it is running. The timer T3585 to be stopped includes the timer T3585 applied for all the PLMNs, if running, and the timer T3585 applied for the registered PLMN, if running.

NOTE 1:	Upon receipt of the PDU SESSION AUTHENTICATION COMMAND message for a PDU session, if the UE provided a DNN (or no DNN) and an S-NSSAI (or no S-NSSAI) when the PDU session is established, timer T3396 associated with the DNN (or no DNN, if no DNN was provided by the UE) is running, and timer T3584 associated with the DNN (or no DNN, if no DNN was provided by the UE) and the S-NSSAI (or no S-NSSAI, if no S-NSSAI was provided by the UE) is running, then the UE stops both the timer T3396 and the timer T3584.

NOTE 2:	Upon receipt of the PDU SESSION AUTHENTICATION COMMAND message for a PDU session, if the UE provided a DNN (or no DNN) and an S-NSSAI (or no S-NSSAI) when the PDU session is established, timer T3585 associated with the S-NSSAI (or no S-NSSAI, if no S-NSSAI was provided by the UE) is running, and timer T3584 associated with the DNN (or no DNN, if no DNN was provided by the UE) and the S-NSSAI (or no S-NSSAI, if no S-NSSAI was provided by the UE) is running, then the UE stops both the timer T3585 and the timer T3584.

Upon receipt of a PDU SESSION AUTHENTICATION COMMAND message and a PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5, the UE passes to the upper layers the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message. Apart from this action and the stopping of timers T3396, T3584 and T3485 (if running), the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

Upon receipt of a PDU SESSION AUTHENTICATION COMMAND message including the Remote user ID IE, the UE shall pass to the upper layer as defined in 3GPP TS 24.554 [19E] the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMMAND message.

* * * Next Change * * * *

6.3.1.2.2	PDU EAP message reliable transport procedure accepted by the UE

The UE shall create a PDU SESSION AUTHENTICATION COMPLETE message when:

a)	the upper layers provide an EAP-response message responding to the received EAP-request message; or

b)	the UE receives an EAP message from the 5G ProSe layer-3 remote UE as defined in 3GPP TS 24.554 [19E].

The UE shall set the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the EAP-response message.

The UE shall set the Remote user ID IE of the PDU SESSION AUTHENTICATION COMPLETE message to the remote user identity from which the EAP message is received if the EAP message is received from the 5G ProSe remote UE.

The UE shall transport the PDU SESSION AUTHENTICATION COMPLETE message and the PDU session ID, using the NAS transport procedure as specified in subclause 5.4.5. Apart from this action, the authentication and authorization procedure initiated by the DN is transparent to the 5GSM layer of the UE.

Upon receipt of a PDU SESSION AUTHENTICATION COMPLETE message, the SMF shall stop timer T3590 and provides the EAP message received in the EAP message IE of the PDU SESSION AUTHENTICATION COMPLETE message to the DN or handles it locally.

* * * Next Change * * * *

6.6.2.3	Remote UE report procedure accepted by the network

Upon receipt of the REMOTE UE REPORT message, the SMF shall send a REMOTE UE REPORT RESPONSE message to the UE.

The SMF shall include the PTI from the REMOTE UE REPORT message.

The SMF shall include the EAP message IE to an EAP-success message or an EAP-failure message to be sent to the UE if the EAP-success message or the EAP-failure message is received from the DN.

The SMF may include the 5G ProSe direct link release indication IE to indicate whether the 5G ProSe layer-3 relay UE should release the 5G ProSe direct link to the 5G ProSe layer-3 remote UE.

The SMF may include the Authorized QoS flow description IE to indicate the QoS flow parameters to the 5G ProSe layer-3 relay UE when the PDU session authentication and authorization procedure for the 5G ProSe layer-3 remote UE is successful.

Upon receipt of the REMOTE UE REPORT RESPONSE message, the UE shall stop timer T3586 and enter the state PROCEDURE TRANSACTION INACTIVE.

Upon receipt of the REMOTE UE REPORT RESPONSE message including the EAP message IE, the UE shall pass the EAP message received in the EAP message IE of the REMOTE UE REPORT RESPONSE message to the 5G ProSe layer-3 remote UE as defined in 3GPP TS 24.554 [19E].

Upon receipt of the REMOTE UE REPORT RESPONSE message including the 5G ProSe direct link release indication IE, if the 5G ProSe direct link release indication IE is set to "Release 5G ProSe direct link", the UE should release the 5G ProSe direct link to the 5G ProSe layer-3 remote UE identified by the Remote UE context connected IE in the REMOTE UE REPORT message as defined in 3GPP TS 24.554 [19E].

* * * Next Change * * * *

8.3.4.1	Message definition

The PDU SESSION AUTHENTICATION COMMAND message is sent by the SMF to the UE for authentication of the UE establishing the PDU session or of the UE participating in the PDU session. See table 8.3.4.1.1.

Message type:	PDU SESSION AUTHENTICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 8.3.4.1.1: PDU SESSION AUTHENTICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMMAND message identity | Message type 9.7 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| xy | Remote user ID | Remote user ID 9.11.4.z | O | TLV | 3-257 |

* * * Next Change * * * *

8.3.4.a	Remote user ID

This IE is included in the message when the network needs to transmit the EAP message for 5G ProSe layer-3 remote UE.

* * * Next Change * * * *

8.3.5.1	Message definition

The PDU SESSION AUTHENTICATION COMPLETE message is sent by the UE to the SMF in response to the PDU SESSION AUTHENTICATION COMMAND message and indicates acceptance of the PDU SESSION AUTHENTICATION COMMAND message. See table 8.3.5.1.1.

Message type:	PDU SESSION AUTHENTICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 8.3.5.1.1: PDU SESSION AUTHENTICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | PDU SESSION AUTHENTICATION COMPLETE message identity | Message type 9.7 | M | V | 1 |
|  | EAP message | EAP message 9.11.2.2 | M | LV-E | 6-1502 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.11.4.6 | O | TLV-E | 4-65538 |
| xy | Remote user ID | Remote user ID 9.11.4.z | O | TLV | 3-257 |

* * * Next Change * * * *

8.3.5.b	Remote user ID

This IE is included in the message when the UE needs to transmit the EAP message for 5G ProSe layer-3 remote UE.

* * * Next Change * * * *

8.3.20.1	Message definition

The REMOTE UE REPORT RESPONSE message is sent by the network to the UE to acknowledge receipt of a remote UE report message. See table 8.3.20.1.

Message type:	REMOTE UE REPORT RESPONSE

Significance:	dual

Direction:	network to UE

Table 8.3.20.1: REMOTE UE REPORT RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 9.2 | M | V | 1 |
|  | PDU session ID | PDU session identity 9.4 | M | V | 1 |
|  | PTI | Procedure transaction identity 9.6 | M | V | 1 |
|  | Remote UE report response message identity | Message type 9.7 | M | V | 1 |
| 78 | EAP message | EAP message 9.11.2.2 | O | TLV-E | 6-1502 |
| xa | 5G ProSe direct link release indication | 5G ProSe direct link release indication 9.11.4.f | O | TLV | 3 |
| 79 | Authorized QoS flow descriptions | QoS flow descriptions 9.11.4.12 | O | TLV-E | 6-65538 |

* * * Next Change * * * *

8.3.20.c	EAP message

This IE is included when the external DN performs authentication and authorization of the UE using EAP.

8.3.20.d	5G ProSe direct link release indication

This IE is included in the message when the network needs to indicate whether the 5G ProSe layer-3 relay UE should release the 5G ProSe direct link to the 5G ProSe layer-3 remote UE.

8.3.20.e	Authorized QoS flow descriptions

This IE is included in the message when the network needs to indicate the QoS flow parameters to the 5G ProSe layer-3 relay UE when the PDU session authentication and authorization procedure for the 5G ProSe layer-3 remote UE is successful.

* * * Next Change * * * *

9.11.4.z	Remote user ID

Editor's note:	It is FFS what are the 5G ProSe remote UE identities that can be included.

9.11.4.f	5G ProSe direct link release indication

The purpose of the 5G ProSe direct link release indication information element is to indicate whether the 5G ProSe layer-3 relay UE should release the 5G ProSe direct link to the 5G ProSe layer-3 remote UE.

The 5G ProSe direct link release indication information element is coded as shown in figure 9.11.4.f.1 and table 9.11.4.f.1.

The 5G ProSe direct link release indication is a type 4 information element with a length of 3 octets.

| 8 | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5G ProSe direct link release indication IEI |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of 5G ProSe direct link release indication IE |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 Spare |  | 0 Spare | 0 Spare | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare | 5PDLRI |  | octet 3 |

Figure 9.11.4.f.1: 5G ProSe direct link release indication information element

Table 9.11.4.f.1: 5G ProSe direct link release indication information element

| 5G ProSe direct link release indication (5PDLRI) (bit 1 of octet 3) |  |
| --- | --- |
| Bit |  |
| 1 |  |
| 0 | Not release 5G ProSe direct link |
| 1 | Release 5G ProSe direct link |

* * * End of Changes * * * *