3GPP TSG-CT WG1 Meeting #136-e	C1-223561

# E-Meeting, 12th – 20th May 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 27.007 | CR | 0776 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GSM congestion re-attempt indicator with ABO bit and CATBO bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GProtoc17 |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 24.501 subclause 9.11.4.21, the 5GSM congestion re-attempt indicator IE is defined with ABO bit and CATBO bit. The ABO bit indicates whether the back-off timer is applied in the registered PLMN or all PLMNs. The CATBO bit indicates whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type. However, current TS 27.007 only defines the 5GSM congestion re-attempt indicator IE with ABO bit. The definition of 5GSM congestion re-attempt indicator IE with CATBO bit is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add definition of 5GSM congestion re-attempt indicator IE with CATBO bit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The back-off timer cannot be applied considering access type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.55, 10.1.56, 10.1.57, 10.1.58 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

10.1.55	S-NSSAI based back-off timer status reporting +CSBTSR

Table 10.1.55-1: +CSBTSR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSBTSR=[<n>] | +CME ERROR: <err> |
| +CSBTSR? | +CSBTSR: <n> |
| +CSBTSR=? | +CSBTSR: (list of supported <n>s) |

Description

Set command controls the presentation of unsolicited result code +CSBTSRI: <S-NSSAI>,<event_type>[,<S-NSSAI_backoff_time>,<5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>[,<procedure>]] reporting the S-NSSAI based back-off timer parameter values from MT to TE if the back-off timer is started, stopped, deactivated or expires. Refer clause 9.2 for possible <err> values.

Read command returns the current S-NSSAI based back-off timer unsolicited result code settings in the MT.

Test command returns values supported as a compound value.

Defined values

<n>: integer type.

0	Disable presentation of the unsolicited result code +CSBTSRI.

1	Enable presentation of the unsolicited result code +CSBTSRI.

<S-NSSAI>: string type in hexadecimal format. Dependent of the form, the string can be separated by dot(s) and semicolon(s). The S-NSSAI is associated with the back-off timer for identifying a network slice in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. Refer parameter <S-NSSAI> in clause 10.1.1. This parameter shall not be subject to conventional character conversion as per +CSCS.

<event_type>: integer type. Indicates the event happened to the back-off timer.

0	The back-off timer is started.

1	The back-off timer is stopped.

2	The back-off timer is expired.

3	The back-off timer is deactivated.

<S-NSSAI_backoff_time>: integer type; indicates the remaining back-off time associated with the <S-NSSAI> in seconds. When the back-off timer is deactivated, the parameter <S-NSSAI_backoff_time> is omitted. When the back-off timer is stopped or expired, 0 is indicated.

<5GSM_congestion_re-attempt_abo_indicator>: integer type. Indicates whether the back-off timer is applied in the registered PLMN or all PLMNs.

0	The back-off timer is applied in the registered PLMN.

1	The back-off timer is applied in all PLMNs.

<5GSM_congestion_re-attempt_catbo_indicator>: integer type. Indicates whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type.

0	The back-off timer is applied in both 3GPP access type and non-3GPP access type.

1	The back-off timer is applied in the current access type.

<procedure>: integer type. Indicates the procedure(s) for which the back-off timer applies. When <procedure>=0 the information returned is associated with timer T3585. When the parameter <procedure> is omitted, the back-off timer is deactivated.

0	All procedures.

Implementation

Optional.

* * * Next Change * * * *

10.1.56	S-NSSAI based back-off timer read dynamic parameters +CSBTRDP

Table 10.1.56-1: +CSBTRDP action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSBTRDP[=<S-NSSAI>] | [+CSBTRDP: <S-NSSAI>[,<S-NSSAI_backoff_time>][,<5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>][,<procedure>] [<CR><LF>+CSBTRDP: <S-NSSAI>[,<S-NSSAI_backoff_time>][,<5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>][,<procedure>] [...]]] |
| +CSBTRDP=? |  |

Description

The execution command returns the relevant information in the MT for the S-NSSAI based back-off timer parameter value <S-NSSAI_backoff_time>, <5GSM_congestion_re-attempt_abo_indicator>, <5GSM_congestion_re-attempt_catbo_indicator> and <procedure> for an <S-NSSAI> if the back-off timer is running.

If the parameter <S-NSSAI> in the execution command is omitted, the relevant information for all S-NSSAIs associated with running back-off timers are returned.

Defined values

<S-NSSAI>: string type in hexadecimal format. Dependent of the form, the string can be separated by dot(s) and semicolon(s). The S-NSSAI is associated with the back-off timer for identifying a network slice in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. When <S-NSSAI> indicates an empty string (""), the following parameter is associated with no S-NSSAI as specified in 3GPP TS 24.501 [161]. Refer parameter <S-NSSAI> in clause 10.1.1. This parameter shall not be subject to conventional character conversion as per +CSCS.

<S-NSSAI_backoff_time>: integer type; indicates the remaining back-off time associated with the <S-NSSAI> in seconds. When the back-off timer is deactivated, the parameter <S-NSSAI_backoff_time> is omitted.

<5GSM_congestion_re-attempt_abo_indicator>: integer type. Indicates whether the back-off timer is applied in the registered PLMN or all PLMNs.

0	The back-off timer is applied in the registered PLMN.

1	The back-off timer is applied in all PLMNs.

<5GSM_congestion_re-attempt_catbo_indicator>: integer type. Indicates whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type.

0	The back-off timer is applied in both 3GPP access type and non-3GPP access type.

1	The back-off timer is applied in the current access type.

<procedure>: integer type. Indicates the procedure(s) for which the back-off timer applies. When <procedure>=0 the information returned is associated with timer T3585. When the parameter <procedure> is omitted, the back-off timer is deactivated.

0	All procedures.

Implementation

Optional.

* * * Next Change * * * *

10.1.57	S-NSSAI and DNN based back-off timer status reporting +CSDBTSR

Table 10.1.57-1: +CSDBTSR parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSDBTSR=[<n>] | +CME ERROR: <err> |
| +CSDBTSR? | +CSDBTSR: <n> |
| +CSDBTSR=? | +CSDBTSR: (list of supported <n>s) |

Description

Set command controls the presentation of unsolicited result code +CSDBTSRI: <S-NSSAI>,<DNN>,<event_type>[,<S-NSSAI_DNN_backoff_time>, <5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>[,<procedure>]] reporting the S-NSSAI and DNN based back-off timer parameter values from MT to TE if the back-off timer is started, stopped, deactivated or expires. Refer clause 9.2 for possible <err> values.

Read command returns the current S-NSSAI and DNN based back-off timer unsolicited result code settings in the MT.

Test command returns values supported as a compound value.

Defined values

<n>: integer type.

0	Disable presentation of the unsolicited result code +CSDBTSRI.

1	Enable presentation of the unsolicited result code +CSDBTSRI.

<S-NSSAI>: string type in hexadecimal format. Dependent of the form, the string can be separated by dot(s) and semicolon(s). The S-NSSAI is associated with the back-off timer for identifying a network slice in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. Refer parameter <S-NSSAI> in clause 10.1.1. This parameter shall not be subject to conventional character conversion as per +CSCS.

<DNN>: string type; indicates the DNN associated with the back-off timer for identifying a data network in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. This parameter shall not be subject to conventional character conversion as per +CSCS.

<event_type>: integer type. Indicates the event happened to the back-off timer.

0	The back-off timer is started.

1	The back-off timer is stopped.

2	The back-off timer is expired.

3	The back-off timer is deactivated.

<S-NSSAI_DNN_backoff_time>: integer type; indicates the remaining back-off time associated with the <S-NSSAI> and <DNN> in seconds. When the back-off timer is deactivated, the parameter <S-NSSAI_DNN_backoff_time> is omitted.

<5GSM_congestion_re-attempt_abo_indicator>: integer type. Indicates whether the back-off timer is applied in the registered PLMN or all PLMNs.

0	The back-off timer is applied in the registered PLMN.

1	The back-off timer is applied in all PLMNs.

<5GSM_congestion_re-attempt_catbo_indicator>: integer type. Indicates whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type.

0	The back-off timer is applied in both 3GPP access type and non-3GPP access type.

1	The back-off timer is applied in the current access type.

<procedure>: integer type. Indicates the procedure(s) for which the back-off timer applies. When <procedure>=0 the information returned is associated with timer T3584. For all other values of <procedure> the information returned is associated with the back-off timer as specified in 3GPP TS 24.501 [161] for the various session management procedures. When the parameter <procedure> is omitted, the back-off timer is deactivated.

0	All procedures.

1	PDU session establishment procedure (see 3GPP TS 24.501 [161], clause 6.4.1)

2	PDU session modification procedure (see 3GPP TS 24.501 [161], clause 6.4.2).

Implementation

Optional.

* * * Next Change * * * *

10.1.58	S-NSSAI and DNN based back-off timer read dynamic parameters +CSDBTRDP

Table 10.1.58-1: +CSDBTRDP action command syntax

| Command | Possible response(s) |
| --- | --- |
| +CSDBTRDP[=<S-NSSAI>,<DNN>] | [+CSDBTRDP: <S-NSSAI>,<DNN>[,<S-NSSAI_DNN_backoff_time>][,<5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>[,<procedure>]]] [<CR><LF>+CSDBTRDP: <S-NSSAI>,<DNN>[,<S-NSSAI_DNN_backoff_time>][,<5GSM_congestion_re-attempt_abo_indicator>,<5GSM_congestion_re-attempt_catbo_indicator>[,<procedure>]]] [...]]] |
| +CSDBTRDP=? |  |

Description

The execution command returns the relevant information in the MT for the S-NSSAI and DNN based back-off timer parameter value <S-NSSAI_DNN_backoff_time>, <5GSM_congestion_re-attempt_abo_indicator>, <5GSM_congestion_re-attempt_catbo_indicator> and <procedure> for the <S-NSSAI> and <DNN> combination if the back-off timer is running.

If the parameter <S-NSSAI> in the execution command is omitted, the DNN specific information for all S-NSSAIs associated with running back-off timers are returned.

If the parameter <DNN> in the execution command is omitted, the S-NSSAI specific information for all DNNs associated with running back-off timers are returned.

If both the parameters <S-NSSAI> and <DNN> are omitted, the relevant information for all S-NSSAI and DNN combinations associated with running back-off timers are returned.

Defined values

<S-NSSAI>: string type in hexadecimal format. Dependent of the form, the string can be separated by dot(s) and semicolon(s). The S-NSSAI is associated with the back-off timer for identifying a network slice in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. When <S-NSSAI> indicates an empty string (""), the parameter <S-NSSAI_DNN_backoff_time> in the response is associated with no S-NSSAI as specified in 3GPP TS 24.501 [161]. Refer parameter <S-NSSAI> in clause 10.1.1. This parameter shall not be subject to conventional character conversion as per +CSCS.

<DNN>: string type; indicates the DNN associated with the back-off timer for identifying a data network in 5GS, see 3GPP TS 23.501 [165] and 3GPP TS 24.501 [161]. When <DNN> indicates an empty string (""), the parameter <S-NSSAI_DNN_backoff_time> in the response is associated with no DNN as specified in 3GPP TS 24.501 [161]. This parameter shall not be subject to conventional character conversion as per +CSCS.

<S-NSSAI_DNN_backoff_time>: integer type; indicates the remaining back-off time associated with the <S-NSSAI> and <DNN> combination in seconds. When the back-off timer is deactivated, the parameter <S-NSSAI_DNN_backoff_time> is omitted.

<5GSM_congestion_re-attempt_abo_indicator>: integer type. Indicates whether the back-off timer is applied in the registered PLMN or all PLMNs.

0	The back-off timer is applied in the registered PLMN.

1	The back-off timer is applied in all PLMNs.

<5GSM_congestion_re-attempt_catbo_indicator>: integer type. Indicates whether the back-off timer is applied in the current access type or both 3GPP access type and non-3GPP access type.

0	The back-off timer is applied in both 3GPP access type and non-3GPP access type.

1	The back-off timer is applied in the current access type.

<procedure>: integer type. Indicates the procedure(s) for which the back-off timer applies. When <procedure>=0 the information returned is associated with timer T3584. For all other values of <procedure> the information returned is associated with the back-off timer as specified in 3GPP TS 24.501 [161] for the various session management procedures. When the parameter <procedure> is omitted, the back-off timer is deactivated.

0	All procedures.

1	PDU session establishment procedure (see 3GPP TS 24.501 [161], clause 6.4.1)

2	PDU session modification procedure (see 3GPP TS 24.501 [161], clause 6.4.2).

Implementation

Optional.

* * * End of Changes * * * *