3GPP TSG-CT WG1 Meeting #137-e	C1-22XXXX

# E-Meeting, 18th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.549 | CR | 0012 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Added description and overview |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2022-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are couple of place holders which need to be filled in. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added text for - Overview clause for Resources and - Description clause Configuration as resurce in Annex A. Minor modification of the text in: - Overview clause for Resources and - Description clause Configuration as resurce in Annex B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing text remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1.2.1, A.2.1.2.2.1, B.2.1.2, B.2.1.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### A.2.1.2.1	Overview

The Resource URI structure of the ETN_Configuration API is as shown in Figure A.2.1.2.1-1:

Figure A.2.1.2.1-1: Resource URI structure of the ETN_Configuration API

Table A.2.1.2.1-1 provides an overview of the resources and applicable HTTP method.

Table A.2.1.2.1-1: Resources and method overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Configuration | /val-services/{valServiceId}/configurations/{configurationId} | PUT (NOTE) | Performs configuration. |
| NOTE: In this release, the only configuration is the slice adaptation as described in 3GPP TS 23.434 [2]. |  |  |  |

* * * Second Change * * * *

##### A.2.1.2.2.1	Description

The Configuration resource allows an SNSCE-C a specific configuration identified by a configuration ID, to send an HTTP request containing:

a)	a list of one or more VAL UEs;

b)	a requested S-NSSAI;

c)	optionally a requested DNN; and

d)	optionally a requested configuration cause,

for a specific VAL service identified by a VAL service ID, toward a SNSCE-S to perform a network triggered slice configuration for the list of one or more VAL UEs for that specific VAL service.

NOTE:	In this release, S-NSSAI and DNN are the only used route selection descriptors of the URSP rules described in 3GPP TS 24.526 [3].

* * * Third Change * * * *

#### B.2.1.2.1	Overview

The Resource URI structure of the ETN_Configuration API is as shown in Figure B.2.1.2.1-1:

Figure B.2.1.2.1-1: Resource URI structure of the ETN_Configuration API

Table B.2.1.2.1-1 provides an overview of the resources and applicable CoAP method.

Table B.2.1.2.1-1: Resources and method overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| Configuration | /val-services/{valServiceId}/configurations/{configurationId} | PUT (NOTE) | Performs configuration. |
| NOTE: In this release, the only configuration is the slice adaptation as described in 3GPP TS 23.434 [2]. |  |  |  |

* * * Fourth Change * * * *

##### B.2.1.2.2.1	Description

The Configuration resource allows an SNSCE-C a specific configuration identified by a configuration ID, to send a CoAP request containing:

a)	a list of one or more VAL UEs;

b)	a requested S-NSSAI;

c)	optionally a requested DNN; and

d)	optionally a requested configuration cause,

for a specific VAL service identified by a VAL service ID, toward a SNSCE-S to perform a network triggered slice configuration for the list of one or more VAL UEs for that specific VAL service.

NOTE:	In this release, S-NSSAI and DNN are the only used route selection descriptors of the URSP rules described in 3GPP TS 24.526 [3].

* * * End of Changes * * * *