3GPP TSG-CT WG1 Meeting #137-e	C1-225049

# E-Meeting, 18th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.379 | CR | 0837 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support providing FAs used by affiliated group members |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMONASTERY2 |  |  |  |  |  | Date: |  |  | 2022-07-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 23.280 specifies a mechanism to get the list of currently affiliated members of an MC service group including the associated functional alias(es) of those members. Existing specs contain a procedure for subscription and notification for dynamic data associated with a group, but the provided info doesn't include list of activated functional alias(es). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend affiliation mechanism to contain the activated FAs of affiliated members, by adding functional alias information to the contents of the group dynamic data |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Deriving the FAs used by affiliated group members is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 9.3.1.2	Syntax

The application/pidf+xml MIME body indicating per-user affiliation information is constructed according to IETF RFC 3863 [52] and:

1)	contains a <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT ID of the MCPTT user;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] per each MCPTT client of the <presence> element;

4)	can contain a <p-id> child element defined in the XML schema defined in table 9.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the MCPTT client ID;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <affiliation> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each MCPTT group in which the MCPTT user is interested at the MCPTT client;

8)	contains a "group" attribute of each <affiliation> element set to the MCPTT group ID of the MCPTT group in which the MCPTT user is interested at the MCPTT client;

9)	can contain a "status" attribute of each <affiliation> element indicating the affiliation status of the MCPTT user to MCPTT group at the MCPTT client; and

10)	can contain an "expires" attribute of each <affiliation> element indicating expiration of affiliation of the MCPTT user to MCPTT group at the MCPTT client.

The application/pidf+xml MIME body indicating per-group affiliation information is constructed according to IETF RFC 3856 [51] and:

1)	contains the <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT group ID of the MCPTT group;

3)	contains one <tuple> child element according to IETF RFC 3863 [52] of the <presence> element;

4)	can contain a <p-id> child element defined in the XML schema defined in table 9.3.1.2-1, of the <presence> element set to an identifier of a SIP PUBLISH request;

5)	contains an "id" attribute of the <tuple> element set to the MCPTT ID of the MCPTT user;

6)	contains one <status> child element of each <tuple> element;

7)	contains one <affiliation> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each MCPTT client at which the MCPTT user is interested in the MCPTT group;

8)	contains one "client" attribute defined in the XML schema defined in table 9.3.1.2-1, of the <affiliation> element set to the MCPTT client ID;

9)	can contain an "expires" attribute defined in the XML schema defined in table 9.3.1.2-1, of the <affiliation> element indicating expiration of affiliation of the MCPTT user to MCPTT group at the MCPTT client;and

10)	can contain one <functionalAlias> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each functional alias the group member has activated with the "functionalAliasID" attribute set to the corresponding functional alias ID.

The application/pidf+xml MIME body indicating per-group dynamic data information is constructed according to IETF RFC 3856 [51] and:

1)	contains the <presence> root element according to IETF RFC 3863 [52];

2)	contains an "entity" attribute of the <presence> element set to the MCPTT group ID of the MCPTT group;

3)	can contain a <groupStatus> child element defined in the XML schema , of the <presence> element set to an appropriate value specified in Table 9.3.1.2-3;

4)	can contain a "ModifiedBy" attribute of the <groupStatus> element;

5)	can contain an <additionalData> child element definied in the XML schema defined in table 9.3.1.2-1, of the <presence> element;

6)	can contain a <groupBroadcastAlias> attribute defined in the XML schema defined in table 9.3.1.2-1 of the <additionalData> element;

7)	can contain a <groupRegroupAlias> attribute defined in the XML schema defined in table 9.3.1.2-1 of the <additionalData> element;

8)	can contain a <groupCallOnoing> attribute defined in the XML schema defined in table 9.3.1.2-1 of the <additionalData> element;

9)	contains one <tuple> child element per affiliated MCPTT ID of the MCPTT group, according to IETF RFC 3863 [52] of the <presence> element;

10)	contains an "id" attribute of the <tuple> element set to the MCPTT ID of the group member;

11)	contains one <status> child element of each <tuple> element;

12)	contains one <affiliation> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each MCPTT client at which the MCPTT user is interested in the MCPTT group;

13)	contains one "client" attribute defined in the XML schema defined in table 9.3.1.2-1, of the <affiliation> element set to the MCPTT client ID;

14)	can contain an "expires" attribute defined in the XML schema defined in table 9.3.1.2-1, of the <affiliation> element indicating expiration of affiliation of the MCPTT user to MCPTT group at the MCPTT client; and

15)	can contain one <functionalAlias> child element defined in the XML schema defined in table 9.3.1.2-1, of the <status> element, for each functional alias the group member has activated with the "functionalAliasID" attribute set to the corresponding functional alias ID.

Table 9.3.1.2-1: XML schema with elements and attributes extending the application/pidf+xml MIME body

<?xml version="1.0" encoding="UTF-8"?>

<xs:schema

targetNamespace="urn:3gpp:ns:mcpttPresInfo:1.0"

xmlns:xs="http://www.w3.org/2001/XMLSchema"

xmlns:mcpttPI10="urn:3gpp:ns:mcpttPresInfo:1.0"

elementFormDefault="qualified" attributeFormDefault="unqualified">

<!-- MCPTT specific child elements of presence element -->

<xs:element name="p-id" type="xs:string"/>

<xs:element name="groupStatus" type="mcpttPI10:groupStatusType"/>

<xs:element name="additionalData" type="mcpttPI10:additionalDataType"/>

<!-- MCPTT specific child elements of tuple element -->

<xs:element name="affiliation" type="mcpttPI10:affiliationType"/>

<xs:complexType name="affiliationType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="group" type="xs:anyURI" use="optional"/>

<xs:attribute name="client" type="xs:anyURI" use="optional"/>

<xs:attribute name="status" type="mcpttPI10:statusType" use="optional"/>

<xs:attribute name="expires" type="xs:dateTime" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

<xs:simpleType name="statusType">

<xs:restriction base="xs:string">

<xs:enumeration value="affiliating"/>

<xs:enumeration value="affiliated"/>

<xs:enumeration value="deaffiliating"/>

</xs:restriction>

</xs:simpleType>

<xs:complexType name="groupStatusType">

<xs:simpleContent>

<xs:extension base="xs:string">

<xs:attribute name="ModifiedBy" type="xs:anyURI" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:simpleContent>

</xs:complexType>

<xs:complexType name="emptyType"/>

<xs:complexType name="additionalDataType">

<xs:complexContent>

<xs:extension base="mcpttPI10:emptyType">

<xs:attribute name="groupBroadcastAlias" type="xs:anyURI" use="optional"/>

<xs:attribute name="groupRegroupAlias" type="xs:anyURI" use="optional"/>

<xs:attribute name="groupCallOngoing" type="xs:boolean" use="optional"/>        <xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:extension>

</xs:complexContent>

</xs:complexType>

<!-- MCPTT specific child elements of status element -->

<xs:element name="functionalAlias" type="mcpttPI10:functionalAliasType"/>

<xs:complexType name="functionalAliasType">

<xs:sequence>

<xs:any namespace="##any" processContents="lax" minOccurs="0" maxOccurs="unbounded"/>

</xs:sequence>

<xs:attribute name="functionalAliasID" type="xs:anyURI" use="optional"/>

<xs:attribute name="expires" type="xs:dateTime" use="optional"/>

<xs:anyAttribute namespace="##any" processContents="lax"/>

</xs:complexType>

</xs:schema>

The application/pidf+xml MIME body refers to namespaces using prefixes specified in table 9.3.1.2-2.

Table 9.3.1.2-2: Assignment of prefixes to namespace names in the application/pidf+xml MIME body

| Prefix | Namespace |
| --- | --- |
| mcpttPI10 | urn:3gpp:ns:mcpttPresInfo:1.0 |
| NOTE: The "urn:ietf:params:xml:ns:pidf" namespace is the default namespace so no prefix is used for it in the application/pidf+xml MIME body. |  |

Table 9.3.1.2-3 ABNF syntax of values of the <groupStatus> element

in-peril-value = %x69.6E.2D.70.65.72.69.6C ; "in-peril"

emergency-value = %x65.6D.65.72.67.65.6E.63.79 ; "emergency"

End of changes