3GPP TSG-CT WG1 Meeting #138-e	C1-225804

# E-Meeting, 10th – 14th October 2022

# Source:	China Mobile, China Southern Power Grid

# Title:	New SID on network selection to support accessing an overlay network via an underlay network

# Document for:	Approval

# Agenda Item:	18.1.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on network selection to support accessing an overlay network via an underlay network

Acronym:	 FS_NetSel_OU

Unique identifier:	XXXX

Potential target Release:	Rel-18

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No |  |  |  |  |  |
| Don't know | X |  | X | X | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

The following requirement has been defined in TS 22.261 by SA1 for non-public networks:

Subject to an agreement between the operators and service providers, operator policies and the regional or national regulatory requirements, the 5G system shall support for non-public network subscribers:

- access to subscribed PLMN services via the non-public network;

- seamless service continuity for subscribed PLMN services between a non-public network and a PLMN;

- access to selected non-public network services via a PLMN;

- seamless service continuity for non-public network services between a non-public network and a PLMN.

To support this stage 1 requirement, SA2 specified underlay/overlay NPN deployment options in TS 23.501.

Since SNPN is for non-public use, the service access and association between SNPNs and PLMNs are different from those between PLMNs for public users. The following scenarios or requirements need to be considered when the UE selects an underlay network for a specific overlay network in real deployment environments.

Issue#1: some SNPNs as an overlay network do not allow accessing its services from a random PLMN.

Issue#2:Some SNPNs as an overlay network prefers specific QoS requirement fulfilled by PLMNs as an underlay network.

Issue#3: Some SNPNS and PLMNsprefer an agreement and SLA to support accessing an overlay network via an underlay network.

In addition, the SLA between an underlay network and an overlay network has already been considered in TS 23.501.

On the other hand, SNPN/PLMN selection procedures in current TS23.122 cannot support the deployments corresponding to above issues thus require enhancements.

Furthermore, in Rel-16, SNPNs as underlay networks are subscribed SNPN, while in Rel-17 SNPNs include non- subscribed SNPNs.Whether a non- subscribed SNPNs can be an underlay networks needs to be considered.

Therefore, the current proposed study item is proposed to fulfill the SNPN selection and PLMN selection for the above deployments. It is suggested to cover the following key issues:

-Whether the decision of accessing an overlay network via an underlay network needs to be made before the network selection of an underlay network in the above deployments;

-How an agreement and a SLA between networks can be reflected in the network selection to support accessing an overlay network via an underlay network in the above deployments;

-Whether a non-subcribed SNPN is not allowed to be selected in the case that the UE has a PLMN service or is going to access a PLMN service via a SNPN in the above deployments.

4	Objective

The study item includes the following tasks:

-Task#1: To study potential solutions on the PLMN selection to support accessing to multiple subscribed SNPNs via a PLMN in the deployments an agreement/SLA between networks required and SNPN particularity considered;

-Task#2: To study potential solutions on the SNPN selection to support accessing to a PLMN via a SNPN in the deployments an agreement/SLA between networks required.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.700-aa | Study on network selection for underlay/overlay access | TSG#99 (March, 2023) | TSG#100 (June, 2023) | Chen Xu, China Mobile, chenxu@chinamobile.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |

6	Work item Rapporteur(s)

Chen Xu, China Mobile, chenxu@chinamobile.com

7	Work item leadership

CT1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Mobile

China Southern Power Grid

Vivo