3GPP TSG-CT WG1 Meeting #138-e	C1-22XXXX

# E-Meeting, 10th – 14th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 0731 | rev | 3 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connectivity for NSWO authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenvo, Nokia, Nokia Shanghi Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSWO_5G |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 6.3.12b in TS 23,501 defines a new list of PLMNs advertized by a discovered WLAN which can be used by the UE with the HPLMN listed in the new list of PLMNs to connect to use the 5G NSWO authentication procedure defined in TS 33.501. This new list of PLMNs is necessary as described in S2-2206732, since otherwise it would be impossible for the UE to determine for the UE whether it initiates the Authentication and Key Agreement procedure specified in TS 33.402 or the NSWO authentication procedure specified in TS 33.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added and identity and definition of a new information element for the list of PLMNs with AAA connectivity to a 5GC. Added Abbreviation for NSWOF. Backward compatibility analysis on the UE: The PLMN List with AAA connectivity to 5GC is not backward compatible to 3GPP TS 24.302 V17.6.0. A UE not having implemented this List of PLMN with AAA connectivity to 5GC, cannot determine how to initiate the NSWO authentication procedure specified in TS 33.501. Limiting the PLMN List IE to AAA connectivity to EPC is backward compatible. A UE not having implemented this List of PLMN with AAA connectivity to 5GC can determine how to initiate the Authentication and Key Agreement procedure specified in TS 33.402 Backward compatibility analysis on the WLAN: The PLMN List with AAA connectivity to 5GC is not backward compatible to 3GPP TS 24.302 V17.6.0. A WLAN not having implemented this List of PLMN with AAA connectivity to 5GC, cannot allocate NSWOF for the UE to perform the NSWO authentication procedure specified in TS 33.501. Limiting the PLMN List IE to AAA connectivity to EPC is backward compatible. A WLAN not having implemented this List of PLMN with AAA connectivity to 5GC can determine how to route the UE's request for the Authentication and Key Agreement procedure specified in TS 33.402 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE with NSWO capability cannot identify PLMNs for connecting to NSWOF to perform 5G NSWO authentication procedure defined in TS 33.501. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, H.2.4.1, H.2.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************* NEXT CHANGE ***********************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAA	Authentication, Authorization and Accounting

ACL	Access Control List

AKA	Authentication and Key Agreement

ANDSF	Access Network Discovery and Selection Function

ANDSF-SN	Access Network Discovery and Selection Function Server Name

ANID	Access Network Identity

ANQP	Access Network Query Protocol

APN	Access Point Name

DHCP	Dynamic Host Configuration Protocol

DM	Device Management

DNS	Domain Name System

DSCP	Differentiated Services Code Point

DSMIPv6	Dual-Stack MIPv6

eAN/PCF	Evolved Access Network Packet Control Function

EAP	Extensible Authentication Protocol

EPC	Evolved Packet Core

ePDG	Evolved Packet Data Gateway

EPS	Evolved Packet System

ERP	EAP Re-authentication Protocol

ESP	Encapsulating Security Payload

FQDN	Fully Qualified Domain Name

GAA	Generic Authentication Architecture

GBA	Generic Bootstrapping Architecture

HA	Home Agent

H-ANDSF	Home-ANDSF

HRPD	High Rate Packet Data

HSGW	HRPD Serving Gateway

IEEE	Institute of Electrical and Electronics Engineers

IFOM	IP Flow Mobility

IKEv2	Internet Key Exchange version 2

IARP	Inter-APN Routing Policy

IPMS	IP Mobility Mode Selection

ISMP	Inter-system Mobility Policy

ISRP	Inter-system Routing Policy

IANA	Internet Assigned Numbers Authority

I-WLAN	Interworking – WLAN

MAPCON	Multi Access PDN Connectivity

MCM	Multi-connection mode

MO	Management Object

NAI	Network Access Identifier

NAP	Network Access Provider

NBIFOM	Network-Based IP Flow Mobility

NBM	Network based mobility management

NSP	Network Service Provider

NSSAI	Network Slice Selection Assistance Information

NSWO	Non-Seamless WLAN Offload

NSWOF	Non-Seamless WLAN Offload Function

OMA	Open Mobile Alliance

OPI	Offload Preference Indicator

PCO	Protocol Configuration Options

P-GW	PDN Gateway

PDU	Protocol Data Unit

PSPL	Preferred Service Provider List

QoS	Quality of Service

SCM	Single-connection mode

S-GW	Serving Gateway

S-NSSAI	Single NSSAI

SPI	Security Parameters Index

TFT	Traffic Flow Template

TSCM	Transparent single-connection mode

UE	User Equipment

UICC	Universal Integrated Circuit Card

V-ANDSF	Visited-ANDSF

W-APN	WLAN APN

WiMAX	Worldwide Interoperability for Microwave Access

WLAN	Wireless Local Area Network

WLANSP	WLAN Selection Policy

WLCP	WLAN Control Protocol

WMF	WiMAX Forum

******************* NEXT CHANGE ***********************

### H.2.4.1	Information Element Identity (IEI)

Indicates the information element identity. The following values for IEI are defined in this version of the specification:

00000000	PLMN List

00000001	PLMN List with S2a connectivity

00000010	PLMN List with trusted 5G connectivity

00000011	PLMN List with trusted 5G connectivity-without-NAS

00000100	PLMN List with AAA connectivity to 5GC

00000101

To

11111111	Reserved

******************* NEXT CHANGE ***********************

### H.2.4.X	PLMN List with AAA connectivity to 5GC IE

The PLMN List with AAA connectivity to 5GC information element is used by the WLAN to indicate the PLMNs deploying NSWOF, with which the WLAN supports AAA connectivity to 5GC and is able to perform NSWO in 5GS procedures as specified in annex S of 3GPP TS 33.501 [78].

The format of the PLMN List with AAA connectivity to 5GC information element is identical to the format of the PLMN List information element defined in clause H.2.4.2.

******************* END OF CHANGES ***********************