3GPP TSG-CT WG1 Meeting #138-e	C1-225834

# E-Meeting, 10th – 14th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.538 | CR | 0012 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of the L3 message format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH |  |  |  |  |  | Date: |  |  | 2022-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some IEs included in the messages defined in A.2.1 doesn’t align with the coding part as defined in A.2.2. Backwards compatibility analysis: There is no modification of the IE coding. Only alignment is proposed. Thus there is no backwards compatible issues based on the change of this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align length or format of the IE included in the L3 message with the IE coding as defined in A.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The length or format of the IE doesn’t align with the IE coding and may cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.1.1, A.2.1.2, A.2.1.3, A.2.1.4, A.2.1.5, A.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

A.2.1.1	for sending a message to MSGin5G Client

For sending a message to MSGin5G Client, the Application Client may use the message content specified in Table A.2.1.1-1.

Message type:	MESSAGE SENDING REQUEST

Significance:	dual

Direction:	the Application Client of the Constrained UE to the MSGin5G Client of the MSGin5G Gateway UE

Table A.2.1.1-1: message content for sending a message to MSGin5G Client

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Target address | Target address A.2.2.2 | M | LV | 6-n |
|  | Message ID | Message ID A.2.2.4 | M | V | 16 |
|  | Payload | Payload A.2.2.5 | M | LV-E | 2-65537 |
| A | Application ID | Application ID A.2.2.3 | O | TV | 3 |
| B | Delivery status required | Delivery status required A.2.2.6 | O | TV | 1 |
| D | Target Type | Target Type A.2.2.7 | O | TV | 1 |

If using the message content specified in table A.2.1.1-1, the Application Client may generate a message according to 6.4.2.3.1 and send the generated message as a UDP message to the MSGin5G Client.

* * * Next Change * * * *

A.2.1.2	for sending a message delivery report to MSGin5G Client

For sending a message delivery status report to MSGin5G Client, the Application Client may use the message content specified in Table A.2.1.2-1.

Message type:	DELIVERY REPORT SENDING REQUEST

Significance:	dual

Direction:	the Application Client of the Constrained UE to the MSGin5G Client of the MSGin5G Gateway UE

Table A.2.1.2-1: message content for sending a message delivery status report to MSGin5G Client

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Delivery Status | Delivery Status A.2.2.8 | M | V | 1 |
|  | Message ID | Message ID A.2.2.4 | M | V | 16 |
|  | Reply-to Message ID | Reply-to Message ID A.2.2.13 | M | V | 16 |

If using the message content specified in table A.2.1.2-1, the Application Client may generate a message according to 6.4.2.3.2 and send the generated message as a UDP message to the MSGin5G Client.

* * * Next Change * * * *

A.2.1.3	for sending a message to Application Client

For sending a message to Application Client, the MSGin5G Client may use the message content specified in Table A.2.1.3-1.

Message type:	MESSAGE RECEIVED REQUEST

Significance:	dual

Direction:	the MSGin5G Client of the MSGin5G Gateway UE to the Application Client of the Constrained UE

Table A.2.1.3-1: message content for sending a message to Application Client

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Message ID | Message ID A.2.2.4 | M | V | 16 |
|  | Payload | Payload A.2.2.5 | M | LV-E | 2-65537 |
| F | Originator Address | Originator Address A.2.2.10 | O | TLV | 3-257 |
| E | Group ID | Group ID A.2.2.11 | O | TLV | 3-257 |
| B | Delivery status required | Delivery status required A.2.2.6 | O | TV | 1 |
| C | Priority | Priority A.2.2.9 | O | TV | 1 |

If using the message content specified in table A.2.1.3-1, the MSGin5G Client may generate a message according to 6.4.2.2.1 and send the generated message as a UDP message to the Application Client.

* * * Next Change * * * *

A.2.1.4	for sending a message delivery status report to Application Client

For sending a message delivery status report to Application Client, the MSGin5G Client may use the message content specified in Table A.2.1.4-1.

Message type:	DELIVERY REPORT RECEIVED REQUEST

Significance:	dual

Direction:	the MSGin5G Client of the MSGin5G Gateway UE to the Application Client of the Constrained UE

Table A.2.1.4-1: message content for sending a message delivery status report to MSGin5G Client

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Delivery Status | Delivery Status A.2.2.8 | M | V | 1 |
|  | Message ID | Message ID A.2.2.4 | M | V | 16 |
|  | Reply-to Message ID | Reply-to Message ID A.2.2.13 | M | V | 16 |

If using the message content specified in table A.2.1.4-1, the MSGin5G Client may generate a message according to 6.4.2.2.3 and send the generated message as a UDP message to the Application Client.

* * * Next Change * * * *

A.2.1.5	for sending a message sending response to Application Client

For sending a message sending response to Application Client, the MSGin5G Client may use the message content specified in Table A.2.1.5-1.

Message type:	MESSAGE SENDING RESPONSE

Significance:	dual

Direction:	the MSGin5G Client of the MSGin5G Gateway UE to the Application Client of the Constrained UE

Table A.2.1.5-1: message content for message sending response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Result | Result A.2.2.11 | M | V | 1 |
| X | Failure Reason | MSGin5G cause A.2.2.17 | O | TV | 2 |

* * * Next Change * * * *

A.2.1.6	for sending a message received response to MSGin5G Client

For sending a message sending response to Application Client, the MSGin5G Client may use the message content specified in Table A.2.1.6-1.

Message type:	MESSAGE RECEIVED RESPONSE

Significance:	dual

Direction:	the MSGin5G Client of the MSGin5G Gateway UE to the Application Client of the Constrained UE

Table A.2.1.6-1: message content for message sending response

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message Type | Message Type A.2.2.1 | M | V | 1 |
|  | Result | Result A.2.2.11 | M | V | 1 |
| X | Failure Reason | MSGin5G cause A.2.2.17 | O | TV | 2 |

If using the message content specified in table A.2.1.6-1, the Application Client may generate a message according to 6.4.2.3.3 and send the generated message as a UDP message to the MSGin5G Client.

* * * End of Change * * * *