3GPP TSG-CT WG1 Meeting #141e	C1-232394

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5284 | rev | - | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The mobility management based network slice usage control – general introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2 agreed CR(S2-2303810), the network slice usage control feature is introduced for Rel-18 to enable the functionality to monitor the usage of slices and trigger the release of a PDU session and/or the deregistration of a slice. This paper mainly captures the mobility management aspect of network slice usage control on the network side and the UE side. The network can inform the supporting UE of the on-demand NSSAI in the configured NSSAI. The UE is required to include the on-demand S-NSSAI(s) in the requested NSSAI only when it is on demand. Then the AMF will manage the de-registration inactivity timer per on-demand S-NSSAI in case of the S-NSSAI becomes the allowed S-NSSAI but has no associated PDU sessions for a certain time. Regarding the management of the de-registration inactivity timer on the UE side, as per the discussion in the C1-232395, this CR proposes that the AMF does not send this timer to the UE and the UE is not required to implement this timer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose a new clause to introduce the mobility management based network slice usage control. Regarding the management of the de-registration inactivity timer on the UE side, this CR proposes that the AMF does not send this timer to the UE and the UE is not required to implement this timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network slice usage control feature is not supported in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.6.2.x	Mobility management based network slice usage control

The UE and the network may support the mobility management based network slice usage control.

The HPLMN can perform the network slice usage control to monitor and control the on-demand NSSAI in case of the S-NSSAI becomes the allowed S-NSSAI but has no associated PDU sessions for a certain time. The period of a certain time is managed by the de-registration inactivity timer per network slice in the network.

The AMF may inform the UE about the on-demand NSSAI if the UE indicates support of the network slice usage control in the 5GMM capability IE in the REGISTRATION REQUEST message. The UE may include the on-demand S-NSSAI(s) in the requested NSSAI. After the AMF receives the on-demand S-NSSAI(s), and:

a)	if the AMF determines the on-demand S-NSSAI(s) is the allowed S-NSSAI, the AMF shall:

1)	include the on-demand S-NSSAI in the allowed NSSAI to the UE during the generic UE configuration update procedure or the registration procedure; and

2)	locally start the de-registration inactivity timer for the on-demand S-NSSAI; or

b)	if the AMF determines the on-demand S-NSSAI(s) is removed from the allowed S-NSSAI or the de-registration inactivity timer expires, the AMF shall:

1)	include a new allowed NSSAI to the UE during the generic UE configuration update procedure; and

2)	locally stop and reset the de-registration inactivity timer for the on-demand S-NSSAI if running; or

c)	if all the PDU sessions associated with the on-demand S-NSSAI(s) are released successfully, the AMF shall locally start the de-registration inactivity timer for the on-demand S-NSSAI; or

d)	if one PDU session associated with the on-demand S-NSSAI(s) is established successfully, the AMF shall locally stop and reset the de-registration inactivity timer for the on-demand S-NSSAI.

The mobility management based network slice usage control is not applicable for the S-NSSAI used for emergency services or priority services.

* * * End of Changes * * * *