3GPP TSG-CT WG1 Meeting #141e	C1-232079v1

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 5180 | rev | 1 | Current version: | 18.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify AMF behaviour when S-NSSAI to be replaced is available |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 23.501 clause 5.15.19, the AMF behavior when the S-NSSAI which has been replaced is available is specified as follows: “When the AMF is notified that the S-NSSAI is available again or the congestion of the S-NSSAI has been mitigated, if the AMF has configured the supporting UE with the Alternative S-NSSAI, and the AMF determines for the UE to use the S-NSSAI again, the AMF reconfigures the supporting UE (e.g. by using UE Configuration Update message) to use the S-NSSAI. If there is an existing PDU Session associated with the Alternative S-NSSAI, the AMF sends updates to the SMF of the PDU Session, e.g. triggering Nsmf_PDUSession_UpdateSMContext service operation, that the PDU Session is to be transferred to the S-NSSAI.” It should be specified in TS 24.501 that when the S-NSSAI which has been replaced is available, the AMF updates the UE with such information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the S-NSSAI which has been replaced is available, the AMF provides updated alternative NSSAI excluding the S-NSSAI which has been replaced and corresponding alternative S-NSSAI to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will never know when the S-NSSAI which has been replaced is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

4.6.2.7	Mobility management based network slice replacement

If the UE and network support network slice replacement, and the AMF determines that an S-NSSAI included in the allowed NSSAI needs to be replaced with an alternative S-NSSAI, the AMF provides the alternative S-NSSAI in the allowed NSSAI (if not included yet) and in the configured NSSAI (if not included yet) and the mapping information between the S-NSSAI to be replaced and the alternative S-NSSAI to the UE during UE configuration update procedure as follows:

a)	for non-roaming UE, the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

b)	for roaming UE:

1)	if the S-NSSAI included in the allowed NSSAI needs to be replaced, the AMF provides the mapping information between the S-NSSAI included in the allowed NSSAI and the alternative S-NSSAI to the UE; and

2)	if the S-NSSAI included in the mapped S-NSSAI(s) for the allowed NSSAI needs to be replaced, the AMF provides the mapping information between the S-NSSAI included in the mapped S-NSSAI(s) for the allowed NSSAI and the alternative S-NSSAI to the UE.

NOTE:	The alternative S-NSSAI may be part of or not part of the subscribed S-NSSAI(s) in the UE subscription.

Editor's note:	(WI: eNS_Ph3, CR: 5069) Whether an S-NSSAI included in the allowed NSSAI can be replaced with more than one alternative S-NSSAI is FFS.

If the AMF determines that the S-NSSAI which has been replaced is available, the AMF provides the updated alternative NSSAI excluding the S-NSSAI which has been replaced and the corresponding alternative S-NSSAI to the UE during the UE configuration update procedure or during the registration procedure.

* * * End of Change * * * *