3GPP TSG-CT WG1 Meeting #141e	C1-232598

# Online 17– 21 April 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.545 | CR | 0069 | rev | 1 | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the location service registration procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GFLS |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to S6-230483, stage2 has added the procedure and information flow for location service registration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of the location service registration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for location service registration are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.x(new), B.3.1.3.1, B.3.1.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.2.x	Location service registration procedure

6.2.x.1	SLM client HTTP procedure

The SLM-C sends a location service registration request when it needs to register the available location service to the SLM-S to report the UE’s location capabilities before the SLM-S requesting the location information. In order to send the location service registration request, the SLM-C shall send an HTTP POST request message according to procedures specified in IETF RFC 7231 [16]. In the HTTP POST request message, the SLM-C:

a)	shall include a Request-URI set to the URI corresponding to the identity of the SLM-S.

b)	shall include an Authorization header field with the "Bearer" authentication scheme set to an access token of the "bearer" token type as specified in IETF RFC 6750 [13];

c)	shall include an application/vnd.3gpp.seal-location-info+xml MIME body and in the <location-info> root element:

1)	shall include a <requested-identity> element with a <VAL-user-id> child element set to the identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE;

2)	may include a <location-capability> element specifying the information of the location capabilities of VAL UE for which the location service is registered. In the <location-capability> element, the SLM-C may include:

i)	a <access-type> child element specifying the identity of the available access type of the VAL UE; and/or

ii)	a <positioning-method>child element specifying the identity of the available positioning methods of the VAL UE;

6.2.x.2	SLM server HTTP procedure

Upon receiving an HTTP POST request containing:

a)	an Accept header field set to "application/vnd.3gpp.seal-location-info+xml";

b)	a Content-Type header field set to "application/vnd.3gpp.seal-location-info+xml";

c)	an application/vnd.3gpp.seal-location-info+xml MIME body with a < location-capability > element included in the <location-info> root element;

the SLM-S:

a)	shall determine the identity of the sender of the received HTTP POST request as specified in clause 6.2.1.1; and

1)	if the identity of the sender of the received HTTP POST request is not authorized to register any location services, shall respond with a HTTP 403 (Forbidden) response to the HTTP POST request and shall skip rest of the steps;

2)	shall support handling an HTTP POST request from a SLM-C according to procedures specified in IETF RFC 4825 [9] "POST Handling";

3)	may authorize the identity of the available access type of the VAL UE if received from SLM-C; and/or

4)	may authorize the identity of the available positioning methods of the VAL UE if received from SLM-C;

b)	shall generate an HTTP 200 (OK) response according to IETF RFC 7231 [16] and send the HTTP 200 (OK) response towards the SLM-C.

6.2.x.3	SLM client CoAP procedure

In order to register the available location services to the SLM-S to report the UE’s location capabilities before the SLM-S requesting the location information, the SLM-C shall send a CoAP POSTrequest message to the SLM-S according to procedures specified in IETF RFC 7252 [21]. In the CoAP POST request, the SLM-C:

a)	shall include a CoAP URI set to the URI corresponding to the identity of the SLM-S as specified in clause B.3.1.2.x;

1)	the "apiRoot" is set to the SLM-S URI;

2)	the "valServiceId" is set to specific VAL service; and

b)	shall include an Accept option set to "application/vnd.3gpp.seal-location-configuration+cbor"; and

b)	may include a "location-capability" object:

1)	may include a "access-type" is set to the identity of the available access type of the VAL UE;

2)	may include a "positioning-method " is set to the identity of the available positioning methods of the VAL UE; and/or

c)	shall send the request protected with the relevant ACE profile (OSCORE profile or DTLS profile) as described in 3GPP TS 24.547 [6].

6.2.x.4	SLM server CoAP procedre

Upon receiving of a CoAP POST request where the CoAP URI of the CoAP POST request identifies a registration as specified in clause B.3.1.2.x, the SLM-S:

a)	shall determine the identity of the sender of the received CoAP POST request as specified in clause 6.2.1.2, and:

1)	if the identity of the sender of the received CoAP POST request is not authorized to register any location services, shall respond with a CoAP 4.03 (Forbidden) response to the CoAP POST request and skip rest of the steps;

b)	may authorize the location-capability including:

1)	the identity of the available access type of the VAL UE if received from SLM-C; and/or

2)	the identity of the available positioning methods of the VAL UE if received from SLM-C;

c)	shall generate a CoAP 2.05 (Content) response according to IETF RFC 7252 [21] and send the CoAP 2.05 (Content) response towards the SLM-C.

* * * Next Change * * * *

B.3.1.2.1	Overview

Figure B.3.1.2.1-1: Resource URI structure of the SU_LocationReporting API provided by SLM-S

Table B.3.1.2.1-1 provides an overview of the resources and applicable CoAP methods.

Table B.3.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| Registration | /val-services/{valServiceId}/registration | POST | Register the available location services of the SLM-C. |
| Trigger Configurations | /val-services/{valServiceId}/trigger-configurations | GET | Retrieve trigger configuration of the SLM-C for a given VAL service, according to query criteria. |
| Location Report | /location-reports/{valTgtUe} | PUT | Report location information of the SLM-C. |
| Locations | /locations | FETCH | Observe the location information of another SLM-C. |
|  |  | GET | Retrieve location information of another SLM-C. |
| Location Area Information | /location-area-info | FETCH | Query the information, e.g., user information based on the specific location area. |

* * * Next Change * * * *

B.3.1.2.x	Resource: Registration

B.3.1.2.x.1	Description

The Registration resource allows a SLM-C to register the available location services to SLM-S.

B.3.1.2.x.2	Resource Definition

Resource URI: {apiRoot}/su-lr/<apiVersion>/val-services/{valServiceId}/registration

This resource shall support the resource URI variables defined in the table B.3.1.2.x.2-1.

Table B.3.1.2.x.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause C.1.1 of 3GPP TS 24.546 [29]. |
| apiVersion | string | See clause B.3.1.1. |
| valServiceId | string | Identifier of a VAL service. |

B.3.1.2.x.3	Resource Standard Methods

B.3.1.2.x.3.1	POST

This operation retrieves the allowed registration.

This method shall support URI query options specified in table B.3.1.2.x.3.1-1, the response data structures and response codes specified in table B.3.1.2.x.3.1-1.

Table B.3.1.2.x.3.1-1: URI query options supported by the POST Request on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| val-tgt-ue | string | M | 1 | The identifier of VAL UE owns the registration. |
| NOTE: Other request options also apply in accordance with normal CoAP procedures. |  |  |  |  |

Table B.3.1.2.x.3.1-2: Data structures supported by the POST Request payload on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationCapability | O | 0..1 | The information of location capability of VAL UE for which the location service is registered. |

* * * Next Change * * * *

B.3.1.3.2.3	Type: LocationCapability

Table B.3.1.3.2.3-1: Definition of type LocationCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| accessType | AccessType | O | 0..1 | The identity of the available access type of the VAL UE. |  |
| positioningMethod | PositioningMethod | O | 0..1 | The identity of the available positioning methods of the VAL UE. |  |

* * * End of changes * * *