3GPP TSG-CT WG1 Meeting #141-e	C1-232377

# E-Meeting, 17th - 21st April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.302 | CR | 0750 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN services for N5CW devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has agreed in CR3821 to 23.501 (see S2-2303635) clause 5.30.2.15 that that N5CW devices may access 5GC in an SNPN via a trusted WLAN access network that supports a TWIF function. A WLAN access network may advertise (e.g. with ANQP), not only the PLMNs with which "5G connectivity-without-NAS" is supported (as specified in clause 6.3.12a.1), but also the SNPNs with which "5G connectivity-without-NAS" is supported, as well as the related parameters and indications defined in clause 5.30.2.2 (i.e. human-readable network name(s), GIN(s), indication whether access using credentials from a Credentials Holder is supported, indication whether SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN): #### 5.30.2.15 Access to SNPN services for N5CW devices Devices that do not support 5GC NAS signalling over WLAN access (referred to as "Non-5G-Capable over WLAN" devices, or N5CW devices for short), may access 5GC in an SNPN via a trusted WLAN access network that supports a TWIF function. To access SNPN services the N5CW device performs the following procedure: - A WLAN access network may advertise (e.g. with ANQP), not only the PLMNs with which "5G connectivity-without-NAS" is supported (as specified in clause 6.3.12a.1), but also the SNPNs with which "5G connectivity-without-NAS" is supported, as well as the related parameters and indications defined in clause 5.30.2.2 (i.e. human-readable network name(s), GIN(s), indication whether access using credentials from a Credentials Holder is supported, indication whether SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN). - The N5CW device initiates the access network selection procedure by sending an ANQP query to each discovered WLAN access network and constructs a list of available SNPNs with which "5G connectivity-without-NAS" is supported. This list contains the SNPNs with which "5G connectivity-without-NAS" is supported as advertised by all the discovered WLAN access networks. - The N5CW device selects an SNPN that is included in the list of available SNPNs with which "5G connectivity-without-NAS" is supported following the procedure in clause 5.30.2.4. - The N5CW device selects a WLAN access network (e.g. an SSID) that supports "5G connectivity-without-NAS" to the selected SNPN and initiates the "Initial Registration and PDU Session Establishment" procedure specified in clause 4.12b.2 of TS 23.502 [3]. If there are multiple WLAN access networks that support "5G connectivity-without-NAS" to the selected SNPN, then the N5CW device selects the highest priority WLAN access network from this list. To determine the priority of a WLAN access network, the N5CW device shall apply the WLANSP rules (if provided), and the procedure specified in clause 6.6.1.3 of TS 23.503 [45], "UE procedure for selecting a WLAN access based on WLANSP rules". If the N5CW device is not provided with WLANSP rules, the N5CW device determines the priority of a WLAN access network by using implementation means. NOTE: How the N5CW device selects credentials to use for SNPN access is implementation specific. Accordingly, this CR adds a new SNPN List with trusted 5G Connectivity without-NAS IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new SNPN List with trusted 5G Connectivity without-NAS IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 3 not aligned with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.2.4.1, H.2.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note to MCC: If 24.302CR0748 and 24.302CR750 are both approved, please implement CR0748 first before CR 0750 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

### H.2.4.1	Information Element Identity (IEI)

Indicates the information element identity. The following values for IEI are defined in this version of the specification:

00000000	PLMN List

00000001	PLMN List with S2a connectivity

00000010	PLMN List with trusted 5G connectivity

00000011	PLMN List with trusted 5G connectivity-without-NAS

00000100	PLMN List with AAA connectivity to 5GC

00000101

To

00011111	Reserved

00100000	SNPN List with trusted 5G connectivity

00100010	SNPN List with trusted 5G connectivity-without-NAS

00100011

To

11111111	Reserved

*** Next change ***

### H.2.4.X	SNPN List with trusted 5G Connectivity-without-NAS IE

The SNPN List with trusted 5G Connectivity-without-NAS information element is used by the network to indicate the SNPNs for which the WLAN provides connectivity to a 5GCN, for devices without NAS capability, that can be selected from the WLAN. The format of the SNPN List with trusted 5G Connectivity-without-NAS information element is identical to the format of the SNPN List with trusted 5G Connectivity information element defined in clause H.2.4.7.

*** End change ***