3GPP TSG-CT WG1 Meeting #146	C1-240380

# Online, 22– 26 January 2024													(revision of C1-240133)

Source:	Huawei, HiSilicon

Title:	IMS Data Channel Interaction with CRS service

Spec:	3GPP TS 24.186 v1.0.0

Agenda item:	18.3.8

Document for:	Decision

1. Introduction

This p-CR provides the content of IMS data channel interaction with the CRS (Customized Ringing Signal) service specified in 3GPP TS 24.183.

This document focuses on the interaction between the data channel media in IMS basic call and the CRS service specified in 3GPP TS 24.183 Release 18, and does not involve usage of data channel media in the enhanced CRS service.

2. Reason for Change

The data channel media in the IMS basic call interaction with supplementary services needs to be defined for new 3GPP TS 24.186.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.186 v1.0.0.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[5]	IETF RFC 5688: "A Session Initiation Protocol (SIP) Media Feature Tag for MIME Application Subtype".

[6]	IETF RFC 6809: "Mechanism to Indicate Support of Features and Capabilities in the Session Initiation Protocol (SIP)".

[7]	IETF RFC 3264: "An Offer/Answer Model with the Session Description Protocol (SDP) ".

[8]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[9]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[10]	3GPP TS 24.173: "IMS Multimedia telephony communication service and supplementary services; Stage 3".

[11]	3GPP TS 24.275: "Management Object (MO) for Basic Communication Part (BCP) of IMS Multimedia Telephony (MMTEL) communication service".

[12]	3GPP TS 22.261: " Service requirements for the 5G System; Stage 1".

[13]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[14]	IETF RFC 8864: "Negotiation Data Channels Using the Session Description Protocol (SDP)".

[15]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[16] 	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[17]	3GPP TS 24.615: "Communication Waiting (CW) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[18]	3GPP TS 29.175: "IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services; Stage 3".

[19]	3GPP TS 29.176: "IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3".

[20]	3GPP TS 32.260: "Telecommunication management; Charging management; IP Multimedia Subsystem (IMS) charging".

[21]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; stage 2".

[22]	3GPP TS 24.647: "Advice Of Charge (AOC) using IP Multimedia (IM) Core Network (CN) subsystem".

[23]	3GPP TS 24.239: "Flexible Alerting (FA) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[24]	3GPP TR 24.174: "Support of multi-device and multi-identity in the IP Multimedia Subsystem (IMS); Stage3.

[25]	3GPP TS 24.642: " Completion of Communications to Busy Subscriber (CCBS) and Completion of Communications by No Reply (CCNR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[x]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) Customized Ringing Signal (CRS); Protocol specification".

* * * Next Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AR	Augmented Reality

AOC	Advice Of Charge

AS	Application Server

CCBS	Completion of Communications to Busy Subscriber

CCNL	Completion of Communications on Not Logged-in

CCNR	Completion of Communications by No Reply

CD	Communication Deflection

CDIV	Communication DIVersion

CFB	Communication Forwarding Busy

CFNL	Communication Forwarding on Not Logged-in

CFNR	Communication Forwarding No Reply

CFNRc	Communication Forwarding on subscriber Not Reachable

CFU	Communication Forwarding Unconditional

CN	Core Network

CONF	Conference

CRS	Customized Ringing Signal

CW	Communication Waiting

DC	Data Channel

FA	Flexible Alerting

IM	IP Multimedia

IMS	IP Multimedia Core Network Subsystem

MF	Media Function

MiD	Multi-iDentity

MRF	Multimedia Resource Function

MuD	Multi-Device

MWI	Message Waiting Indication

OIP	Originating Identification Presentation

OIR	Originating Identification Restriction

TIP	Terminating Identification Presentation

TIR	Terminating Identification Restriction

UE	User Equipment

* * * Next Change * * * *

10.x Customized Ringing Signal (CRS)

10.x.1	General

The CRS service is an operator specific service specified in 3GPP TS 24.183 [x], which describes three models of CRS service:

-	Download and play model: it has no interaction with IMS data channel.

-	Gateway model: it has no impact to IMS data channel.

-	Early session model: it has no impact to IMS data channel.

As specified in 3GPP TS 24.183 [x], the CRS media can consist of music, voice, text, video or other customized ringing signals.

10.x.2	Actions on the originating UE

When the originating UE is configured with IMS_DC_configuration node specified in 3GPP TS 24.275 [11] and the DC_Setup_Option leaf indicates that the IMS data channel is to be setup simultaneously while establishing an IMS session, the data channel media can be negotiated in the initial INVITE request and its corresponding response.

In early session model, if the originating UE initiates an MMTel session with an IMS data channel, the UE shall contain the data channel media description in the SDP offer of the initial SIP INVITE, as specified in the clause 9.3.2.1.2.

10.x.3	Actions on the CRS AS

Upon receiving the SIP requests and responses containing data channel SDP media descriptions, the CRS AS shall ignore them and just transmit them transparently.

* * * End of Changes * * * *