3GPP TSG-CT WG1 Meeting #146	C1-24XXXX

# Online, 22– 26 January 2024

Source:	Huawei, HiSilicon

Title:	EN resolution on defining additional LCS-UPP procedures

Spec:	3GPP TS 24.572 v1.0.0

Agenda item:	18.2.19

Document for:	Agreement

1. Introduction

This p-CR is to resolve an EN on defining additional UPP-CM procedures.

2. Reason for Change

In the current TS 24.572 sub 7.1, below EN is remained. Actually, the whole section 7 is defining procedures for LCS-UPP, not for LCS-UP connection management, which is covered in section 6. Hence, the wording "e.g. for LCS-UP connection management" in the EN is very confusing and needs to be removed.

<Editor’s note:	Additional LCS-UPP procedures between the LMF and the UE, e.g. for LCS-UP connection management, is FFS>

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.572 v1.0.0.

* * * First Change * * * *

7.1	Overview

The main function of the Location Services User Plane protocol (LCS-UPP) is to support generic transport between the UE and the LMF of messages of positioning related protocols:

a)	LPP messages; and

b)	location supplementary services messages, only including:

1)	messages for EventReport operations (see 3GPP TS 24.080 [11]);

2)	messages for PeriodicTriggeredInvoke operations (see 3GPP TS 24.080 [11]); and

3)	messages for MSCancelDeferredLocation operations (see 3GPP TS 24.080 [11]).

LCS-UPP procedures are performed between a Location Services User Plane (LCS-UP) entity in a UE and an LCS-UP entity in the LMF.

The following UE-initiated LCS-UPP procedures are specified:

a)	uplink LCS-UP transport procedure.

The following LMF-initiated LCS-UPP procedures are specified:

a)	downlink LCS-UP transport procedure.

Editor’s note:	Additional LCS-UPP procedures between the LMF and the UE is FFS.

The uplink LCS-UP transport procedures and the downlink LCS-UP transport procedures can be performed on the user plane connection of a PDU session via 3GPP access in 5GS as specified in clause 7.3.

LCS-UPP messages are transported in an IP packet according to clause 7.2.1.

LCS-UPP is a standard L3 protocol according to 3GPP TS 24.007 [7], LCS-UPP messages are standard L3 messages according to 3GPP TS 24.007 [7] and error behaviour specified for L3 protocol according to 3GPP TS 24.007 [7] applies for LCS-UPP.

* * * End of Changes * * * *