3GPP TSG-CT WG1 Meeting #146	C1-240302

# Online , 22– 26 January 2024

Source:	Lenovo

Title:	Restructuring of resource URI for ADAES

Spec:	3GPP TS 24.559V0.3.1

Agenda item:	18.2.29

Document for:	Agreement

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Resource URI needs to be modified to cover all ADAE features

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.559V0.3.1.

* * * First Change * * * *

#### 7.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.3.1-1 depicts the resource URI structure of the ADAE_ServiceConfiguration API.

Figure 7.1.3.1-1: Resource URI structure of the ADAE_ServiceConfiguration API

Table 7.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| Application performance event subscription | /application-performance | POST | Subscription to the VAL performance analytics event |
| Individual application performance event subscription | /application-performance/{appPerfId} | DELETE | Deletes an individual VAL performance analytics event |
| Edge load event subscription | /edge-load | POST | Subscription to the edge load data collection event |
| Individual edge load event subscription | /edge-load/{edgeLdId} | DELETE | Deletes an individual edge load data collection subscription |
| Service experience | /service-experience | POST | Configure triggers for reports on the service experience information |
| Individual service experience | /service-experience/{srvTrigId} | DELETE | Delete an individual trigger-configuration for service experience report |

* * * End of Changes * * * *