3GPP TSG-CT WG1 Meeting #159	C1-260338

Goa, India; 9th – 13th Feburary 2026

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.548 | CR | 0108 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resource representation and APIs for store and forward provided by SNRM-C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2026-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The application enablers provisioning the satellite S&F configuration to the VAL UE and expose the satellite S&F events information to the VAL server is introduced in SA6. CT1 has completed HTTP related encoding and procedures, but CoAP encoding and procedures are missing. This CR is proposed to add the resource representation and APIs for store and forward provided by SNRM-C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the resource representation and APIs for store and forward provided by SNRM-C. Backwards compatibility analysis The CR is backwards compatible since supporting CoAP procedures will not impact the existing Store and forward HTTP procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource representation and APIs for store and forward provided by SNRM-C is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.a(new),A.a.1(new), A.a.1.1(new), A.a.1.2(new), A.a.1.2.1(new), A.a.1.2.2(new), A.a.1.2.2.1(new), A.a.1.2.2.2(new), A.a.1.2.2.3(new), A.a.1.2.3(new), A.a.1.2.3.1(new), A.a.1.2.3.2(new), A.a.1.2.3.3(new), A.a.1.3(new), A.a.1.3.1(new), A.a.1.3.2(new), A.a.1.3.2.1(new), A.a.1.3.2.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

A.a	Resource representation and APIs for store and forward provided by SNRM-C

A.a.1	SU_StoreForward API provided by SNRM-C

A.a.1.1	API URI

The CoAP URIs used in CoAP requests from SNRM-C towards the SNRM-S shall have the Resource URI structure as defined in clause C.1.1 of 3GPP TS 24.546 [31] with the following clarifications:

-	The <apiName> shall be "su-sf".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause A.a.1.2

A.a.1.2	Resources

A.a.1.2.1	Overview

Figure A.a.1.2.1-1: Resource URI structure of the SU_StoreForward API

Table A.a.1.2.1-1 provides an overview of the resources and applicable CoAP methods.

Table A.a.1.2.1-1: Resources and methods overview

| Resource name | Resource URI | CoAP method | Description |
| --- | --- | --- | --- |
| Notification | /val-services/{valServiceId}/notification | PUT | Notify the requested satellite S&F events. |
| Report | /val-services/{valServiceId}/report | PUT | Report the satellite inforamtion of the SNRM-C for a given VAL service. |

A.a.1.2.2	Resource: Notification

A.a.1.2.2.1	Description

The Report resource allows a SNRM-C to notify the stallite store and forward event to SNRM-S.

A.a.1.2.2.2	Resource Definition

Resource URI: {apiRoot}/su-sf/<apiVersion>/val-services/{valServiceId}/notification

This resource shall support the resource URI variables defined in the table A.a.1.2.2.2-1.

Table A.a.1.2.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See Annex C.1.1 of 3GPP TS 24.546 [29]. |
| apiVersion | string | See clause A.a.2.1. |
| valServiceId | string | Identifier of a VAL service. |

A.a.1.2.2.3	Resource Standard Methods

A.a.1.2.2.3.1	PUT

This operation sends the requested S&F events and trigger criterion.

This method shall support the response data structures and response codes specified in table A.a.1.2.2.3.1-1.

Table A.a.1.2.2.3.1-1: Data structures supported by the PUT Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Notification | M | 1 | 2.04 Changed | The requested satellite S&F event. |

A.a.1.2.3	Resource: Report

A.a.1.2.3.1	Description

The Report resource allows a SNRM-C to report the stallite store and forward information to SNRM-S.

A.a.1.2.3.2	Resource Definition

Resource URI: {apiRoot}/su-sf/<apiVersion>/val-services/{valServiceId}/report

This resource shall support the resource URI variables defined in the table A.a.1.2.3.2-1.

Table A.a.1.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See Annex C.1.1 of 3GPP TS 24.546 [29]. |
| apiVersion | string | See clause A.a.2.1. |
| valServiceId | string | Identifier of a VAL service. |

A.a.1.2.3.3	Resource Standard Methods

A.a.1.2.3.3.1	PUT

This operation sends the satellite S&F information.

This method shall support the response data structures and response codes specified in table A.a.1.2.3.3.1-1.

Table A.a.1.2.3.3.1-1: Data structures supported by the PUT Response payload on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| Report | M | 1 | 2.04 Changed | The satellite S&F information. |

A.a.1.3	Data Model

A.a.1.3.1	General

This clause specifies the application data model supported by the API. Data types listed in clause C.1.4 of 3GPP TS 24.546 [31] apply to this API.

Table A.a.1.3.1-1 specifies the data types defined specifically for the SU_ StoreForward API service provided by SNRM-C.

Table A.a.1.3.1-1: SU_ StoreForward API provided by SNRM-C specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| Notification | A.a.1.3.2.1 | The requested satellite S&F event. |  |
| Report | A.a.1.3.2.2 | The satellite store and forward events report information. |  |

A.a.1.3.2	Structured data types

A.a.1.3.2.1	Type: Notification

Table A.a.1.3.2.1-1: Definition of type Notification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| storeForwardEvents | StoreForwardEvents | M | 1..N | One or more the requested store and forward event(s). |  |
| storeForwardTrigger | StoreForwardTrigger | O | 0..N | One or more the trigger condition(s). |  |
| timeStamp | DateTime | O | 0..1 | Indicate the timestamp of the notification. |  |

A.a.1.3.2.2	Type: Report

Table A.a.1.3.2.2-1: Definition of type Report

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ueType | UEType | O | 0..1 | Indicate the UE type. |  |
| satelliteInfo | SatelliteInfo | O | 0..1 | The satellite S&F information. |  |

* * * End of Changes * * * *