3GPP TSG-CT Meeting #111	CP-260abc

# Fukuaoka, Japan; 09th – 10th March 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.560 | CR | 0001 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology alignment with SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2026-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | LS C1-260031 from SA on completion of Study on AI/ML consistency alignment with TR 22.850. This LS informs various groups including CT1 of the outcome of the SA-level study on AI/ML consistency alignment, which resulted in the addition of definitions related to AI/ML in TS 21.905. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All occurrences of AIML have been modified to AI/ML. Other small corrections. Backward compatibility analysis: No backward compatibility issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS 24.560 is not aligned with definitions related to AI/ML in TS 21.905. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 3.1,3.2, 3.3, 4.1, 5.1, 5.2.1, 5.2.2.1, 5.2.2.2.1, 5.2.2.2.2, 5.3.2.2.2, 5.3.2.3.2, 5.3.2.4.2, 5.3.2.5.2, 5.3.2.6.2, 5.4.2.2.1, 5.4.2.2.2, 5.5.1, 5.5.2, 5.5.2.2.1, 5.6, 5.6.1, 5.6.2.1, 5.6.2.2.1, 5.6.2.2.2, 5.6.2.3.1, 5.6.2.3.2, 5.6.2.4.1, 5.6.2.4.2, 5.6.2.5.1, 5.6.2.5.2, 5.9.2.2.2, 5.10.2.2.2, 5.11.1, 5.11.2.2.2, 5.12, 5.12.1, 5.12.2.1, 5.12.2.2.1, 5.12.2.2.2, 5.12.2.3.1, 5.12.2.3.2, 5.13, 5.13.1, 5.13.2.1, 5.13.2.2.1, 5.13.2.2.2, 5.13.2.3.1, 5.13.2.3.2, 6.1.4.1, 6.1.4.2, 6.1.4.2.1, 6.1.4.2.2, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.3, 6.1.6.2.4, 6.2.6.2.2, 6.2.6.2.3, 6.3.6.1, 6.3.6.2.4, 6.3.6.2.6, 6.3.6.2.9, 6.3.6.3.4, 6.3.6.3.5, 6.3.6.3.9, 6.5.1, 6.5.3.1, 6.5.4.1, 6.5.4.2, 6.5.4.2.1, 6.5.6.1, 6.6.6.2.6, 6.6.6.3.6, 6.8.6.1, 6.8.6.2.2, 6.8.6.2.4, 6.8.6.2.7, 6.10.6.1, 6.10.6.2.2, 6.10.6.2.3, 6.10.6.2.4, 6.10.6.2.5, 6.11.1, 6.11.4.1, 6.11.4.2, 6.11.4.2.1, 6.11.4.2.2, 6.11.4.3, 6.11.4.3.1, 6.11.4.3.2, 6.11.6.1, 6.11.6.2.2, 6.11.6.2.3, 6.11.6.2.4, 6.11.6.3.3, 6.12.1, 6.12.4.1, 6.12.4.2, 6.12.4.2.1, 6.12.4.2.2, 6.12.4.3, 6.12.4.3.2, 6.12.6.1, 6.12.6.2.2, 6.12.6.2.3, 6.12.6.2.4, 6.12.6.2.5, 6.12.6.2.7, 6.12.6.3.3, A.2, A.3, A.4, A.5, A.6, A.7, A.8, A.9, A.10, A.11, A.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible correction to the following APIs: TS24560_Aimlec_AIMLEClientParticipation.yaml TS24560_Aimlec_HFLTraining.yaml TS24560_Aimles_AIMLEClientRegistration.yaml TS24560_Aimles_SplitOpPipeline.yaml TS24560_Aimlec_FLGroupIndication.yaml TS24560_Aimlec_ClientDataProcessing.yaml TS24560_Aimlec_MLModTngCapEva.yaml T24560_Aimles_UeTLModelSelectionAssistance.yaml TS24560_Aimlec_AIMLEClientServiceOperations.yaml TS24560_Aimlec_AimlTaskTransfer.yaml TS24560_Aimles_AimlTaskTransfer.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 2: C1-260486 agreed in CT1#159 meeting is revised to correct statement related to the impacted yaml files and to remove duplicated clauses A.4. |  |  |  |  |  |  |  |  |

==============First change==============

# 1	Scope

The present document specifies the stage 3 protocol and data model for the AI/ML enabling SEAL services. It provides stage 3 protocol definitions and message flows and specifies the API for each service offered by the AIMLE server.

The stage 2 architecture and procedures are specified in 3GPP TS 23.482 [4].

The common protocol and interface aspects for API definition are specified in clause 5.2 of 3GPP TS 29.122 [5].

The present document is applicable to the user equipment (UE) supporting AI/ML enabling SEAL services functionalities as described in 3GPP TS 23.482 [4], to the application server supporting AI/ML enabling SEAL services functionalities as described in 3GPP TS 23.482 [4], and to the application server supporting the vertical application server (VAL server) functionality as defined in specific vertical application service (VAL service) specification.

NOTE:	The specification of the VAL server for a specific VAL service is out of the scope of the present document.

==============Next change==============

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [2].

AIMLE client: An AI/ML enablement layer entity (see 3GPP TS 23.482 [4] clause 5) which is an AI/ML endpoint, and performs client-side operations.

AI/ML intermediate model: An AI/ML intermediate model defined in 3GPP TS 23.482 [4].

AI/ML operation: An AI/ML operation defined in 3GPP TS 23.482 [4].

AIMLE server: An AI/ML enablement layer entity (see 3GPP TS 23.482 [4] clause 5) which is an AI/ML endpoint, and performs server-side operations.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.434 [3] apply:

SEAL service

VAL client

VAL server

VAL service

Vertical

Vertical application

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.482 [4] apply:

AIMLE service

FL member

FL client

FL server

==============Next change==============

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

==============Next change==============

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2].

ADAE	Application Data Analytics Enablement

AI/ML	Artificial Intelligence / Machine Learning

AIMLE	AI/ML Enablement

API	Application Programming Interface

AS	Application Server

CAPIF	Common API Framework

FL	Federated Learning

HFL	Horizontal Federated Learning

ML	Machine Learning

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SEAL	Service Enabler Architecture Layer for verticals

TL	Transfer Learning

VAL	Vertical Application Layer

VFL	Vertical Federated Learning

==============Next change==============

## 4.1	Overview

Artificial intelligence machine learning enablement (AIMLE) SEAL services enable the AIMLE server to communicate with the AIMLE client over the AI/ML-UU reference points. The detailed specification of AIMLE is provided in 3GPP TS 23.482 [4].

The present document specifies the APIs in detail, needed to support the AIMLE services offered by the AIMLE client and the AIMLE services offered by the AIMLE server.

==============Next change==============

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API descriptions

| Service Name | Clause | Description | OpenAPI specification File | API name | Annex |
| --- | --- | --- | --- | --- | --- |
| Aimlec_AIMLEClientParticipation | 5.2 | AIMLE client participation service | TS24560_Aimlec_AIMLEClientParticipation.yaml | aimlec-cp | A.2 |
| Aimlec_HFLTraining | 5.3 | AIMLE client HFL training service | TS24560_Aimlec_HFLTraining.yaml | aimlec-hfl-trng | A.3 |
| Aimles_AIMLEClientRegistration | 5.4 | AIMLE client registration service | TS24560_Aimles_AIMLEClientRegistration.yaml | aimles-client-reg | A.4 |
| Aimles_SplitOpEvent | 5.5 | AIMLE server split operation event | TS29482_Aimles_SplitOpEvent.yaml | aimles-splitopevent | NOTE |
| Aimles_SplitOpPipeline | 5.6 | AIMLE server split operation pipeline | TS24560_Aimles_SplitOpPipeline.yaml | aimles-sopl | A.5 |
| Aimlec_FLGroupIndication | 5.7 | AIMLE client FL group indication service | TS24560_Aimlec_FLGroupIndication.yaml | aimlec-flgi | A.6 |
| Aimlec_ClientDataProcessing | 5.8 | AIMLE client data processing service | TS24560_Aimlec_ClientDataProcessing.yaml | aimlec-data-proc | A.7 |
| Aimlec_MLModTngCapEva | 5.9 | ML model training capability evaluation service | TS24560_Aimlec_MLModTngCapEva.yaml | aimlec-ml-mtce | A.8 |
| Aimles_UeTLModelSelectionAssistance | 5.10 | TL enablement service | TS24560_Aimles_UeTLModelSelectionAssistance.yaml | aimles-ue-tl-msa | A.9 |
| Aimlec_AIMLEClientServiceOperations | 5.11 | AIMLE client service operations service | TS24560_Aimlec_AIMLEClientServiceOperations.yaml | aimlec-serv-ops | A.10 |
| Aimlec_AimlTaskTransfer | 5.12 | AIMLE client AI/ML task transfer service | TS24560_Aimlec_AimlTaskTransfer.yaml | aimlec-task-transfer | A.11 |
| Aimles_AimlTaskTransfer | 5.13 | AIMLE server AI/ML task transfer service | TS24560_Aimles_AimlTaskTransfer.yaml | aimles-task-transfer | A.12 |
| Aimles_MLModelRetrieval | 5.14 | AIMLE server ML model retrieval | TS29482_Aimles_MLModelRetrieval.yaml | aimles-mlmr | NOTE |
| Aimles_MLModelUpdate | 5.15 | AIMLE client ML model update | TS29482_Aimles_MLModelUpdate.yaml | TBD | NOTE |
| NOTE: The API and OpenAPI are defined in 3GPP TS 29.482 [7]. |  |  |  |  |  |

NOTE:	When 3GPP TS 29.122 [5] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the AIMLE service producer (AIMLE server or AIMLE client) takes the role of the SCEF and the AIMLE service consumer (AIMLE client or AIMLE server) takes the role of the SCS/AS.

==============Next change==============

### 5.2.1	Service description

The AIMLE client participation service enables the AIMLE server to verify and manage the participation of AIMLE clients in AI/ML operations as defined in 3GPP TS 23.482 [4].

==============Next change==============

#### 5.2.2.1	Introduction

The service operation defined for Aimlec_AIMLEClientParticipation API is shown in the table 5.2.2.1-1.

Table 5.2.2.1-1: Operations of AIMLE client participation service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimlec_AIMLEClientParticipation_Request | This service operation is used by the AIMLE server to request for participation of the AIMLE client for the AI/ML operations. | AIMLE server |

==============Next change==============

##### 5.2.2.2.1	General

The Aimlec_AIMLEClientParticipation_Request service operation is used by AIMLE server to verify and manage the participation of AIMLE clients in AI/ML operations.

==============Next change==============

##### 5.2.2.2.2	AIMLE client participation service operation

To manage the AIMLE client for AI/ML operations, the AIMLE server shall send an HTTP POST request (custom operation: "Participation for AI/ML operations") with a Request-URI set to "{apiRoot}/aimlec-cp/<apiVersion>/participation" and the request body including the AimlecParticipationReq data structure as specified in clause 6.1.6.2.2.

Upon receipt of the HTTP POST request:

a)	the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized to request the AIMLE client to participate to the AI/ML operations; and

b)	if the AIMLE server:

1)	is not authorized, the AIMLE client shall respond to the AIMLE server with an appropriate error status code; or

2)	is authorized, the AIMLE client shall respond to the AIMLE server with:

i)	if the HTTP POST request is handled successfully, an HTTP "200 OK" status code with the response body including the AimlecParticipationResp data structure as specified in clause 6.1.6.2.3; or

ii)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.1.7.

The AIMLE client may respond to the AIMLE server with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.3.2.2.2	AI/ML operation for HFL training subscription

To subscribe to the HFL training event with the AIMLE client, the AIMLE server shall send to the AIMLE the HTTP POST request containing the HflTrngSub data type as specified in clause 6.2.6.2.2.

Upon receipt of the HTTP POST request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to subscribe to the HFL training event, the AIMLE client shall respond to the AIMLE server with an appropriate error status code as specified in clause 6.2.7; or

b)	is authorized to subscribe to the HFL training event, the AIMLE client shall:

1)	create the HFL training event subscription and store the subscription information; and

2)	respond the AIMLE server by including in the response:

i)	an HTTP "Location" header containing the URI of the newly created resource; and

ii)	the HflTrngSub data type as specified in clause 6.2.6.2.2.

==============Next change==============

##### 5.3.2.3.2	AI/ML operation for HFL training subscription retrieval

To retrieve an individual HFL training event subscription with the AIMLE client, the AIMLE server shall send the HTTP GET request to the AIMLE client.

Upon receipt of the HTTP GET request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to retrieve the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code as specified in clause 6.2.7; or

b)	is authorized to retrieve the existing individual HFL training event subscription, the AIMLE client shall include the HflTrngSub data type as specified in clause 6.2.6.2.2 in the response.

The AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.3.2.4.2	AI/ML operation for HFL training subscription update

To update an existing individual HFL training event subscription with the AIMLE client, the AIMLE server shall send to the AIMLE client:

-	the HTTP PATCH request (for partial update) containing the HflTrngSubPatch data type as specified in clause 6.2.6.2.3; or

-	the HTTP PUT request (for full replacement) containing the HflTrngSub data type as specified in clause 6.2.6.2.2.

Upon receipt of the HTTP PATCH request or HTTP PUT request from the AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to update the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code as specified in clause 6.2.7; or

b)	is authorized to update the existing individual HFL training event subscription, the AIMLE client:

1)	shall update the existing individual HFL training event subscription with the updated subscription information; and

2)	may include the HflTrngSub data type as specified in clause 6.2.6.2.2 in the response.

The AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.3.2.5.2	AI/ML operation for HFL training unsubscription

To unsubscribe an existing individual HFL training event subscription with the AIMLE client, the AIMLE server shall send an HTTP DELETE request to the AIMLE client.

Upon receipt of the HTTP DELETE request from AIMLE server, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized for the request. If the AIMLE server:

a)	is not authorized to unsubscribe for the existing individual HFL training event subscription, the AIMLE client shall respond to the AIMLE server with an appropriate error status code as specified in clause 6.2.7; or

b)	is authorized to update the existing individual HFL training event subscription, the AIMLE client shall:

1)	unsubscribe the existing individual HFL training event subscription; and

2)	respond to the AIMLE server with an HTTP "204 No Content" status code.

The AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.3.2.6.2	AI/ML operation for HFL training notification

To notify about the HFL training event to the AIMLE server, the AIMLE client shall send to the AIMLE server an HTTP POST request containing the HflTrngNotify data type as specified in clause 6.2.6.2.4.

Upon receipt of the HTTP POST request from the AIMLE client, the AIMLE server shall verify the identity of the AIMLE client to determine that the AIMLE client is authorized for the request. Upon verification of the AIMLE client, the AIMLE server shall respond to the AIMLE client with:

a)	if the HTTP POST request is handled successfully, an HTTP "204 No Content" status code; or

b)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.2.7.

The AIMLE server may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE server towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

==============Next change==============

##### 5.4.2.2.1	General

The Aimles_AIMLEClientRegistration_Request service operation is used by the AIMLE client to register to the AIMLE server. The AIMLE server stores the AIMLE client information for future interactions e.g. to discover and select suitable AIMLE clients for requested AI/ML operations.

==============Next change==============

##### 5.4.2.2.2	AIMLE client registration request

To register itself at the AIMLE server, the AIMLE client shall send an HTTP POST request to the AIMLE server targeting the "AIMLE client registrations" collection resource, with the request body including the AimleClientRegInfo data structure as specified in clause 6.3.6.2.3.

Upon reception of the HTTP POST registration request, the AIMLE server shall perform an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. If the AIMLE client:

1)	is authorized to register at the AIMLE server, the AIMLE server shall:

a)	create a new "Individual AIMLE client registration" resource with the received registration information; and

b)	respond with an HTTP "201 Created" status code with the response body including the AimleRegistration data structure and an HTTP "Location" header field containing the URI of the created resource, as specified in clause 6.3.3.2.3.1; or

2)	is not authorized to register at the AIMLE server, the AIMLE server shall take proper error handling actions, as specified in clause 6.3.7, and respond with an appropriate error status code.

If an "expTime" attribute indicating the expiration time for the AIMLE client registration was included in the AimleRegistration data structure as part of the created resource representation in step 1b above, then a periodic registration is required and to maintain the registration at the AIMLE server, the AIMLE client shall send a registration update request (as defined in clause 5.4.2.3) before the expiration of the current registration. If the AIMLE server did not receive the registration update request before the expiration of the current registration, then the AIMLE server shall delete the corresponding "Individual AIMLE client registration" resource.

NOTE:	Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository.

==============Next change==============

### 5.5.1	Service description

The split operation event subscription service enables an AIMLE client to subscribe, unsubscribe, and to update an existing subscription with the AIMLE server for events notification related to split AI/ML operation.

==============Next change==============

#### 5.5.2.1	Introduction

The service operation defined for Aimles_SplitOpEvent API is shown in the table 5.5.2.1-1.

Table 5.5.2.1-1: Operations of split operation event subscription

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimles_SplitOpEvent_Subscribe | This service operation is used by the AIMLE client to subscribe with the AIMLE server to be notified of events related to split AI/ML operation. | AIMLE client |
| Aimles_SplitOpEvent_Notify | This service operation is used by the AIMLE client to receive split operation notification from AIMLE server. | AIMLE server |
| Aimles_SplitOpEvent_UpdateSubscription | This service operation is used by the AIMLE client to update a subscription with the AIMLE server. | AIMLE client |
| Aimles_SplitOpEvent_Unsubscribe | This service operation is used by the AIMLE client to delete a subscription with the AIMLE server. | AIMLE client |

==============Next change==============

##### 5.5.2.2.1	General

This service operation is used by AIMLE client to subscribe with the AIMLE server to be notified of events related to split AI/ML operation.

==============Next change==============

## 5.6	Split AI/ML operation pipeline service

==============Next change==============

### 5.6.1	Service description

The split AI/ML operation pipeline service enables an AIMLE client to obtain information about available instances of split operation pipeline or processing the nodes of interest.

==============Next change==============

#### 5.6.2.1	Introduction

The service operation defined for Aimles_SplitOpPipeline API is shown in the table 5.6.2.1-1.

Table 5.6.2.1-1: Operations of split AI/ML operation pipeline service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimles_SplitOpPipeline_Discover | This service operation is used by the AIMLE client or VAL server to communicate with the AIMLE server for split AI/ML operation pipeline discovery. | AIMLE client |
| Aimles_SplitOpPipeline_Create | This service operation is used by the AIMLE client to create an instance of a split operation pipeline at the AIMLE server. | AIMLE client |
| Aimles_SplitOpPipeline_Update | This service operation is used by the AIMLE client to update an instance of a split operation pipeline at the AIMLE server. | AIMLE client |
| Aimles_SplitOpPipeline_Delete | This service operation is used by the AIMLE client to delete an instance of a split operation pipeline at the AIMLE server. | AIMLE client |

==============Next change==============

##### 5.6.2.2.1	General

This service operation is used by AIMLE client to discover instance(s) of split AI/ML operation pipeline or processing nodes from the AIMLE server.

==============Next change==============

##### 5.6.2.2.2	 AI/ML operation for pipeline discovery

To discover instance(s) of split AI/ML operation pipeline or processing nodes, the AIMLE client shall send an HTTP POST request to AIMLE server as specified in clause 6.5.4.2. The body of the POST message shall include the SplitOpPipelineDiscoverReq data structure as specified in clause 6.5.6.2.5.

Upon receipt of the HTTP POST request from the AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request; and

b)	if the AIMLE client:

1)	is not authorized to request split operation pipeline discovery, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation pipeline discovery;

i)	the AIMLE server may determine existing instance(s) of a split operation pipeline satisfy the request parameters; and

ii)	if no instance of a split operation pipeline satisfies the request parameters, the AIMLE server shall determine whether an instance of a split operation pipeline can be created and creates a split operation profile as defined in clause 5.6.2.3.

==============Next change==============

##### 5.6.2.3.1	General

This service operation is used by AIMLE client to create an instance of split AI/ML operation pipeline at the AIMLE server.

==============Next change==============

##### 5.6.2.3.2	AI/ML operation for pipeline creation

To create an instance of split AI/ML operation pipeline, the AIMLE client shall send an HTTP POST request to AIMLE server as specified in clause 6.5.3.2. The body of the POST message shall include the SplitOpPipelineCreateReq data structure as specified in clause 6.5.6.2.2.

Upon receipt of the HTTP POST request from the AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request; and

b)	if the AIMLE client:

1)	is not authorized to request split operation pipeline creation, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation pipeline creation then the AIMLE server shall determine if an instance of a split operation pipeline can be created or not. If the requested instance of split operation pipeline:

i)	can be created, the AIMLE server shall create a split operation profile and in the response the AIMLE server shall include an indication that split operation pipeline was successful and the corresponding split operation profile; or

ii)	cannot be created, the AIMLE server in the response shall include an indication that the split operation pipeline was unsuccessful with an appropriate error status code.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.6.2.4.1	General

This service operation is used by AIMLE client to update instance(s) of split AI/ML operation pipeline or processing nodes from the AIMLE server.

==============Next change==============

##### 5.6.2.4.2	AI/ML operation for pipeline update

To update instance(s) of split AI/ML operation pipeline or processing nodes, the AIMLE client shall send an HTTP PATCH request (for partial update) or HTTP PUT request (for full replacement) to AIMLE server as specified in clause 6.5.3.3.3. The body of the HTTP PATCH request message or HTTP PUT request message shall include the SplitOpPipelinePatch data structure as specified in clause 6.5.6.2.4 or HTTP PUT request message shall include the SplitOpPipelineCreateReq data structure as specified in clause 6.5.6.2.2.

Upon receipt of the HTTP PATCH request or HTTP PUT request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request; and

b)	if the AIMLE client:

1)	is not authorized to request split operation pipeline update, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to request split operation pipeline update;

i)	has updated the instance, then in the response the AIMLE server shall include an indication of success, and may include an expiration time; or

ii)	has not updated the instance, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.5.7.

If the AIMLE server determined the received HTTP PATCH request or the received HTTP PUT request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP PATCH request or the HTTP PUT request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.6.2.5.1	General

This service operation is used by AIMLE client to delete an instance of split AI/ML operation pipeline at the AIMLE server.

==============Next change==============

##### 5.6.2.5.2	AI/ML operation for pipeline deletion

To delete an instance of split AI/ML operation pipeline, the AIMLE client shall send an HTTP DELETE request to AIMLE server as specified in clause 6.5.3.3.3.

Upon receipt of the HTTP DELETE request from AIMLE client:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized for the request;

b)	if the AIMLE client:

1)	is not authorized to delete instance(s) of split AI/ML operation pipeline, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized to delete instance(s) of split AI/ML operation pipeline, the AIMLE server may delete the instance(s). If the AIMLE server:

i)	has deleted an instance of split AI/ML operation pipeline, then in the response the AIMLE server shall include an indication of success; or

ii)	has not deleted an instance of split AI/ML operation pipeline, then in the response the AIMLE server shall include an indication of failure and may include an appropriate error response as specified in clause 6.5.7.

If the AIMLE server determined the received HTTP DELETE request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP DELETE request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.9.2.2.2	Perform ML model training capability evaluation

To request the AIMLE client to perform the ML model training capability evaluation, the AIMLE server shall send an HTTP POST request (custom operation: "ML model training capability evaluation request") to the AIMLE client, with the request URI set to "{apiRoot}/aimlec-ml-mtce/<apiVersion>/request", as specified in clause 6.8.4.2 and include in a body of the HTTP POST request a MlModTngCapEvalReq data structure defined in clause 6.8.6.2.2.

Upon reception of the HTTP POST request, the AIMLE client:

1)	if the "availTime" attribute is received, shall evaluate availability for the requested time to support FL operation;

2)	if the "testTask" attribute is received, shall run the task for test ML model training capability;

3)	if "modelInfo" attribute is received, shall evaluate own capability for the indicated AI/ML model information and model parameters for use in the FL training process;

4)	if the "dataSetReq" attribute is received, shall determine a data alignment between the datasets of the different domains;

5)	based on the performed actions in steps 1 – 4, shall evaluate own capability and availability to join the FL training process; and

6)	shall respond to the AIMLE server with an HTTP "200 OK" status code and include in the response body a MlModTngCapEvalResp data structure which:

a)	shall contain the outcome of the ML model training capability evaluation in the "capEvalOut" attribute; and

b)	if the "capEvalOut" attribute indicates:

i)	the ability to join the FL training process, shall contain the test result of the ML model training capability evaluation in the "testResult" attribute (e.g. successfully completed test task, available for the requested time to support FL operation, requirement on dataset successfully executed); or

ii)	the inability to join the FL training process, shall contain the "evalFailInd" attribute which specifies the reason for not joining the FL training process (e.g. test task not completed, unavailable for the requested time to support FL operation, requirement on dataset cannot be executed, indicated model parameter for use in the FL training process not supported).

On failure, the AIMLE client shall take proper error handling actions, as specified in clause 6.8.7, and respond with an appropriate error status code.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

The ML model training capability result can be used by the AIMLE server to select FL members for FL training process (e.g. HFL, VFL).

==============Next change==============

##### 5.10.2.2.2	AI/ML operation for TL model selection assistance request

To request the AIMLE server to provide one or more pre-trained ML models for the TL enablement service, the AIMLE client shall send an HTTP POST request (custom operation: "TL model selection assistance") with a Request-URI set to "{apiRoot}/aimles-ue-tl-msa/<apiVersion>/assist-tlms" and with a body containing data type TlModelSelectAssistReq as defined in clause 6.10.6.2.2.

Upon receipt of the HTTP POST request:

a)	the AIMLE server shall verify the identity of the AIMLE client and determine if the AIMLE client is authorized to request one or more pre-trained ML models for the TL enablement service; and

b)	if the AIMLE client:

1)	is not authorized, the AIMLE server shall respond to the AIMLE client with an appropriate error status code; or

2)	is authorized, the AIMLE sever:

i)	shall request ML repository one or more pre-trained ML models for the TL enablement service as described in 3GPP TS 29.482 [7]; and

ii)	shall respond to the AIMLE server with:

A)	if the HTTP POST request is handled successfully, an HTTP "200 OK" response code with the response body containing data type TlModelSelectAssistResp as defined in clause 6.9.6.2.3 which includes one or more pre-trained ML models for the TL enablement service; and

B)	if the HTTP POST request is not handled successfully, an appropriate error response as specified in clause 6.9.7.

The AIMLE client may respond to the AIMLE server with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the request shall be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

### 5.11.1	Service description

The AIMLE client service operations service enables the communication between the AIMLE client (e.g., AI/ML capable UE) and the AIMLE server for AIMLE client service operations as defined in 3GPP TS 23.482 [4]. The AIMLE client service operations service is provided by the AIMLE client.

==============Next change==============

##### 5.11.2.2.2	Perform AIMLE client service operation

To request the AIMLE client to perform the AIMLE client service operation, the AIMLE server shall send an HTTP POST request (custom operation: "AIMLE service operation request") to the AIMLE client, with the request URI set to "{apiRoot}/aimlec-serv-ops/<apiVersion>/perform" and the request body including the AimleClientServOpReq data structure, as specified in clause 6.10.4.2.

Upon reception of the HTTP POST request, the AIMLE client:

1)	shall perform the service operation mode received in the "servOpMode" attribute for the requested AI/ML service operation received in the "servOpId" attribute;

2)	if the "servOpModeCfg" attribute is received, shall configure and monitor the AI/ML service operation in accordance with the received AI/ML service operation mode configuration requirements;

3)	if the "servOpModeStatRptg" attribute is received, shall determine whether a periodic or event-based reporting of the AI/ML service operation mode status shall be applied towards the AIMLE server; and

4)	shall respond to the AIMLE server with an HTTP "200 OK" status code and indicate the current service operation mode status within the "servOpModeStatus" attribute contained in the response body AimleClientServOpResp data structure.

On failure, the AIMLE client shall take proper error handling actions, as specified in clause 6.10.7, and respond with an appropriate error status code.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

### 5.12	AIMLE client AI/ML task transfer service

==============Next change==============

### 5.12.1	Service description

The AIMLE client AI/ML task transfer service enables the communication between the AIMLE client (e.g., AI/ML capable UE) and the AIMLE server for AI/ML task transfer operations as defined in 3GPP TS 23.482 [4]. The AIMLE client AI/ML task transfer service is provided by the AIMLE client.

==============Next change==============

#### 5.12.2.1	Introduction

The service operations defined for Aimlec_AimlTaskTransfer API are shown in the table 5.12.2.1-1.

Table 5.12.2.1-1: Operations for AIMLE client AI/ML task transfer service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimlec_AIMLTaskTransfer_Request | This service operation is used by the AIMLE server to request AI/ML task transfer. | AIMLE server |
| Aimlec_DirectAIMLTaskTransfer_Request | This service operation is used by the source AIMLE client to request direct AI/ML task transfer. | AIMLE client |

==============Next change==============

##### 5.12.2.2.1	General

The Aimlec_AIMLTaskTransfer_Request service operation is used by the AIMLE server to request the AIMLE client to perform the AI/ML task transfer operation.

==============Next change==============

##### 5.12.2.2.2	Requesting AI/ML task transfer

To request the AIMLE client to perform the AI/ML task transfer operation, the AIMLE server shall send an HTTP POST request (custom operation: "AI/ML task transfer") to the AIMLE client, with the request URI set to "{apiRoot}/aimlec-task-transfer/<apiVersion>/request" and the request body including the AimleClientTaskTransferReq data structure, as specified in clause 6.11.6.2.2, which:

1)	shall contain an identity of the AIMLE client from which the AI/ML task is to be transferred within the "sourceAimlId" attribute;

2)	shall contain type of the AI/ML operation to be transferred within the "aimlTaskType" attribute;

3)	shall contain type of the AI/ML information in the AI/ML task to be transferred within the "aimlInfoType" attribute; and

4	may contain the requested time for the AI/ML task transfer within the "aimlTaskTransferTime" attribute.

Upon reception of the HTTP POST request, the AIMLE client shall verify the identity of the AIMLE server and determine if the AIMLE server is authorized to request AI/ML task transfer. If the AIMLE server:

1)	is not authorized or a failure occurs during HTTP request processing, the AIMLE client shall take proper error handling actions, as specified in clause 6.11.7, and respond with an appropriate error status code; or

2)	is authorized, the AIMLE client shall respond to the AIMLE server with:

a)	an HTTP "200 OK" status code and indicate the time for the AI/ML task transfer within the "aimlTaskTransferTime" attribute contained in the response body AimleClientTaskTransferRes; or

b)	an HTTP "204 No Content" status code if the AIMLE client does not provide the AI/ML task transfer information in the response.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.12.2.3.1	General

The Aimlec_DirectAIMLTaskTransfer_Request service operation is used by the source AIMLE client to request the AIMLE client to perform the direct AI/ML task transfer operation.

==============Next change==============

##### 5.12.2.3.2	Requesting direct AI/ML task transfer

To request the AIMLE client to perform the AI/ML task transfer operation, the source AIMLE client shall send an HTTP POST request (custom operation: "Direct AI/ML task transfer") to the AIMLE client, with the request URI set to "{apiRoot}/aimlec-task-transfer/<apiVersion>/request-direct" and the request body including the AimleClientDirectTransferReq data structure, as specified in clause 6.11.6.2.4, which:

1)	shall contain type of the AI/ML operation to be transferred within the "aimlTaskType" attribute;

2)	shall contain type of the AI/ML information in the AI/ML task to be transferred within the "aimlInfoType" attribute; and

3	may contain the requested time for the AI/ML task transfer within the "aimlTaskTransferTime" attribute.

Upon receipt of the HTTP POST request, the AIMLE client shall perform an authentication and authorization check to determine whether the source AIMLE client is permitted to request the AI/ML task transfer operation. If the source AIMLE client:

1)	is not authorized or a failure occurs during HTTP request processing, the AIMLE client shall take proper error handling actions, as specified in clause 6.11.7, and respond with an appropriate error status code; or

2)	is authorized, the AIMLE client shall respond with an HTTP "204 No Content" status code.

If the AIMLE client determined the received HTTP POST request needs to be redirected, the AIMLE client may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE client towards which the HTTP POST request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

## 5.13	AIMLE server AI/ML task transfer service

==============Next change==============

### 5.13.1	Service description

The AIMLE server AI/ML task transfer service enables the communication between the AIMLE client (e.g., AI/ML capable UE) and the AIMLE server for the AI/ML task transfer operations as defined in 3GPP TS 23.482 [4]. The AIMLE server AI/ML task transfer service is provided by the AIMLE server.

==============Next change==============

#### 5.13.2.1	Introduction

The service operations defined for the Aimles_AimlTaskTransfer API are shown in the table 5.13.2.1-1.

Table 5.13.2.1-1: Operations for AIMLE server AI/ML task transfer service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Aimles_AIMLTaskTransferAssist_Request | This service operation is used by the AIMLE client to request the AIMLE server to assist in an AI/ML task transfer. | AIMLE client |
| Aimles_AIMLESControlledAIMLTaskTransfer_Request | This service operation is used by the AIMLE client to request the AIMLE server to perform the AIMLE server controlled AI/ML task transfer. | AIMLE client |

==============Next change==============

##### 5.13.2.2.1	General

The Aimles_AIMLTaskTransferAssist_Request service operation is used by the AIMLE client to request the AIMLE server to assist in AI/ML task transfer operation.

==============Next change==============

##### 5.13.2.2.2	Requesting AI/ML task transfer assist

To get assistance from the AIMLE server, the AIMLE client shall send an HTTP POST request (custom operation: "AI/ML task transfer assist") to the AIMLE server, with the request URI set to "{apiRoot}/aimles-task-transfer/<apiVersion>/assist-tt" and the request body including the AimlesTaskTransferAssistReq data structure as specified in clause 6.12.6.2.2.

Upon reception of the HTTP POST request, the AIMLE server, based on the content of the received AimlesTaskTransferAssistReq data structure, shall discover other AIMLE clients, select one or more target AIMLE clients and determine which the transfer mode shall be applied. The AIMLE server shall send a "200 OK" response to the HTTP POST request. The AIMLE server shall include in the response body the AimlesTaskTransferAssistResp data structure, as specified in clause 6.12.6.2.3, which:

-	shall contain a time window for assistance in the AI/ML task transfer in the "assistanceTime" attribute;

-	shall contain the list of selected AIMLE clients in the "targetAimlIds" attribute; and

-	may contain the transfer mode to be applied in the "transferMode" attribute.

On failure, the AIMLE server shall take proper error handling actions, as specified in clause 6.12.7, and respond with an appropriate error status code.

If the AIMLE server determined the received HTTP DELETE request needs to be redirected, the AIMLE server may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE server towards which the HTTP DELETE request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

##### 5.13.2.3.1	General

The Aimles_AIMLESControlledAIMLTaskTransfer_Request service operation is used by the AIMLE client to communicate with the AIMLE server to request AIMLE server-controlled AI/ML task transfer from the AIMLE client.

==============Next change==============

##### 5.13.2.3.2	Requesting AIMLE server controlled AI/ML task transfer

To request the AIMLE server to perform the AIMLE server controlled AI/ML task transfer, the AIMLE client shall send an HTTP POST request (custom operation: "Controlled AI/ML task transfer") to the AIMLE server, with the request URI set to "{apiRoot}/aimles-task-transfer/<apiVersion>/request-ctld " and the request body including the AimlesControlledTaskTransferReq data structure as specified in clause 6.12.6.2.4.

Upon receipt of the HTTP POST request, the AIMLE server shall perform an authentication and authorization check to determine whether the AIMLE client is permitted to communicate with the AIMLE server. If the AIMLE client:

1)	is authorized to communicate with the AIMLE server, the AIMLE server shall check the availability of the target AIMLE client. If the target AIMLE client is available, the AIMLE server shall send a "200 OK" response to the HTTP POST request. The AIMLE server shall include in the response body the AimlesControlledTaskTransferResp data structure, as specified in clause 6.12.6.2.5, which shall contain a time window for assistance in the AI/ML task transfer in the "assistanceTime" attribute; or

2)	is not authorized or the target AIMLE client is not available, the AIMLE server shall take proper error handling actions, as specified in clause 6.12.7, and respond with an appropriate error status code.

If the AIMLE server determined the received HTTP DELETE request needs to be redirected, the AIMLE server may respond with an HTTP "307 Temporary Redirect" status code or an HTTP "308 Permanent Redirect" status code including an HTTP "Location" header containing an alternative URI representing the end point of an alternative AIMLE server towards which the HTTP DELETE request should be sent. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5].

==============Next change==============

#### 6.1.4.1	Overview

Table 6.1.4.1-1 provides an overview of the custom operations and applicable HTTP methods defined for the Aimlec_AIMLEClientParticipation API.

Table 6.1.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Participation for AI/ML operations | /participation | POST | Used by the AIMLE server to request the AIMLE client for participation of the AI/ML operations. |

==============Next change==============

#### 6.1.4.2	Operation: Participation for AI/ML operations

==============Next change==============

##### 6.1.4.2.1	Description

The custom operation enables the AIMLE server to request the AIMLE client to join or depart one or more AI/ML operations.

==============Next change==============

##### 6.1.4.2.2	Operation definition

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1, 6.1.4.2.2-2, 6.1.4.2.2-3 and 6.1.4.2.2-4.

Table 6.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimlecParticipationReq | M | 1 | Request for participation the AI/ML operations. |

Table 6.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimlecParticipationResp | M | 1 | 200 OK | Successful case. The requested "participation the AI/ML operation" shall be responded. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| NOTE: The mandatory HTTP error status code for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.1.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.1.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

==============Next change==============

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Aimlec_AIMLEClientParticipation API.

Table 6.1.6.1-1: Aimlec_AIMLEClientParticipation API specific data types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlecParticipationReq | 6.1.6.2.2 | Represents the participation request for the AI/ML operations. |  |
| AimlecParticipationResp | 6.1.6.2.3 | Represents the participation response for the AI/ML operations. |  |
| ClienSetPart | 6.1.6.3.3 | Represents the participation request for the AIMLE client set. |  |
| SchedAimlOperation | 6.1.6.2.4 | Represents the scheduled AI/ML participation type. |  |

Table 6.1.6.1-2 specifies data types re-used by the Aimlec_AIMLEClientParticipation API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_AIMLEClientParticipation API.

Table 6.1.6.1-2: Aimlec_AIMLEClientParticipation API re-used data types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlOperation | 6.3.6.3.5 | Represents the type of the AI/ML operation. |  |
| DatasetRequirement | 3GPP TS 29.482 [7] | Represents the dataset requirements. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [5] | Represents an offered scheduled communication time. |  |
| ServiceRequirement | 3GPP TS 29.482 [7] | Represents the service requirements. |  |

==============Next change==============

##### 6.1.6.2.2	Type: AimlecParticipationReq

Table 6.1.6.2.2-1: Definition of type AimlecParticipationReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requesterId | string | M | 1 | Contains the identifier of the service consumer. For the AIMLE client, e.g. unique client identifier. For the AIMLE server, e.g., FQDN, URI. |  |
| clientSetId | string | M | 1 | Identifies the identity of the AIMLE client set. |  |
| clientSetPart | ClientSetPart | M | 1 | Identifies the participation request for the AIMLE client set. |  |
| mlModelId | string | M | 1 | Identifies the identity of the ML model for AI/ML operation. |  |
| schedAimlOperations | array(SchedAimlOperation) | M | 1..N | Identifies the list of AI/ML operations which are required to be performed. |  |
| dataSetReq | DatasetRequirement | M | 1 | Identifies the dataset requirements. |  |
| serviceReq | ServiceRequirement | M | 1 | Identifies the service requirements. |  |

==============Next change==============

##### 6.1.6.2.3	Type: AimlecParticipationResp

Table 6.1.6.2.3-1: Definition of type AimlecParticipationResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientStatus | boolean | M | 1 | A "true" value indicates the willingness of the AIMLE client to be added to or to be removed from the AIMLE client list to perform AI/ML operations. A "false" value indicates the denial of the AIMLE client to be added to or to be removed from the AIMLE client list to perform AI/ML operations. |  |

==============Next change==============

##### 6.1.6.2.4	Type: SchedAimlOperation

Table 6.1.6.2.4-1: Definition of type SchedAimlOperation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlOperation | AimlOperation | M | 1 | Identifies the type of the AI/ML operation. |  |
| aimlOperSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for the AI/ML operation. |  |

==============Next change==============

##### 6.2.6.2.2	Type: HflTrngSub

Table 6.2.6.2.2-1: Definition of type HflTrngSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requesterId | string | M | 1 | Contains the identifier of the service consumer. For the AIMLE client, e.g. unique client identifier. For the AIMLE server, e.g., FQDN, URI. |  |
| notifUri | Uri | M | 1 | Identifies the URI towards which the notification should be delivered. |  |
| aimlMdlInfo | MlModelInfo | M | 1 | Identifies the information about the AI/ML model and the model parameters which are to be used for the HFL training. This attribute shall be provided in the HTTP request. |  |
| dataId | string | M | 1 | Identifies the dataset which is to be used for the HFL training. This attribute shall be provided in the HTTP request. |  |
| noDataSamp | integer | M | 1 | Identifies the required number of samples for a round of the HFL training. This attribute shall be provided in the HTTP request. |  |
| operSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for when the HFL training is to occur. |  |
| notifReqs | ReportingRequirements | O | 0..1 | Identifies the requirements for the notifications. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of optional features. This attribute shall be present only if feature negotiation in clause 6.2.8 needs to take place. This attribute shall be provided in the HTTP request and in the HTTP response of successful resource creation. |  |
| vaSrvId | string | M | 1 | Identifies the VAL service for the AIMLE HFL training operation. This parameter shall be supplied by the AMILE client in the HTTP response. |  |
| subID | string | O | 0..1 | Identifies the subscription. |  |

==============Next change==============

##### 6.2.6.2.3	Type: HflTrngSubPatch

Table 6.2.6.2.3-1: Definition of type HflTrngSubPatch

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notifUri | Uri | O | 0..1 | Identifies the URI towards which the notification should be delivered. |  |
| aimlMdlInfo | MlModelInfo | O | 0..1 | Identifies the information about the AI/ML model and the model parameters which are to be used for the HFL training. |  |
| dataId | string | O | 0..1 | Identifies the dataset which is to be used for the HFL training. |  |
| noDataSamp | integer | O | 0..1 | Identifies the required number of samples for a round of the HFL training. |  |
| operSched | ScheduledCommunicationTime | O | 0..1 | Identifies the schedule for when the HFL training is to occur. |  |
| notifReqs | ReportingRequirements | O | 0..1 | Identifies the requirements for the notifications. |  |

==============Next change==============

#### 6.3.6.1	General

This clause specifies the application data model supported by the Aimles_AIMLEClientRegistration API.

Table 6.3.6.1-1 specifies the data types defined for the Aimles_AIMLEClientRegistration API.

Table 6.3.6.1-1: Aimles_AIMLEClientRegistration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleClientProfile | 6.3.6.2.6 | Contains the AIMLE client capability information e.g. supported AI/ML model types, AI/ML service operation type. |  |
| AimleClientRegInfo | 6.3.6.2.3 | Contains the AIMLE client registration information. |  |
| AimleRegistration | 6.3.6.2.2 | Represents an individual AIMLE client registration resource. |  |
| AimlModelType | 6.3.6.3.4 | Represents the AI/ML model type. |  |
| AimlOperation | 6.3.6.3.5 | Represents the AI/ML service operation type. |  |
| ClientCapability | 6.3.6.2.7 | Contains the AIMLE client capability information. |  |
| DataCapability | 6.3.6.3.8 | Contains a list of data capabilities. |  |
| DataSetAvailability | 6.3.6.2.8 | Represents a dataset availability. |  |
| LocationConfig | 6.3.6.2.9 | To be checked if needed. |  |
| MlApplicationType | 6.3.6.3.6 | Represents the ML application type. |  |
| ResourceUsageLevel | 6.3.6.3.7 | Represents the resource usage level. |  |
| ServiceData | 6.3.6.2.5 | Contains VAL service identifier with the corresponding service permission. |  |
| ServicePermissionLevel | 6.3.6.3.3 | Represents the service permission level. |  |
| SupportedProfile | 6.3.6.2.4 | Contains AIMLE client profiles and supported service information. |  |
| TaskCapability | 6.3.6.3.9 | Contains the AI/ML task performing capabilities. |  |

Table 6.3.6.1-2 specifies data types re-used by the Aimles_AIMLEClientRegistration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimles_AIMLEClientRegistration API.

Table 6.3.6.1-2: Aimles_AIMLEClientRegistration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [5] | Used to indicate an expiration time of the AIMLE client registration. |  |
| LocationArea5G | 3GPP TS 29.122 [5] | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |
| ScheduledCommunicationTime | 3GPP TS 29.571 [11] | Used to indicate the availability schedule of the AIMLE client for the AI/ML service. |  |
| SupportedFeatures | 3GPP TS 29.571 [11] | Used to negotiate the applicability of the optional features defined in table 6.3.8-1. |  |
| Uri | 3GPP TS 29.122 [5] | Used to indicate a URI. |  |
| ValSvcAreaId | 3GPP TS 29.549 [9] | Used to indicate the VAL Service Area identifier. |  |
| ValTargetUe | 3GPP TS 29.549 [9] | Unique identifier of a VAL user or a VAL UE. |  |

==============Next change==============

##### 6.3.6.2.4	Type: SupportedProfile

Table 6.3.6.2.4-1: Definition of type SupportedProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientProfile | AimleClientProfile | C | 0..1 | Contains the AIMLE client capability information e.g. supported AI/ML model types, AI/ML service operation type. (NOTE) |  |
| suppServices | array(ServiceData) | C | 1..N | Contains the list of VAL services identifiers with corresponding service permissions. (NOTE) |  |
| NOTE: This attribute shall be included in the HTTP POST request for the AIMLE client registration. |  |  |  |  |  |

==============Next change==============

##### 6.3.6.2.6	Type: AimleClientProfile

Table 6.3.6.2.6-1: Definition of type AimleClientProfile

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleClientUri | Uri | M | 1 | URI information of the AIMLE client. |  |
| aimlModelTypes | array(AimlModelType) | O | 1..N | Contains AI/ML model types supported by the AIMLE client (e.g., decision tree, linear regression, neural network). |  |
| aimlOperations | array(AimlOperation) | M | 1..N | Contains AI/ML operations supported by the AIMLE client (e.g., training, model transfer, model inference, model offload, model split, continue performing intermediate AI/ML operation). |  |
| clientCap | ClientCapability | M | 1 | Contains the AIMLE client capability information (e.g. ML application type, allowed resource usage level). |  |
| availTimeSchedCfgs | array(ScheduledCommunicationTime) | O | 1..N | Contains the availability schedule of the AIMLE client for the AI/ML service, e.g., the AIMLE client is available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |  |
| unavblTimeSchedCfgs | array(ScheduledCommunicationTime) | O | 1..N | Contains the unavailability schedule of the AIMLE client for the AI/ML service, e.g., the AIMLE client is not available to participate in the AI/ML operations in the given time slot(s) and/or day(s) of the week. |  |
| availLocCfgs | array(LocationConfig) | O | 1..N | Contains the available location-based configurations of the AIMLE client for the AI/ML service, e.g., the AI/ML member is available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |  |
| unavblLocCfgs | array(LocationConfig) | O | 1..N | Contains the unavailable location-based configurations of the AIMLE client for the AI/ML service, e.g., the AI/ML member is not available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |  |
| dataSetAvail | DataSetAvailability | O | 0..1 | Contains a dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |  |
| dataCap | array(DataCapability) | O | 1..N | Contains a list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis (EAD) functions. |  |
| taskCaps | array(TaskCapability) | O | 1..N | Contains the AI/ML task performing capabilities i.e. compute capabilities (e.g., high, low), task performance preference capabilities (e.g., low costs). |  |

==============Next change==============

##### 6.3.6.2.9	Type: LocationConfig

Table 6.3.6.2.9-1: Definition of type LocationConfig

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientLoc | LocationArea5G | O | 0..1 | Contains the location-based configurations of the AIMLE client for the AI/ML service, e.g., locations represented by coordinates, civic addresses, and network area information. |  |
| valSvcAreaId | ValSvcAreaId | O | 0..1 | Contains the VAL service area identifier. |  |

==============Next change==============

##### 6.3.6.3.4	Enumeration: AimlModelType

The enumeration AimlModelType represents AI/ML model types. It shall comply with the provisions defined in table 6.3.6.3.4-1.

Table 6.3.6.3.4-1: Enumeration AimlModelType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DECISION_TREE | Indicates the decision tree type of the AI/ML model. |  |
| LINEAR_REGRESSION | Indicates the linear regression type of the AI/ML model. |  |
| NEURAL_NETWORK | Indicates the neural network type of the AI/ML model. |  |
| OTHER_MODEL_TYPE | Indicates the other type of the AI/ML model. |  |

==============Next change==============

##### 6.3.6.3.5	Enumeration: AimlOperation

The enumeration AimlOperation represents the type of the AI/ML operation. It shall comply with the provisions defined in table 6.3.6.3.5-1.

Table 6.3.6.3.5-1: Enumeration AimlOperation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MODEL_INFERENCE | Indicates the model inference type of the AI/ML operation. |  |
| MODEL_OFFLOAD | Indicates the model offload type of the AI/ML operation. |  |
| MODEL_SPLIT | Indicates the model split type of the AI/ML operation. |  |
| MODEL_TRANSFER | Indicates the model transfer type of the AI/ML operation. |  |
| MODEL_TRAINING | Indicates the model training type of the AI/ML operation. |  |
| CONTINUE_PERFORM_INTERMEDIATE | Indicates the ability to continue performing of the intermediate AI/ML operation. |  |

==============Next change==============

##### 6.3.6.3.9	Enumeration: TaskCapability

The enumeration TaskCapability represents AI/ML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities. It shall comply with the provisions defined in table 6.3.6.3.9-1.

Table 6.3.6.3.9-1: Enumeration TaskCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| HIGH_COMPUTE_CAPABILITY |  |  |
| LOW_COMPUTE_CAPABILITY |  |  |
| LOW_COSTS_PERFORMANCE |  |  |
| GREEN_TASK_PERFORMANCE (NOTE) |  |  |
| ENERGY_EFFICIENT_PERFORMANCE (NOTE) |  |  |
| NOTE: The client shall not include GREEN_TASK_PERFORMANCE and ENERGY_EFFICIENT_PERFORMANCE. |  |  |

==============Next change==============

### 6.5.1	Introduction

The AI/ML split operation pipeline shall use the Aimles_SplitOpPipeline API.

The API URI of the Aimles_SplitOpPipeline API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [5].

-	The <apiName> shall be "aimles-sopl".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.5.3.

==============Next change==============

#### 6.5.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.5.3.1-1 depicts the resource URIs structure for the Aimles_SplitOpPipeline API.

Figure 6.5.3.1-1: Resource URI structure of the Aimles_SplitOpPipeline API

Table 6.5.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.5.3.1-1: Resources and methods overview

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| AI/ML split operation pipeline creation | /request | POST | Used by the AIMLE client to create an instance of a split operation pipeline at the AIMLE server. |
| Individual AI/ML split operation pipeline creation | /request/{requestId} | PUT | Used by the AIMLE client to fully update an instance of a split operation pipeline at the AIMLE server |
|  |  | DELETE | Used by the AIMLE client to delete an instance of a split operation pipeline at the AIMLE server |
|  |  | PATCH | Used by the AIMLE client to partially fully update an instance of a split operation pipeline at the AIMLE server. |

==============Next change==============

#### 6.5.4.1	Overview

Table 6.5.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| AI/ML split operation discovery | /discovery | POST | Used by the AIMLE client or VAL server to communicate with the AIMLE server for split AI/ML operation pipeline discovery. |

==============Next change==============

#### 6.5.4.2	Operation: AI/ML split operation discovery

==============Next change==============

##### 6.5.4.2.1	Description

The custom operation enables the AIMLE client to request the AIMLE server to perform the AI/ML split operation discovery.

==============Next change==============

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the Aimles_SplitOpPipeline API.

Table 6.5.6.1-1: Aimles_SplitOpPipeline API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| SplitOpPipelineCreateReq | 6.5.6.2.2 | Represents the AIMLE Split Operation Pipeline Create request. |  |
| SplitOpPipelineCreateResp | 6.5.6.2.3 | Represents the AIMLE Split Operation Pipeline Create response |  |
| SplitOpPipelinePatch | 6.5.6.2.4 | Represents the AIMLE Split Operation Pipeline Patch request. |  |
| SplitOpPipelineDiscReq | 6.5.6.2.5 | Represents the AIMLE Split Operation Pipeline Discovery request. |  |
| SplitOpPipelineDiscResp | 6.5.6.2.6 | Represents the AIMLE Split Operation Pipeline Discovery response. |  |
| SplitOpRequirements | 6.5.6.2.7 | Represents the AIMLE Split Operation Pipeline requirements |  |

Table 6.5.6.1-2 specifies data types re-used by the Aimles_SplitOpPipeline API service.

Table 6.5.6.1-2: Aimles_SplitOpPipeline API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Uri | 3GPP TS 29.122 [5] | Used to indicate the notification URI. |  |
| UsageInformation | 3GPP TS 29.482 [7] | Represents the usage information for the split AI/ML model. |  |
| DiscFilters | 3GPP TS 29.482 [7] | Represents the discovery filters to determine matching split operation profile or nodes. |  |
| SplitOpPipelineInfo | 3GPP TS 29.482 [7] | Represents split operation pipeline information. |  |
| SplitOpProfile | 3GPP TS 29.482 [7] | Represents the split operation profile that VAL server participates to. |  |
| StageInfo | 3GPP TS 29.482 [7] | Represents the information related to each stage in split operation pipeline. |  |
| Endpoint | 3GPP TS 29.558 [10] | Represent the endpoint information of a node. |  |

==============Next change==============

##### 6.6.6.2.6	Type: FlGroupDeletionInfo

Table 6.6.6.2.6-1: Definition of type FlGroupDeletionInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cause | FlGroupDelCause | O | 0..1 | Represents the cause for the FL group deletion. |  |
| expTime | TimeWindow | O | 0..1 | Represents the expiration time of the FL members group deletion (e.g., due to AI/ML service termination or group UE mobility to different service area). If the expTime attribute is not included, the deletion of the group is instant. |  |

==============Next change==============

##### 6.6.6.3.6	Enumeration: FlGroupDelCause

The enumeration FlGroupDelCause represents the deletion cause for the FL group. It shall comply with the provisions defined in table 6.6.6.3.6-1.

Table 6.6.6.3.6-1: Enumeration FlGroupDelCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SRV_TERMINATION | Indicates the AI/ML service termination. |  |
| OUT_OF_SRV_AREA | Indicates the UE has moved out of the service area. |  |

==============Next change==============

#### 6.8.6.1	General

This clause specifies the application data model supported by the Aimlec_MLModTngCapEva API.

Table 6.8.6.1-1 specifies the data types defined for the Aimlec_MLModTngCapEva API.

Table 6.8.6.1-1: Aimlec_MLModTngCapEva API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlModelData | 6.8.6.2.4 | Contains the AI/ML model information and model parameters for use in FL training. |  |
| CapEvalOutcome | 6.8.6.3.3 | Indicates the outcome of the ML model training capability evaluation. |  |
| DataSetRequirements | 6.8.6.2.5 | Contains requirements on data set for FL training. |  |
| DomainFeatures | 6.8.6.2.6 | Contains a list of features for each data domain(s) of the datasets at the UE. |  |
| MlModTngCapEvalReq | 6.8.6.2.2 | Contains the ML model training capability evaluation request information. |  |
| MlModTngCapEvalResp | 6.8.6.2.3 | Contains the ML model training capability evaluation response information. |  |

Table 6.8.6.1-2 specifies data types re-used by the Aimlec_MLModTngCapEva API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_MLModTngCapEva API.

Table 6.8.6.1-2: Aimlec_MLModTngCapEva API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlModelType | 6.3.6.3.4 | Used to indicate a type of the AI/ML model. |  |
| MLModelProfile | 3GPP TS 29.482 [7] | Used to indicate an ML model profile. |  |
| TimeWindow | 3GPP TS 29.122 [5] | Used to indicate a time window. |  |

==============Next change==============

##### 6.8.6.2.2	Type: MlModTngCapEvalReq

Table 6.8.6.2.2-1: Definition of type MlModTngCapEvalReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleServerId | string | M | 1 | The AIMLE server identifier. |  |
| availTime | TimeWindow | O | 0..1 | Indicates the requested available time to support FL operation. |  |
| testTask | string | O | 0..1 | Represents the task for test ML model training capability. (NOTE) |  |
| modelInfo | AimlModelData | O | 0..1 | Contains the AI/ML model information and model parameters for use in the FL training process. |  |
| dataSetReq | DataSetRequirements | O | 0..1 | Contains requirements on data set for FL training. |  |
| NOTE: The detail content of the "testTask" attribute is implementation dependent. |  |  |  |  |  |

==============Next change==============

##### 6.8.6.2.4	Type: AimlModelData

Table 6.8.6.2.4-1: Definition of type AimlModelData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlModels | array(AimlModelInfo) | O | 1..N | Contains information about the AI/ML model. (NOTE 1) |  |
| mlModelParams | array(string) | O | 1..N | Contains model parameters for use in FL training. (NOTE 2) |  |
| NOTE 1: For the HFL only one AI/ML model shall be present. For the VFL more than one AI/ML model may be present. NOTE 2: The detail content of the "mlModelParams" attribute is implementation dependent. |  |  |  |  |  |

==============Next change==============

##### 6.8.6.2.7	Type: AimlModelInfo

Table 6.8.6.2.7-1: Definition of type AimlModelInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlModelType | AimlModelType | O | 0..1 | Contains AI/ML model type (e.g., decision tree, linear regression, neural network). |  |
| mlModelProf | MLModelProfile | O | 0..1 | Contains ML model profile data e.g. ML model profile identifier, ML model address, ML model information, AIMLE server identifier, ML repository identifier. |  |

==============Next change==============

#### 6.10.6.1	General

This clause specifies the application data model supported by the Aimlec_AIMLEClientServiceOperations API.

Table 6.10.6.1-1 specifies the data types defined for the Aimlec_AIMLEClientServiceOperations API.

Table 6.10.6.1-1: Aimlec_AIMLEClientServiceOperations API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleClientServOpReq | 6.10.6.2.2 | Contains the AIMLE client service operation request information. |  |
| AimleClientServOpResp | 6.10.6.2.3 | Contains the AIMLE client service operation response information. |  |
| ServiceOperationInfo | 6.10.6.2.4 | Contains the AI/ML service operation information. |  |
| ServiceOperationMode | 6.10.6.3.3 | Represents service operation modes. |  |
| ServiceOpModeConfiguration | 6.10.6.2.5 | Contains the AI/ML service operation mode configuration. |  |

Table 6.10.6.1-2 specifies data types re-used by the Aimlec_AIMLEClientServiceOperations API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_AIMLEClientServiceOperations API.

Table 6.10.6.1-2: Aimlec_AIMLEClientServiceOperations API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DurationSec | 3GPP TS 29.122 [5] | Used to indicate a time duration expressed in units of seconds. |  |
| ReportingRequirements | 3GPP TS 29.549 [9] | Used to indicate the reporting configuration of the AI/ML service operation status. |  |
| Uint32 | 3GPP TS 29.571 [11] | Used to indicate the latency. |  |
| Uri | 3GPP TS 29.122 [5] | Used to indicate a URI. |  |

==============Next change==============

##### 6.10.6.2.2	Type: AimleClientServOpReq

Table 6.10.6.2.2-1: Definition of type AimleClientServOpReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimleServerId | string | M | 1 | The AIMLE server identifier. |  |
| valServiceId | string | O | 0..1 | Represents the VAL service identifier. |  |
| servOpId | string | M | 1 | Contains the AI/ML service operation identifier. |  |
| servOpMode | ServiceOperationMode | M | 1 | Contains the service operation mode. |  |
| servOpInfo | ServiceOperationInfo | O | 0..1 | Contains the AI/ML service operation information (e.g. AI/ML service model container, URI of the model to fetch the model from a repository, AI/ML service aggregator URI). |  |
| servOpModeCfg | ServiceOpModeConfiguration | O | 0..1 | Contains the AI/ML service operation mode configuration (e.g. network utilization (like stop the AI/ML service when latency is worse than x milliseconds, time limit threshold (like stop the AI/ML service after 24 hours), model performance (like stop the AI/ML service when model accuracy is 99% achieved)). |  |
| servOpModeStatRptg | ReportingRequirements | O | 0..1 | Indicates the reporting configuration of the AI/ML service operation status. |  |

==============Next change==============

##### 6.10.6.2.3	Type: AimleClientServOpResp

Table 6.10.6.2.3-1: Definition of type AimleClientServOpResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServiceId | string | O | 0..1 | Represents the VAL service identifier. |  |
| servOpId | string | M | 1 | Contains the AI/ML service operation identifier. |  |
| servOpModeStatus | ServiceOperationMode | M | 1 | Indicates the service operation mode status. |  |

==============Next change==============

##### 6.10.6.2.4	Type: ServiceOperationInfo

Table 6.10.6.2.4-1: Definition of type ServiceOperationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mlMdlContainer | string | O | 0..1 | Represents the AI/ML service model container. |  |
| mlMdlUri | Uri | O | 0..1 | Represents the URI of the ML model to be retrieved from the model repository. |  |
| mlMdAggregUri | Uri | O | 0..1 | Represents the ML model aggregator URI to send the model updates. |  |
| maxConvgTime | DurationSec | O | 0..1 | Indicates the maximum convergence time used in the AI/ML service operation optimization assistance. |  |

==============Next change==============

##### 6.10.6.2.5	Type: ServiceOpModeConfiguration

Table 6.10.6.2.5-1: Definition of type ServiceOpModeConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| maxLatency | Uint32 | O | 0..1 | Indicates the latency threshold value in milliseconds to stop the AI/ML service operation. |  |
| maxDurHour | integer | O | 0..1 | Indicates the maximum duration time of the AI/ML service operation expressed in hours. |  |
| modelAccuracy | integer | O | 0..1 | Indicates the threshold value of the model accuracy expressed as a percentage to stop the AI/ML service operation. |  |

==============Next change==============

### 6.11.1	Introduction

The AIMLE client AI/ML task transfer shall use the Aimlec_AimlTaskTransfer API.

The API URI of the Aimlec_AimlTaskTransfer API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [5].

-	The <apiName> shall be "aimlec-task-transfer".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.11.4.

==============Next change==============

#### 6.11.4.1	Overview

Table 6.11.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| AI/ML task transfer | /request | POST | Used by the AIMLE server to request the AIMLE client to perform AI/ML task transfer. |
| Direct AI/ML task transfer | /request-direct | POST | Used by the AIMLE client to request the target AIMLE client to perform AI/ML task transfer. |

==============Next change==============

#### 6.11.4.2	Operation: AI/ML task transfer

==============Next change==============

##### 6.11.4.2.1	Description

The custom operation enables the AIMLE server to request the AIMLE client to perform the AI/ML task transfer operation.

==============Next change==============

##### 6.11.4.2.2	Operation definition

This operation shall support the response data structures and response codes specified in tables 6.11.4.2.2-1 and 6.11.4.2.2-2.

Table 6.11.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimleClientTaskTransferReq | M | 1 | Contains the AIMLE client task transfer request information. |

Table 6.11.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimleClientTaskTransferRes | M | 1 | 200 OK | Successful case. The AIMLE client AI/ML task transfer is performed. |
| n/a |  |  | 204 No Content | Successful case. The AIMLE client AI/ML task transfer is performed and no content is returned in the response body. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.11.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.11.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

==============Next change==============

#### 6.11.4.3	Operation: Direct AI/ML task transfer

==============Next change==============

##### 6.11.4.3.1	Description

The custom operation enables the AIMLE client to request an AIMLE client to perform the direct AI/ML task transfer operation.

==============Next change==============

##### 6.11.4.3.2	Operation definition

This operation shall support the response data structures and response codes specified in tables 6.11.4.3.2-1 and 6.11.4.3.2-2.

Table 6.11.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimleClientDirectTransferReq | M | 1 | Contains the AIMLE client direct task transfer request information. |

Table 6.11.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case. The AIMLE client direct AI/ML task transfer is performed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE client. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.11.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

Table 6.11.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE client. |

==============Next change==============

#### 6.11.6.1	General

This clause specifies the application data model supported by the Aimlec_AimlTaskTransfer API.

Table 6.11.6.1-1 specifies the data types defined for the Aimlec_AimlTaskTransfer API.

Table 6.11.6.1-1: Aimlec_AimlTaskTransfer API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimleClientDirectTransferReq | 6.11.6.2.4 | Contains the AIMLE client direct task transfer request information. |  |
| AimleClientTaskTransferReq | 6.11.6.2.2 | Contains the AIMLE client task transfer request information. |  |
| AimleClientTaskTransferRes | 6.11.6.2.3 | Contains the AIMLE client task transfer response information. |  |
| AimlInfoType | 6.11.6.3.3 | Represents the AI/ML information type. |  |

Table 6.11.6.1-2 specifies data types re-used by the Aimlec_AimlTaskTransfer API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimlec_AimlTaskTransfer API.

Table 6.11.6.1-2: Aimlec_AimlTaskTransfer API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlOperation | 6.3.6.3.5 | Represents a type of the AI/ML operation (e.g., training, model transfer, model inference, model offload, model split, continue performing intermediate AI/ML operation). |  |
| TimeWindow | 3GPP TS 29.122 [5] | Represents a time window. |  |
| ValTargetUe | 3GPP TS 29.549 [9] | Unique identifier of a VAL user or a VAL UE. |  |

==============Next change==============

##### 6.11.6.2.2	Type: AimleClientTaskTransferReq

Table 6.11.6.2.2-1: Definition of type AimleClientTaskTransferReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | string | M | 1 | Contains the identifier of the service consumer. For the AIMLE client, e.g. unique client identifier. For the AIMLE server, e.g., FQDN, URI. |  |
| sourceAimlId | ValTargetUe | M | 1 | The identifier of the VAL UE i.e. the source AIMLE client. |  |
| aimlTaskType | AimlOperation | M | 1 | The type of the AI/ML operation (e.g. ML model training). |  |
| aimlInfoType | AimlInfoType | M | 1 | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |  |
| aimlTaskTransferTime | TimeWindow | O | 0..1 | Information on the requested time or time window for the AI/ML task transfer. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the AI/ML task transfer request. |  |

==============Next change==============

##### 6.11.6.2.3	Type: AimleClientTaskTransferRes

Table 6.11.6.2.3-1: Definition of type AimleClientTaskTransferRes

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlTaskTransferTime | TimeWindow | M | 1 | Information on the time or time window for the AI/ML task transfer. |  |

==============Next change==============

##### 6.11.6.2.4	Type: AimleClientDirectTransferReq

Table 6.11.6.2.4-1: Definition of type AimleClientDirectTransferReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | ValTargetUe | M | 1 | The identifier of the VAL UE i.e. the source AIMLE client. |  |
| aimlTaskType | AimlOperation | M | 1 | The type of the AI/ML operation (e.g. ML model training). |  |
| aimlInfoType | AimlInfoType | M | 1 | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |  |
| aimlTaskTransferTime | TimeWindow | O | 0..1 | Information on time or time window for the AI/ML task transfer. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the request. |  |

==============Next change==============

##### 6.11.6.3.3	Enumeration: AimlInfoType

The enumeration AimlInfoType represents the type of the AI/ML information. It shall comply with the provisions defined in table 6.11.6.3.3-1.

Table 6.11.6.3.3-1: Enumeration AimlInfoType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INTERMEDIATE_AIML_OP_RESULTS | Indicates the intermediate AI/ML operation results type of the AI/ML information. |  |
| INTERMEDIATE_AIML_OP_STATUS | Indicates the intermediate AI/ML operation status type of the AI/ML information. |  |
| OTHER_AIML_INFO_TYPE | Indicates other types of the AI/ML information. |  |

==============Next change==============

### 6.12.1	Introduction

The AIMLE server AI/ML Task Transfer shall use the Aimles_AIMLTaskTransfer API.

The API URI of the Aimles_AIMLTaskTransfer API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 5.2.4 of 3GPP TS 29.122 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 5.2.4 of 3GPP TS 29.122 [5].

-	The <apiName> shall be "aimles-task-transfer".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.12.4.

==============Next change==============

#### 6.12.4.1	Overview

Table 6.12.4.1-1: Custom operations without associated resources

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| AI/ML task transfer assist | /assist-tt | POST | Used by the AIMLE client to request the AIMLE server to perform task transfer assist. |
| Controlled AI/ML task transfer | /request-ctld | POST | Used by the AIMLE client to request the AIMLE server to perform AIMLE server controlled task transfer. |

==============Next change==============

#### 6.12.4.2	Operation: AI/ML task transfer assist

==============Next change==============

##### 6.12.4.2.1	Description

The custom operation enables the AIMLE client to request the AIMLE server to perform the AI/ML task transfer assist operation.

==============Next change==============

##### 6.12.4.2.2	Operation definition

This operation shall support the response data structures and response codes specified in tables 6.12.4.2.2-1 and 6.12.4.2.2-2.

Table 6.12.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimlesTaskTransferAssistReq | M | 1 | Contains the AIMLE server task transfer assist request information. |

Table 6.12.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimlesTaskTransferAssistResp | M | 1 | 200 OK | Successful case. The AIMLE server AI/ML task transfer assist is performed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.12.4.2.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE server. |

Table 6.12.4.2.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE server. |

==============Next change==============

#### 6.12.4.3	Operation: Controlled AI/ML task transfer

==============Next change==============

##### 6.12.4.3.2	Operation definition

This operation shall support the response data structures and response codes specified in tables 6.12.4.3.2-1 and 6.12.4.3.2-2.

Table 6.12.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| AimlesControlledTaskTransferReq | M | 1 | Contains the AIMLE server controlled task transfer request information. |

Table 6.12.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| AimlesControlledTaskTransferResp | M | 1 | 200 OK | Successful case. The AIMLE server controlled AI/ML task transfer is performed. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [5]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.12.4.3.2-3: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE server. |

Table 6.12.4.3.2-4: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI located in an alternative AIMLE server. |

==============Next change==============

#### 6.12.6.1	General

This clause specifies the application data model supported by the Aimles_AIMLTaskTransfer API.

Table 6.12.6.1-1 specifies the data types defined for the Aimles_AIMLTaskTransfer API.

Table 6.12.6.1-1: Aimles_AIMLTaskTransfer API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlesControlledTaskTransferReq | 6.12.6.2.4 | Contains the AIMLE server controlled task transfer request information. |  |
| AimlesControlledTaskTransferResp | 6.12.6.2.5 | Contains the AIMLE server controlled task transfer response information. |  |
| AimlesTaskTransferAssistReq | 6.12.6.2.2 | Contains the AIMLE server task transfer assist request information. |  |
| AimlesTaskTransferAssistResp | 6.12.6.2.3 | Contains the AIMLE server task transfer assist response information. |  |
| AimlIntermediateInfo | 6.12.6.2.7 | Contains the AI/ML intermediate information for intermediate AI/ML operation. |  |
| AimlRmngTrainingReq | 6.12.6.2.6 | Contains requirements for AI/ML model training. |  |
| TransferMode | 6.12.3.3.3 | Represents the transfer mode. |  |

Table 6.12.6.1-2 specifies data types re-used by the Aimles_AIMLTaskTransfer API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the Aimles_AIMLTaskTransfer API.

Table 6.12.6.1-2: Aimles_AIMLTaskTransfer API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimlInfoType | 6.11.6.3.3 | Represents the AI/ML information type. |  |
| AimlOperation | 6.3.6.3.5 | Represents a type of the AI/ML operation (e.g., training, model transfer, model inference, model offload, model split, continue performing intermediate AI/ML operation). |  |
| TimeWindow | 3GPP TS 29.122 [5] | Represents a time window. |  |
| ValTargetUe | 3GPP TS 29.549 [9] | Unique identifier of a VAL user or a VAL UE. |  |

==============Next change==============

##### 6.12.6.2.2	Type: AimlesTaskTransferAssistReq

Table 6.12.6.2.2-1: Definition of type AimlesTaskTransferAssistReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | ValTargetUe | M | 1 | The identifier of source AIMLE client. |  |
| valServiceId | string | O | 0..1 | The identifier of the VAL service for which the assistance information is requested. |  |
| aimlTaskType | AimlOperation | M | 1 | The type of the AI/ML operation (e.g. ML model training). |  |
| aimlInfoType | AimlInfoType | M | 1 | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |  |
| aimlRmngTrainReq | AimlRmngTrainingReq | C | 0..1 | Contains requirements for AI/ML model training including, required remaining training resource, required remaining training number of iterations. (NOTE) |  |
| aimlImdInfo | AimlIntermediateInfo | C | 0..1 | Contains the AI/ML intermediate information for intermediate AI/ML operation, including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations. (NOTE) |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the request. |  |
| NOTE: This attribute may be present only if the aimlTaskType attribute is set to value "MODEL_TRAINING". |  |  |  |  |  |

==============Next change==============

##### 6.12.6.2.3	Type: AimlesTaskTransferAssistResp

Table 6.12.6.2.3-1: Definition of type AimlesTaskTransferAssistResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| assistanceTime | TimeWindow | M | 1 | Indicates a time window for assistance in the AI/ML task transfer. |  |
| targetAimlIds | array(ValTargetUe) | M | 1..N | List of the target AIMLE clients. |  |
| transferMode | TransferMode | O | 0..1 | Indication of the transfer mode (e.g., direct transfer). |  |

==============Next change==============

##### 6.12.6.2.4	Type: AimlesControlledTaskTransferReq

Table 6.12.6.2.4-1: Definition of type AimlesControlledTaskTransferReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestorId | ValTargetUe | M | 1 | The identifier of the source AIMLE client. |  |
| aimlTaskType | AimlOperation | M | 1 | The type of the AI/ML operation (e.g. ML model training). |  |
| aimlInfoType | AimlInfoType | M | 1 | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |  |
| aimlTaskTransferTime | TimeWindow | M | 1 | Information on time or time window for the AI/ML task transfer. |  |
| timeValidity | TimeWindow | O | 0..1 | The time validity of the request. |  |

==============Next change==============

##### 6.12.6.2.5	Type: AimlesControlledTaskTransferResp

Table 6.12.6.2.5-1: Definition of type AimlesControlledTaskTransferResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| assistanceTime | TimeWindow | M | 1 | Indicates a time window for assistance in the AI/ML task transfer. |  |

==============Next change==============

##### 6.12.6.2.7	Type: AimlIntermediateInfo

Table 6.12.6.2.7-1: Definition of type AimlIntermediateInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| aimlImdModel | AimlRmngTrainingReq | O | 0..1 | Indicates the AI/ML intermediate model. |  |
| aimlUsedTrainTime | TimeWindow | O | 0..1 | Indicates the AI/ML intermediate model used training time. |  |
| usedTrainResource | string | O | 0..1 | Indicates used training resource. |  |
| usedTrainIterNum | integer | O | 0..1 | Indicates used training number of iterations. |  |

==============Next change==============

##### 6.12.6.3.3	Enumeration: TransferMode

The enumeration TransferMode represents the mode of transfer. It shall comply with the provisions defined in table 6.12.6.3.3-1.

Table 6.12.6.3.3-1: Enumeration TransferMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DIRECT | Directly from the source AI/ML member to the target AI/ML member. |  |
| SERVER_CONTROLLED | Transfer with AIMLE server controlled. |  |

==============Next change==============

## A.2	Aimlec_AIMLEClientParticipation API

openapi: 3.0.0

info:

title: Aimlec_AIMLEClientParticipation

version: 1.0.1

description: |

API for Client Participation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-cp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/participation:

post:

summary: >

Used by AIMLE server to manage the participation of AIMLE clients in AI/ML operations.

operationId: ClientParticipation

tags:

- AIMLE client for participation of the AI/ML operations request

requestBody:

description: Contains information of the AIMLE client participates the AI/ML operations.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationReq'

responses:

'200':

description: Contains the outcome of the successful AIMLE client participation.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimlecParticipationReq:

description: Represents the participation request for the AI/ML operations.

type: object

required:

- requesterId

- clientSetId

- clientSetPart

- mlModelId

- schedAimlOperations

- dataSetReq

- serviceReq

properties:

requesterId:

description: Represents the requester identity.

type: string

clientSetId:

description: Represents the AIMLE client set identity.

type: string

clientSetPart:

$ref: '#/components/schemas/ClientSetPart'

mlModelId:

description: Identifies the identity of the ML model for AI/ML operation.

type: string

schedAimlOperations:

type: array

items:

$ref: '#/components/schemas/SchedAimlOperation'

minItems: 1

dataSetReq:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/DatasetRequirement'

serviceReq:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ServiceRequirement'

AimlecParticipationResp:

description: Represents the participation response for the AI/ML operations.

type: object

required:

- clientStatus

properties:

clientStatus:

description: true indicates the willingness and false indicates the denial.

type: boolean

SchedAimlOperation:

description: Represents the scheduled AI/ML participation type.

type: object

required:

- aimlOperation

properties:

aimlOperation:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlOperSched:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

# Enumerations

ClientSetPart:

anyOf:

- type: string

enum:

- JOIN

- DEPART

- type: string

description: >

This string provides the participation request for the AIMLE client set.

description: |

Identifies the type of request for participating the AIMLE client set.

Possible values are:

- JOIN: Request to join the AIMLE client set.

- DEPART: Request to depart from the AIMLE client set.

==============Next change==============

## A.3	Aimlec_HFLTraining API

openapi: 3.0.0

info:

title: Aimlec_HFLTraining

version: 1.0.1

description: |

API for Horizontal Federated Learning (HFL) training Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-hfl-trng/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: >

Used by AIMLE server to subscribe with the AIMLE client for HFL training event.

operationId: HFLTraningSubs

tags:

- HFL training service API event subscriptions (Collection)

requestBody:

description: Contains information for HFL training subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

responses:

'201':

description: Created. Service API event subscription resource created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notifUri:

'{$request.body#/notifUri}':

post:

requestBody:  # contents of the callback message

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngNotify'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: Identifier of the individual Events Subscription.

required: true

schema:

type: string

get:

description: Retrieve an existing "Individual HFL training subscription" resource.

operationId: RetrieveHFLTraningSubs

tags:

- Individual HFL training Subscription (Document)

responses:

'200':

description: The individual HFL training subscription.

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: >

Update the "Individual HFL training subscription" resource.

operationId: UpdateHFLTrainingSubs

tags:

- Individual HFL training subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

responses:

'200':

description: >

OK. Individual HFL training subscription resource is successfully updated,

and representation of updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

'204':

description: >

No Content. Individual HFL training subscription resource is successfully updated,

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Modify the "Individual HFL training subscription" resource.

operationId: ModifyHFLTrainingSubs

tags:

- Individual HFL training subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/HflTrngSubPatch'

responses:

'200':

description: >

OK. The Individual HFL training subscription resource is successfully modified,

and representation of the modified resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/HflTrngSub'

'204':

description: >

No Content. The Individual HFL training subscription resource is successfully modified,

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual HFL training ubscription.

operationId: DeleteHFLTraningSubs

tags:

- Individual HFL training Subscription (Document)

responses:

'204':

description: >

The individual HFL training Subscription matching the subscriptionId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

HflTrngSub:

description: Represents data type for HFL training subscription and its update.

type: object

required:

- requesterId

- notifUri

- aimlMdlInfo

- dataId

- noDataSamp

- vaSrvId

properties:

requesterId:

description: Represents the requester identity.

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

aimlMdlInfo:

$ref: 'TS29482_AIMLES_MLModelTraining.yaml#/components/schemas/MlModelInfo'

dataId:

description: Identifies the dataset which is to be used for the HFL training.

type: string

noDataSamp:

description: Identifies the required number of samples for a round of the HFL training.

type: integer

operSched:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

notifReqs:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ReportingRequirements'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

vaSrvId:

description: Identifies the VAL service for the AIMLE HFL training operation.

type: string

subId:

description: Identifies the subscription.

type: string

HflTrngSubPatch:

description: Represents data type for partial update of the HFL training subscription.

type: object

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

aimlMdlInfo:

$ref: 'TS29482_AIMLES_MLModelTraining.yaml#/components/schemas/MlModelInfo'

dataId:

description: Identifies the dataset which is to be used for the HFL training.

type: string

noDataSamp:

description: Identifies the required number of samples for a round of the HFL training.

type: integer

operSched:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

notifReqs:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ReportingRequirements'

HflTrngNotify:

description: Represent the information from the HFL training event notification.

type: object

required:

- hflTrngOut

- vaSrvId

properties:

vaSrvId:

description: Identifies the VAL service for the AIMLE HFL training operation.

type: string

hflTrngOut:

$ref: 'TS29482_AIMLES_MLModelTraining.yaml#/components/schemas/PerfParams'

hflTrngErr:

$ref: 'TS29482_AIMLES_MLModelTraining.yaml#/components/schemas/TrainingErr'

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

==============Next change==============

## A.4	Aimles_AIMLEClientRegistration API

openapi: 3.0.0

info:

title: Aimles_AIMLEClientRegistration

version: 1.0.1

description: |

API for AIMLE Client Registration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimles-client-reg/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/registrations:

post:

summary: Registers the AIMLE client at the AIMLE server.

operationId: RegAimleClient

tags:

- AIMLE client registrations

requestBody:

description: >

Contains information for the creation of a new individual AIMLE client registration

resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientRegInfo'

responses:

'201':

description: Represents an individual AIMLE client registration resource.

content:

application/json:

schema:

$ref: '#/components/schemas/AimleRegistration'

headers:

Location:

description: >

Contains the URI of the newly created individual AIMLE client registration

resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

put:

summary: Update an Individual AIMLE client registration resource.

operationId: UpdateAimleClientReg

tags:

- Individual AIMLE client registration (Document)

parameters:

- name: registrationId

description: >

String identifying the individual AIMLE client registration resource at the AIMLE server.

in: path

required: true

schema:

type: string

requestBody:

description: >

Contains information for the update of individual AIMLE client registration resource.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimleRegistration'

responses:

'200':

description: >

An individual AIMLE client registration resource is updated, and a representation of

that resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/AimleRegistration'

'204':

description: No Content. An individual AIMLE client registration resource is updated.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Removes an Individual AIMLE client registration resource.

operationId: DelAimleClientReg

tags:

- Individual AIMLE client registration (Document)

parameters:

- name: registrationId

description: >

String identifying the individual AIMLE client registration resource at the AIMLE server.

in: path

required: true

schema:

type: string

responses:

'204':

description: An individual AIMLE client registration resource is removed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimleRegistration:

description: Represents an individual AIMLE client registration resource.

type: object

required:

- regData

properties:

regData:

$ref: '#/components/schemas/AimleClientRegInfo'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AimleClientRegInfo:

description: Contains the AIMLE client registration information.

type: object

required:

- aimleClientId

- suppProfiles

properties:

aimleClientId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

suppProfiles:

description: Contains a list of supported service information and AIMLE client profiles.

type: array

items:

$ref: '#/components/schemas/SupportedProfile'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

SupportedProfile:

description: Contains AIMLE client profiles and supported service information.

type: object

properties:

clientProfile:

$ref: '#/components/schemas/AimleClientProfile'

suppServices:

description: >

Contains the list of VAL services identifiers with corresponding service permissions.

type: array

items:

$ref: '#/components/schemas/ServiceData'

minItems: 1

ServiceData:

description: Contains VAL service identifier with the corresponding service permission.

type: object

required:

- valServiceId

properties:

valServiceId:

description: Represents the VAL service identifier.

type: string

servPermLevel:

$ref: '#/components/schemas/ServicePermissionLevel'

AimleClientProfile:

description: >

Contains the AIMLE client capability information e.g. supported AI/ML model types,

AI/ML service operation type.

type: object

required:

- aimleClientUri

- aimlOperations

- clientCap

properties:

aimleClientUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

aimlModelTypes:

description: Contains AI/ML model types supported by the AIMLE client.

type: array

items:

$ref: '#/components/schemas/AimlModelType'

minItems: 1

aimlOperations:

description: Contains AI/ML operations supported by the AIMLE client.

type: array

items:

$ref: '#/components/schemas/AimlOperation'

minItems: 1

clientCap:

$ref: '#/components/schemas/ClientCapability'

availTimeSchedCfgs:

description: >

Contains the availability schedule of the AIMLE client for the AI/ML service.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

unavblTimeSchedCfgs:

description: >

Contains the unavailability schedule of the AIMLE client for the AI/ML service.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

availLocCfgs:

description: >

Contains the available location-based configurations of the AIMLE client for the

AI/ML service.

type: array

items:

$ref: '#/components/schemas/LocationConfig'

minItems: 1

unavblLocCfgs:

description: >

Contains the unavailable location-based configurations of the AIMLE client for the

AI/ML service.

type: array

items:

$ref: '#/components/schemas/LocationConfig'

minItems: 1

dataSetAvail:

$ref: '#/components/schemas/DataSetAvailability'

dataCap:

description: >

Contains a list of data capabilities such as the type of data that can be collected,

supported data processing capabilities and supported exploratory data analysis (EAD)

functions.

type: array

items:

$ref: '#/components/schemas/DataCapability'

minItems: 1

taskCaps:

description: Contains the AI/ML task performing capabilities.

type: array

items:

$ref: '#/components/schemas/TaskCapability'

minItems: 1

ClientCapability:

description: Contains the AIMLE client capability information.

type: object

required:

- mlAppType

- rsrcUsageLvl

properties:

mlAppType:

$ref: '#/components/schemas/MlApplicationType'

rsrcUsageLvl:

$ref: '#/components/schemas/ResourceUsageLevel'

DataSetAvailability:

description: Represents a dataset availability.

type: object

required:

- dataSetIds

properties:

dataSetIds:

description: Contains a list of dataset identifiers.

type: array

items:

type: string

minItems: 1

size:

description: Represents the dataset size e.g., number of entries in dataset.

type: integer

age:

description: Represents the dataset age e.g. data set usage in number of days.

type: integer

features:

description: Contains a list of dataset features.

type: array

items:

type: string

minItems: 1

LocationConfig:

description: >

Indicates the location-based configurations of the AIMLE client for the AI/ML service.

type: object

properties:

clientLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

valSvcAreaId:

$ref: 'TS29549_SS_VALServiceAreaConfiguration.yaml#/components/schemas/ValSvcAreaId'

# Simple data types

# Enumerations

ServicePermissionLevel:

anyOf:

- type: string

enum:

- PREMIUM_RESOURCE_USAGE

- STANDARD_RESOURCE_USAGE

- LIMITED_RESOURCE_USAGE

- OTHER_SERVICE_PERMISSION_LEVEL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents a service permission level.

Possible values are:

- PREMIUM_RESOURCE_USAGE: Indicates a premium resource usage level.

- STANDARD_RESOURCE_USAGE: Indicates a standard resource usage level.

- LIMITED_RESOURCE_USAGE: Indicates a limited resource usage level.

- OTHER_SERVICE_PERMISSION_LEVEL: Indicates other service permission level.

AimlModelType:

anyOf:

- type: string

enum:

- DECISION_TREE

- LINEAR_REGRESSION

- NEURAL_NETWORK

- OTHER_MODEL_TYPE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the AI/ML model types.

Possible values are:

- DECISION_TREE: Indicates the decision tree type of the AI/ML model.

- LINEAR_REGRESSION: Indicates the linear regression type of the AI/ML model.

- NEURAL_NETWORK: Indicates the neural network type of the AI/ML model.

- OTHER_MODEL_TYPE: Indicates the other types of the AI/ML model.

AimlOperation:

anyOf:

- type: string

enum:

- MODEL_INFERENCE

- MODEL_OFFLOAD

- MODEL_SPLIT

- MODEL_TRANSFER

- MODEL_TRAINING

- CONTINUE_PERFORM_INTERMEDIATE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the type of the AI/ML operation.

Possible values are:

- MODEL_INFERENCE: Indicates the model inference type of the AI/ML operation.

- MODEL_OFFLOAD: Indicates the model offload type of the AI/ML operation.

- MODEL_SPLIT: Indicates the model split type of the AI/ML operation.

- MODEL_TRANSFER: Indicates the model transfer type of the AI/ML operation.

- MODEL_TRAINING: Indicates the model training type of the AI/ML operation.

- CONTINUE_PERFORM_INTERMEDIATE: Indicates the ability to continue performing of

the intermediate AI/ML operation.

MlApplicationType:

anyOf:

- type: string

enum:

- FEDERATED_LEARNING

- TRANSFER_LEARNING

- SPLIT_LEARNING

- OTHER_ML_APPLICATION_TYPE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the ML application types.

Possible values are:

- FEDERATED_LEARNING: Indicates the federated learning ML application type.

- TRANSFER_LEARNING: Indicates the transfer learning ML application type.

- SPLIT_LEARNING: Indicates the split learning ML application type.

- OTHER_ML_APPLICATION_TYPE: Indicates the other ML application types.

ResourceUsageLevel:

anyOf:

- type: string

enum:

- PREMIUM_RESOURCE_USAGE

- STANDARD_RESOURCE_USAGE

- LIMITED_RESOURCE_USAGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents a resource usage level.

Possible values are:

- PREMIUM_RESOURCE_USAGE: Indicates a premium resource usage level.

- STANDARD_RESOURCE_USAGE: Indicates a standard resource usage level.

- LIMITED_RESOURCE_USAGE: Indicates a limited resource usage level.

DataCapability:

anyOf:

- type: string

enum:

- RAW_DATA

- STRUCURED_DATA

- SEMI_STRUCTURED_DATA

- UNSTRUCTURED_DATA

- PROCESSED_DATA

- EXPLOATORY_DATA_ANALYSIS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the data capabilities.

Possible values are:

- RAW_DATA: Indicates the raw data.

- STRUCURED_DATA: Indicates the structured data.

- SEMI_STRUCTURED_DATA: Indicates the semi-structured data.

- UNSTRUCTURED_DATA: Indicates the unstructured data.

- PROCESSED_DATA: Indicates the processed data.

- EXPLOATORY_DATA_ANALYSIS: Indicates the exploratory data analysis function.

match.

TaskCapability:

anyOf:

- type: string

enum:

- HIGH_COMPUTE_CAPABILITY

- LOW_COMPUTE_CAPABILITY

- LOW_COSTS_PERFORMANCE

- GREEN_TASK_PERFORMANCE

- ENERGY_EFFICIENT_PERFORMANCE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the AI/ML task performing capabilities.

Possible values are:

- HIGH_COMPUTE_CAPABILITY: Indicates a high compute capability.

- LOW_COMPUTE_CAPABILITY: Indicates a low compute capability.

- LOW_COSTS_PERFORMANCE: Indicates a low cost performance.

- GREEN_TASK_PERFORMANCE: Indicates a green task performance.

- ENERGY_EFFICIENT_PERFORMANCE: Indicates an energy efficient performance.

==============Next change==============

## A.5	Aimles_SplitOpPipeline API

openapi: 3.0.0

info:

title: Aimles_SplitOpPipeline

version: 1.0.1

description: |

API for split AI/ML operation pipeline service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimles-sopl/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: >

Used by AIMLE client to create an instance of a split operation pipeline at the AIMLE

server.

operationId: SplitOpPipeCreate

tags:

- Split operation pipeline creation API (Collection)

requestBody:

description: Contains information of the AI/ML split operation pipelie creation.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineCreateReq'

responses:

'201':

description: Created. Split operation pipeline creation resource is created successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineCreateResp'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request/{requestId}:

parameters:

- name: requestId

in: path

description: Identifier of the individual AI/ML split operation pipeline.

required: true

schema:

type: string

put:

description: >

Update an individual split operation pipeline resource.

operationId: SplitOpPipeUpdate

tags:

- Individual split operation pipeline (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineCreateReq'

responses:

'200':

description: >

OK. Individual AIMLE split operation pipeline resource is successfully updated,

and representation of updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineCreateResp'

'204':

description: >

No Content. Individual AIMLE split operation pipeline resource is successfully updated,

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: >

Modify the "Individual AIMLE split operation pipeline" resource.

operationId: SplitOpPipeModify

tags:

- Individual AIMLE split operation pipeline (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/SplitOpPipelinePatch'

responses:

'200':

description: >

OK. The Individual AIMLE split operation pipeline resource is successfully modified,

and representation of the modified resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineCreateResp'

'204':

description: >

No Content. The Individual AIMLE split operation pipeline resource is successfully

modified, and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual AIMLE split operation pipeline.

operationId: SplitOpPipeDelete

tags:

- Individual AIMLE split operation pipeline (Document)

responses:

'204':

description: >

The individual AIMLE split operation pipeline matching the requestId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/discovery:

post:

summary: >

Used by consumer to request the AIMLE server for split AI/ML operation pipeline discovery

operationId: SplitOpPipeDiscovery

tags:

- AIMLE split operation pipeline discovery

requestBody:

description: Contains information of the AIMLE split operation pipeline discovery.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineDiscReq'

responses:

'200':

description: Contains successful outcome of AIMLE split operation pipeline discovery.

content:

application/json:

schema:

$ref: '#/components/schemas/SplitOpPipelineDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SplitOpPipelineCreateReq:

description: Represents AIMLE split operation pipeline create and its update.

type: object

required:

- requestorId

- notifUri

- splitOpRequirements

properties:

requestorId:

description: Identifies the service consumer.

type: string

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

splitOpRequirements:

$ref: '#/components/schemas/SplitOpRequirements'

SplitOpPipelineCreateResp:

description: Represents response to AIMLE split operation pipeline create and its update.

type: object

properties:

splitOpProfile:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/SplitOpProfile'

SplitOpPipelinePatch:

description: Represents AIMLE split operation pipeline update.

type: object

required:

- notifUri

- splitOpPipelineId

- splitOpPipelineInfo

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

splitOpPipelineId:

description: Identifies split operation pipeline.

type: string

splitOpPipelineInfo:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/SplitOpPipelineInfo'

SplitOpPipelineDiscReq:

description: Represents the AIMLE Split Operation Pipeline Discovery request.

type: object

required:

- notifUri

- discFilters

properties:

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

discFilters:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/DiscFilters'

SplitOpPipelineDiscResp:

description: Represents the AIMLE Split Operation Pipeline Discovery response.

type: object

properties:

discoveredNodes:

type: array

items:

type: string

minItems: 1

splitOpProfiles:

type: array

items:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/SplitOpProfile'

minItems: 1

anyOf:

- required: [discoveredNodes]

- required: [splitOpProfiles]

SplitOpRequirements:

description: Represents the AIMLE Split Operation Pipeline requirements.

type: object

required:

- stageInfo

properties:

stageInfo:

type: array

items:

$ref: 'TS29482_AIMLES_SplitOpEvent.yaml#/components/schemas/StageInfo'

minItems: 1

usageInfo:

$ref: 'TS29482_AIMLES_SplitOpNodeRegistration.yaml#/components/schemas/UsageInformation'

notificationTarget:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

==============Next change==============

## A.6	Aimlec_FLGroupIndication API

openapi: 3.0.0

info:

title: Aimlec_FLGroupIndication

version: 1.0.1

description: |

API for AIMLE Client Federated Learning Group Indication Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services –

Service enabler Architecture Layer for Verticals (SEAL) Protocol Specification; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/24_series/24.560/

servers:

- url: '{apiRoot}/aimlec-flgi/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/indicate:

post:

summary: Indicates FL group information to FL group member

operationId: IndicateFLMemberInfo

tags:

- Indicate FL group

requestBody:

description: Contains the FL group member information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/IndFlMember'

responses:

'204':

description: No Content (Success)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

# Structured data types

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

IndFlMember:

description: Indicates the FL member the information on FL member group.

type: object

properties:

serverId:

type: string

description: Identifier of the indicating AIMLE server.

valServiceId:

type: string

description: Identifier of the VAL service for which the grouping indication is applied.

mlModelId:

type: string

description: Identifier of the ML model for which the grouping indication is applied.

analyticsId:

type: string

description: >

Identifier of the ADAE analytics service, the FL grouping is based on, if

the FL process is used for that ADAE analytics service.

flGroupIds:

type: array

items:

$ref: '#/components/schemas/FlGroupInfo'

minItems: 1

description: >

List of the AIMLE identifiers of that created or modified FL group for the FL process.

flGroupDelInfo:

$ref: '#/components/schemas/FlGroupDeletionInfo'

required:

- serverId

- flGroupIds

FlGroupInfo:

description: Represents the FL group information.

type: object

required:

- flGroupId

properties:

flGroupId:

description: Contains the FL group identifier.

type: string

flMembers:

type: array

items:

$ref: '#/components/schemas/FlMemberData'

minItems: 1

description: Contains FL member data.

FlMemberData:

description: Represents the FL group member data e.g. FL member identifier, address.

type: object

properties:

flMemberId:

type: string

description: Identifier of the FL members

flMemberAddr:

$ref: 'TS29549_SS_NetworkResourceAdaptation.yaml#/components/schemas/ValUeAddrInfo'

flMemberInfo:

$ref: '#/components/schemas/FlMemberInfo'

FlMemberInfo:

description: Represents the FL member information e.g. availability, constraint, FL role.

type: object

properties:

availability:

$ref: '#/components/schemas/FlMemberAvailability'

constraints:

description: Represents the FL group member constraints.

type: array

items:

$ref: '#/components/schemas/FlMemberConstraint'

minItems: 1

role:

$ref: '#/components/schemas/FlMemberRole'

FlGroupDeletionInfo:

description: Indicates the FL group deletion information.

type: object

properties:

cause:

$ref: '#/components/schemas/FlGroupDelCause'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

# Simple data types

# Enumerations

FlMemberAvailability:

anyOf:

- type: string

enum:

- AVAILABLE

- STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

But is not used to encode content defined in the present version of this API.

description: |

Represents the information regarding FL member availability of the VAL UE.

Possible values are:

- AVAILABLE: The FL member is available.

- NOT_AVAILABLE: The FL member is not available.

FlMemberConstraint:

anyOf:

- type: string

enum:

- LIMITED_MEMORY

- LIMITED_PROCCESSING

- LIMITED_ACCESS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

But is not used to encode content defined in the present version of this API.

description: |

Represents the FL member constraint information of the VAL UE.

Possible values are:

- LIMITED_MEMORY: Indicates a limited memory load.

- LIMITED_PROCCESSING: Indicates a limited processing power.

- LIMITED_ACCESS: Indicates a limited access to only the local data.

FlMemberRole:

anyOf:

- type: string

enum:

- FL_CLIENT

- FL_SERVER

- FL_AGGREGATOR

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

But is not used to encode content defined in the present version of this API.

description: |

Represents the FL member role of the VAL UE.

Possible values are:

- FL_CLIENT: Indicates an FL client role.

- FL_SERVER: Indicates an FL server role.

- FL_AGGREGATOR: Indicates an FL aggregator role.

FlGroupDelCause:

anyOf:

- type: string

enum:

- SRV_TERMINATION

- OUT_OF_SRV_AREA

- type: string

description: >

This string provides information on the FL group deletion cause.

description: |

Represents the information regarding FL group deletion cause as AI/ML service termination or

UE has moved out of the service area.

Possible values are:

- SRV_TERMINATION: Indicates the AI/ML service termination.

- OUT_OF_SRV_AREA: Indicates the UE has moved out of the service area.

==============Next change==============

## A.7	Aimlec_ClientDataProcessing API

openapi: 3.0.0

info:

title: Aimlec_ClientDataProcessing

version: 1.0.1

description: |

API for Transfer Learning (TL) Enablement Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-data-proc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/trigger:

post:

summary: >

Used by AIMLE server to trigger AIMLE client to request client data processing procedure.

operationId: ClientDataProcessing

tags:

- Client data processing procedure request

requestBody:

description: Contains information to trigger client data processing procedure.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CltDataProcReq'

responses:

'200':

description: Contains the outcome of the successful client data processing.

content:

application/json:

schema:

$ref: '#/components/schemas/CltDataProcResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

CltDataProcReq:

description: Contains information to trigger client data processing procedure.

type: object

required:

- requesterId

- dataProc

properties:

requesterId:

description: Represents the requester identity.

type: string

dataProc:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/DataMgmtOp'

dataPrepReqs:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/DataProcessReqs'

dataAnalysisReqs:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/DataProcessReqs'

dataProcSched:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

CltDataProcResp:

description: Contains the outcome of the successful client data processing.

type: object

properties:

dataPrepOutputs:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/AggregatedDataPrepOutputs'

dataAnalysisOutputs:

$ref: 'TS29482_AIMLES_DataManagement.yaml#/components/schemas/AggregatedDataAnaOutputs'

timestamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

anyOf:

- required: [dataPrepOutputs]

- required: [dataAnalysisOutputs]

==============Next change==============

## A.8	Aimlec_MLModTngCapEva API

openapi: 3.0.0

info:

title: Aimlec_MLModTngCapEva

version: 1.0.1

description: |

API for ML Model Training Capability Evaluation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-ml-mtce/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: >

Enables the AIMLE server to request the AIMLE client to perform ML model training

capability evaluation service operation.

operationId: MlModTrainCapEvaReq

tags:

- ML model training capability evaluation request

requestBody:

description: Contains the ML model training capability evaluation request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MlModTngCapEvalReq'

responses:

'200':

description: Contains the ML model training capability evaluation response information.

content:

application/json:

schema:

$ref: '#/components/schemas/MlModTngCapEvalResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

MlModTngCapEvalReq:

description: Contains the ML model training capability evaluation request information.

type: object

required:

- aimleServerId

properties:

aimleServerId:

description: Represents the AIMLE server identifier.

type: string

availTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

testTask:

description: Represents the task for test ML model training capability.

type: string

modelInfo:

$ref: '#/components/schemas/AimlModelData'

dataSetReq:

$ref: '#/components/schemas/DataSetRequirements'

MlModTngCapEvalResp:

description: Contains the ML model training capability evaluation response information.

type: object

required:

- capEvalOut

properties:

capEvalOut:

$ref: '#/components/schemas/CapEvalOutcome'

testResult:

description: Represents the test result of the ML model training capability evaluation.

type: string

evalFailInd:

description: Represents the reason for inability to join the FL training process.

type: string

AimlModelData:

description: Contains the AI/ML model information and model parameters for use in FL training.

type: object

properties:

aimlModels:

description: Contains information about the AI/ML model.

type: array

items:

$ref: '#/components/schemas/AimlModelInfo'

minItems: 1

mlModelParams:

description: Contains model parameters for use in FL training.

type: array

items:

type: string

minItems: 1

DataSetRequirements:

description: Contains requirements on data set for FL training.

type: object

properties:

commonFtIds:

description: >

Contains a list of the features identifiers of the required features common to

the dataset of the different data domains.

type: array

items:

type: string

minItems: 1

domainFts:

description: >

Contains a list of features for each data domain(s) of the datasets at the UE.

type: array

items:

$ref: '#/components/schemas/DomainFeatures'

minItems: 1

dataSource:

description: >

Represents the identifier of a data source for the FL training (e.g. SEAL server,

SEAL client, other NF entity, etc.).

type: string

DomainFeatures:

description: Contains a list of features for each data domain(s) of the datasets at the UE.

type: object

required:

- domain

- featureIds

properties:

domain:

description: >

Represents a data domain i.e. a specific category of data or logical groupings of

data that all relate together (e.g. customer data, product data, etc.).

type: string

featureIds:

description: >

Represents a list of the features identifiers for the data domain of the datasets

at the UE.

type: array

items:

type: string

minItems: 1

AimlModelInfo:

description: Contains information about the AI/ML model.

type: object

properties:

aimlModelType:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlModelType'

mlModelProf:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelProfile'

# Simple data types

# Enumerations

CapEvalOutcome:

anyOf:

- type: string

enum:

- ABILITY_TO_JOIN

- FIRST_MATCH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the outcome of the ML model training capability evaluation.

Possible values are:

- ABILITY_TO_JOIN: Indicates ability to join the training process.

- INABILITY_TO_JOIN: Indicates inability to join the training process.

==============Next change==============

## A.9	Aimles_UeTLModelSelectionAssistance API

openapi: 3.0.0

info:

title: Aimles_UeTLModelSelectionAssistance

version: 1.0.1

description: |

API for Transfer Learning (TL) Enablement Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimles-ue-tl-msa/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/assist-tlms:

post:

summary: >

Used by the AIMLE client to request the AIMLE server to perform TL enablement service.

operationId: UeTlModSelectAssistReq

tags:

- UE transfer learning model selection assistance request

requestBody:

description: Contains information to trigger TL enablement service.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TlModelSelectAssistReq'

responses:

'200':

description: Contains one or more pre-trained ML models for the TL enablement service.

content:

application/json:

schema:

$ref: '#/components/schemas/TlModelSelectAssistResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

TlModelSelectAssistReq:

description: Contains information to trigger TL enablement service.

type: object

required:

- serverId

- valSrvId

- tlCriteria

properties:

serverId:

description: Represents the AIMLE server identity.

type: string

valSrvId:

description: Represents the VAL service for the TL enablement service.

type: string

tlCriteria:

$ref: '#/components/schemas/TlCriteria'

mlTaskId:

description: Represents the task for the TL enablement service.

type: string

adaeAnalyticsId:

description: Represents the ADAE analytics for the TL enablement service.

type: string

mlModelProfile:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModelProfile'

mlModelReq:

description: Represents the requirements for the ML model for the TL enablement service.

type: array

items:

type: string

minItems: 0

valUeIds:

description: Represents VAL UEs associated with the ML task for the TL enablement service.

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 0

mlModelRateReq:

description: Represents requirements for rating the ML model for TL enablement service.

type: array

items:

type: string

minItems: 0

TlModelSelectAssistResp:

description: Contains the pre-trained ML models for the TL enablement service.

type: object

required:

- mlModelList

properties:

mlModelList:

description: Represents the selected pre-trained ML models for the TL enablement service.

type: array

items:

$ref: '#/components/schemas/MlModel'

minItems: 1

TlCriteria:

description: Contains the criteria for the TL enablement service.

type: object

properties:

reqFeatures:

description: Represents the required features for a pre-trained model.

type: array

items:

type: string

minItems: 0

dataReq:

description: Represents the training data requirements.

type: array

items:

type: string

minItems: 0

tlType:

$ref: '#/components/schemas/TlType'

environment:

$ref: '#/components/schemas/EnvironmentType'

access:

description: Permissions and restrictions to access the pre-trained model.

type: array

items:

$ref: '#/components/schemas/AccessType'

minItems: 0

MlModel:

description: Contains the pre-trained ML models for the TL enablement service.

type: object

required:

- mlRepositoryId

- mlModelInfo

properties:

mlRepositoryId:

description: Represents the unique repository identity of an ML model.

type: string

mlModelInfo:

$ref: 'TS24560_Aimlec_MLModTngCapEva.yaml#/components/schemas/AimlModelInfo'

mlModelRating:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

AccessType:

description: Contains permission and restriction to access the pre-trained model.

type: object

properties:

modelLicense:

description: Represents the license of the pre-trained ML model.

type: string

dataTrainLicense:

description: Represents the license of the dataset for the pre-trained ML model.

type: string

codeTrainLicense:

description: Represents the license of the code for the pre-trained ML model.

type: string

# Enumerations

TlType:

anyOf:

- type: string

enum:

- INDUCTIVE

- TRANSDUCTIVE

- UNSUPERVISED

- type: string

description: >

This string provides the type of transfer learning.

description: |

Identifies how the pre-trained ML models can be related to the trained ML models for

the target ML task.

Possible values are:

- INDUCTIVE: For different task than the target ML task

- TRANSDUCTIVE: For similar task as target ML task but different data distributions

- UNSUPERVISED: For similar tasks as target ML task but with unlabelled data

EnvironmentType:

anyOf:

- type: string

enum:

- DOMAIN_SHIFT

- SIMULATED_REAL

- DYNAMIC

- HETEROGENEOUS

- ROBOTICS

- SMART

- type: string

description: >

This string provides the type of the environment.

description: |

Identifies how the environment and data distributions, suited for pre-trained ML models

in relationship with the trained ML model for the target ML task.

Possible values are:

- DOMAIN_SHIFT: Different data distributions

- SIMULATED_REAL: Simulated data vs. real data

- DYNAMIC: Data distributions changes over time

- HETEROGENEOUS: Significantly different data distributions

- ROBOTICS: Suitable for the target ML task for robotic platforms

- SMART: Suitable for the target ML task for smart platforms

==============Next change==============

## A.10	Aimlec_AIMLEClientServiceOperations API

openapi: 3.0.0

info:

title: Aimlec_AIMLEClientServiceOperations

version: 1.0.1

description: |

API for AIMLE Client Service Operations Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-serv-ops/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/perform:

post:

summary: >

Enables the AIMLE server to request the AIMLE client to perform the AIMLE client

service operation.

operationId: AimleServOperReq

tags:

- AIMLE service operation request

requestBody:

description: Contains the AIMLE client service operation request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientServOpReq'

responses:

'200':

description: Contains the AIMLE client service operation response information.

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientServOpResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimleClientServOpReq:

description: Contains the AIMLE client service operation request information.

type: object

required:

- aimleServerId

- servOpId

- servOpMode

properties:

aimleServerId:

description: Represents the AIMLE server identifier.

type: string

valServiceId:

description: Represents the VAL service identifier.

type: string

servOpId:

description: Represents the AI/ML service operation identifier.

type: string

servOpMode:

$ref: '#/components/schemas/ServiceOperationMode'

servOpInfo:

$ref: '#/components/schemas/ServiceOperationInfo'

servOpModeCfg:

$ref: '#/components/schemas/ServiceOpModeConfiguration'

servOpModeStatRptg:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ReportingRequirements'

AimleClientServOpResp:

description: Contains the AIMLE client service operation response information.

type: object

required:

- servOpId

- servOpModeStatus

properties:

valServiceId:

description: Represents the VAL service identifier.

type: string

servOpId:

description: Represents the AI/ML service operation identifier.

type: string

servOpModeStatus:

$ref: '#/components/schemas/ServiceOperationMode'

ServiceOperationInfo:

description: Contains the AI/ML service operation information.

type: object

properties:

mlMdlContainer:

description: Represents the AI/ML service model container.

type: string

mlMdlUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

mlMdAggregUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

maxConvgTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

ServiceOpModeConfiguration:

description: Contains the AI/ML service operation mode configuration.

type: object

properties:

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxDurHour:

description: >

Indicates the maximum duration time of the AI/ML service operation expressed in hours.

type: integer

modelAccuracy:

description: >

Indicates the threshold value of the model accuracy expressed as a percentage to

stop the AI/ML service operation.

type: integer

# Simple data types

# Enumerations

ServiceOperationMode:

anyOf:

- type: string

enum:

- START

- STOP

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

But is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation modes.

Possible values are:

- START: Indicates a request to start the AIMLE service operation or status of the AIMLE

service operation.

- STOP: Indicates a request to stop the AIMLE service operation or status of the AIMLE

service operation.

==============Next change==============

## A.11	Aimlec_AimlTaskTransfer API

openapi: 3.0.0

info:

title: Aimlec_AimlTaskTransfer

version: 1.0.1

description: |

API for AIMLE Client AI/ML Task Transfer Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-task-transfer/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request:

post:

summary: Enables the AIMLE server to request the AIMLE client to perform AI/ML task transfer.

operationId: AimlTaskTransf

tags:

- AI/ML task transfer

requestBody:

description: Contains the AIMLE client task transfer request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientTaskTransferReq'

responses:

'200':

description: Contains the AIMLE client task transfer response information.

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientTaskTransferRes'

'204':

description: No Content. The AIMLE client AI/ML task transfer is performed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-direct:

post:

summary: >

Enables the AIMLE client to request the target AIMLE client to perform AI/ML task transfer.

operationId: DirAimlTaskTransf

tags:

- Direct AI/ML task transfer

requestBody:

description: Contains the AIMLE client direct task transfer request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimleClientDirectTransferReq'

responses:

'204':

description: No Content. The AIMLE client direct AI/ML task transfer is performed.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimleClientTaskTransferReq:

description: Contains the AIMLE client task transfer request information.

type: object

required:

- requestorId

- sourceAimlId

- aimlTaskType

- aimlInfoType

properties:

requestorId:

description: Represents the identifier of the AIMLE server.

type: string

sourceAimlId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

aimlTaskType:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlInfoType:

$ref: '#/components/schemas/AimlInfoType'

aimlTaskTransferTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

AimleClientTaskTransferRes:

description: Contains the AIMLE client task transfer response information.

type: object

properties:

aimlTaskTransferTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

AimleClientDirectTransferReq:

description: Contains the AIMLE client direct task transfer request information.

type: object

required:

- requestorId

- aimlTaskType

- aimlInfoType

properties:

requestorId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

aimlTaskType:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlInfoType:

$ref: '#/components/schemas/AimlInfoType'

aimlTaskTransferTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

# Simple data types

# Enumerations

AimlInfoType:

anyOf:

- type: string

enum:

- INTERMEDIATE_AIML_OP_RESULTS

- INTERMEDIATE_AIML_OP_STATUS

- FIRST_MATCH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Represents the type of the AI/ML information.

Possible values are:

- INTERMEDIATE_AIML_OP_RESULTS: Indicates the intermediate AI/ML operation results type

of the AI/ML information.

- INTERMEDIATE_AIML_OP_STATUS: Indicates the intermediate AI/ML operation status type of

the AI/ML information.

- OTHER_AIML_INFO_TYPE: Indicates other types of the AI/ML information.

==============Next change==============

## A.12	Aimles_AimlTaskTransfer API

openapi: 3.0.0

info:

title: Aimles_AimlTaskTransfer

version: 1.0.1

description: |

API for AIMLE Server AI/ML Task Transfer Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence / Machine Learning (AI/ML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimles-task-transfer/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/assist-tt:

post:

summary: >

Enables the AIMLE client to request the AIMLE server to perform task transfer assist.

operationId: AimlTtAssist

tags:

- AI/ML task transfer assist

requestBody:

description: Contains the AIMLE server task transfer assist request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlesTaskTransferAssistReq'

responses:

'200':

description: Contains the AIMLE server task transfer assist response information.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlesTaskTransferAssistResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/request-ctld:

post:

summary: >

Enables the AIMLE client to request the AIMLE server to perform AIMLE server controlled

task transfer.

operationId: CtldAimlTt

tags:

- Controlled AI/ML task transfer

requestBody:

description: Contains the AIMLE server controlled task transfer request information.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlesControlledTaskTransferReq'

responses:

'200':

description: Contains the AIMLE server controlled task transfer response information.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlesControlledTaskTransferResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimlesTaskTransferAssistReq:

description: Contains the AIMLE server task transfer assist request information.

type: object

required:

- requestorId

- aimlTaskType

- aimlInfoType

properties:

requestorId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

valServiceId:

description: >

Represents identifier of the VAL service for which the assistance information

is requested.

type: string

aimlTaskType:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlInfoType:

$ref: 'TS24560_Aimlec_AimlTaskTransfer.yaml#/components/schemas/AimlInfoType'

aimlRmngTrainReq:

$ref: '#/components/schemas/AimlRmngTrainingReq'

aimlImdInfo:

$ref: '#/components/schemas/AimlIntermediateInfo'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

AimlesTaskTransferAssistResp:

description: Contains the AIMLE server task transfer assist response information.

type: object

required:

- assistanceTime

- targetAimlIds

properties:

assistanceTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

targetAimlIds:

description: List of the target AIMLE clients.

type: array

items:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

minItems: 1

transferMode:

$ref: '#/components/schemas/TransferMode'

AimlesControlledTaskTransferReq:

description: Contains the AIMLE server controlled task transfer request information.

type: object

required:

- requestorId

- aimlTaskType

- aimlInfoType

- aimlTaskTransferTime

properties:

requestorId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

aimlTaskType:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlInfoType:

$ref: 'TS24560_Aimlec_AimlTaskTransfer.yaml#/components/schemas/AimlInfoType'

aimlTaskTransferTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

timeValidity:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

AimlesControlledTaskTransferResp:

description: Contains the AIMLE server controlled task transfer response information.

type: object

required:

- assistanceTime

properties:

assistanceTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

AimlRmngTrainingReq:

description: Contains requirements for AI/ML model training.

type: object

properties:

reqRmngTrainResorce:

description: Indicates required remaining training resource.

type: string

reqRmngTrainIterNum:

description: Indicates required remaining training number of iterations.

type: integer

AimlIntermediateInfo:

description: Contains the AI/ML intermediate information for intermediate AI/ML operation.

type: object

properties:

aimlImdModel:

$ref: '#/components/schemas/AimlRmngTrainingReq'

aimlUsedTrainTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

usedTrainResource:

description: Indicates used training resource.

type: string

usedTrainIterNum:

description: Indicates used training number of iterations.

type: integer

# Simple data types

# Enumerations

TransferMode:

anyOf:

- type: string

enum:

- DIRECT

- SERVER_CONTROLLED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

But is not used to encode content defined in the present version of this API.

description: |

Represents the mode of transfer.

Possible values are:

- DIRECT: Directly from the source AI/ML member to the target AI/ML member.

- SERVER_CONTROLLED: Transfer with AIMLE server controlled.

==============End of change==============