TSG-CT WG3 Meeting #110-e	C3-203382

E-Meeting, 2nd – 11th June 2020													(Revision of C3-203xyz)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.525 | CR | 0102 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supported headers, Resource Data type, Operation Name and yaml mapping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc16 |  |  |  |  |  | Date: |  |  | 2020-05-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - New tables are added to show the mapping between the APIs defined in the specification and the Yaml, to align with the new tables added in 29.502 at the meeting #96e(C4-200934) - New tables are added to capture supported headers for response codes 2xx and 3xx as coded in the yaml file, to align with the new tables added in 29.518 at the meeting#97e (C4-202380) - Capture the Data type for each Resource listed in the table allowing for a more optimized logic for the parser, to align with CT4 meeting #97e(C4-202421) - Capture the Operation Name in the Custom Operations table allowing for a more optimized logic for the parser, to align with CT4 meeting #97e(C4-202452) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change #1: Add a table in clause 4.2.1 to show a mapping of the corresponding API files descriptions for this specification Change#2: Added tables to capture supported headers for response code 201 in clause 5.3.2.3.1. Change#3: Added a new column “Data type” in clause 5.3.2.2, 5.3.3.2. Change#4: Added a new column “Operation Name” in the tables of clause 5.3.3.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause misoperation or misunderstanding for implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 5.3.2.2, 5.3.2.3.1, 5.3.3.2, 5.3.3.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Thee CR does not impact the OpenAPI file |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

…

# Proposed changes:

# *** 1st Change ***

4.2.1	Introduction

Table 4.2.1-1: Operations of the Npcf_UEPolicyControl Service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Npcf_UEPolicyControl_Create | Creates a UE Policy Association. | NF consumer (AMF, V-PCF in roaming case) |
| Npcf_UEPolicyControl_Update | Updates of an UE Policy Association and provides corresponding policies to the NF consumer when the policy control request trigger is met or the AMF is relocated due to the UE mobility and the old PCF is selected. | NF consumer (AMF, V-PCF in roaming case) |
| Npcf_UEPolicyControl_UpdateNotify | Provides the updated policy control request triggers to the AMF by the (V-)PCF in the non-roaming or roaming case; Provides updated UE policy and policy control request trigger to the V-PCF by the H-PCF; or initiates the UE Policy association termination towards to the NF consumer by the NF producer. | PCF (H-PCF and V-PCF in roaming case) |
| Npcf_UEPolicyControl_Delete | Provides means for the NF consumer to delete the UE Policy Association. | NF consumer (AMF, V-PCF in roaming case) |

Table 4.2.1-2 summarizes the corresponding APIs defined in this specification.

Table 4.2.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Npcf_UEPolicyControl | 5 | UE Policy Control Service | TS29525_Npcf_UEPolicyControl.yaml | npcf-ue-policy-control | A.2 |

# *** Next Change ***

5.3.2.2	Resource definition

Resource URI: {apiRoot}/npcf-ue-policy-control/v1/policies/

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1 |

# *** Next Change ***

5.3.2.3.1	POST

This method shall support the URI query parameters specified in table 5.3.2.3.1-1.

Table 5.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 5.3.2.3.1-2 and the response data structures and response codes specified in table 5.3.2.3.1-3.

Table 5.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| PolicyAssociationRequest | M | 1 | Input parameters for the creation of a policy association. |

Table 5.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| PolicyAssociation | M | 1 | 201 Created | Policy association was created and policies are being provided. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [5] also apply. |  |  |  |  |

Table  5.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId} |

# *** Next Change ***

5.3.3.2	Resource definition

Resource URI: {apiRoot}/npcf-ue-policy-control/v1/policies/{polAssoId}

This resource shall support the resource URI variables defined in table 5.3.2.2-1.

Table 5.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 5.1. |
| polAssoId | string | Identifier of a policy association. |

# *** Next Change ***

5.3.3.4.1	Overview

Table 5.3.3.4.1-1: Custom operations

| Operation name | Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- | --- |
| Update | {apiRoot} /npcf-ue-policy-control/v1/ policies/ {polAssoId}/update | POST | Report observed event trigger and obtain updated policies. |

*** End of Changes ***